PF/OH Inductance Calculations

C Neumeyer 4/3/97

General:

PPPL Magnetics Library program ICC5.f is used to calculate the mutual matrix for the NSTX circuits. In addition it computes the resistance vector, which is cross checked against the PF Coils Spreadsheet. The OH self inductance calculation is checked against a standard formula for solenoids.

References:

"Documentation of EAD Magnetics Codes", EAD-3705

"Inductance Calculations" , F W Grover, Dover Publications, 1973, p. 105

NSTX PF Coils spreadsheet 4/3/97

Method:

Essential data is obtained from the NSTX PF Coils spreadsheet...

| Coil | R (center) | ∆R | Z (center) | ∆Z | Turns | Fill |
| --- | --- | --- | --- | --- | --- | --- |

|  | (cm) | (cm) | (cm) | (cm) |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| OH | 13.19 | 4.49 | 106.57 | 213.13 | 482.00 | 0.7120 |
| --- | --- | --- | --- | --- | --- | --- |

| PF1a | 18.03 | 4.16 | 144.83 | 53.88 | 48.00 | 0.7177 |
| --- | --- | --- | --- | --- | --- | --- |

| PF1b | 30.48 | 8.48 | 181.88 | 17.13 | 28.00 | 0.6457 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2a | 79.92 | 16.27 | 193.35 | 6.80 | 14.00 | 0.7409 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2b | 79.92 | 16.27 | 185.26 | 6.80 | 14.00 | 0.7409 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3a | 149.45 | 18.64 | 163.35 | 6.80 | 15.00 | 0.6928 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3b | 149.45 | 18.64 | 155.26 | 6.80 | 15.00 | 0.6928 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4a | 178.27 | 6.78 | 72.75 | 6.80 | 5.00 | 0.6349 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4b | 179.46 | 9.15 | 64.66 | 6.80 | 8.00 | 0.7525 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4c | 180.65 | 11.53 | 56.57 | 6.80 | 9.00 | 0.6723 |
| --- | --- | --- | --- | --- | --- | --- |

File "iccin.d" is created...

10,3,10

1,2,3,4,5,7,9,11,13,16

0.1319  1.0657  2.1313  0.0449  482.0  2  1.7240  0.7120 0  0

0.1803  1.4483  0.5388  0.0416   48.0  1  1.7240  0.7177 0  0

0.1803 -1.4483  0.5388  0.0416   48.0  1  1.7240  0.7177 0  0

0.3048 -1.8188  0.1713  0.0848   28.0  1  1.7240  0.7331 0  0

0.7992  1.9335  0.0680  0.1627   14.0  1  1.7240  0.7363 0  0

0.7992  1.8526  0.0680  0.1627   14.0  1  1.7240  0.7363 0  0

0.7992 -1.9335  0.0680  0.1627   14.0  1  1.7240  0.7363 0  0

0.7992 -1.8526  0.0680  0.1627   14.0  1  1.7240  0.7363 0  0

1.4945  1.6335  0.0680  0.1864   15.0  1  1.7240  0.6886 0  0

1.4945  1.5526  0.0680  0.1864   15.0  1  1.7240  0.6886 0  0

1.4945 -1.6335  0.0680  0.1864   15.0  1  1.7240  0.6886 0  0

1.4945 -1.5526  0.0680  0.1864   15.0  1  1.7240  0.6886 0  0

1.7827  0.7275  0.0680  0.0678    5.0  2  1.7240  0.6310 0  0

1.7946  0.6466  0.0680  0.0915    8.0  2  1.7240  0.7479 0  0

1.8065  0.5657  0.0680  0.1153    9.0  2  1.7240  0.6682 0  0

0.8000  0.0000  0.0100  0.1000    1.0  1  1.0000  0.9999 1  0

10*0

Line 1: # Coil groups, 3 for MKS units, Coil Group # of Plasma

Line 2:  Number of coil beginning each coil group

Subsequent Lines per coil (or symmetric coil pair) and plasma:

R, Z, ∆Z, ∆R, turns, symmetry (1 for single coil, 2 for mirror image coils 	w.r.t. midplane, -2 with mirror image coils w/-turns below midplane),

resistivity * e8  in Ω-m (e.g. 1.742 for copper), fill factor (conducting 	area/coil area), ncc (0 for rectangular coil), alf (0 for ncc=0 type coils)

Last line: 10 zeros to indicate end of file

File cntmtx.d is obtained (mΩ @ 20C, mH)...

|  | OH | PF1au | PF1al | PF1b | PF2u | PF2l | PF3u | PF3l | PF4 | Plasma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| OH | 1.30E+01 | 7.31E-01 | 7.31E-01 | 3.74E-01 | 2.76E-01 | 2.76E-01 | 2.95E-01 | 2.95E-01 | 5.05E-01 | 1.32E-02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1au | 7.31E-01 | 3.76E-01 | 1.95E-04 | 2.26E-04 | 7.23E-02 | 1.35E-03 | 6.04E-02 | 5.29E-03 | 3.90E-02 | 5.22E-04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1al | 7.31E-01 | 1.95E-04 | 3.76E-01 | 9.01E-02 | 1.35E-03 | 7.23E-02 | 5.29E-03 | 6.04E-02 | 3.90E-02 | 5.22E-04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1b | 3.74E-01 | 2.26E-04 | 9.01E-02 | 5.36E-01 | 1.66E-03 | 1.86E-01 | 6.61E-03 | 1.01E-01 | 4.93E-02 | 4.96E-04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF2u | 2.76E-01 | 7.23E-02 | 1.35E-03 | 1.66E-03 | 1.98E+00 | 1.03E-02 | 7.31E-01 | 4.11E-02 | 3.08E-01 | 2.68E-03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF2l | 2.76E-01 | 1.35E-03 | 7.23E-02 | 1.86E-01 | 1.03E-02 | 1.98E+00 | 4.11E-02 | 7.31E-01 | 3.08E-01 | 2.68E-03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF3u | 2.95E-01 | 6.04E-02 | 5.29E-03 | 6.61E-03 | 7.31E-01 | 4.11E-02 | 5.18E+00 | 1.66E-01 | 1.31E+00 | 9.32E-03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF3l | 2.95E-01 | 5.29E-03 | 6.04E-02 | 1.01E-01 | 4.11E-02 | 7.31E-01 | 1.66E-01 | 5.18E+00 | 1.31E+00 | 9.32E-03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF4 | 5.05E-01 | 3.90E-02 | 3.90E-02 | 4.93E-02 | 3.08E-01 | 3.08E-01 | 1.31E+00 | 1.31E+00 | 8.56E+00 | 3.45E-02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Plasma | 1.32E-02 | 5.22E-04 | 5.22E-04 | 4.96E-04 | 2.68E-03 | 2.68E-03 | 9.32E-03 | 9.32E-03 | 3.45E-02 | 9.77E-05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| R | 97.435 | 2.7973 | 2.7973 | 2.4307 | 4.1659 | 4.1659 | 8.3465 | 8.3465 | 14.717 | 6.72E-06 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Cross checking:

Resistance vector (20C) from ICC agrees well with PF Coils spreadsheet...

|  | R(ICC) | R(SS) |
| --- | --- | --- |

| OH | 9.74E-02 | 9.74E-02 |
| --- | --- | --- |

| PF1au | 2.80E-03 | 2.80E-03 |
| --- | --- | --- |

| PF1al | 2.80E-03 | 2.80E-03 |
| --- | --- | --- |

| PF1b | 2.43E-03 | 2.76E-03 |
| --- | --- | --- |

| PF2u | 4.17E-03 | 4.14E-03 |
| --- | --- | --- |

| PF2l | 4.17E-03 | 4.14E-03 |
| --- | --- | --- |

| PF3u | 8.35E-03 | 8.30E-03 |
| --- | --- | --- |

| PF3l | 8.35E-03 | 8.30E-03 |
| --- | --- | --- |

| PF4 | 1.47E-02 | 1.46E-02 |
| --- | --- | --- |

Self Inductance of OH coil agrees well with calc using formula from Grover...

L = 0.050138 * (2a/b)N^2aK' µH

| a | 5.19291339 | in |
| --- | --- | --- |

| b | 167.818898 | in |
| --- | --- | --- |

| c | 1.76771654 | in |
| --- | --- | --- |

| 2a/b | 0.06188711 |  |
| --- | --- | --- |

| K | 0.974985 |  |
| --- | --- | --- |

| c/2a | 0.1702047 |  |
| --- | --- | --- |

| c/b | 0.01053348 |  |
| --- | --- | --- |

| k | 0.10361259 |  |
| --- | --- | --- |

| K' | 0.87137241 |  |
| --- | --- | --- |

| Inductance | 13.0477663 | mH |
| --- | --- | --- |

(see attached sheets for Grover reference and look-up of k and K'

Current swing required of OH coil for 0.6 volt-sec...

L = Nφ/I

∆I = N∆φ/L

∆I = 964 * 0.6 / 13.0e-3 = 44492 amp

or + 24000 to -20492 amp

Max available flux swing  +24 to -24kA...

∆φ= L∆I/N

= 13.0e-3 * 48000/964

= 0.65 volt-sec

ICC Code Listing: Attached