TF Inductance Calculation

C Neumeyer 1/29/97

The field is...

B = µ0NI/2πr

The problem is to solve for...

φ = ∫B*ds

Then the inductance...

L = Nφ/I

The attached "TF Inductance Calc" spreadsheet integrates φ = ∫B*ds from the outer edge of the TF coil inner leg bundle to the inside of the outer leg. 100 ∆r slices are used.

Reference for the TF coil outer leg shape is attached (PPL.LM.NSTX.TF-3-Param).

The final result is L = 4.176 mH.

This agrees well with A Brooks earlier estimate for the 72 turn coil (17mH), based on an n^2 scaling:

L' = (72/36)^2*4.17 = 16.7 mH