FIRE

TF

Parametric

Analysis

C Neumeyer

5/31/00

Objective

• Quantify TF performance with BeCu and Cu using simple model to guide choice of conductor material and peak field

Method

• Spreadsheet model

- Include BeCu and Cu specific heat and resistance variation with T

- Include nuclear heating

- Neglect non-uniform current density effects

Modeling

• Coil dimensions from T Brown

• Inductance from R Woolley (TFCUR model) but confirmed by spreadsheet calculation

• BeCu (C17510) and Cu (pure) specific heat and resistivity data from NIST Monograph 177, “Properties of Copper and Copper Alloys at Cryogenic Temperatures”

ELECTRICAL CIRCUIT

Calculation

• Simple Euler integration ∆Ii=(Vi-Ii-1*Ri-1)*∆t/L with R(T) and ∆T(dissipation, specific heat (Ti))

• PS Voltage set to full during current rise, set to V=IR during flat top, set to 0 during current fall

• EOFT declared when inner leg T and stored magnetic energy such that L/R decay will lead to max allowable T (assumed 370C)

• Nuclear heating set to 30MW, applied to inner legs

• Added PF and Heating power to TF power, PF(Bt(Ip)

SPREADSHEET I/O

| LINNER | 3.74806 | m |
| --- | --- | --- |
| AINNER | 0.012419 | m^2 |
| VINNER | 0.046547157 | m^3 |
| DINNER | 8750 | kg/m^3 |
| MINNER | 407.287625 | kg |
| RES293INNER | 2.53E-08 | ohm-m |
| COEFFINNER | 0.0025 | 1/degK |
| M0INNER | 16.4940 |  |
| M1INNER | 3.3738 |  |
| M2INNER | -0.0105 |  |
| M3INNER | 1.1643E-05 |  |
| RINNER293 | 7.6338E-06 | ohm |
| LOUTER | 5.8261 | m |
| AOUTER | 0.0218 | m^2 |
| VOUTER | 0.1272 | m^3 |
| DOUTER | 8940.0000 | kg/m^3 |
| MOUTER | 1137.1498 | kg |
| RES293OUTER | 1.7200E-08 | ohm-m |
| COEFFOUTER | 0.0041 | 1/degK |
| M0OUTER | -82.3620 |  |
| M1OUTER | 4.9545 |  |
| M2OUTER | -0.0188 |  |
| M3OUTER | 2.4957E-05 |  |
| ROUTER293 | 4.5899E-06 | ohm |
| TURNS/COIL | 15.0000 |  |
| COILS | 16.0000 |  |
| TURNS | 240.0000 |  |
| R0 | 2.0000 | m |

| %IACSINNER | 68 | % |
| --- | --- | --- |
| TMAX | 370 | degK |
| B(R0) | 10 | T |
| VNL | 1300 | volt |
| REG | 10 | % |
| PNUCINNER | 30.0 | MW |
| PPF@10T/6.4MA | 250.0 | MW |
| PAUX | 60.0 | MW |
| PMAXTF= | 489.8 | MW |
| PPEAKTOT | 799.8 | MW |
| DTRAMP= | 17.3 | sec |
| DTFLAT= | 21.5 | sec |
| WDISS= | 11.1 | GJ |

## BeCu Coil (1.3kV,10T w/NH)

Cu Coil (0.65kV, 10T w/NH)

Summary of Data

|  | BeCu |  |  |  |  |  | Cu |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Conductivity | 77 | 77 | 68 | 68 | 68 | 68 | 100 | 100 | 100 | 100 |
| Bt | 10 | 10 | 10 | 10 | 12 | 12 | 10 | 10 | 11.5 | 11.5 |
| Nuc Heat | 0 | 30 | 0 | 30 | 0 | 30 | 0 | 30 | 0 | 30 |
| PT Flat top | 31 | 21 |  | 18.5 |  | 12 |  |  |  |  |
| CN Flat top | 29 | 25 | 24 | 21 | 8 | 9 | 60 | 45 | 36 | 30 |
| Ptf | 490 | 490 | 490 | 490 | 587 | 587 | 276 | 266 | 334 | 327 |

## Conclusions

• All Cu design improves performance, reducing peak TF power requirement by ~ factor of 2 while increasing flat top ~ factor of 2 @ 10T, factor of 3 @ upgrade level (~ 12T)

• All Cu design would reduce AC/DC converter cost by ~ $25M

• All Cu design would improve chances of powering FIRE exclusively from grid, and then saving another ~ $30M