I. Physics Investigation

The physics studies for NSTX have covered the areas of achievable shapes, confinement and transport, MHD stability, heating and current drive, and possible operational scenarios.

A. Confinement and Performance

Operation in START has suggested interesting possibilities for enhanced confinement operation at low aspect ratio. In particular, the START confinement data seem to agree well with some popular L-mode scalings, most notably the Lackner-Gottardi scaling. This scaling has parametric dependences generally similar to those of ITER89-P, the more accepted L-mode scaling expression.  As little is known about confinement scaling at low aspect ratio, an experiment such as NSTX is essential for determining the confinement characteristics at low R/a.

The expected behavior of drift-type instabilities was studied using a comprehensive kinetic toroidal microinstability code which takes into account high-n effects, all resonances, finite beta effects, and finite-Larmor-radius and banana orbit effects. The study was performed in the electrostatic limit for plasmas at fixed q and RBt, over the range of aspect ratio from 1.25 to 2.5. The results indicated a reduction in the orbit-averaged curvature function (the instability drive), with decreasing aspect ratio, with the linear growth rate of the mode becoming negative (stability) for R/a ≤ 1.5.  The effect is more pronounced in the more collisionless plasmas, again indicating the need for high power heating to achieve low collisionality.  The ability to produce low collisionality plasmas with strong auxiliary heating as well as the ability to operate over a range of aspect ratio allows NSTX to explore the benefit of low aspect ratio in suppressing transport-inducing microinstabilities.

A more general approach can be taken to assessing the variation in performance with aspect ratio. Various performance indicators can be formulated as functions of aspect ratio only with an appropriate number of constraints. For this exercise, one natural choice of constraints is fixed TF coil current density, βn, qedge, and Pinput. Given these constraints, the variation in confinement (ITER89-P scaling), β, Pfusion/Pcenter (the fusion power production normalized by the power dissipated in the center stack), and the (inverse) TF field utilization with aspect ratio is shown in Figure 2.  What is plotted in the figure are normalized values (normalized to 1 at R/a~1.5). It is the magnitude of the variation with aspect ratio that is important to note, and for this, the various parameters not only increase with decreasing aspect ratio, but the rate of increasing performance actually increases as the aspect ratio decreases, indicating large performance gains in the ultra-low-aspect-ratio region (R/a<1.3).  The TF field utilization factor, Ip/ITF, indicates the amount of TF that needs to be supplied; lower TF requirements are necessary with increasing TF utilization (decreasing ITF/Ip).

The performance of low aspect ratio devices, based on the energetics calculations shown here, indicate that ultra-low-aspect-ratio is a promising approach for a compact Volumetric Neutron Source and/or reactor. Performance increases with decreasing aspect ratio, low R/a devices have only modest TF requirements, meaning that standard Cu coils could be envisaged, and studies have shown potential factor of two decreases in capital cost and COE over conventional fusion reactors, making an ULART reactor competitive with fission reactor COE.

There are, however, several issues that need to be addresses for the development of a steady-state spherical torus reactor, most notably MHD stability and Current Drive.

B. MHD Stability and Beta Limit

In order for a compact spherical torus reactor to be attractive, it must necessarily operate at high beta, with beta values typically > 30%.  Because of this, MHD stability is a key issue for ultra-low-aspect-ratio.  Here, we examine the MHD stability for "conventional" pressure and current profiles with and without a conducting wall.  Without a wall, the ULART stability is limited by low-n modes, with a β-limit of only near 25% (βn = 4).  The low-n modes are effectively suppressed with the presence of a wall, as shown in the upper curve where it is now high-n ballooning that is the β-limiting factor. With the wall, the βt increases to approximately 33% (βn > 5).

An even higher βt value is obtained with profile optimization.  This is shown in Fig. 3.  The results of this profile optimization show two main elements. The first is that by raising q0 to ~3.4, the ballooning limit is raised to approximately 43% (βn = 8.7).  Of crucial importance in this case as well as the previous one is the presence of the conducting wall, which effectively stabilizes the low-n modes once again. The other key feature of this profile optimization is the large amount of current carried by the bootstrap and diamagnetic currents, with 77% of the current due to bootstrap, this value increasing to 85% of the total current when other pressure gradient dependent currents are also considered.  This self-driven current is of great importance for reactor operation without an OH solenoid.

C. Heating and Current Drive

A spherical torus reactor design precludes an OH solenoid for two essential reasons. The first is that the center stack must be made as skinny as possible to minimize aspect ratio.  The second is the neutron damage on insulating materials that would inevitably occur. In addition to these, a compact spherical torus reactor requires high current, typically 20 to 30 MA.  These reasons make non-inductive current drive an essential element of an ST reactor. The non-inductive current drive can come from four sources; neutral beams, helicity injection, RF, and bootstrap current.  The bootstrap current, which can account for up to 85% of the total current, was discussed in the last section.

1. Neutral Beam Injection - Neutral beam current drive has been estimated for 5 MW of near-tangential injection into a low aspect ratio configuration.  With Do ⇒ D+ injection and the temperature typical of NBI at the 5 MW level, Te(0) = 2.5 keV and Ti(0) = 5 keV, we find that about half of the fast ions are trapped at these low aspect ratios, and these, therefore, do not contribute to the current.  The remaining passing particles impart enough momentum to electrons to drive approximately 0.3 MA of current. While this is not enough to sustain the 1 MA current fully non-inductively, it can play a significant role together with a high bootstrap fraction.

2.  Helicity Injection (HI) - This technique has been demonstrated on the HIT device at plasma current up to 200 kA. It can be extended to 1 MA for use in the current initiation phase of NSTX, and HI may also be a viable technique to control the edge plasma current density during steady state.

3. High Harmonic Fast Wave - The NSTX plasmas present special challenges for radio-frequency heating and current drive, noting that the plasma dielectric constant ωpe2/ωce2 is about 45  for typical NSTX parameters (this value is ≈ 1 for conventional tokamak plasmas.).  At this value of ωpe2/ωce2, both Lower Hybrid and Electron Cyclotron waves have severe accessibility problems.  Lower Hybrid accessibility is limited to high n|| (>10), meaning interaction between the waves and low energy electrons, limiting the amount of possible current drive to an almost negligible value. There is virtually no access for Electron Cyclotron wave propagation for this value of the dielectric constant.  (Recent theoretical calculations indicate that accessibility for ECH may be greatly improved using mode-conversion to electron Bernstein waves.)

An intriguing candidate for RF heating and current drive is High Harmonic Fast Wave (HHFW). HHFW shows strong, single pass absorption for high-β operation, consistent with the high density, low TF regime of NSTX. The wave absorption scales as ne/BT3.  Fast Waves have good accessibility up to βi values of ~ 50%, at which point the waves mode convert to the Ion Bernstein branch.  The advantage of the Fast Waves is that their electron heating can lead to higher bootstrap fractions, and the power deposition can be localized for current profile control.  It is estimated that between 0.2 and 0.9 MA of current can be driven by Fast Waves in NSTX equilibria.

The following summarizes the advantages for using HHFW for NSTX:

(a). Strong single pass absorption (90 – 100%),  Im k⊥ ∝ ne/B3 ∝ β /B.

(b) FW Electron Heating synergistic with advanced ST regime.

•	Higher neTe and lower ν* ⇒ higher bootstrap current.

•	Higher Te ⇒ higher current drive efficiency.

•	Localized heating and current drive ⇒ j profile control.

(c). Due to strong absorption, the current drive efficiency can be high.

•	ICD(MA) ≈ 6 Prf(MW) ne-1(1013cm-3) R(m)-1 nˆ-2.

e.g., NSTX High Beta - 80% Bootstrap Fraction Discharges:

ne(0) ≈ 5 x 1013cm-3, R = 0.8 m, Prf = 7.5 MW.

Central CD (nˆ = 4):  ICD(MA) ≈ 700 kA.

Off-axis CD (nˆ = 6): ICD(MA) ≈ 300 kA.

(d). Good wave accessibility (ion beta limit of ≈ 50 % and no electron beta limit).

(e). High power, long-pulse transmitters available at PPPL (7.5 MW, 5 sec).

D. NSTX Operational Scenarios

The attractive features of high-beta/high confinement and potential for fully non-inductive current drive are combined in NSTX for a MA-level proof-of-principle ST physics experiment.  The tokamak simulation code (TSC) was used to model NSTX discharge start-up and maintenance, with an aim toward producing a fully non-inductively current driven discharge. For this case, a 0.5 MA target plasma was produced using the OH capability that is built into the NSTX solenoid, but the OH current is brought to zero, and maintained at zero once the current flattop (1 MA) is reached 0.2 sec into the discharge. A HHFW power of 5 MW is turned on and

maintained once the plasma has reached the 0.5 MA target level. The HHFW power deposition profile is parameterized as a function of poloidal flux, Ψ, based on results from a slab, wave-damping calculation, with an additional 1-Ψ falloff to take into account the amount of trapped electrons that do not contribute to the current.

Figure 4 shows the first 0.5 sec of time evolution for an NSTX discharge with HHFW current drive.  The 0.5 MA target plasma has a relatively low electron temperature, Te(0)~0.5 keV.  Therefore, when the HHFW heating and current drive is turned on at this time, on average about 0.5 MA of current is driven.  Later in time, the bootstrap current becomes more important reaching for this case to be 50%.  Therefore, with HHFW and bootstrap current, it is possible to drive the entire 1 MA current.  In the bottom panel of Figure 4 is shown the resistive surface voltage. Note that although the HHFW and bootstrap account for the total 1 MA current, there is still some residual positive resistive voltage; this is because the discharge has not settled down to a steady-state. The total current profile is not well-aligned with those of the bootstrap and Fast Wave profiles, and some voltage is therefore needed to effect this alignment.  This indicates that some form of plasma profile control may be needed to optimize the bootstrap current fraction.  The combination of core current drive with NBI (an upgrade option), profile control current drive via HHFW, and surface current drive via HI should make this possible.

E. NSTX Physics Summary

From above theoretical investigations, the most promising regime for spherical torus and so for NSTX is identified to be the ULART regime, R/a ≈ 1.25 with significant natural elongation of κ ≈ 2.2.  Moreover, to be reactor relevant, a fully non-inductively sustained high β ST regime with a sufficiently long pulse length of ~5 sec must be demonstrated.  From the theoretical investigations, it appears to be feasible to achieve a non-inductively sustained high-β (≤ 40 %) / high-bootstrap-fraction (≥ 80%) regime with helicity injection and high harmonic fast wave heating and current drive at the reactor relevant 1 MA level low collisionality plasmas  These regimes have sufficient plasma density ( > 3.5 x 1019 m-3) to ensure adequate divertor operation and plasma cleanliness.  The NSTX model discharge is prototypical of the ones used for the high performance ST reactor studies.  With this challenging physics goal in mind, the NSTX device is being designed for maximum flexibility and plasma accessibility in a cost effective way by utilizing existing facilities and infrastructure at PPPL.  Due to the wide range of challenging physics issues, active national participations in the NSTX physics program is planned.

Technical Progress

NSTX Physics Activities

The Physics team has worked closely with the Engineering team to address the key physics issues that impact critical design items such as the center stack, PF coil system and associated power supply requirements, RF power systems, and plasma facing components.  Equilibrium calculations for both the up-down symmetric (Double Null and Natural Divertor) and Single Null (for Coaxial Helicity Injection) configurations have defined the required positions and shapes of the PF1a and PF1b coils, along with

the coil current levels required to produce the range of possible plasma shapes. Furthermore, shaping requirements have been key in setting the height of the new OH solenoid. Calculations have been performed to determine the axisymmetric stray fields from such sources as the OH current during startup and induced eddy currents in the vessel and passive plates, TF leads, TF loops, and toroidal eddy currents induced in the TF center stack conductor due to ramping of the OH current. Only the first source of stray field (OH and eddy currents), was found to be significant.  Calculations have been performed to determine the PF current levels required to compensate these stray fields and allow plasma initiation.

Other areas in which Physics has contributed include Tokamak Simulation Code (TSC) scenario development for inductive plasma operation from near initiation to final OH current shut-down.  This is important input into Engineering for determining the I2t heating of the various coils and the PF power supply voltage requirements.  Results from these runs have also been used as input to RF coupling calculations to assess heating and current drive possibilities.  Simulations in TSC and TRANSP have been carried out for Neutral Beam Injection scenario development, and this work is ongoing.  The effect of the fast ion population on the RF heating of the plasma is also an area of ongoing work.

TSC has been used to develop radial and vertical disruption scenarios; code output is used as a basis for determining stresses on the center stack due to induced and halo currents.  Finally, 0D calculations have been used to estimate the expected heat loads on the center stack and divertor plates for all three plasma configurations.  One and 2D heat flux calculations are underway.

Future Accomplishments (i.e., FY97/98/99)