The eqdsk files, named eqdsk_Beta_Scan.MW or eqdsk_Beta_Scan.PDF are a concatenation of three eqdsk files written in Microsoft Word 5.1 or PDF format respectively. The three eqdsk files were generated by Dennis Strickler of ORNL, and they constitute a beta scan The files were originally prepared for RF code benchmarking purposes. The global parrameters for the beta scan are:

R-a=0.175 m

R+a=1.533 m

R/a=1.26

Ip=1.0 MA

Bt=0.3 T

li=0.2

kappa=2.0

delta=0.35

The three beta values are 5% (0203c), 25 %(0122c), and 40%  (0203a). Note that labels indicating the beta values have been placed in the eqdsk_Beta_Scan files; the EQDSK read format (see below) pertains to the lines in these files starting with "EQDSK". For simplicity, you might want to uncancatenate the files and strip out the labels before reading them.

A prescription for determining the temperature and density profiles from the pressure profiles in the eqdsk files is as follows

Assume a parabolic shape for the electron and ion

>temperatures

>

>T(r)=Tlim+(T0-Tlim)(1-(r/a)^2)

>

>and assume Zeff=2 (flat across the profile), and Zimp=6. Then, the ne(r) and

>ni(r) can be backed out of the EQDSK.

>

>ni=(Zimp-Zeff)*ne/(Zimp-1),

>

>and neTe+niTi=Ptot (from EQDSK), and the above relation allows you to

>determine the ne(r) from Te(r), Ti(r), Zeff, Zimp, and Ptot .

>

>The four cases to examine are:

>

>

>A) 5% BETA : Te0= 1.0 KeV, Ti0=1.0Kev, Tlim=30eV

>        (0203C) kz=nphi/Rantenna=12/160=.075 m^-1

>

>B)  5% BETA : Te0= 0.5 KeV, Ti0=0.5Kev, Tlim=30eV

>        (0203C)  kz=nphi/Rantenna=12/160=.075 m^-1

>

>C) 25% BETA: Te0= 4.0 KeV, Ti0=1.0Kev, Tlim=30eV

>        (0122C)  kz=nphi/Rantenna=4/160=.0254 m^-1

>

>D) 40% BETA: Te0= 5.0 KeV, Ti0=1.4Kev, Tlim=30eV

>        (0203A)   kz=nphi/Rantenna=4/160=.025 m^-1

>

S. Kaye

PPPL

(609) 243-3162

skaye@pppl.gov

EQDSK Format

>The EQDSK files are: 0203c (5% beta), 0122c (25%), and 0203a (40%).

>

>The code that is needed to read the EQDSK files is:

>

>

>c

>c________________________________________________________________________

>c

>c     EQDSK FORMAT

>c

>c     jm            no. of radial mesh points

>c     km            no. of vertical mesh points

>c     xdm      m    radial dimension of mesh

>c     zdm      m    vertical dimension of mesh

>c     rcnt     m    radius of mesh center

>c     redg     m    radius of inboard edge  of mesh

>c     ra       m    r - magnetic axis

>c     za       m    z - magnetic axis

>c     psia     Wb   flux - axis

>c     psil     Wb   flux - limiter (or X-pt)

>c     bt       T    vacuum toroidal field at rcnt

>c     plcx     amp  plasma current

>c     psix(1)  Wb   flux - upper X-point

>c     psix(2)  Wb   flux - lower X-point

>c     rxp(1)   m    r - upper X-point

>c     rxp(2)   m    r - lower X-point

>c     zxp(1)   m    z - upper X-point

>c     zxp(2)   m    z - lower X-point

>c     fgf     T-m   toroidal flux function F = RBt

>c     pgf   pascal  pressure function P

>c     ffpgf         = FF'

>c     ppgf          = P'

>c     psi     Wb    poloidal flux function

>c     jphi    amp   toroidal current density

>c     nc            no. of (PF coil) filaments

>c     rc       m    r - PF filaments

>c     zc       m    z - PF filaments

>c     cc      amp   current - PF filaments

>c

>      write(9,3000) ipestg, jm, km, iout

>      write(9,3100) xdm, zdm, rcnt, redg

>      write(9,3100) ra, za, psia, psil, bt

>      write(9,3100) plcx, psix(1), psix(2), rxp(1), rxp(2)

>      write(9,3100) zxp(1), zxp(2), betav, qsrf(1), qsrf(npsi)

>      write(9,3100) (fgf(j), j=1,jm)

>      write(9,3100) (pgf(j), j=1,jm)

>      write(9,3100) (ffpgf(j), j=1,jm)

>      write(9,3100) (ppgf(j), j=1,jm)

>      write(9,3100) ((psi(j,k), j=1,jm), k=1,km)

>      write(9,3100) ((jphi(j,k),j=1,jm), k=1,km)

>      write(9,3200) nc

>      write(9,3100) (rc(i), i=1,nc)

>      write(9,3100) (zc(i), i=1,nc)

>      write(9,3100) (cc(i), i=1,nc)

>c

> 3000 format(' EQDSK',42x,4i4)

> 3100 format(5e16.9)

> 3200 format(i4)

>c

>