TRANSP profiles for NBI heating case

In this folder are multiple PDF files containing plots and text files in UFILE format for output from a TRANSP simulation of a beam heated NSTX case. This plots were taken from TRANSP run # 11112p09 in RUNDATA:[TRANSP.NSTX.97]. This was a run to equilibrium; that is, the TRANSP was run just long enough for the beam parameters to reach equilibrium.

Given are files of the following information

PDF Plot File	Text File		Description

Btor (z=0)      Z11112.BT0	Toroidal field vs R

Bplo (z=0)                     .BP0     	Poloidal field vs R

Br contours                  .BR         2-D Br contours (radial)

By contours                  .BY         2-D By contours (vertical)

mod B contours            .MDB      2-D mod total B contours

mod Btor contour        .MBT       2-D mod toroidal B contours

mod Bpol contour        . MBP      2-D mod poloidal B contours

xi contours                    .XI           2-D sqrt(tor. flux) contours

Ti profile                         .TI            Ti vs xi (1-D)

Ti profile vs R                  .TIR	   Ti vs R (1-D)

Te profile			     .TE            Te vs xi (1-D)

Te profile vs R                 .TER         Te vs R (1-D)

ne profile                         .NE	    ne vs xi (1-D)

ne profile vs R                 .NER        ne vs R (1-D)

q profile                            .QSI          q vs xi (1-D)

q profile vs R 		       .QR           q vs R (1-D)

The text files, in UFILE format, can be read using either standard UFILE read routines or by more simplife code. For the 1-D UFILEs, the independent and dependent variable names are given in the headers, as are the # (N) of data points. The first N points are the independent variable values sequentially, and the second N points are the

dependent variable values. The 2_D UFILEs are structured similarly, but they have two independent variables (in these cases R and Y).

The discharge parameters for this case are close to the nominal NSTX parameters:

Ip=1 MA

Bt=0.3 T

kappa=2

delta=0.35

R=0.85 m

a=0.68 m

Beams were 5 MW, 80 keV D0 injected into D+ plasmas at an Rtan of 0.7 m. Approximately 200 kA of beam driven current was generated.

The density profile was assumed to be relatively flat, relfecting H-mode like profiles.

The temperature profiles were assumed to be parabolic, and the density profile sqrt of parabolic.

S. Kaye

PPPL

(609) 243-3162

skaye@pppl.gov