Now that the issue of equilibrium profiles is resolved, I have re-done

the HHFW scenarios for the four cases of NSTX equilibria using CURRAY.

Assumptions remain the same as presented in our recent PPPL meeting.

f = 30 MHz,  Prf = 6.0 MW

Antenna poloidal length = 0.75 m,  launch location : outboard midplane

Initial poloidal wavenumber = 0.

n0/<n> = 1.13 - 1.23,   n0/n(a) = 0.05

T0/<T> = 1.77 - 1.82,   T(a) = 30 eV

Case    beta(%) N_phi   Teo(keV)  Ti0(keV)   ne0,13     Pe/Prf  Irf(kA)

----    ------- -----   --------  --------   ------     ------  -------

a       5       12      1.0       1.0        1.182      1.0     389

b       5       12      0.5       0.5        2.364      1.0     170

c       25      4       4.0       1.0        2.16       0.58    267

d       40      4       5.0       1.4        2.676      0.42    59

d'      40      8       5.0       1.4        2.676      0.50    209

Notes:

Cases (a), (b) and (c) have driven current profiles peaked on axis.

Case (d) results in negative current drive over a narrow region at

psi=0.3.  This partially explains the low CD efficiency.

Cases (c) and (d) both record significant ion damping.  In case (c).

it occurs at psi < 0.3, and in case (d), psi < 0.6.

Case (d') uses a much higher initial toroidal wavenumber resulting in

lower ion damping, eliminates net negative CD, and higher

CD efficiency.

Wave penetration is as deep as psi = 0.1 in both cases (d) and (d').

If you are interested in looking at the plots of these results, please

let me know and I'll be happy to provide the PS files.

Regards,

T.K.

TAK KUEN MAU                           Fusion Energy Research Program

mau@fusion.ucsd.edu                    Rm. 460, EBU-II, UC-San Diego

(619)534-2880                          9500 Gilman Drive

(619)534-7716 (Fax)                    La Jolla, CA 92093-0417