NSTX CALCULATION FORM

NOTE:  All signatures on this form require a printed name, signature, and date.

WBS	13		Calculation #13-13	Revision #0

Title	Center Stack Stress Analysis

Originator’s signature	John Spitzer

Purpose	The purpose of these analyses was to determine the deflection/stress levels on the center stack casing resulting from halo current loads as well as the deflections and stresses in the casing and support as a result of bakeout and operational heat loads.

References	NSTX Memos:

1) 13-961010-JMB-01, Estimate of Halo Loads for Inner Leg of the Vacuum Vessel

2) 13-970217-AWB-02, Loads on CS from Plasma Disruption

3) 13-970214-AWB-02, Thermal Analysis of VV/CS during Normal Operation

4) 13-970214-AWB-01, Bakeout of Vacuum Vessel and Internals

5) 13-970214-JMS-01, Centerstack Halo Current Stress Analysis

6) 13-970214-JMS-02, Stress Analysis of the Centerstack Case and Support Structure from Bakeout and Operational aThermal Loads

Assumptions	-The loads on the center stack as a result of halo currents are calculated in Reference 1.  Thermal loads for normal operation and bakeout are given in References 3 and 4.

-The center stack dimensions used in the analysis were those that were accurate at the time the halo loads were calculated.  The casing is modeled as 4 mm thick Inconel.

-  For the halo analysis the casing is assumed to be fixed at the base and guided at the top of the casing.  No support structure is explicitly modeled.

-For the thermal analysis plates representing the support structure from the upper plate of the lower hub assembly are included in the model.  The base of these plates is assumed to be fixed and the top of the casing is guided.  Since there are thermal gradients throughout the structure, the material properties used in these models are chosen at an average temperature.

Calculation	All analyses were performed using ANSYS 5.2  Model input files reside on /work1/jspitzer on ead01.pppl.gov.

Model input files for halo analyses:

cshlwgcns.tf1 - centered non-symm. halo current interaction with tf

cshlwgcns.pf1 - centered non-symm. halo current interaction with pf

cshlwgcns.pftf1 - centered non-symm. halo current interaction w/ tf&pf

cshlwgdis.pftf1 - displaced non-symm. halo current interaction w/ tf&pf

Model input files for thermal analyses:

cswgbo - temperature distribution during bakeout

cswgbo1np - structural response to bakeout temperature distribution

cswgop - temperature distribution during operation

cswgop1np - structural response to operating temperature distribution

Conclusion	The results of the various analyses are documented in References 5 and 6.  These analyses indicate that the stresses in all instances are well below the allowable stress level of 2/3 of the yield stress of Inconel at temperature.  The predicted deflections in all analyses are considered insignificant.  No formal analysis of the loads resulting from plasma disruption were performed since these loads were considered benign.  While some slight dimensional changes have taken place since that time these are not expected to have any significant impact on the result.

I have reviewed this calculation and to my professional satisfaction it is properly performed and correct.

Checker’s signature