NSTX GRD Revision 1

Explanation of Changes

C. Neumeyer

11 August 1997

Only those changes with non-trivial cost or technical impact are descibed herein. See marked up GRD for details of all changes.

Deletions are indictated in red strikethru, additions are indicated in blue italics.

1) Upgrade list (1.1.2.b)

"b. Upgrades presently contemplated include the following:

• EC start-up upgrade

• Coaxial Helicity Injection (CHI) current sustainment

• Addition of Neutral Beam injection (NBI)

• Long pulse operation (with center stack replacement)

• Addition of pellet injector

• Various pPlasma diagnostics upgrades

- electron temperature profile measurement

- plasma current profile measurement

• Automatic control for Glow Discharge Cleaning (GDC) between 	pulses"

Discussion: The above clarifies the main diagnostics upgrades which have evolved since the first issue of the GRD. It also includes provision for automatic control of GDC between pulses. GDC design decisions will consider the latter but only zero cost features will be included.

Cost/Schedule Implications: None

2) NBI Upgrade Features (Table 1.1.2-1)

"Table 1.1.2-1: Plan for NBI Upgrade Features

| Design Feature | Schedule |
| --- | --- |

| Plasma Facing Components (PFC) additional power dissipation* | U |
| --- | --- |

| PFC protective armor for NB shine-through and inadvertent loading | U |
| --- | --- |

| PFC beam dumps | U |
| --- | --- |

| Vacuum Vessel (VV) additional power dissipation** | U |
| --- | --- |

| VV port compatibility and availability (up to two beamlines) | B |
| --- | --- |

| Vacuum Pumping System (VPS) exhaust capability | B |
| --- | --- |

| Cooling Water System (CWS) NB source cooling capacity | U |
| --- | --- |

| Power Systems (PS) AC power (up to two beamlines) | U B |
| --- | --- |

| Central I&C additional control, data acquisition, and data network capacity | U |
| --- | --- |

| Facility space and provision for installation of NBI and related services | B |
| --- | --- |

B = To be included in baseline

U = to be deferred until upgrade, unless convenient to include in baseline at minimal cost

* = may be required as an upgrade if total (NBI + RF) required energy deposition during a pulse exceeds the baseline requirement During combined NBI + RF operations if the heating power exceeds the baseline then the pulse duration and repetition period shall be limited in accordance with the capability of the baseline PFCs, unless a PFC upgrade is undertaken.

** = may be required as an upgrade if total (NBI + RF) required average power deposition during a sequence of pulses exceeds the baseline requirement During combined NBI + RF operations if the heating power exceeds the baseline then the pulse duration and/or repetition period shall be limited such that the average heat load is within the cooling capability of the baseline VV design, unless a VV cooling upgrade is undertaken."

Discussion:

Possibility of two beamlines is introduced.

Power systems is required to support two beamlines as part of its baseline design (this is corrected from the first issue, should have been this way all along).

Clarification is provided with respect to the fact that in case the NBI upgrades are undertaken the baseline PFC and VV power handling capabilites will be in effect unless the PFC and VV cooling systems are also upgraded.

Cost/Schedule Implications:

The anticipation of a second beamline requires that a second custom port be included in the baseline VV.

J Spitzer to elaborate.

3) Material Requirements (1.1.4 c & d)

"c. All materials utilized within the primary vacuum boundary shall be on the PPPL Vacuum Committee approved list, or shall be approved by the committee.

d. All materials utilized within the primary vacuum boundary shall be designed to withstand the anticipated temperatures during operation and bakeout (section 2.3.2)."

Discussion:  These additions provide clarification.

Cost/Schedule Implications: None

4) Grounding of VV (1.2.2.g & h)

"g. The lower dome of the  outer section of the Vacuum Vessel shall be electrically grounded with via eight toroidally symmetric connections to a single point connection to the facility ground, which is designed to permit opening for the purpose of testing of the integrity of electrical isolation between the Vacuum Vessel and other components and structures. Isolation shall be rated to withstand a one minute AC hipot test at 2kV. Grounding connections shall be sized to carry the current during CHI operations as well as the return of the current during bakeout heating of the center stack casing.

h. Two toroidally symmetric connection points shall be provided at the top and bottom of the center stack casing. Connections shall be sized to carry the current during CHI operations as well as the return of the current during bakeout heating of the center stack casing. "

Discussion:  These additions provide clarification.

Cost/Schedule Implications: None

5) Deletion of Requirment for Galvanically Separate Circuits (1.2.3.1.c)

"c. The PF coils listed in Table 1.2.3.1-1 shall be connected in series groups in galvanically separate circuits with independent current control as indicated in Table 1.2.3.1-2."

Discussion:  It has been decided to allow a galvanic connection between the circuits which provide power to the upper and lower coil pairs of PF1a, PF2, and PF3. This is a cost savings measure since, instead of four cable runs from the TFTR test cell basement "waterfall" area to the NSTX Test Cell, only three are required.  The same functionality is maintained. The only technical difference is that the voltage to ground seen by the coils is increased (from 1kV to 2kV during normal operation, from 2kV to 4kV in case of ground fault. However, the coil insulation is easily able to withstand this voltage (the new PF1a coil uses the same ground insulation as the 6kV OH coil, and the outer PF coils from S1 are rated for a considerably higher voltage).

Cost/Schedule Implications:

Savings are realized by eliminating three cable runs.

S Ramakrishnan to elaborate.

6) Deletion of requirment for sharing HHFW sources w/PBX-M (1.3.1.a)

"a. The HHFW system shall utilize RF power produced by the six existing Tokamak Fusion Test Reactor (TFTR) Ion Cyclotron Range of Frequencies (ICRF) sources. The sources and associated controls and interlocks shall be designed to accommodate shared usage between NSTX and the Poloidal Beta Experiment- Modified (PBX-M)."

Discussion:  Although no specific changes are anticipated which would prevent shared usage the inclusion of this wording as a requirement is inappropriate since PBX-M is not an operating machine.

Cost/Schedule Implications: None

7) Fixed Frequency RF (1.3.1.c)

"c. The set of six sources (and twelve antennas) shall be capable of operation in subsets with up to three different frequencies during an NSTX discharge. Operating frequencies shall be in the range of 30 - 50 operating at a frequency of 30MHz."

Discussion:  Modification of the existing equipment to operate at variable frequency would be extremely expensive. Four of the existing sources as they exist are designed for fixed frequency operation @ 30MHz. The other two will be modified for NSTX from 40MHz to 30MHz, fixed frequency.

Cost/Schedule Implications: None

8) Addition of GDC to WBS3 (1.4.5)

"a. NSTX shall be equipped with a GDC system for cleaning, under zero field conditions, the internal Vacuum Vessel surfaces via the bombardment of ions formed during the glow process."

Discussion:  The GRD talks about GDC on NSTX but does not list it as scope within WBS3. Very early on GDC was viewed as an add-on which would come during the operations phase. But GDC has been part of the assumed construction project scope for a long time (it was included in the July 1996 review) and is included in the baseline cost estimate. Therefore it shall be included in the GRD. Also the PEP has been revised accordingly.

Cost/Schedule Implications: None

9) Revised Diagnostics List (Table 1.5-1)

Table 1.5-1: Baseline NSTX Diagnostics Set

| Diagnostic | Function |
| --- | --- |

| Plasma current Rogowski coils | Total plasma current |
| --- | --- |

| Eddy current Rogowski coils | Halo current monitoring |
| --- | --- |

| Flux loops | Poloidal flux for plasma control |
| --- | --- |

| Bθ, Bρ coils | Plasma control/magnetic fluctuations |
| --- | --- |

| Mirnov coils | Magnetic fluctuations |
| --- | --- |

| Visible TV camera | External shape for plasma control |
| --- | --- |

| IR camera | Heat loads |
| --- | --- |

| “Slow” diamagnetic loop (using toroidal field coil) | Stored energy |
| --- | --- |

| Multichannel bolometer | Radiated power profile |
| --- | --- |

| 1 mm microwave interferometer | Line-integrated plasma density |
| --- | --- |

| Survey spectrometer (SPRED) | Plasma impurities |
| --- | --- |

| Soft X-ray imaging system | Plasma instabilities and fluctuations |
| --- | --- |

| Hα detectors | Edge recycling |
| --- | --- |

| CHERS1 | Ion temperature & toroidal rotation |
| --- | --- |

| High-throughput CHERS background array | Background subtraction for CHERS and Zeff(r) from visible continuum |
| --- | --- |

| X-ray pulse height analysis | Core electron temperature |
| --- | --- |

| Neutral particle analyzer | Core ion temperature and fast ions |
| --- | --- |

| Visible spectrometer | Edge/divertor spectroscopy |
| --- | --- |

| Langmuir probes/thermocouples2 | Divertor parameters |
| --- | --- |

Discussion:  The revised table represents the diagnostics set which has evolved since the first issue of the GRD based on NSTX PAC and physics community input.

Cost/Schedule Implications: None

10) Revised center stack hipot requirement (1.5.a)

"b. Diagnostic elements which must be located in the center stack between the OH coil ground plane and the center stack casing (e.g. Rogowski coils, Flux loops) shall be referenced to ground and shall be insulated to withstand the full CHI voltage. The insulation shall be rated for a one minute AC DC hipot test at 5kV rms.

Discussion:  The revised requirement reflects the fact that the peak CHI voltage (as enforced by the overvoltage protection equipment) will be 2kV DC, and therefore the hipot of related equipment should be at 2E+1kV = 5kV DC.

Cost/Schedule Implications: None

11) PF3 bipolarity requirement (1.6.3.b)

"b. All PF circuits except for OH and PF3 shall be unipolar during a pulse. OH bipolarity is required maximize the flux swing provided by the OH coil. PF3 bipolarity is required to provide cancellation of the stray field from the OH coil during plasma initiation, after which time a current of the opposite polarity is required for plasma equilibrium and shaping."

Discussion:  Analysis of plasma initiation (53-970609-CLN) indicates that the required field null can be best accomplished using PF3. However, the required current polarity is opposite of that required for the scenarios and equilibria. Therefore the PF3 power supply must be designed to supply both polarities of current.

Cost/Schedule Implications:

S Ramakrishnan to elaborate.

12) Pulses and Operating Lifetime (2.1.2.3.c,e,f)

"c. The NSTX machine shall be designed to operate for a period of 10 years. However, new equipment related to the D-site infrastructure (building and utilities) shall be designed for a 25 year operating life.

e. For design purposes the distribution of pulses shall be assumed as follows:

• 1/3 of the pulses will be double-swung OH covering inductive only, short pulse operation

• 2/3 of the pulses will be single-swung OH covering non-inductive current drive studies

f. Approximately 25% of of all the shots shall be at high toroidal field (6 kG). This proportion will be split into double-swung and single-swung OH in the same proportion as 3 kG operation."

Discussion:  Clarification of design requirements.

Cost/Schedule Implications: None.

13) CHI requirement (2.1.3.2.a)

2.1.3.2 CHI Current Sustainment Power Supply Upgrade

a. The CHI current sustainment power supply would be required to provide a current of 20.0 kA at a power level of up to 15.0 MW, for the full duration SOD to EOD.

Discussion:  Resistance projected for CHI has changed, 15MW is no longer correct. Since effective resistance seen by CHI power supply is not well known, it is more appropriate to specify the current injection only at this time.

Cost/Schedule Implications: None.

14) NBI Upgrade up to two beamlines (2.1.3.3.a & 2.2.4.a)

"2.1.3.3 Neutral Beam Injection (NBI) Upgrade

a. The NSTX design shall consider an upgrade which would, initially, place one TFTR Neutral Beam Injector  on NSTX, operating in the range of 80 to 110kV with corresponding power to the plasma of 5.33 to 7.33MW, for 5 seconds once every 300 seconds. Provision shall also be included for the possible implementation of a second TFTR Neutral Beam Injector at a later date.

2.2.4  NBI Upgrade

a. The NBI system (if the initial one beam line upgrade is undertaken) shall be assumed to deliver 6MW to the plasma at 80kV for 5 seconds once every 300 seconds. The total power to be accomodated in the case of a two beam line upgrade is TBD."

Discussion:  Provision is to be included for up to two NB lines.

Cost/Schedule Implications: See item 2)

15) CHI Parameters (2.2.2.b,c)

b. For purposes of sizing the CHI power system components, the plasma (viewed from the external circuit terminals) may be modeled by an inductance of 0.180.0 µH, and a ratio of Ip to Ichi of 10 may be assumed. resistance of 7.0mΩ.

c. Based on the nominal discharge current and equivalent circuit parameters above, the nominal voltage across the CHI electrodes is approximately 1kV. However, to provide flexibility, the design voltage level shall be 2kV. All related electrical insulation shall withstand a one minute, 5kV DC hipot test.

Discussion:  The parameters given reflect the final estimate as projected by D. Orvis of Univ. of Washington. In paragraph c, the detailed electrical isolation requirements are specified.

Cost/Schedule Implications: None.

16) GDC Requirements (2.2.3.a)

"a. The NSTX shall be capable of operation in equipped to provide glow discharge cleaning (GDC) mode with DC glow in in 10 -2 Torr  the range of 5 ~ 30 mtorr of D or He at voltages up to ~450 V and currents up to ~20 A for indefinitely long periods with a throughput of 44 torr-l/s."

Discussion:  GDC to be provided (see item 6)), and minor change in performance specifications.

Cost/Schedule Implications: None.

17) Pre-shot temperature (2.3.4.a)

"a.  In-vessel components shall be maintained at temperatures not less than 50°C between discharges."

Discussion:  New specification.

Cost/Schedule Implications: None.

18) Limiter requirements (2.5.a,b)

a. The inner wall shall act as the inboard limiter.

b. Passive plates and HHFW antenna shielding shall act as the outboard limiter.

Discussion:  Clarification of limiter features.

Cost/Schedule Implications: None.

19) PFC High Heat Flux Regions

2.6.1 Divertor and Limiter High Heat Flux Regions

a. For all DN X-point scenarios, the heat load on the inboard divertors shall be deposited at locations on upper and lower surfaces, each described by vertical and horizontal lines swept through an angle φ = 0 to 360o. The vertical lines connect between points (R1, Z1) and (R2, Z2) where R1 = R2 = 0.2562 m, Z1 = +/- 1.2608 m, and Z2 = 1.6030 m. The horizontal lines connect between points (R2, Z2) and (R3, Z3) where R2 = 0.2562 m, Z2 = Z3 = +/- 1.6030 m, and R3 = 0.4756 m.

b. For all DN X-point scenarios the heat load on the outboard divertors shall be deposited at locations on upper and lower conical surfaces, each  described by a line between points (R1, Z1) and (R2, Z2), swept through an angle φ = 0 to 360o, where R1 = 0.5880 m, Z1 = +/- 1.6648 m, R2 = 0.9842 m, Z2 = +/- 1.5352 m.

c. For all SN X-point scenarios the heat load shall be deposited on the inboard and outboard divertor plates as per h. and i. above, except only on the lower surfaces.

d. The region of the inner wall which will act as an inboard limiter for Natural Divertor plasmas and receive the heat loads associated therewith is defined as that extending between Z = +/- 1.0 m.

Discussion:  It was decided to remove the precise specifications of heat load deposition regions from the GRD.  Instead they are being specified (in greater detail than the above) in the PFC SRD.

Cost/Schedule Implications: None.

20) PFC Heat Loads (Table 2.6.2-1)

Table 2.6.2-1: PFC Heat Loads

| Scenario | Parameter | Inner Div | Outer Div | Inner Wall | Total |  |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | Upper | Lower | Upper | Lower |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Natural Divertor | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 0.0 | 1.1 | 1.1 | 2.1 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.000 | 0.094 | 0.094 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 0.0 | 4.3 | 4.3 | 1.9 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Double Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.4 | 0.4 | 1.7 | 1.7 | 0.0 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.038 | 0.038 | 0.038 | 0.038 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 5.1 | 5.1 | 17.1 | 17.1 | 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Single Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 3.5 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 0.4 1.4 | 0.0 | 1.8 2.8 | 0.3 0.0 | 2.5 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.092 | 0.000 | 0.100 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 2.0 7.0 | 0.0 | 7.0 10.8 | 0.3 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  Physics has recommended adopting the more pessimistic of the two cases for single null power partitioning as given in "NSTX Divertor Heat Flux Estimates", S. Kaye, 10/24/96

Cost/Schedule Implications: None. The baseline PFC tiles are able to withstand the given loads.

21) Breakdown field null target (2.6.1.a)

a. The NSTX PF/OH magnets and power systems shall be designed to produce a loop voltage ≥ 3.0V and a field null (≤ 2 Gauss) region ≥ 40 cm diameter for ≥ 2 mS at a target located at r=0.8 m on the midplane.

Discussion:  Nominal target location is specified.

Cost/Schedule Implications: None.

22) Field Errors (2.6.5)

a. To avoid locked modes from non-axisymmetric fields (including those from coil misalignment, bus work, coil feeds, coil cross-overs, and magnetic materials), the m=2 n=1 component of the vacuum error field (at the q=2 surface) shall be limited to 5 gauss. If necessary, compensation coils must be installed.

Discussion:  New requirement per PRD.

Cost/Schedule Implications: The ability of the present coil design, given alignment tolerances, to satisfy the requirement is presently under evaluation.  It may or may not prove necessary to install a field error compensation coil and power supply set.

23) Disruptions (2.7.a)

"• a frequency of occurrence of 10 50 % in any phase or mode of operation."

Discussion:  Nominal disruption frequency is changed to be in accordance with the PRD.

Cost/Schedule Implications: None.