National Spherical Tokamak Experiment

NSTX

GENERAL REQUIREMENTS DOCUMENT

NSTX-RQMTS-GRD

Revision 0

May 28, 1996

________________________________      	   ______________________________

Charles Neumeyer	Masa Ono

NSTX Project Engineering Manager	NSTX Project Manager

Record of Revisions

| Revision | Date | MRB | Description of Change |
| --- | --- | --- | --- |

Preface			1

1 Functions, Configuration & Essential Features	2

1.1 General		2

1.1.1 Location	2

1.1.2 Design Basis and Upgrades	2

1.1.3 Directions of Fields & Currents	3

1.1.4 Material Selection	4

1.2 WBS 1 Tokamak Systems	4

1.2.1 Plasma Facing Components	4

1.2.2 Vacuum Vessel & Support Structure	5

1.2.3 Magnets	6

1.3 WBS 2 Plasma Heating & Current Drive Systems	7

1.3.1 High Harmonic Fast Wave (HHFW)	7

1.3.2 Coaxial Helicity Injection (CHI)	8

1.3.3 Electron Cyclotron Heating (ECH)	8

1.4 WBS 3 Auxiliary Systems	8

1.4.1 Vacuum Pumping System (VPS)	8

1.4.2 Cooling Water Systems (CWS)	9

1.4.3 Gas Delivery System (GDS)	9

1.4.4 Bakeout System	9

1.5 WBS 4 Plasma Diagnostics	10

1.6 WBS 5 Power Systems	10

1.6.1 AC Power Systems	11

1.6.2 TF Power Conversion System	11

1.6.3 PF Power Conversion System	11

1.6.4 CHI Power Conversion System	12

1.7 WBS 6 Central Instrumentation & Control (I&C) System	12

1.8 WBS 72 Project Physics	13

2 Performance & Operational Requirements	15

2.1 General		15

2.1.2 Baseline Requirements	18

2.1.3 Upgrades	23

2.2 Plasma Heating and Current Drive	24

2.2.1 High Harmonic Fast Wave (HHFW) Heating & Current Drive	24

2.2.2 Coaxial Helicity Injection (CHI)	24

2.2.3 Electron Cyclotron Heating (ECH)	25

2.2.4  NBI Upgrade	25

2.3 Vacuum and Wall Conditioning	25

2.3.1 Base Pressure	25

2.3.2 Bakeout	25

2.3.3 Glow Discharge Cleaning	26

2.3.4 Boronization	26

2.3.4 Pre-shot Temperature	26

2.4 Fueling	26

2.4.1 Gas Injection	26

2.5 Power Handling and Particle Control Plasma Boundaries	26

2.5.1 Scrape-off Layer (SOL) Envelopes	26

2.5.2 Maximum Heating Power	27

2.5.3 Divertor Requirements	27

2.5.2 Limiter Requirements	27

2.5.2.1 Limiter Materials and Power Handling	28

2.6 Plasma Startup and Equilibrium Control	28

2.6.1 Radial Plasma Position Control	28

2.6.2 Vertical Plasma Position Control	28

2.6.3 Divertor Strike Point Position Control (Scenarios I and III only)	28

2.6.4 Plasma Startup	29

2.7 Plasma Disruptions	29

3 Reliability, Availability, and Maintainability Requirements	30

3.1 Mission Reliability	30

3.2 Operational Availability	30

3.3 Shot Reliability	30

3.4 Maintainability	31

3.5 Scheduled Maintenance	31

4 Design Criteria		32

4.1 General Design Guidelines	32

4.2 Facilities Design Criteria	32

5 Environmental, Safety, and Health (ES&H) Requirements	35

5.1 Policy Statement	35

5.2 Radiological Design Objectives	35

6 Decontamination and Decommissioning Requirements	37

Preface

The mission of the National Spherical Tokamak Experiment (NSTX) is to assess the physics performance of the Spherical Tokamak (ST) concept, in which the aspect ratio (ratio of major radius (R0) to minor radius (a), R0/a) is much lower than most tokamaks built to date. Supporting objectives are to:

• Exploit  techniques for non-inductive current drive and profile control that are consistent with efficient continuous operation of a tokamak fusion reactor without a central solenoid.

• Maximize the use of existing facilities and components so as to minimize the cost of the project.

Top-level performance requirements for NSTX, along with cost and schedule objectives, are included in the Project Definition Statement (PDS).

Operational requirements and physics performance objectives are given in the Project Requirements Document (PRD).

The General Requirements Document (GRD) defines the overall engineering requirements.

System Requirements Documents (SRD) and System Design Descriptions (SDD) shall be written for each major element of the Work Breakdown Structure (WBS) and shall provide the detailed basis for the engineering design of the machine.

The order of precedence for these requirements is the PDS, PRD, GRD, SRD, SDD.

1 Functions, Configuration & Essential Features

The functions, configuration and essential features of the NSTX machine and the various hardware elements of the WBS are outlined in the following.

1.1 General

1.1.1 Location

a. The NSTX machine shall be installed at the PPPL C-site C-cell (formerly occupied by the Princeton Large Torus (PLT)).

1.1.2 Design Basis and Upgrades

a. This GRD provides the baseline requirements for NSTX and provides additional information concerning anticipated upgrades. The baseline design of NSTX shall not preclude the possibility of the upgrades, and shall facilitate the upgrades whenever possible and cost effective.

b. The anticipated upgrades include the following:

• addition of neutral beam injection (NBI)

• long pulse operation (with center stack replacement)

c. Design features related to the NBI upgrade and the schedule for their implementation shall be as indicated in Table 1.1.2-1.

Table 1.1.2-1: Schedule for NBI Upgrade Features

| Design Feature | Schedule |
| --- | --- |

| Plasma Facing Components (PFC) additional power dissipation | B |
| --- | --- |

| PFC protective armor for NB shine-through and inadvertent loading | U |
| --- | --- |

| PFC beam dumps | U |
| --- | --- |

| Vacuum Vessel (VV) additional power dissipation | B |
| --- | --- |

| VV port compatibility and availability | B |
| --- | --- |

| Vacuum Pumping System (VPS) exhaust capability | B |
| --- | --- |

| Cooling Water System (CWS) NB source cooling capacity | U |
| --- | --- |

| Power Systems (PS) AC power | U |
| --- | --- |

| Central I&C additional control, data acquisition, and data network capacity | U |
| --- | --- |

| Facility space and provision for installation of NBI and related services | B |
| --- | --- |

B = To be included in baseline

U = to be deferred until upgrade, unless convenient to include in baseline at minimal cost

d. Design features related to the long pulse upgrade and the schedule for their implementation shall be as indicated in Table 1.1.2-2.

Table 1.1.2-2: Schedule for Long Pulse Upgrade Features

| Design Feature | Schedule |
| --- | --- |

| PFC active cooling | U |
| --- | --- |

| Vacuum Vessel (VV) active cooling | U |
| --- | --- |

| Toroidal Field (TF) magnet inner legs | U |
| --- | --- |

| Toroidal Field (TF) magnet outer legs | B |
| --- | --- |

| Ohmic Heating (OH) magnet | not req'd |
| --- | --- |

| Poloidal Field (PF) magnets | B |
| --- | --- |

| Plasma Heating & Current Drive long pulse capability | U |
| --- | --- |

| Cooling Water System (CWS) additional cooling capacity | U |
| --- | --- |

| Gas Delivery System (GDS) long pulse capacity | U |
| --- | --- |

| Plasma Diagnostics long pulse capability | U |
| --- | --- |

| Power Systems (PS) AC and DC long pulse power supply | U |
| --- | --- |

| Central I&C long pulse control and data acquisition capability | U |
| --- | --- |

B = To be included in baseline

U = to be deferred until upgrade, unless convenient to include in baseline at minimal cost

1.1.3 Directions of Fields & Currents

a. Figure 1.1.3-1 defines the positive sense of the right-handed coordinate system which shall be used on NSTX.

NSTX Coordinate System

Figure 1.1.3-1

Referring to Figure 1.1.3-1 the definition of positive sense of currents and fields on NSTX shall be as indicated in Table 1.1.3-1.

Table 1.1.3-1: NSTX Current/Field (+) Directions

| Current/Field | Nominal Direction |
| --- | --- |

| Plasma Current | (+) φ |
| --- | --- |

| Poloidal Coil Currents | (+) φ |
| --- | --- |

| Toroidal Field | (-) φ |
| --- | --- |

| Radial Field | (+) R |
| --- | --- |

| Vertical Field | (+) Z |
| --- | --- |

b. All components of NSTX shall be designed to permit operation with plasma current in either φ direction, with toroidal field always in opposite φ direction.

1.1.4 Material Selection

a. All materials to be used in the tokamak and peripheral equipment (R ≤ 3.0 m, |Z| ≤ 3.0m) must have a relative magnetic permeability ≤ 1.02 unless otherwise authorized by the Project.

1.2 WBS 1 Tokamak Systems

1.2.1 Plasma Facing Components

a. All surfaces which face the plasma (Plasma Facing Components (PFCs)) shall consist of carbon based materials designed to absorb the heat and radiation flux from the plasma, to minimize the influx of impurities to the plasma, and to withstand the electromagnetic forces associated with plasma disruption.

b. The PFCs shall include inboard and outboard divertor plates for absorption of the high heat flux due to plasma operation with Double Null (DN) and Single Null (SN) separatrices (X-points).

c. The PFCs shall include passive stabilizers consisting of one upper and one lower plate oriented in the toroidal direction but with electrical breaks to avoid conducting toroidal loops. The precise passive stabilizer geometry requirements and electrical characteristics shall be established as part of the design evolution based on inputs provided by the NSTX Project Physics Group.

d. The PFCs shall include toroidal and poloidal limiters to constrain the boundary of cross sectional space occupied by the plasma.

e. All PFCs shall be designed to accommodate a high temperature bakeout mode to liberate trapped impurities.

f. All PFCs shall be designed to accommodate a helium glow discharge cleaning mode.

g. All PFCs shall be electrically connected to the VV via their mounting structures. An exception is that the center stack casing and inner wall of the bore of the machine, including the inboard divertor plates, shall be electrically isolated from the remainder of the VV to permit the biasing of this structure with respect to the VV for Coaxial Helicity Injection (CHI).

h. Passive plates shall be designed to be removable after the machine has been assembled.

1.2.2 Vacuum Vessel & Support Structure

a. The two halves of the Spheromak-1 (S-1) VV shall be used to form the upper and lower domes of the NSTX VV. A new cylinder shall be constructed to connect the upper and lower domes.

b. The support structure of the VV and the machine shall utilize the existing PLT support structure.

c. Sufficient ports shall be provided with dimensions compatible with the requirements of all types of Heating & Current drive systems (refer to section 1.3) as well as diagnostic systems. Additional openings shall be provided for personnel access.

d. VV ports for High Harmonic Fast Wave (HHFW, refer to section 1.3.1) heating and current drive antennae shall be of adequate size to permit the insertion and removal of the antennae, and shall be located in accordance with the phasing requirements of the HHFW system.

e. The VV shall be electrically grounded with a single point connection to the facility ground, which is designed to permit opening for the purpose of testing of the integrity of electrical isolation between the VV and other components and structures.

1.2.3 Magnets

1.2.3.1 Poloidal Field (PF) Magnets

a. The PF magnets shall consist of an Ohmic Heating (OH) central solenoid magnet, shaping field magnets (PF1a and PF1b), and outer PF magnets (PF2a, 2b, 3a,3b,4a,4b, and 4c) as listed in Table 1.2.3.1-1. Nominal current directions shall be as indicated.

Table 1.2.3.1-1: PF Magnets

| Designation | Source |
| --- | --- |

| OH | New |
| --- | --- |

| PF1a | New |
| --- | --- |

| PF1b | New |
| --- | --- |

| PF2a | S-1, EF-1a |
| --- | --- |

| PF2b | S-1, EF-1b |
| --- | --- |

| PF3a | S-1, EF-2a |
| --- | --- |

| PF3b | S-1, EF-2b |
| --- | --- |

| PF4a | S-1, EF-3a |
| --- | --- |

| PF4b | S-1, EF-3b |
| --- | --- |

| PF4c | S-1, EF-3c |
| --- | --- |

b. The PF coil system shall be symmetric about the mid plane, defined as the horizontal plane which passes through the elevation of maximum plasma radial extent.

c. The PF coils listed in Table 1.2.3.1-1 shall be connected in series groups in galvanically separate circuits with independent current control as indicated in Table 1.2.3.1-2.

Table 1.2.3.1-2: Galvanically Separate PF Circuits

| PF Circuit | Coil Grouping |
| --- | --- |

| 1 | OH |
| --- | --- |

| 2 | PF1a, upper & lower |
| --- | --- |

| 3 | PF1b, upper & lower |
| --- | --- |

| 4 | PF2a, upper & lower |
| --- | --- |

| 5 | PF2b upper |
| --- | --- |

| 6 | PF2b lower |
| --- | --- |

| 7 | PF3a, PF3b, upper & lower |
| --- | --- |

| 8 | PF4a, PF4b, PF4c, upper & lower |
| --- | --- |

d. All series coil connections indicted in Table 1.2.3.1-2 shall result in current flow which is equal in magnitude, and in the same φ direction, in the upper and lower coils.

e. All aspects of the PF coil design shall be compatible with NSTX operation with plasma current in either the (+) and (-) φ direction, with toroidal field φ direction opposite to plasma current direction.

f. PF coil geometries and current scenarios shall be established as part of the design evolution based on inputs provided by the NSTX Project Physics Group, consistent with the requirements for start up field null quality and loop voltage generation, as well as plasma equilibrium control.

1.2.3.2 Toroidal Field (TF) Magnets

a. The TF magnets shall consist of separate inner leg and outer leg components, all connected in series. The utilization of cross sectional area in the center stack of the machine shall be optimized to maximize the conductor cross sectional area.

b. All aspects of the TF coil design shall be compatible with NSTX operation with plasma current in either the (+) and (-) φ direction, with toroidal field φ direction opposite to plasma current direction.

1.3 WBS 2 Plasma Heating & Current Drive Systems

1.3.1 High Harmonic Fast Wave (HHFW)

a. The HHFW system shall utilize RF power produced by the six existing Tokamak Fusion Test Reactor (TFTR) Ion Cyclotron Range of Frequencies (ICRF) sources. The sources and associated controls and interlocks shall be designed to accommodate shared usage between NSTX, TFTR and the Poloidal Beta Experiment- Modified (PBX-M).

b. The HHFW system shall utilize twelve antennae mounted on the vacuum vessel wall in the gap between the upper and lower passive stabilizing plates. The antennae shall be arranged toroidally between the NSTX vacuum vessel mid plane ports. Each antenna shall consist of a current strap, a back plane, a Faraday shield, and a local (bumper) limiter structure. The antennae shall be compatible with bakeout and glow discharge modes of operation.

c. The set of six sources (and twelve antennae) shall be capable of operation in subsets with up to three different frequencies during an NSTX discharge. Operating frequencies shall be in the range of 30 - 50 MHz. All RF generators operating at any single frequency shall be phase locked to provide control over the wave number spectrum excited in the plasma at that frequency.

d. Switches shall be provided to isolate HHFW power from the NSTX test cell. These switches shall be interlocked with the test cell access control system.

1.3.2 Coaxial Helicity Injection (CHI)

a. The center stack casing and inner wall of the bore of the machine, including the inboard divertor plates, shall be electrically isolated from the remainder of the VV to permit the biasing of this structure with respect to the VV, to drive a poloidal component of plasma current.

b. The PFCs which form the electrodes from which the CHI current flows shall be designed to dissipate the local power generation due to the CHI current, during normal and fault conditions, and shall be designed to withstand the electromagnetic forces due the fault current deliverable by the CHI power supply system prior to crow bar.

1.3.3 Electron Cyclotron Heating (ECH)

a. The ECH system shall provide microwave RF power in the electron cyclotron range of frequencies. A new system shall be provided, dedicated for use on NSTX.

b. TBD ECH launchers shall be installed at an outboard location in the VV to direct the microwave radiation toward the desired absorption location. The launchers must be compatible with bakeout and glow discharge modes. Each launcher shall have a vacuum window connecting it to the external transmission system. This window must have low electrical loss at microwave frequencies and satisfy the thermal and mechanical requirements for vacuum interfaces.

c. Switches shall be provided to isolate ECH power from the NSTX test cell. These switches shall be interlocked with the test cell access control system.

1.4 WBS 3 Auxiliary Systems

1.4.1 Vacuum Pumping System (VPS)

a. The VPS shall provide the required high vacuum environment for plasma operations. It shall exhaust the spent plasma constituents after each pulse, and shall exhaust the impurities associated with the bakeout and glow discharge cleaning of the PFCs.

b. The VPS shall be equipped with redundant Residual Gas Analyzers (RGA).

1.4.2 Cooling Water Systems (CWS)

a. The CWS shall provide the required cooling water of the required quality, temperature, pressure and flow for all NSTX systems. Maximum use shall be made of existing equipment at C-site.

b. Instrumentation shall be provided to monitor the flow rate in all individual magnet coil cooling paths, with sufficient sensitivity and speed of response to permit power supply shutdown and avoid magnet damage due to overheating.

c. Instrumentation shall be provided to monitor the outlet water temperature of a subset of the magnet coil cooling paths, with provision for alarming both high and low temperature conditions.

d. The CWS and NSTX facility Heating, Ventilation, and Air Conditioning (HVAC) system shall include dew point controls to prevent condensation on water cooled components in the NSTX test cell.

1.4.3 Gas Delivery System (GDS)

a. The GDS shall provide the required gas injection for NSTX for hydrogen, deuterium, and helium as required for conditioning and plasma operations.

b. Three piezoelectric valves shall be provided to control gas injection to the VV. The valves shall be located on the mid plane at three locations separated by 120 degrees. The valves shall be supplied by manifolds which connect to four source tanks in such a way that any valve can be supplied from any source tank.

1.4.4 Bakeout System

a. The bakeout system shall include means for heating the surfaces of the PFCs to the required temperatures for an indefinite period while maintaining the VV and components connected thereto to temperatures within their temperature ratings.

1.5 WBS 4 Plasma Diagnostics

a. The baseline plasma diagnostics set shall consist of the elements listed in Table 1.5-1.

Table 1.5-1: Baseline NSTX Diagnostics Set

| Diagnostic | Purpose |
| --- | --- |

| Multichannel TVTS (single pulse) | Te(r), ne(r) |
| --- | --- |

| CHERS (with neutral probe beam) | Ti(r), vf(r) |
| --- | --- |

| MSE (with neutral probe beam) | j(r) |
| --- | --- |

| Multichannel bolometer | Prad(r) |
| --- | --- |

| Visible continuum array | Zeff(r) |
| --- | --- |

| 1 mm microwave interferometer | nel |
| --- | --- |

| Survey spectrometer (SPRED) | Impurities |
| --- | --- |

| Soft X-ray imaging system | Fluctuations |
| --- | --- |

| Mirnov coils | Fluctuations |
| --- | --- |

| Flux loops | Fluctuations |
| --- | --- |

| Diamagnetic loop | Beta |
| --- | --- |

| Rogowski coils | Ip |
| --- | --- |

| Flux loops | Magnetics |
| --- | --- |

| Ha detectors | Edge recycling |
| --- | --- |

| Langmuir probes | Edge parameters |
| --- | --- |

| Visible camera | Plasma control |
| --- | --- |

| IR camera | Heat loads |
| --- | --- |

| Residual gas analyzers | In-vessel gases |
| --- | --- |

b. Diagnostic elements which must be located on the center stack (e.g. Rogowski coils, Flux loops) shall be referenced to ground potential, and shall be adequately insulated to withstand the voltage associated with the biasing of the center stack casing for CHI

c. All diagnostic signals shall be isolated via optical or magnetic means prior to exiting the test cell boundary. The isolation shall be rated to withstand a one minute AC hipot test at TBD kV.

1.6 WBS 5 Power Systems

a. The C-site power supply resources shall be utilized to the maximum extent for NSTX operation.

b. Transfer switches shall be provided to allow the shared usage of power supplies which serve both NSTX and PBX-M.

c. Controls and interlocks shall be designed to accommodate shared usage between NSTX and PBX-M.

1.6.1 AC Power Systems

a. The AC Power Systems shall provide all electrical power required for the NSTX facility.

b. Experimental operation of the NSTX facility shall not be coincident with the operation of other major machines at PPPL (TFTR, PBX-M). Provision shall be included to prevent simultaneous operation to the extent that limits on power delivery to PPPL could be exceeded.

1.6.2 TF Power Conversion System

a. The TF Power Conversion System shall be designed to provide a constant current in the TF magnets during an NSTX discharge based on a reference current input signal provided in real time.

b. Bus links or other means shall be provided in order that the current in the TF magnets can be driven in either direction. However the toroidal field direction shall always be opposite to the plasma current direction. The polarity reversal procedure shall require 4 hours or less.

c. No-load disconnect switches shall be provided to isolate the TF Power System from the connections to the TF magnets. In addition, no-load grounding switches shall be provided to ground the terminals of the connections to the TF magnets. These switches shall be interlocked with test cell access control.

d. Redundant measurements of TF current shall be provided.

e. The TF Power Conversion System shall include protection to avoid the delivery of current which would overheat the TF magnets, assuming that their coolant conditions are nominal.

1.6.3 PF Power Conversion System

a. PF Power System shall be designed to provide a variable current in the PF magnet circuits (listed in Table 1.2.3.1-2) during an NSTX discharge based on a reference current input signal provided in real time.

b. All PF circuits except for OH shall be unipolar during a pulse.

c. Bus links or other means shall be provided in order that the current in the PF magnets can be driven in either direction. The polarity reversal procedure shall require 4 hours or less.

d. No-load disconnect switches shall be provided to isolate the PF Power System from the connections to the PF magnets. In addition, no-load grounding switches shall be provided to ground the terminals of the connections to the PF magnets. These switches shall be interlocked with test cell access control.

e. Redundant measurements of each PF current shall be provided.

f. The PF Power Conversion System shall include protection to avoid the delivery of current which would overheat the PF magnets, assuming that their coolant conditions are nominal.

1.6.4 CHI Power Conversion System

a. CHI Power System shall be designed to provide a capacitor bank discharge followed a controlled current in the CHI circuit during an NSTX discharge based on a reference current input signal.

b. A fast acting crow-bar circuit shall be provided to divert the capacitor bank discharge from the NSTX machine on command.

b. Bus links or other means shall be provided in order that the current in the CHI circuit can be driven in either direction. The polarity reversal procedure shall require 4 hours or less.

c. No-load disconnect switches shall be provided to isolate the CHI Power System from the connections to the CHI magnets. In addition, no-load grounding switches shall be provided to ground the terminals of the connections to the CHI magnets. These switched shall be interlocked with test cell access control.

d. Redundant measurements of CHI current shall be provided.

1.7 WBS 6 Central Instrumentation & Control (I&C) System

a. The Central I&C system shall provide all supervisory control and monitoring of the NSTX facility, including:

• Plant Control and Monitoring (asynchronous routine control and monitoring)

• Synchronization System (synchronization of triggered actions from master clock events)

• Critical Data Network (high speed synchronous facility-wide network for real time 	control)

• Safety Interlock System (master supervisory control of experiment)

• Access Control System (control and monitoring of access to hazardous areas)

• Data Acquisition System (periodic sampling, acquisition and display of regularly sampled 	data, and acquisition and display of data sampled and stored by remote devices)

b. The Central I&C shall provide reference signals to local I&C systems (provided by responsible WBS elements) for local feedback control, but is not required to close the loop for feedback control of plant processes.

c. The NSTX Control Room shall be installed in the location previously occupied by the PLT Control Room.

d. The design of the I&C system shall facilitate access to archived NSTX data by parties outside of the NSTX control room (e.g. remote collaborators, physicists inside and outside of PPPL) in a read-only mode.

e. The Central I&C system shall provide sixteen (16) electrically isolated, general purpose signal channels between the test cell and the control room. These signals shall be displayed in the control room in real-time.

1.8 WBS 72 Project Physics

a. NSTX Project Physics shall be responsible for the implementation of all Plasma Control hardware and software.

b. NSTX Project Physics shall provide the requirements to the other WBS elements as listed in this document which are directly related to plasma behavior including the following.

• PF coil current centers required to achieve NSTX mission

• PF coil current scenarios and equilibria which are required to achieve NSTX 		mission including:

- establishment of required field null quality for plasma initiation

- establishment of required loop voltage for plasma initiation

- reference plasma scenarios

- plasma shape and profile flexibility equilibria

- feed back control of radial and vertical position, as well as divertor 			strike point position

• PF coil voltage requirements due to:

- basic scenarios

- feedback control of radial and vertical position, as well as divertor

strike point position

• Geometric and electrical characteristics of passive plates

• Description of cross sectional boundary of space occupied by plasma for all modes of 	operation

• Maximum heat loads on PFCs, including divertor plates and limiters, due to plasma 	operation and CHI

• Currents and voltages in passive structures and PF coils due to plasma disruption.

• Effective impedance presented to CHI power supply circuit

2 Performance & Operational Requirements

2.1 General

2.1.1 Definitions

a. The events listed in Table 2.1.1-1 are defined for the purposes of requirements definition.

Table 2.1.1-1: Definition of Events

| Name | Definition |
| --- | --- |

| SOP | Start of Pulse (start of power supply energization) |
| --- | --- |

| SOPTF | Start of Pulse TF (start of TF current) |
| --- | --- |

| SOPPF | Start of Pulse PF (start of PF current) |
| --- | --- |

| SOFTF | Start of Flat Top TF |
| --- | --- |

| SOI | Start of Initiation |
| --- | --- |

| EOI | End of Initiation |
| --- | --- |

| SOD | Start of Plasma Discharge |
| --- | --- |

| SOF | Start of Plasma Flat Top |
| --- | --- |

| SOX | Start of X-point |
| --- | --- |

| EOX | End Start of X-point |
| --- | --- |

| EOF | End of Plasma Flat Top |
| --- | --- |

| EOD | End of Plasma Discharge |
| --- | --- |

| EOFTF | End of Flat Top TF |
| --- | --- |

| EOPPF | End of Pulse PF (end of PF current) |
| --- | --- |

| EOPTF | End of Pulse TF (end of TF current) |
| --- | --- |

| EOP | End of Pulse (end of power supply energization) |
| --- | --- |

b. The plasma major radius (R0) is defined as the radius from the major axis of the tokamak to the center of the plasma on the mid plane.  The plasma minor radius (a) is defined as the half width of the plasma on the mid plane.

c. All required flux swings (volt-seconds) are measured at a major radius of 0.8 meters, on the mid plane.

d. All required loop voltages are measured at a major radius of 0.8 meters, on the mid plane at the EOI event time.

e. For limiter (natural divertor) plasmas, the plasma edge is defined as the flux surface which is tangent with the inboard limiting surface at the mid plane. The Scrape Off Layer (SOL, which contains approximately 1- ε-3 ≈ 95% of the particle power loss from the plasma) is defined as the region between the plasma edge and the flux surface which passes through the mid plane at a location TBD cm outboard from the plasma edge. A fraction of the SOL impinges on the inboard limiting surface; the remainder impinges on other PFCs. Refer to Figure 1.1.2-1.

Limiter (Natural Divertor) Plasma

Figure 2.1.1-1

f. For double null X-point divertor plasmas, the inboard and outboard plasma edges are defined as the flux surfaces which pass through the X-points and which intersect the mid plane at R0+/- a. These flux surfaces extend beyond the X-point and impinge on the inboard and outboard divertors at the inboard and outboard strike points. The inboard SOL shall be defined as the region between the inboard plasma edge and the flux surface which passes through the mid plane at R0-a-2.0 cm. The outboard SOL shall

be defined as the region between the outboard plasma edge and the flux surface which passes through the mid plane at R0+a+2.0 cm. Refer to Figure 2.1.1-2.

Double Null Plasma

Figure 2.1.1-2

g. For single null X-point divertor plasmas, the plasma edge is defined as the flux surfaces which passes through the X-point and which intersect the mid plane at R0+/- a. This flux surface extends beyond the X-point and impinges on the inboard and outboard divertors at the inboard and outboard strike points. The SOL shall be defined as the region between the outboard plasma edge and the flux surface which passes through the mid plane at R0+a+2.0 cm. Refer to Figure 2.1.1-3.

Single Null Plasma

Figure 2.1.1-2

2.1.2 Baseline Requirements

2.1.2.1 Plasma Scenarios

a. The baseline NSTX facility shall be designed to accommodate the following scenarios:

I. Inductive Scenario with Natural Divertor, 0.5 second flat top

II. Non-Inductive Scenario with Natural Divertor, 5.0 second flat top

III. Non-Inductive Scenario with Double Null (DN) X-point Divertor, 5.0 second flat top2

IV. Non-inductive Scenario with Single Null (SN) X-point Divertor, 5.0 second flat top2

b. For all reference scenarios described herein the TF flat top shall extend for the full duration of plasma current, i.e. SOFTF ≤ SOI, and EOD ≤ EOFTF.

c. For all reference scenarios described herein the nominal toroidal field shall be 3.2kG at a major radius of 0.8 m, in a direction opposite to the plasma current. However, it shall be possible to operate with a toroidal field at 6.4kG with a TF flat top (SOFTF to EOFTF) duration of 0.6 seconds.

d. The peak-to-average toroidal field ripple shall be ≤ 0.5% over the entire plasma cross section.

e. For all reference scenarios described herein the PF current distributions at SOI, EOI, SOD, SOF, SOX, EOF, and EOD shall be specified by the NSTX Project Physics group.

f. For all reference scenarios described herein the gap between the outboard plasma edge at the mid plane and the surface of the HHFW antennae shall be TBD cm.

g. For all reference scenarios described herein the heating/current drive power shall be applied to the plasma for the full duration SOF to EOF, except for the ECH which shall be applied only during the interval SOI to EOI, with a maximum duration of 100mS.

2.1.2.1.1 Scenario I (Inductive with Natural Divertor)

a. The Inductive Natural Divertor scenario is characterized by plasma initiation via inductive loop voltage, inductive ramp up, flat top, and ramp down of plasma current using the PF coils. The plasma is inboard limited. Performance requirements are summarized in Table 2.1.2.1.1-1.

Table 2.1.2.1.1-1: Scenario I (Inductive with Natural Divertor)

| Requirement | Value | Units |
| --- | --- | --- |

| Plasma Initiation Voltage | 6.0 | volts |
| --- | --- | --- |

| Duration SOI/EOI | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOI/EOI | TBD | volt-seconds |
| --- | --- | --- |

| Duration SOD/SOF | 0.2 | seconds |
| --- | --- | --- |

| Flux Swing SOD/SOF | TBD | volt-seconds |
| --- | --- | --- |

| Plasma Flat Top Current | 1.0 | Meg-Ampere |
| --- | --- | --- |

| Duration SOF/EOF | 0.5 | seconds |
| --- | --- | --- |

| Flux Swing SOF/EOF | TBD | volt-seconds |
| --- | --- | --- |

| Duration EOF/EOD | TBD | seconds |
| --- | --- | --- |

| Flux Swing EOF/EOD | TBD | volt-seconds |
| --- | --- | --- |

| HHFW Power/Plasma SOF/EOF | 0.0 | MW |
| --- | --- | --- |

| ECH Power/Plasma SOF/EOFSOI/EOI | 0.2 | MW |
| --- | --- | --- |

| CHI Power/Plasma SOF/EOF | 0.0 | MW |
| --- | --- | --- |

| Inboard Limiter Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

| Upper Inboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Upper Outboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Lower Inboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Lower Outboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Other PFC Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

2.1.2.1.2 Scenario II (Non-Inductive with Natural Divertor)

a. The Partial Non- Inductive Natural Divertor scenario is characterized by plasma initiation via inductive loop voltage ECH and CHI, partial inductive and partial CHI ramp up, plasma ramp up, flat top, and ramp down current drive via CHI, HHFW, ECH, and bootstrap, and inductive ramp down. The plasma is inboard limited. Performance requirements are summarized in Table 2.1.2.1.2-1.

Table 2.1.2.1.2-1: Scenario II (Non- Inductive with Natural Divertor)

| Requirement | Value | Units |
| --- | --- | --- |

| Plasma Initiation Voltage | 0.0 | volts |
| --- | --- | --- |

| Duration SOI/EOI | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOI/EOI | TBD | volt-seconds |
| --- | --- | --- |

| Duration SOD/SOF | 0.2 | seconds |
| --- | --- | --- |

| Flux Swing SOD/SOF | TBD | volt-seconds |
| --- | --- | --- |

| Plasma Flat Top Current | 1.0 | Meg-Ampere |
| --- | --- | --- |

| Duration SOF/EOF | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOF/EOF | TBD | volt-seconds |
| --- | --- | --- |

| Duration EOF/EOD | TBD | seconds |
| --- | --- | --- |

| Flux Swing EOF/EOD | TBD | volt-seconds |
| --- | --- | --- |

| HHFW Power/Plasma SOF/EOF | 6.0 | MW |
| --- | --- | --- |

| ECH Power/Plasma SOF/EOFSOI/EOI | 0.2 | MW |
| --- | --- | --- |

| CHI Power/Plasma SOF/EOF | 0.0 | MW |
| --- | --- | --- |

| Inboard Limiter Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

| Upper Inboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Upper Outboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Lower Inboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Lower Outboard Divertor Power Density SOX/EOX | TBD | MW/m2 |
| --- | --- | --- |

| Other PFC Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

2.1.2.1.3 Scenario III (Partial Non- Inductive with Double Null X-Point)

a. The Partial Non-Inductive Double Null X-Point scenario is characterized by plasma initiation via inductive loop voltage ECH and CHI, partial inductive and partial CHI ramp up, plasma ramp up, flat top, and ramp down current drive via CHI, HHFW, ECH, and bootstrap, and inductive ramp down.  Performance requirements are summarized in Table 2.1.2.1.3-1.

Table 2.1.2.1.3-1: Scenario III (Partial Non- Inductive with Double Null X-Point)

| Requirement | Value | Units |
| --- | --- | --- |

| Plasma Initiation Voltage | 0.0 | volts |
| --- | --- | --- |

| Duration SOI/EOI | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOI/EOI | TBD | volt-seconds |
| --- | --- | --- |

| Duration SOD/SOF | 0.2 | seconds |
| --- | --- | --- |

| Flux Swing SOD/SOF | TBD | volt-seconds |
| --- | --- | --- |

| Plasma Flat Top Current | 1.0 | Meg-Ampere |
| --- | --- | --- |

| Duration SOF/EOF | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOF/EOF | TBD | volt-seconds |
| --- | --- | --- |

| Duration EOF/EOD | TBD | seconds |
| --- | --- | --- |

| Flux Swing EOF/EOD | TBD | volt-seconds |
| --- | --- | --- |

| HHFW Power/Plasma SOF/EOF | 6.0 | MW |
| --- | --- | --- |

| ECH Power/Plasma SOF/EOFSOI/EOI | 0.2 | MW |
| --- | --- | --- |

| CHI Power/Plasma SOF/EOF | 0.0 | MW |
| --- | --- | --- |

| Inboard Limiter Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

| Upper Inboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Upper Outboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Lower Inboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Lower Outboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Other PFC Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

b. The inboard SOL shall intersect the inboard divertors at locations on upper and lower conical surfaces, each  described by a line between points R1, Z1 and R2, Z2, swept through an angle φ = 0 to 360o, where R1 = TBD m, Z1 = +/- TBD m, R2 = TBD m, Z2 = +/- TBD m.

c. The outboard SOL shall intersect the outboard board divertors at locations on upper and lower conical surfaces, each  described by a line between points R1, Z1 and R2, Z2, swept through an angle φ = 0 to 360o, where R1 = TBD m, Z1 = +/- TBD m, R2 = TBD m, Z2 = +/- TBD m.

2.1.2.1.4 Scenario IV (Non-Inductive with Single Null X-Point)

a. The Non-Inductive Single Null X-Point scenario is characterized by plasma initiation via ECH and CHI,  partial inductive and partial CHI ramp up, plasma ramp up, flat top, and ramp down current drive via CHI, HHFW, ECH, and bootstrap, and inductive ramp down. Performance requirements are summarized in Table 2.1.2.1.3-1.

Table 2.1.2.1.3-1: Scenario IV (Non-Inductive with Double Null X-Point)

| Requirement | Value | Units |
| --- | --- | --- |

| Plasma Initiation Voltage | 0.0 | volts |
| --- | --- | --- |

| Duration SOI/EOI | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOI/EOI | TBD | volt-seconds |
| --- | --- | --- |

| Duration SOD/SOF | 0.2 | seconds |
| --- | --- | --- |

| Flux Swing SOD/SOF | TBD | volt-seconds |
| --- | --- | --- |

| Plasma Flat Top Current | 1.0 | Meg-Ampere |
| --- | --- | --- |

| Duration SOF/EOF | TBD | seconds |
| --- | --- | --- |

| Flux Swing SOF/EOF | TBD | volt-seconds |
| --- | --- | --- |

| Duration EOF/EOD | TBD | seconds |
| --- | --- | --- |

| Flux Swing EOF/EOD | TBD | volt-seconds |
| --- | --- | --- |

| HHFW Power/Plasma SOF/EOF | 6.0 | MW |
| --- | --- | --- |

| ECH Power/Plasma SOF/EOFSOI/EOI | 0.2 | MW |
| --- | --- | --- |

| CHI Power/Plasma SOF/EOF | 15.0 | MW |
| --- | --- | --- |

| Inboard Limiter Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

| Upper Inboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Upper Outboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Lower Inboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Lower Outboard Divertor Power Density SOX/EOX | 6.0 | MW/m2 |
| --- | --- | --- |

| Other PFC Power Density SOD/EOD | TBD | MW/m2 |
| --- | --- | --- |

b. The inboard SOL shall intersect the lower inboard divertor at locations on a conical surface described by a line between points R1, Z1 and R2, Z2, swept through an angle φ = 0 to 360o, where R1 = TBD m, Z1 = +/- TBD m, R2 = TBD m, Z2 = +/- TBD m.

c. The outboard SOL shall intersect the lower outboard divertor at locations on a conical surface described by a line between points R1, Z1 and R2, Z2, swept through an angle φ = 0 to 360o, where R1 = TBD m, Z1 = +/- TBD m, R2 = TBD m, Z2 = +/- TBD m.

2.1.2.2 Flexibility

a. In addition to the basic scenarios described in section 2.1.2.1 the design of the NSTX machine shall provide flexibility in terms of variation of plasma shape during the interval between SOF and EOF, which require PF current distributions different from those associated with the basic scenarios. Plasma edge, SOL, and PFC heat load requirements described for the basic scenarios will still apply, however.

b. For all flexibility points the PF current distributions shall be specified by the NSTX Project Physics group.

2.1.2.3 Pulse Repetition Period & Operating Life

a. For the Inductive Scenario (Scenario I) the baseline NSTX machine shall be designed for a pulse repetition period of 600 seconds and a maximum of 50 pulses per 24 hour period.

b. For the Non-Inductive Scenarios (Scenario II, III, and IV) the baseline NSTX machine shall be designed for a pulse repetition period of 300 seconds and a maximum of 100 pulses per 24 hour period, .

c. The NSTX machine shall be designed to operate for a period of 10 years.

d. NSTX operation shall be assumed to consist of 100 pulses per day, 5 days per week, 3 weeks per month, 9 months per year, for 10 years (total of 135,000 pulses). Non-operating times are available for maintenance.

2.1.3 Upgrades

a. The upgrades described herein are envisioned as possibilities which may or may not be undertaken, depending on experience gained with NSTX operation, and other factors. The baseline design of NSTX should facilitate the upgrades when possible without significant cost impact. As a minimum, the baseline design shall permit eventual upgrade and the upgrade path shall be identified as part of the baseline design process.

2.1.3.1 Neutral Beam Injection (NBI) Upgrade

a. The NSTX design shall consider an upgrade which would place the four existing PBX-M sources on NSTX.

b. The baseline design for all NSTX components shall identify an upgrade path for the neutral beam upgrade.

2.1.3.2 Long Pulse Upgrade

a. The NSTX design shall consider an upgrade which would remove the baseline TF and OH magnets, and place a new TF magnet in the center stack. The NSTX plasma pulse length shall be assumed to be 60 seconds for this upgrade. The long pulse repetition period shall be 3600 seconds.

b. The baseline design for all NSTX components shall identify an upgrade path for the long pulse operation.

2.2 Plasma Heating and Current Drive

2.2.1 High Harmonic Fast Wave (HHFW) Heating & Current Drive

a. The HHFW system shall be designed to deliver 6 MW of power to the plasma.

b. A Faraday shield shall be provided between the launcher and plasma.  At operating temperature, the front surface of the Faraday shield shall conform (±2mm) to the surface of a circular toroid located on the mid plane with major radius TBD m and minor radius TBD m within TBD m of the mid plane.

2.2.2 Coaxial Helicity Injection (CHI)

a. The CHI capacitor bank discharge shall be designed to provide a 1/2 cycle sinusoidal current waveform with a peak current of 50.0 kA  at 5mS. TBD ohms at TBD Hz. The delivered energy shall be 3.0 MJoule.

b. The CHI power supply shall be designed to provide a current of 200.0 kA at a power level of up to 15.0 MW.

c. For purposes of sizing the CHI power conversion system, the plasma may be modeled by an inductance of 0.1µH, and a ratio of Ip to Ichi of 10 may be assumed.

2.2.3 Electron Cyclotron Heating (ECH)

a. The ECH system shall be designed to deliver 200kW of power to the plasma.

2.2.4  NBI Upgrade

a. The NBI system (if the upgrade is undertaken) shall be assumed to deliver 5MW to the plasma.

2.3 Vacuum and Wall Conditioning

2.3.1 Base Pressure

a. The NSTX shall operate in high vacuum conditions with a base partial pressure of ≤1x10-7 torr for fuel gases (Z ≤ 2) and a base partial pressure of ≤1x10-9 torr for impurity gases (Z > 2) at 65°C.

b. The impurity gas load during pump down to base pressure shall not exceed 1x10-5 torr-l/s.

c. The vacuum pumping system shall be capable of evacuating the VV from atmospheric pressure to 10-3 torr in less than 3 hours.

d. The vacuum pumping system shall be capable of evacuating the VV from 2 x 10 -3 Torr to 1 x 10 -7 Torr during the interval between each NSTX pulse with a fuel gas load of TBD liters.

e. The vacuum pumping system shall maintain a vacuum of 10 -4 Torr during Glow Discharge Cleaning (GDC).

2.3.2 Bakeout

a. The surfaces of the PFCs shall be heated during bakeout to 350°C.

b. The temperature of the inner VV surfaces shall be heated to 150°C.

c. Bakeout heaters shall be designed to heat the VV and PFCs to the required temperature in 48 hours.

2.3.3 Glow Discharge Cleaning

a. The NSTX shall be capable of operation in a glow discharge cleaning (GDC) mode under zero field conditions with DC glow in 10 -4 Torr of H, D, or He for indefinite periods with a throughput of 44 torr-l/s.

2.3.4 Pre-shot Temperature

In-vessel components shall be maintained at temperatures not less than 50°C between discharges.  The vacuum vessel and internal components shall be maintained at the same temperatures during the interval between discharges.

2.4 Fueling

2.4.1 Gas Injection

a. The NSTX shall be provided with a programmable gas injection system capable of supplying hydrogen, deuterium, and helium gases to the outboard mid plane with a maximum throughput of 250 Torr-l/s.

2.5 Plasma Boundaries

2.5.1 Scrape-off Layer (SOL) Envelopes

a. Considering all of the X-Point scenarios (scenarios III and IV) described herein, all internal hardware (except for the divertor plates) shall be located outside of the SOLs, over the full range of flexibility equilibria. This region of exclusion shall be defined by Project Physics.

2.5.2 Limiter Requirements

2.5.2.1 Limiter Geometry

a. Two outboard toroidal limiters shall be provided. Location, geometry, and other characteristics to be provided by Project Physics.

b. One outboard poloidal limiter shall be provided which conforms to but extends TBD mm beyond the geometry of the Faraday Shield on the HHFW launcher.

c. The inboard wall which faces the plasma shall serve as the inboard toroidal limiter for plasma start up and for defining the plasma edge during natural divertor plasma operation. The surface shall be cylindrical with R = TBD cm extended between Z = +/- TBD cm.

d. Protective armor shall be provided to protect the outboard passive structure and components between the passive structure and vacuum vessel wall from energetic particle losses due to field ripple.

2.5.2.2 Limiter Materials and Power Handling

a. The plasma-facing surfaces of the limiter assemblies shall be made of carbon-based materials.

b. Maximum limiter surface temperatures shall not exceed 1200 °C.

c.Iinboard toroidal limiter power handling requirement is given in section 2.1.2.1. Other limiters shall be designed to handle TBD MW/m2.

2.6 Plasma Startup and Equilibrium Control

2.6.1 Radial Plasma Position Control

a. The NSTX machine shall include a feedback equilibrium control capability which will provide radial plasma position control. Coil current and voltage requirements (in addition to those required to accomplish the basic scenario) shall be specified by NSTX Project Physics.

2.6.2 Vertical Plasma Position Control

a. Conducting structure (passive plates) shall be provided which decrease the growth rate of the vertical instability. Precise geometry and required electrical characteristics of passive plates shall be provided by NSTX Project Physics.

b. The NSTX machine shall include a feedback equilibrium control capability which will provide vertical position control. Coil current and voltage requirements (in addition to those required to accomplish the basic scenario) shall be specified by NSTX Project Physics.

2.6.3 Divertor Strike Point Position Control

a. The NSTX machine shall include a feedback equilibrium control system which will provide outboard strike point position control such that the location of the outboard strike point can be maintained within ±TBD cm of nominal. Coil current and voltage requirements (in addition to those required to accomplish the basic scenario) shall be specified by NSTX Project Physics.

2.6.4 Plasma Startup

a. Requirements for field null quality will be addressed by NSTX Project Physics when determining the coil positions and current scenarios.

2.7 Plasma Disruptions

a. The device shall be designed to accommodate major disruptions characterized by:

• possible vertical displacement

• a thermal quench occurring in 0.1-1 ms

• a current decay with a time-varying decay rate, such that the peak rate is ≤1.25 MA/ms, and the average rate (determined by the time to decay from 90% to 10% of the pre-disruption current) is ≤0.5 MA/ms.

• a peak poloidal halo current up to 33% of the maximum plasma current prior to the disruption, with a toroidal peaking factor of 1.5:1; that is, the toroidal dependence of the halo current is [1 + 0.5*cos (f - f0)], for all toroidal phase angles f0  from 0 to 2*π.

• a frequency of occurrence of 50% in any phase or mode of operation.

b. The device shall be designed to also accommodate major disruptions characterized by:

• vertical displacement

• a thermal quench occurring in 0.1-1 ms

• a peak poloidal halo current up to 40% of the maximum plasma current prior to the disruption, with a toroidal peaking factor of 2:1; that is, the toroidal dependence of the halo current is [1 + 1.0*cos (f - f0)], for all toroidal phase angles f0  from 0 to 2*π.

• a frequency of occurrence of 1% in any phase or mode of operation.

3 Design Criteria

3.1 General Design Guidelines

a. General design guidelines for various operating conditions, based on probability of occurrence, are provided in Table 4.1-1.

Table 3.1-1: General Design Guidelines

| Operating Condition | Description | P, Probability Of Occurrence In A Year | General Design Guidelines |
| --- | --- | --- | --- |

| Normal Events | Events that are planned to occur regularly in the course of facility operation | P=1 | Provide safe and reliable operation |
| --- | --- | --- | --- |

| Anticipated Events | Events of moderate frequency which may occur once or more in the lifetime of a facility | 1>P≥10-2 | The facility should be capable of returning to operation without extensive corrective action or repair |
| --- | --- | --- | --- |

| Unlikely Events | Events which are not anticipated but which may occur during the lifetime of a facility | 10-2>P≥10-4 | The facility should be capable of returning to operation following potentially extensive corrective actions or repairs, as necessary |
| --- | --- | --- | --- |

| Extremely Unlikely Events | Events which are limiting faults and are not expected to occur during the lifetime of a facility but are postulated because of their safety consequences | 10-4>P≥10-6 | Facility damage may preclude returning to operation |
| --- | --- | --- | --- |

| Incredible Events | Events of extremely low probability of occurrence or of non-mechanistic origin | P<10-6 | Not considered in the design |
| --- | --- | --- | --- |

3.2 Facilities Design Criteria

b. The NSTX tokamak structure shall be designed to satisfy the Department of Energy (DOE) standard for natural phenomena hazard (NPH) events.  Only the effects of earthquake shall be considered for the NSTX tokamak structure.  DOE requires the use of Performance Categories (PC) to specify the relative risk, environmental impact, importance, and cost of each facility.  The assessment for seismic loading and evaluation for seismic response shall be followed to determine that the design of the structure is acceptable with respect to the performance goals. There are no safety class items associated with the NSTX machine and since it will be fueled only by hydrogen and deuterium its failure would not result in the release of hazardous materials.  On this basis the seismic performance goal for NSTX tokamak structure is to maintain worker safety and it shall be placed in NPH Performance Category 1 (PC-1).  Those SSCs whose failure would adversely effect the performance of the NSTX tokamak structure or create a threat to worker safety shall also be placed in PC-1.  All other systems shall be placed in PC-0 and will thus have no seismic design requirements.

The DOE design criteria5 allow the PC-1 structures, systems, and components (SSC) to be designed using the simplified approaches specified in building code, such as Uniform Building Code (UBC).  The NSTX tokamak structure shall be installed in the PLT facility room.  The seismic design shall consider the response to the motion of the machine floor rather than the ground motion.  According to UBC code, static analysis approach may be used for determining the seismic effects.  For PC-1 SSCs the design forces may be based on the total lateral seismic forces Fp given by UBC provisions:

Fp = Z I Cp WP = 0.135 Wp

Where	Z	= a seismic zone factor.  For PPPL , Z=0.09g

I	= an importance factor. For PC-1, I = 1.00

Cp	= a horizontal force factor. Cp = 1.5 for non-rigid elements.

Wp	= the weight of element or component

The lateral force shall be distributed in proportion to the mass distribution of the machine.  Forces shall be applied in the horizontal directions that result in the most critical loadings for design.

b. NSTX conventional facilities shall be designed in accordance with DOE 6430.1A General Design Criteria and applicable national codes referenced therein.

4 Environmental, Safety, and Health (ES&H) Requirements

4.1 Policy Statement

a. The design, manufacture, fabrication, construction, installation, test, operation, maintenance, modification, and eventual decontamination and decommissioning of the NSTX shall be accomplished in a manner that will protect personnel, visitors, the public, property and the environment from injury.  Pursuant to this policy, the NSTX project shall:

• Comply with all applicable Federal, State, Local, and PPPL ES&H regulations;

• Assess and minimize the risks inherent in the NSTX program;

• Actively encourage ES&H awareness on the part of NSTX personnel and visitors.

In particular, NSTX shall be designed and operated in accordance with DOE Orders and PPPL Environment, Safety and Health Directives.  In the event of conflicts between DOE Orders and PPPL Environment, Safety and Health Directives, DOE Orders shall take precedence.

4.2 Radiological Design Objectives

a. All operations must be planned to incorporate the radiation safety guidelines, practices, and procedures included in PPPL EHSD 5008, Section 10.

b. Radiological design objectives and regulatory limits to be observed in the design and operation of the NSTX facility are defined in Table 4.2-1.

Table 4.2-1: Radiological Design Objectives

|  |  | P, Probability | Public Exposure |  | Occupational Exposure |  |
| --- | --- | --- | --- | --- | --- | --- |

| Condition |  | Of Occurrence In A Year | Regulatory Limit (rem per yr) | Design Objective (rem per yr) | Regulatory Limit (rem per yr) | Design Objective (rem per yr) |
| --- | --- | --- | --- | --- | --- | --- |

| Routine Operation | Normal Operations | P≥1 | 0.1 total 0.01 airborne 0.004 drinking water | 0.01 total | 5 | 1 |
| --- | --- | --- | --- | --- | --- | --- |

|  | Anticipated Events | 1>P≥10-2 | 0.5 total (including normal operation) | 0.05 per event |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| Accidents | Unlikely Events | 10-2>P≥10-4 | 2.5 | 0.5 | ref | ref13 |
| --- | --- | --- | --- | --- | --- | --- |

|  | Extremely Unlikely Events | 10-4>P≥10-6 | 25 | 5 | ref | ref15 |
| --- | --- | --- | --- | --- | --- | --- |

|  | Incredible Events | P<10-6 | NA | NA | NA | NA |
| --- | --- | --- | --- | --- | --- | --- |

5 Decontamination and Decommissioning Requirements

a. A plan for the decontamination and decommissioning (D&D) of NSTX shall be developed in accordance with DOE Orders 5820.2A Radioactive Waste Management, and 6430.1A General Design Criteria.  DOE Order 5820.2A requires that planning for facility decommissioning be initiated during the design phase.  DOE Order 6430.1A requires that features and measures be incorporated in the design to simplify decontamination and decommissioning.

b. The NSTX D&D plan shall meet the requirements of WHC-EP-0063-1 Hanford Radioactive Solid Waste Acceptance Criteria.

c. None of the waste packages projected in the D&D plan to be generated during the end of life disposal of NSTX (including a small amount of mixed waste) shall exceed Class A low level waste limits as defined in existing 10CFR61 regulations.

Flat top time corresponds to interval between SOF and EOF

Flat top time corresponds to interval between SOFTF and EOFTF

The definition of operating conditions and general design guidelines are consistent with BNL 51444 Rev. 1

U.S. Department of Energy, "Natural Phenomena Hazards Performance Categorization Criteria for Structures, Systems, and Components', DOE-STD-1021-93,  July. 1993

U.S. Department of Energy, "Natural Phenomena Hazards Design and Evaluation Criteria for Department of Energy Facilities", DOE-STD-1020-934  April. 1994

'Uniform Building Code', 1991 Edition, International Conference of Building Officials, Whittier, CA 1991

U.S. Department of Energy, "Guidelines for Use of Probabilistic Seismic Hazard Curves at Department of Energy Sites", DOE-STD-1024-92  December, 1992

All operations must be planned to incorporate the radiation safety guidelines, practices, and procedures included in PPPL EHSD 5008, Section 10.

Evaluated at the PPPL site boundary.

Dose equivalent to an individual from routine operations (rem per year unless otherwise indicated)

Compliance with this limit is to be determined by calculating the highest effective dose equivalent to any member of the public at any offsite point where there is a residence, school, business, or office.

Dose equivalent to an individual from an accidental release (rem per event)

Refer to PPPL EHSD Section 10, Chapter12 for emergency personnel exposure limits.

For design basis accidents (DBAs), i.e., postulated accidents or natural forces and resulting conditions for which the confinement structure, systems, components, and equipment must meet their functional goals, the design objective is 0.5 rem.

Refer to PPPL EHSD Section 10, Chapter 12 for emergency personnel exposure limits.