National Spherical Tokamak Experiment

NSTX

PROJECT REQUIREMENTS DOCUMENT

NSTX-RQMTS-PRD

Draft A

April 30, 1996

________________________________      	   ______________________________

Stanley Kaye	Masayuki Ono

NSTX Project Engineering Manager	NSTX Project Manager

________________________________      	   ______________________________

Martin Peng	John Schmidt

NSTX Program Manager	PPPL Advanced Projects Dept. Head

________________________________

Robert Goldston

PPPL Associate Director for Research

Record of Revisions

| Revision | Date | ECP | Description of Change |
| --- | --- | --- | --- |

Table of Contents

Preface	1

0 New Regimes of Physics

1 Plasma Performance Requirements	2

1.1 Reference Scenarios	2

1.2 Flexibility	7

1.3 Power Handling and Particle Control	10

1.3.1 Scrape-off Layer (SOL) Envelopes	10

1.3.2 Maximum Heating Power	11

1.3.3 Divertor Requirements	12

1.3.3.1 Divertor Configuration	12

1.3.3.2 Divertor Materials	13

1.3.3.3 Divertor Power Handling	13

1.3.4 Limiter Requirements	13

1.3.4.1 Limiter Configuration	13

1.3.4.2 Limiter Materials	14

1.3.4.3 Limiter Power Handling	14

1.3.5 Neutral Beam Armor Requirements	14

1.3.6 Pumping Speed and Throughput	14

1.4 Plasma Heating and Current Drive	15

1.5 Fueling	18

1.5.1 Gas Injection	18

1.5.2 Pellet Injection	18

1.6 TF Ripple	18

1.7 Field Errors	18

1.8 Plasma Startup and Equilibrium Control	19

1.8.1 Radial Plasma Position Control	19

1.8.2 Vertical Plasma Position Control	19

1.8.3 Separatrix Position Control	20

1.8.4 Plasma Startup	21

1.8.5 External Kink Mode Control	22

1.9 Vacuum and Wall Conditioning	22

1.9.1 Base Pressure	22

1.9.2 Bakeout Temperature	22

1.9.3 Glow Discharge Cleaning	23

1.9.4 Boronization	23

1.9.5 Pre-shot Temperature	23

1.10 Disruption Handling	23

1.11.1 Baseline diagnostics	25

1.12 Instrumentation, Control, and Data Acquisition	26

1.13 Material Selection	27

2 Plasma Operational Requirements	28

2.1 Pulse Length	28

2.2 Pulse Repetition Rate	28

2.3 Annual Operating Limits	29

2.4 Lifetime Operating Limits	29

2.5 Neutron Source Rates/Yields	30

PREFACE

The mission of the National Spherical Tokamak Experiment (NSTX) is to address the spherical tokamak physics principles of

•	Non-inductive start-up, current sustainment and profile control;

•	Confinement and transport;

•	Pressure limits and pressure gradient driven currents;

•	Scrape-off layers and divertors; and

•	Stability and disruption-free operation.

These principles are to be addressed in the scientifically interesting regimes relevant to the volume neutron source (VNS) and future power plants characterized by

•	High average βt (plasma pressure/applied toroidal field pressure) ~ 25 - 45 %;

•	High pressure gradient driven current fraction ~ 50 - 90%;

•	Fully relaxed, non-inductively sustained current profile;

•	Collisionless plasmas with high temperatures and densities; and

•	Low aspect ratios R/a ≥1.25 and high elongation k ≥ 2.

Top-level performance requirements for NSTX, along with cost and schedule objectives, are included in the Project Definition Statement (PDS).  Detailed physics performance requirements are summarized here in the Project Requirements Document (PRD), engineering performance requirements in the General Requirements Document (GRD), and component performance requirements for the elements of the Work Breakdown Structure (WBS) in the System Requirements Documents (SRD’s).  The order of precedence for these requirements is the PDS, PRD, GRD, SDD, followed by the subsystem/equipment design specifications.  The basis for the PRD is provided in the Physics Design Description (PDD).

NSTX shall comply with all applicable federal, state, and local regulations.  The NSTX facility shall be designed and operated in accordance with DOE Orders which are applicable to the Princeton Plasma Physics Laboratory as determined by DOE and concurred by the Laboratory Director.  The design and operation of the NSTX facility shall accommodate subsequent changes not covered by written exemptions granted by DOE.

0 NEW REGIMES OF PHYSICS

NSTX will permit experimental investigations of plasma physics in previously unexplored parameter regimes (see mission) that are scientifically attractive for developing fusion energy.  The physics design requirements for NSTX are therefore focused on the capabilities needed to access these regimes.  The key elements of these requirements are:

•	Full inductive capability to allow testing of a number of potentially attractive means of noninductive initiation (Electron Cyclotron Heating ECH − 200 kW delivered) and startup (Coaxial Helicity Injection CHI − 3 MJ at supply);

•	Dominating plasma shaping via low aspect ratio (R/a ≥ 1.25), high elongation (κ ≥ 2), triangularity (δ ∼ 0.5), and safety factor (q95 ≥ 10) at full current leading to a shaping parameter S (≡q95Ip/aBt0) up to 80 MA/T-m, which is one order of magnitude beyond the best achieved so far in tokamaks;

•	Magnitudes for a (= 64 cm), Bt0 (≥ 0.32 T), and plasma current (Ip = 1 MA) adequate for good confinement to produce collisionless plasmas of high density (∼5 x1013 cm−3) and temperature (several keV);

•	Auxiliary heating techniques and power (High-Harmonics Fast Wave HHFW − 6 MW delivered, and as upgrade Neutral Beam Injection NBI − 5 MW delivered) capable of achieving high average toroidal βt (= 25 − 45%, 3 − 4 times the best achieved so far in tokamaks) and pressure-gradient driven current fraction (= 50 − 90%);

•	Stabilizing shell to allow access to high-βt;

•	Current drive techniques and power (CHI − 15 MW at supply in addition to HHFW) capable of producing the balance of full plasma current beyond the self-driven fraction under high βt conditions;

•	Pulse lengths (≤ 5 s) for the plasma, the device, and the auxiliary systems adequate for testing fully relaxed current profiles under these conditions;

•	Flexibility in producing plasma exhaust and helicity injection configurations including double-null (DN), single-null (SN), and naturally diverted (ND) scrape-off layer (SOL); and

•	High plasma edge density (≤ 2 x 1013 cm−3) and temperature (≤ keV) to determine the physics principles of the SOL plasma for these plasma exhaust configurations.

These combined capabilities will allow experimental tests of a number of plasma operating scenarios that may prove to be attractive for the development of fusion energy.  The database produced by the planned NSTX research program, together with other planned spherical tokamak tests, is expected to provide the scientific basis for a small fusion core (R0 ∼ 1 m) long-pulse, burning DT plasma, producing reactor relevant fusion

plasma and nuclear conditions (fusion power ∼ 50 MW, fusion neutron wall loading of ∼ 1 MW/m2).

This PDR defines the physics requirements stemming from the above which form the basis for the engineering design of NSTX provided in the General Design Requirements (GDR).  The basis for the PDR is provided in the NSTX Physics Design Description (PDD).

1 PLASMA PERFORMANCE REQUIREMENTS

The tokamak, auxillary systems, and facilities shall be designed to satisfy the requirements attendant with the reference scenarios (Section 1.1).  The tokamak shall also be designed to provide flexibility in βp-li and plasma configuration (Section 1.2).  Such flexibility may require modifications to ancillary systems and facilities.

1.1 Reference Scenarios

The tokamak, auxillary systems, and facilities shall be shall be designed to satisfy the current drive requirements attendant with the following scenarios:

• an ECH +  Ohmic scenario (ECH + OH)

• a coaxial helicity injection + Ohmic  (CHI + OH) scenario

• a coaxial helicity + fast wave current drive (CHI+FW) scenario

In order to promote reliable plasma initiation and operational flexibility, the ECH + OH, CHI + OH, and CHI + FW scenarios shall be initiated in an identical fashion, with a loop voltage of at least 6V.  For non-inductive start-up with co-axial helicity injection (CHI), a SN scenario will be used.  In all three scenarios, a portion of the full 1 MA plasma current is expected to be driven non-inductively by varying amounts of bootstrap current.

1.1.1 ECH + OH Scenario

The ECH+ OH scenario is characterized by inductively ramping the initial ECH preionized plasma current to a full current of 1 MA, holding the current flat, and then ramping the plasma current down.  Auxiliary heating will be applied during the ramp-up phase and current flattop phase to increase the plasma stored energy and temperature, and to attain high-βt and maintain this state during the current flattop.  βt will be ramped down with the plasma current.  Approximately 1.6 volt-seconds are supplied between the start and end of the plasma pulse..

1.1.2 CHI + OH Scenario

The CHI + OH scenario is characterized by establishing a 0.5 MA target plasma using coaxial helicity injection, and then ramping the current to 1 MA using a one-half OH swing.   Auxiliary heating will be applied during the 0.5 to 1.0 MA ramp-up phase and during the current flattop phase to increase the plasma temperature, stored energy, and to maximize βt, maintaining high-βt during the current flattop period.

1.1.3 CHI + FW Scenario

The CHI + FW scenario is characterized by fully non-inductive operation. A target plasma of 0.5 MA will be created using CHI, and then the plasma current will be ramped up and maintained at the 1.0 MA level using a combination of FW, CHI, and bootstrap currents.  Auxiliary heating will be applied during the ramp-up phase and during the current flattop phase to increase the plasma temperature, stored energy, and to maximize βt (maximizing bootstrap current), and maintaining high-βt during the current flattop phase.

1.2 Flexibility

1.2.1 Aspect Ratio/Elongation

The tokamak, PF systems, and ancillary systems and facilities shall be capable of producing plasmas characterized by the limiting scenarios where:

• R/a≈1.25, κx≤2.2, δx≤0.5, and

• R/a≈1.40, κx≤3.0, δx≤0.5

in the ND, DN, or SN configuration.

1.2.1.1 R/a≈1.25, κx≤2.2, δx≤0.5

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subjecto to the following constraints for the ND and DN configurations:

• R0+a=1.533 m

• R0-a=0.174 m (assuming a zero scrapeoff layer thickness on the inboard side)

• Zx=+/-1.50 m (vertical height of the X-points)

• the 2 cm outboard flux surface passes between the plasma and the passive 	stabilizer plates

• the 2 cm outboard flux surface strikes the outer divertor target plate

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subjecto to the following constraints for the SN configuration:

• R0+a=1.533 m

• Zx=-1.50 m (vertical height of the lower, active X-point)

• the vacuum flux surface outside the plasma, Ψ∗, can vary from 0 to 20% of the 	normalized separatrix flux surface value (see below)

• Ψ∗ completely encloses the plasma without intersecting a material surface except 	at the footprints

• the Ψ∗ footprints intersect the outboard and inboard divertor plates respectively

1.2.1.2 R/a≈1.40, κx≤3.0, δx≤0.5

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subjecto to the following constraints for the ND configuration:

• R0+a=1.20 m

• R0-a=0.174 m (assuming a zero scrapeoff layer thickness on the inboard side)

• the 2 cm outboard flux surface passes between the plasma and the passive 	stabilizer plates

• the 2 cm outboard flux surface strikes the outer divertor target plate

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subjecto to the following constraints for the DN configuration:

• R0+a=1.20 m

• R0-a=0.20 m (assuming a 2 cm midplane scrapeoff layer thickness on the inboard 	side)

• Zx=+/-1.50 m (vertical height of the X-points)

• the 2 cm outboard flux surface passes between the plasma and the passive 	stabilizer plates

• the 2 cm outboard flux surface strikes the outer divertor target plate

• the 2 cm inboard flux surface strikes the inner divertor target plate

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subjecto to the following constraints for the SN configuration:

• R0+a=1.20 m

• Zx=-1.50 m (vertical height of the lower, active X-point)

• the vacuum flux surface outside the plasma, Ψ∗, can vary from 0 to 20% of the 	normalized separatrix flux surface value

• Ψ∗ completely encloses the plasma without intersecting a material surface except 	at the footprints

• the Ψ∗ footprints intersect the outboard and inboard divertor plates respectively

1.2.2 Toroidal Field/Plasma Current Directionality

The tokamak shall be initially configured to drive plasma current in the positive direction with a negative toroidal field.

The figure below illustrates the right-handed coordinate system used on NSTX.  A positive plasma current is in the f-direction (counter-clockwise direction) and provides a positive vertical field in the Z-direction across the centerline of the vessel.  A positive TF current (in the TF inner leg) is in the Z-direction and provides a positive toroidal field in the f-direction (counter-clockwise direction).  Positive radial fields and currents are in the R-direction, radially outward from the Z-axis, the major axis of the tokamak.

Plasma facing components and rf launchers shall be designed for two quadrant operation, i.e.

• the toroidal field shall be bi-directional and

• the plasma current shall always be in the opposite direction as the toroidal field.

All other equipment shall be designed for four quadrant operation, i.e.

• the toroidal field shall be bi-directional and

• plasma current in the same direction and in the opposite direction as the toroidal field can be accommodated.

1.3 Power Handling and Particle Control

1.3.1 Scrape-off Layer (SOL) Envelopes

The +2 cm (outer) flux surface shall intercept only the outboard divertor targets for the range of equilibria defined in Section 1.2.1.1 and 1.2.1.2.  All internal hardware must be located accordingly.

The -2 cm (inner) flux surface shall intercept only the inboard divertor targets for the range of equilibria defined in Section 1.2.1.2.  All internal hardware must be located accordingly.

1.3.2 Maximum Heating Power

Plasma facing components shall be designed to accommodate heat loads associated with a maximum heating power of 12 MW for 5 sec.

1.3.3 Divertor Requirements

1.3.3.1 Divertor Configuration

The divertor configuration shall accommodate the heat loads resulting from the heating power and pulse duration given in Sec. 1.3.2. Flexibility to reconfigure the divertor assemblies shall be provided.

1.3.3.2 Divertor Materials

The plasma-facing surfaces of the divertor assembly shall be made of carbon-based materials.

The minimum as-manufactured thickness of the graphite armor shall be 1.0cm between the plasma facing surface and the cooling tube interface.

The maximum divertor surface temperature shall not exceed 1400°C.

The NSTX facility shall be capable of being modified to accommodate alternate materials for plasma-facing surfaces including beryllium, tungsten, and molybdenum.

1.3.3.3 Divertor Power Handling

The inboard and outboard divertor targets shall be designed for handling peak heat fluxes  of 6 MW/m2 for pulse lengths up to 5 sec.

1.3.4 Limiter Requirements

1.3.4.1 Limiter Configuration

Toroidal and poloidal limiters shall be provided outboard of the plasma for startup and to protect the outboard passive structure and equipment in the port region from particle and heat loads during operation and disruptions.

Two outboard toroidal limiters shall be provided.  They shall remain outside the +2cm SOL envelope as prescribed in Section 1.3.1.

The outboard poloidal limiter horizon shall conform to the geometry of the Faraday Shield on the ICH/FWCD launcher.

Protective armor shall be provided to protect the inboard structures from particle and heat loads during operation and disruptions. The inner wall will act as the inner limiter.

Protective armor shall be provided to protect the outboard passive structure and components between the passive structure and vacuum vessel wall from energetic particle losses due to field ripple.

1.3.4.2 Limiter Materials

The plasma-facing surfaces of the limiter assemblies shall be made of carbon-based materials.

Maximum limiter surface temperatures shall not exceed 1200°C.  For toroidally continuous limiters, the maximum toroidally averaged surface temperature shall not exceed 1000°C.

The NSTX facility shall be capable of being modified to accommodate alternate materials for plasma-facing surfaces including beryllium, tungsten, and molybdenum.

1.3.4.3 Limiter Power Handling

The inboard protective armor shall be designed to accommodate local heat loads due to NB shine-through, radiation, and plasma startup.

The outboard toroidal limiters shall be designed to accommodate local heat loads due to ripple-trapped particles, radiation, and plasma startup.

1.3.6 Pumping Speed and Throughput

The NSTX facility shall be designed for maximum pumping rates of

• 50 torr-l/s when operated at the pulse repetition rates specified in Section 2.2 and

• 200 torr-l/s when operated at lower pulse repetition rates.

The NSTX facility shall be capable of being upgraded for maximum pumping rates of

• 100 torr-l/s when operated at the pulse repetition rates specified in Section 2.2 and

• 250 torr-l/s when operated at lower pulse repetition rates.

The NSTX facility shall be capable of being upgraded for a maximum pumping rate of 100 torr-l/s in steady state operation.

The NSTX facility shall provide a minimum pumping speed of 65,000 l/sec.

The NSTX facility shall be capable of being upgraded to provide a minimum pumping speed of 85,000 l/sec.

A variable aperture on the vacuum pumping ducts shall be provided to control the pumping speed.

1.4 Plasma Heating and Current Drive

The NSTX facility shall include a heating and current drive system based on multiple technologies (ECH, CHI, and FWH/FWCD), each subsystem expandable to greater power levels.  The NSTX facility shall also provide flexibility in the control of current density profiles.

1.4.1 ECH

1.4.2. CHI

Co-axial Helicity Injection is required to provide non-inductive discharge start-up to current levels of approximately 0.5 MA, and to provide non-inductive currend drive at levels of approximately 0.2 MA in the outer portion of the plasma during the current sustainment phase.

The device and facility shall be configured initially with 3 MJ of energy and 15 MW of power of CHI to the plasma.

1.4.3 Fast Wave Heating/Fast Wave Current Drive (FWH/FWCD)

FWH/FWCD is required to provide electron heating and current drive.

The device and facility shall initially be configured to provide 6 MW of FWH/FWCD power to the plasma.

The FWH/FWCD power shall initially be coupled to the plasma through one 6-strap launcher.

The FWH/FWCD antenna shall be moveable and capable of coupling power to the plasma in both the R/a=1.25 and R/a=1.40 scenarios.

A Faraday shield shall be provided between the launcher and plasma.  At operating temperature, the front surface of the Faraday shield shall conform (±2mm) to the surface of a circular toroid located on the midplane with major radius 1.832 m and minor radius 0.938 m within 0.4m of the midplane.  This surface bounds the family of +2cm flux surfaces over the required range of flexibility in β-li space for both the R/a=1.25 and R/a=1.40 scenarios.

The capability to move the FWH/FWCD launcher out 0.1m from the innermost position specified above shall be provided.

1.5 Fueling

1.5.1 Gas Injection

The device shall be provided with a programmable gas injection system capable of supplying H, D, and other gases to the outboard midplane and various locations in the divertor regions with a maximum throughput of 250 Torr-l/s.

1.5.2 Pellet Injection

The device and facility shall be capable of accommodating a pellet injection system capable of injecting H or D pellets with a maximum steady state fueling rate of 85 Torr-l/s.

1.6 TF Ripple

The peak-to-average TF ripple shall be ≤0.5% over the entire plasma cross section.

1.7 Field Errors

1.8 Plasma Startup and Equilibrium Control

1.8.1 Radial Plasma Position Control

The device shall include a feedback equilibrium control system which will provide radial plasma position control such that:

• the radial location of the separatrix at the outboard midplane can be maintained within 1 cm (rms displacement) of the nominal location, and

• the radial location of the separatrix at the outboard midplane can be moved by ±2 cm from nominal in 50 msec, in response to needs for RF coupling

for the reference scenarios defined in Section 1.1.

PF coils shall be designed to satisfy radial position control requirements for the full range of equilibrium conditions identified in Section 1.2.

Power systems shall be designed to satisfy radial position control requirements attendant with the reference scenarios per Section 1.1.  Power systems shall be capable of of being ugraded to satisfy radial position control requirements for the full range of equilibrium conditions identified in Section 1.2.

1.8.2 Vertical Plasma Position Control

A conducting structure shall be provided which makes the growth rate of the vertical instability, less than 0.2ts, where ts is the L/R time constant of the lowest order, up-down anti-symmetric mode in the conducting structure for all the equilibrium conditions required per Section 1.2.

The device shall include a feedback equilibrium control system which will provide vertical position control such that:

• the plasma can be vertically positioned to any location within ±3cm of the midplane with a response time of 0.5s using one pair of external PF coils and

• the plasma vertical position can be controlled within 1cm (rms displacement) of the nominal location on the time scale of the vertical instability growth time.

The PF coils shall satisfy the vertical position control requirements for the full range of equilibrium conditions identified in Section 1.1 and 1.2.

Power systems shall satisfy vertical position control requirements attendant with the reference scenarios per Section 1.1 and 1.2.

1.8.3 Separatrix Position Control

The device shall include a feedback equilibrium control system which will provide outboard strike point position control such that:

• the location of the outboard  strike point can be moved ±5cm along the divertor surface with a response time of 0.5s and

• the location of the outboard strike point can be maintained within ±1 cm of nominal

for the reference scenarios defined in Section 1.1 and 1.2.

PF coils shall satisfy the outboard strike point control requirements for the full range of equilibrium conditions identified in Section 1.1 and 1.2.

Power systems shall satisfy the outboard strike point control control requirements attendant with the reference scenarios per Section 1.1 and 1.2.

1.8.4 Plasma Startup

The conducting structure shall include resistive breaks to provide sufficient toroidal resistance for penetration of the startup voltage.

The device and facility must be capable of inductively initiating the reference discharge scenarios with a common, preprogrammed startup sequence meeting the following specifications:

• The applied loop voltage shall rise to at least 6 V in the absence of plasma.

• Starting from the time of breakdown, the plasma current shall increase to 500kA within 0.1s under the following assumptions:

- The plasma’s resistive voltage rises instantaneously at the time of breakdown to the breakdown voltage and then decays toward an asymptotic level greater than or equal to 1V.

- The plasma inductance is that of a circular torus centered at the breakdown null point and extending to the nearest limiter.

• Feedback control of the plasma current and radial equilibrium shall be established when the plasma current reaches 100 kA.

1.8.5 External Kink Mode Control

Conducting structure shall be provided for the purpose of passively stabilizing the ideal external kink mode for all the equilibrium conditions required per Section 1.2.

The conducting structure shall be compatible with the initial complements of heating systems per Section 1.4.

Flexibility to reconfigure the structure by removing and re-installing elements of it, including the ability to make it compatible with the upgrade complement of heating systems per Section 1.4, shall be provided.

1.9 Vacuum and Wall Conditioning

1.9.1 Base Pressure

The device and facility shall produce high vacuum conditions with a base partial pressure of ≤1x10-7 torr for fuel gases (Z ≤ 2) and a base partial pressure of ≤1x10-9 torr for impurity gases (Z > 2) at 65°C.

The impurity gas load during pumpdown to base pressure shall not exceed 1x10-5 torr-l/s.

1.9.2 Bakeout Temperature

The vacuum vessel shell and all components internal to the vacuum vessel shall be bakeable at 350°C±25°C.

Vacuum pumping ducts and large horizontal ports shall have nominal bakeout temperatures of 350°C at the interface to the vacuum vessel and 150°C at the interface to the torus vacuum isolation valve.

1.9.3 Glow Discharge Cleaning

The device shall provide a glow discharge cleaning (GDC) capability with DC glow in 10-20 mtorr of H, D, or He at 400 V and 20 A for indefinitely long periods with the vacuum vessel at 350°C and a throughput 44 torr-l/s (He).

The device shall provide the capability to perform GDC between shots with the vacuum vessel at 150°C.

1.9.4 Boronization

The device shall provide the capability for boronization for all surfaces with line-of-sight to the plasma by a glow discharge technique with the interior surfaces at 350°C.

1.9.5 Pre-shot Temperature

In-vessel components shall be maintained at temperatures not less than 50°C between discharges.  The vacuum vessel and internal components shall be maintained at the same temperatures during the interval between discharges.

1.10 Disruption Handling

The device shall be designed to accommodate major disruptions characterized by:

• possible vertical displacement

• a thermal quench occurring in 0.1-1 ms

• a current decay with a time-varying decay rate, such that the peak rate is ≤0.5 MA/ms• a peak poloidal halo current up to 400 kA, 40% of the maximum plasma current prior to the disruption, with a toroidal peaking factor of 2:1; that is, the toroidal dependence of the halo current is [1 + 1.0*cos (f - f0)], for all toroidal phase angles f0  from 0 to 2*π.

• a frequency of occurrence of 1% in any phase or mode of operation.

1.11 Diagnostics

The NSTX machine shall incorporate a system of diagnostics to provide the measurements of the plasma behavior that are necessary to fulfill the physics mission of the NSTX project.  In addition, certain measurements shall be provided for plasma control and operational purposes.  The principal categories of the measurements are as follows:

•  Plasma Control  An array of magnetic diagnostics shall be provided to measure the plasma and vacuum vessel currents, the radial and vertical plasma position, the outboard divertor strike point, and the plasma shape with time resolution and accuracy sufficient to satisfy the associated control and operational requirements in Section 1.8.  Profiles of the plasma current density and the plasma pressure shall be provided to support the NSTX advanced tokamak mission through active control of the plasma profiles.  Magnetic measurements of MHD and locked mode activity shall be provided in support of disruption handling requirements in Section 1.10.  The line-averaged density (<n> ≤ 1.0x1020 m-3) shall be measured to provide feedback control of the plasma density.

•  Confinement  Core profiles of the electron density (neo ≤ 2x1020 m-3), electron temperature (Teo ≤ 6 keV), ion temperature (Tio ≤ 6 keV), average impurity concentration (Zeff ~ 1-5), and the radiated power shall be measured.  Measurements of the plasma poloidal beta (0.01<βp<5.0) and the low-Z and high-Z impurity concentrations shall be provided.  Internal and external fluctuations shall be measured over a range of frequencies adequate to encompass MHD activity and plasma instabilities.

•  Divertor/Edge Plasma  The divertor and limiter surface temperatures shall be measured from ~ 300 oC to 3000 oC with maximum coverage consistent with viewing access to provide operational information needed for wall conditioning, impurity control, and protection of plasma facing components.  Measurements of the electron density, ion/electron temperatures, neutral pressure, and the Hα and impurity line radiation in the SOL and divertor plasma region shall be provided.

•  Fusion Yield  The total neutron source rate and the radial profile of the neutron source emission shall be provided, as well as measurements of the charged fusion products.

All diagnostics shall be implemented consistent with achievable viewing access and state-of-the-art measurement techniques.

Diagnostics shall be designed for D operation only.

1.11.1 Baseline diagnostics

The device and facility shall include the following diagnostics:

| Machine Diagnostics | Physics Diagnostics | Divertor/Edge Diagnostics |
| --- | --- | --- |

| Magnetic Loops Hard X-ray Detectors Millimeter-wave Interferometer UV Survey Spectrometer Plasma/Infrared TV Visible Survey Spectrometer Residual Gas Analyzer Torus Ion Gauges Glow Discharge Probes Inspection Illumination | Visible Filterscopes Visible Bremsstrahlung Charge Exchange Spectroscopy Thomson Scattering MotionalStarkEffect (MSE) Soft X-ray Arrays Epithermal Neutron Detectors ECE Heterodyne Radiometer Bolometer Arrays Fixed Edge Probes | Divertor IR TV Visible Ha TV Foil Bolometers Divertor Bolometer Arrays Multichord Visible Spectrometer Fast Neutral Pressure Gauges Ha Monitors Thermocouples |
| --- | --- | --- |

1.11.2 Upgrade diagnostics

The device and facility shall be capable of accommodating diagnostic upgrades as follows:

| Machine Diagnostics | Physics/Fluctuation Diagnostics | Divertor/Edge Diagnostics |
| --- | --- | --- |

| Steady State Magnetics | Infrared Polarimetry Impurity Pellet Injector IR/FIR Multichannel Interferometer ECE Michelson Interferometer ECE Grating Polychromator Millimeter-waveReflectometer Collective Wave Scattering Beam Emission Spectroscopy (BES) X-ray Crystal Spectrometer Soft X-ray Spectrometer X-ray Pinhole Camera X-ray Pulse Height Analysis Multichanel Neutron Collimator Neutron Flucuation Detectors Charged Fusion Products Charge Exchange Neutral Analyzer Diagnostic Neutral Beam | Moveable Edge Probes Divertor Thomson Scattering |
| --- | --- | --- |

1.12 Instrumentation, Control, and Data Acquisition

The NSTX facility shall have an instrumentation, control, and data acquisition (I&C) system that allows for coordination of operations and the acquisition, archiving, and display of all pertinent information.

The I&C system will be designed for operation of NSTX, and it will provide supervision, remote control, monitoring, data acquisition, and data handling for subsystems.

The national perspective of NSTX requires that researchers located at the NSTX site and at other laboratories and universities have access to the facility in support of the remote operation, physics planning, analysis, and coordination of experimental objectives.

The computations architecture must support a distributed computing environment spanning a minimum of four geographically separate sites inclusive of Princeton Plasma Physics Laboratory, the local site.  This architecture will incorporate the distributed computational facilities as important elements in the analysis, storage, and display of control and diagnostic information.  The selected sites will be connected via a high performance, wide area computer network, with sufficient bandwidth to allow transmission

of information including file transfers, graphical displays, and video information with minimal latency consistent with available technology.

1.13 Material Selection

The materials to be used in the tokamak and peripheral equipment shall be limited based on vacuum, impurity control, magnetic permeability, and activation considerations.

All materials to be used in the tokamak and peripheral equipment must have a relative magnetic permeability less than 1.02 unless otherwise authorized by the Project.

All materials to be used within the plasma vacuum boundary must be approved by the Project.

2 OPERATIONAL REQUIREMENTS

Operational limits for D operation shall be established such that the environmental, safety, and health effects determined for PBX-M, as presented in the Environmental Assessment for PBX-M, shall not be exceeded during normal operation of NSTX.

2.1 Pulse Length

The NSTX facility shall initially be configured to provide 5 s pulses with H and D plasmas for the reference scenarios per Section 1.1 with initial neutron source rates per Section 2.5 and pulse repetition rates per Section 2.2.

The tokamak shall be designed for and the facility be capable of being upgraded for 60 sec operation with H and D plasmas for the full range of flexibility specified in Section 1.2 with the ultimate neutron source rates specified in Section 2.5.

2.2 Pulse Repetition Rate

In its initial configuration, the NSTX facility shall be capable of providing 96 - 5 sec. pulses in a 24 hour period when operated per the reference scenarios.

2.3 Annual Operating Limits

The design shall allow for up to

• 13,000 pulses

• 6.5x104s of operation

2.4 Lifetime Operating Limits

All NSTX systems, with the exception of the divertor modules, shall be designed for the number of cycles and operating time attendant with a 10 year operating life.  New building and utilities shall be designed for a 25 year operating life from the start of plasma operations.

The design shall allow for up to

• 130,000 pulses

• 6.5x105s of D operation

when operated per the reference scenarios based on the factors for fatigue life specified in the NSTX Structural Design Criteria Document.

The design shall allow for up to 6,000 charge-discharge cycles of the toroidal field coils (which includes cycling required for GDC).

2.5 Neutron Source Rates/Yields

The NSTX facility shall be designed to accommodate heat loads attendant with the peak neutron source rate initially envisioned for D operation at the pulse repetition rates specified in Section 2.2.

Tokamak systems shall be designed for and the facility be capable of being upgraded for the heat loads attendant with the peak neutron source rate ultimately envisioned for D operation in steady state operation.

|  | Units | DD Neutrons | DT Neutrons |
| --- | --- | --- | --- |

| D Operation |  |  |  |
| --- | --- | --- | --- |

| Peak neutron source rate |  |  |  |
| --- | --- | --- | --- |

| Initial capability | s-1 | 5.0x1016 | 1.0x1015 |
| --- | --- | --- | --- |

| Ultimate capability | s-1 | 7.5x1016 | 2.25x1015 |
| --- | --- | --- | --- |

| Average annual neutron source rate |  |  |  |
| --- | --- | --- | --- |

| Year 1 | s-1 | 0.75x1016 | 0.75x1014 |
| --- | --- | --- | --- |

| Year 2 | s-1 | 1.50x1016 | 2.25x1014 |
| --- | --- | --- | --- |

| Year 3 | s-1 | 2.25x1016 | 4.50x1014 |
| --- | --- | --- | --- |

| Years 4-10 | s-1 | 3.00x1016 | 6.00x1014 |
| --- | --- | --- | --- |

| Maximum annual neutron yield |  |  |  |
| --- | --- | --- | --- |

| Year 1 |  | 0.5x1020 | 0.5x1018 |
| --- | --- | --- | --- |

| Year 2 |  | 2.5x1020 | 3.75x1018 |
| --- | --- | --- | --- |

| Year 3 |  | 1.5x1021 | 3.0x1019 |
| --- | --- | --- | --- |

| Year 4 |  | 3.0x1021 | 6.0x1019 |
| --- | --- | --- | --- |

| Years 5-10 |  | 6.0x1021 | 1.2x1020 |
| --- | --- | --- | --- |

| Maximum total neutron yield |  | 4x1022 | 8x1020 |
| --- | --- | --- | --- |

| DT Operation |  |  |  |
| --- | --- | --- | --- |

| Peak neutron source rate | s-1 | 6.4x1016 | 5.3x1018 |
| --- | --- | --- | --- |

The NSTX facility includes the tokamak, ancillary systems, and facilities.

Per December 11, 1992 memo from Davidson to Johnson

(Ψ∗-Ψs)/(Ψs−Ψmag) = 0.0 to 0.2, where Ψ∗ is the vacuum flux surface, Ψs is the separatrix flux surface value, and Ψmag is the flux value at the magnetic axis.

Unless otherwise qualified, the pulse length refers to the flattop time. i.e. the period of constant plasma current.

Peak neutron source rate initially envisioned for D operation

Peak neutron source rate ultimately envisioned for D operation