National Spherical Tokamak Experiment

NSTX

PROJECT REQUIREMENTS DOCUMENT

NSTX-RQMTS-PRD-025

Draft B

June 5, 1996

________________________________      	   ______________________________

Stanley Kaye	Masayuki Ono

NSTX Project Physics Manager	NSTX Project Manager

________________________________      	   ______________________________

Martin Peng	John Schmidt

NSTX Program Manager	PPPL Advanced Projects Dept. Head

________________________________

Robert Goldston

PPPL Associate Director for Research

Record of Revisions

| Revision | Date | ECP | Description of Change |
| --- | --- | --- | --- |

| 0 | 4/30/96 |  | Original |
| --- | --- | --- | --- |

| 1 | 5/20/96 |  | Peng, Reierson, Ono, Neumeyer comments |
| --- | --- | --- | --- |

Table of Contents

Preface	1

0 New Regimes of Physics

1 Plasma Performance Requirements	2

1.1 Reference Scenarios	2

1.2 Flexibility	7

1.3 Power Handling and Particle Control	10

1.3.1 Scrape-off Layer (SOL) Envelopes	10

1.3.2 Maximum Heating Power	11

1.3.3 Divertor Requirements	12

1.3.3.1 Divertor Configuration	12

1.3.3.2 Divertor Materials	13

1.3.3.3 Divertor Power Handling	13

1.3.4 Limiter Requirements	13

1.3.4.1 Limiter Configuration	13

1.3.4.2 Limiter Materials	14

1.3.4.3 Limiter Power Handling	14

1.3.5 Neutral Beam Armor Requirements	14

1.3.6 Pumping Speed and Throughput	14

1.4 Plasma Heating and Current Drive	15

1.5 Fueling	18

1.5.1 Gas Injection	18

1.5.2 Pellet Injection	18

1.6 TF Ripple	18

1.7 Field Errors	18

1.8 Plasma Startup and Equilibrium Control	19

1.8.1 Radial Plasma Position Control	19

1.8.2 Vertical Plasma Position Control	19

1.8.3 Separatrix Position Control	20

1.8.4 Plasma Startup	21

1.8.5 External Kink Mode Control	22

1.9 Vacuum and Wall Conditioning	22

1.9.1 Base Pressure	22

1.9.2 Bakeout Temperature	22

1.9.3 Glow Discharge Cleaning	23

1.9.4 Boronization	23

1.9.5 Pre-shot Temperature	23

1.10 Disruption Handling	23

1.11.1 Baseline diagnostics	25

1.12 Instrumentation, Control, and Data Acquisition	26

1.13 Material Selection	27

2 Plasma Operational Requirements	28

2.1 Pulse Length	28

2.2 Pulse Repetition Rate	28

2.3 Annual Operating Limits	29

2.4 Lifetime Operating Limits	29

2.5 Neutron Source Rates/Yields	30

PREFACE

The mission of the National Spherical Tokamak Experiment (NSTX) is to address the spherical tokamak physics principles of

•	Non-inductive start-up, current sustainment and profile control;

•	Confinement and transport;

•	Pressure limits and pressure gradient driven currents;

•	Scrape-off layers and divertors; and

•	Stability and disruption-free operation.

These principles are to be addressed in the scientifically interesting regimes relevant to the volume neutron source (VNS) and future power plants characterized by

•	High average βt (plasma pressure/applied toroidal field pressure) ~ 25 - 45 %;

•	High pressure gradient driven current fraction ~ 50 - 90%;

•	Fully relaxed, non-inductively sustained current profile;

•	Collisionless plasmas with high temperatures and densities; and

•	Low aspect ratios, R/a ≥1.25, and high elongation, κ ≥ 2.

Top-level performance requirements for NSTX, along with cost and schedule objectives, are included in the Project Definition Statement (PDS).  Detailed physics performance requirements are summarized here in the Project Requirements Document (PRD), engineering performance requirements in the General Requirements Document (GRD), and component performance requirements for the elements of the Work Breakdown Structure (WBS) in the System Requirements Documents (SRD’s).  The order of precedence for these requirements is the PDS, PRD, GRD, SDD, followed by the subsystem/equipment design specifications.  The basis for the PRD is provided in the Physics Design Description (PDD).

NSTX shall comply with all applicable federal, state, and local regulations.  The NSTX facility shall be designed and operated in accordance with DOE Orders which are applicable to the Princeton Plasma Physics Laboratory as determined by DOE and concurred by the Laboratory Director.  The design and operation of the NSTX facility shall accommodate subsequent changes not covered by written exemptions granted by DOE.

0 NEW REGIMES OF PHYSICS

NSTX will permit experimental investigations of plasma physics in previously unexplored parameter regimes (see mission) that are scientifically attractive for developing fusion energy.  The physics design requirements for NSTX are therefore focused on the capabilities needed to access these regimes.  The key elements of these requirements are:

•	Full inductive capability to allow testing of a number of potentially attractive means of noninductive initiation (Electron Cyclotron Heating ECH − 200 kW delivered) and startup (Coaxial Helicity Injection CHI − 3 MJ at supply);

•	Very strong plasma shaping at low aspect ratio (R/a ≥ 1.25), high elongation (κ ≥ 2), triangularity (δ ∼ 0.5), and safety factor (q95 ≥ 10) at full current leading to a shaping parameter S (≡q95Ip/aBt0) up to 80 MA/T-m, which is one order of magnitude beyond the best achieved so far in tokamaks;

•	Magnitudes for a (= 64 cm), Bt0 (≥ 0.32 T), and plasma current (Ip = 1 MA) adequate for good confinement to produce collisionless plasmas at high densities (∼5 x1013 cm−3) and temperatures (several keV);

•	Auxiliary heating techniques with sufficient power (High-Harmonics Fast Wave HHFW − 6 MW delivered, and as upgrade Neutral Beam Injection NBI − 5 MW delivered) to achieve high average toroidal βt (= 25 − 45%, 3 − 4 times the best achieved so far in tokamaks) and pressure-gradient driven current fraction (= 50 − 90%);

•	Stabilizing shell to allow access to high-βt;

•	Current drive techniques with sufficient power (CHI − 15 MW at supply in addition to HHFW) to produce the balance of full plasma current beyond the self-driven fraction under high βt conditions;

•	Pulse lengths (≤ 5 s) for the plasma discharge, the device, and the auxiliary systems adequate for testing fully relaxed current profiles under these conditions;

•	Flexibility in producing plasma exhaust and helicity injection configurations including double-null (DN), single-null (SN), and naturally diverted (ND) scrape-off layer (SOL); and

•	High plasma edge density (∼2 x 1013 cm−3) and temperature (∼keV) to determine the physics principles of the SOL plasma for these plasma exhaust configurations.

These capabilities combined will allow experimental tests of a number of plasma operating scenarios that may prove to be attractive for the development of fusion energy.  The database produced by the planned NSTX research program, together with other planned spherical tokamak tests, is expected to provide the scientific basis for a small, (R0

∼ 1 m) D-T fueled, long-pulse, fusion core that produces reactor relevant fusion plasma and nuclear conditions (fusion power ∼ 50 MW, fusion neutron wall loading of ∼ 1 MW/m2).

This PDR defines the physics requirements stemming from the above, and it provides the basis for the engineering design of NSTX described in the General Design Requirements (GDR).  The basis for the PDR is provided in the NSTX Physics Design Description (PDD).

1 PLASMA PERFORMANCE REQUIREMENTS

The tokamak, auxiliary systems, and facilities shall be designed to satisfy the requirements attendant with the reference scenarios (Section 1.1).  The tokamak shall also be designed to provide flexibility in βp-li and plasma configuration (Section 1.2).  Modifications to ancillary systems and facilities, which may be required by such flexibility, will be defined.

1.1 Reference Scenarios - Plasma Initiation

The tokamak, auxiliary systems, and facilities shall be designed to satisfy the current drive requirements attendant with the following plasma initiation scenarios:

• an ECH + Ohmic (ECH + OH) scenario

• an ECH + coaxial helicity injection + Ohmic (ECH + CHI + OH) scenario

• an ECH + coaxial helicity + fast wave current drive (ECH + CHI + FW) scenario

In order to promote reliable plasma initiation and operational flexibility, ECH is applied in all three scenarios to provide initial plasma break down and to facilitate sufficient current rampup rate. For the first two scenarios, which utilize OH, the OH loop voltage shall be at least 6V.  For the scenarios that utilize co-axial helicity injection (CHI), a SN configuration will be used during the portion of the discharge in which CHI is applied.  In all three scenarios, a portion of the full 1 MA plasma current is expected to be driven non-inductively by varying amounts of bootstrap current.

1.1.1 ECH + OH Scenario

The ECH+ OH scenario is characterized by inductively ramping the initial ECH preionized plasma current to a full current of 1 MA in approximately 0.2 sec, holding the current flat for approximately 0.5 sec, and then ramping the plasma current down in approximately 0.2 sec.  Auxiliary heating will be applied during the ramp-up phase and current flattop phase to increase the plasma stored energy and temperature, and to attain high-βt and maintain this state during the current flattop.  βt will be ramped down with the plasma

current.  Approximately 1.6 volt-seconds are supplied between the start and end of the plasma pulse.  The high harmonic heating and current drive technique will be developed in this scenario. Plasmas will be in either the Natural Divertor or Double Null Divertor configuration.

1.1.2 ECH + CHI + OH Scenario

The ECH + CHI + OH scenario is characterized by establishing a 0.5 MA target plasma using coaxial helicity injection and ECH preionization, and then ramping the current to 1 MA using a one-half OH swing.  Auxiliary heating will be applied during the 0.5 to 1.0 MA ramp-up phase and during the current flattop phase to increase the plasma temperature, stored energy, and to maximize βt, maintaining high-βt during the current flattop period. The plasma will be in a Single Null Divertor Configuration during the time the CHI is applied, and in either a Natural Divertor or Double Null Divertor configuration thereafter.

1.1.3 ECH + CHI + FW Scenario

The ECH + CHI + FW scenario is characterized by fully non-inductive operation.  A target plasma of 0.5 MA will be created using CHI and ECH preionization, and then the plasma current will be ramped up and maintained at the 1.0 MA level using a combination of FW, CHI, and bootstrap currents.  Auxiliary heating will be applied during the ramp-up phase and during the current flattop phase to increase the plasma temperature, stored energy, and to maximize βt (maximizing bootstrap current), and maintaining high-βt during the current flattop phase. The plasma will be in a Single Null Divertor configuration during the time the CHI is applied. If the 1 MA plasma current can be maintained soley by FW current drive and bootstrap current, the configuration will be relaxed into one of Natural Divertor or Double Null Divertor.

1.2 Flexibility

1.2.1 Aspect Ratio/Elongation

The tokamak, PF systems, and ancillary systems and facilities shall be capable of producing plasmas characterized by the limiting scenario where:

• R/a≈1.25, κx≤2.2, δx≤0.5, for li from 0.2 to 1.0

in the Natural Divertor (ND), Double-Null (DN), or Single-Null (SN) configuration. The Natural Divertor configuration is formed by applying only equilibrium fields to ensure radial stability. In the ND case, the κx and δx are defined by the last closed flux surface.

Equilibrium conditions shall be sustainable for pulse lengths (TF flattop duration) up to 5 sec. subject to the following constraints for the ND and DN configurations:

• R0 + a = 1.533 m

• R0 - a = 0.175 to 0.195 m (assuming a zero to 2 cm scrapeoff layer thickness on 	the inboard side)

• Zx=+/-1.50 m (vertical height of the X-points)

• the 5 cm outboard flux surface passes between the plasma and the passive 	stabilizer plates

• the 5 cm outboard flux surface strikes the outer divertor target plate

Equilibrium conditions shall be sustainable for pulse lengths up to 5 sec. subject to the following constraints for the SN configuration:

• R0 + a=1.533 m

• Zx =-1.50 m (vertical height of the lower, active X-point)

• the vacuum flux surface outside the plasma, Ψ∗, can vary from 0 to 20% of the 	normalized separatrix flux surface value (see below). This is a requirement 	defined by the application of CHI.

• Ψ∗ completely encloses the plasma without intersecting a material surface except 	at the divertor footprints

• the Ψ∗ footprints intersect the outboard and inboard divertor plates respectively

1.2.2 Toroidal Field/Plasma Current Directionality

The tokamak shall be initially configured to drive plasma current in the positive direction with a negative toroidal field.

The figure below illustrates the right-handed coordinate system used on NSTX.  A positive plasma current is in the φ-direction (counter-clockwise direction) and provides a positive vertical field in the Z-direction across the centerline of the vessel.  A positive TF current (in the TF inner leg) is in the Z-direction and provides a positive toroidal field in the φ-direction (counter-clockwise direction).  Positive radial fields and currents are in the R-direction, radially outward from the Z-axis, the major axis of the tokamak.

Plasma facing components and rf launchers shall be designed for two quadrant operation, i.e.

• the toroidal field shall be bi-directional and

• the plasma current shall always be in the opposite direction as the toroidal field for proper helicity.

All other equipment shall be designed for four quadrant operation, i.e.

• the toroidal field shall be bi-directional and

• plasma current in the same direction and in the opposite direction as the toroidal field can be accommodated.

1.3 Power Handling and Particle Control

1.3.1 Scrape-off Layer (SOL) Envelopes

The +5 cm (outer) flux surface shall intercept only the outboard divertor targets for the range of equilibria defined in Section 1.2.1.  All internal hardware must be located accordingly.

The inner flux surface shall intercept only the inboard divertor targets for the SN equilibria defined in Section 1.2.1.  All internal hardware must be located accordingly.

1.3.2 Maximum Heating Power

Plasma facing components shall be designed to accommodate heat loads associated with a maximum heating power of 12 MW ( 6 MW for HHFW and 5 MW for NBI-upgrade + 1 MW for OH) for 5 sec.  Up to half of the input power is expected to flow into the divertor

plates .  Therefore, the divertor plate should be able to handle up to 30 MJ during the pulse.  It should be noted that the CHI power requirement is not well established at the present time.  The initial CHI plasma start-up should take place in a short time (<< 100 msec) and the heat load to the PFC is small (< 3 MJ).  The CHI edge current drive is a part of the physics research program, and the PFC capability may be enhanced or modified in accordance with the results of CHI edge current drive experiment in the latter time.

What power density do the PFCs (other than the divertors and limiters) need to be designed for? (CN) - this is being addressed by the ORNL group (SK).

1.3.3 Divertor Requirements

1.3.3.1 Divertor Configuration

The divertor configuration shall accommodate the heat loads resulting from the heating power and pulse duration given in Sec. 1.3.2. Flexibility to reconfigure the divertor assemblies shall be provided.

What is the power density for SN and DN configurations?(CN) - this is being addressed by the ORNL group (SK).

1.3.3.2 Divertor Materials

The plasma-facing surfaces of the divertor assembly shall be made of carbon-based materials.

The minimum as-manufactured thickness of the graphite armor shall be 1.0cm between the plasma facing surface and the cooling tube interface.

The maximum divertor surface temperature shall not exceed 2000°C.

The NSTX facility shall be capable of being modified to accommodate alternate materials for plasma-facing surfaces including beryllium, tungsten, and molybdenum.

1.3.3.3 Divertor Power Handling

The inboard and outboard divertor targets shall be designed for handling peak heat fluxes  of 6 MW/m2 for pulse lengths up to 5 sec.

1.3.4 Limiter Requirements

1.3.4.1 Limiter Configuration

Poloidal limiters shall be provided outboard of the plasma for startup and to protect the outboard passive structure and equipment in the port region from particle and heat loads during operation and disruptions.

The outboard poloidal limiter horizon shall conform to the geometry of the Faraday Shield on the FW launcher.

Protective armor shall be provided to protect the inboard structures from particle and heat loads during operation and disruptions. The inner wall will act as the inner limiter.

Protective armor shall be provided to protect the outboard passive structure and components between the passive structure and vacuum vessel wall from energetic particle losses due to field ripple.

1.3.4.2 Limiter Materials

The plasma-facing surfaces of the limiter assemblies shall be made of carbon-based materials.

Maximum limiter surface temperatures shall not exceed 2000°C.

The NSTX facility shall be capable of being modified to accommodate alternate materials for plasma-facing surfaces including beryllium, tungsten, and molybdenum.

1.3.4.3 Limiter Power Handling

The inboard protective armor shall be designed to accommodate local heat loads due to NB shine-through, radiation, and plasma startup.

The outboard toroidal limiters shall be designed to accommodate local heat loads due to ripple-trapped particles, radiation, and plasma startup.

What power density will the limiters have to handle in the natural divertor configuration? Being determined by ORNL

1.3.6 Pumping Speed and Throughput

The NSTX facility shall be designed for maximum pumping rates of

• 50 torr-l/s when operated at the pulse repetition rates specified in Section 2.2 and

• 200 torr-l/s when operated at lower pulse repetition rates.

The NSTX facility shall be capable of being upgraded for maximum pumping rates of

• 100 torr-l/s when operated at the pulse repetition rates specified in Section 2.2 and

• 250 torr-l/s when operated at lower pulse repetition rates (see Section 2.2).

The NSTX facility shall provide a minimum pumping speed of 65,000 l/sec.

A variable aperture on the vacuum pumping ducts shall be provided to control the pumping speed.

1.4 Plasma Heating and Current Drive

The NSTX facility shall include a heating and current drive system based on multiple technologies (ECH, CHI, and HHFWH/HHFWCD), each subsystem expandable to greater power levels.  The NSTX facility shall also provide flexibility in the control of current density profiles.

1.4.1 ECH

ECH system is required to provide up to 200 kW of power to plasma.  The preionization ECH power requirement is 50 kW.  The preionization is required for OH start-up and there fore the ECH system must be able to deliver 50 kW reliably in the base NSTX operation.  The ECH tokamak formation is part of the physics experiment which will require the ECH power as needed up to the maximum 200 kW.

1.4.2. CHI

Co-axial Helicity Injection is required to provide non-inductive discharge start-up to current levels of approximately 0.5 MA as the base operation, and to provide non-inductive current drive at levels of approximately 0.2 MA in the outer portion of the plasma during the current sustainment phase as the physics reserach program.

The device and facility shall be configured initially with 3 MJ of energy and up to 15 MW of power of CHI to the plasma.

1.4.3 High Harmonic Fast Wave Heating/Current Drive (HHFWH/HHFWCD)

HHFWH/HHFWCD is required to provide electron heating and current drive.

The device and facility shall initially be configured to provide 6 MW of HHFWH/HHFWCD power to the plasma.

The HHFWH/HHFWCD power shall initially be coupled to the plasma through two 6-strap launchers.

A Faraday shield shall be provided between the launcher and plasma.  At operating temperature, the front surface of the Faraday shield shall conform (±2mm) to the surface of a circular toroid located on the midplane with major radius 1.832 m and minor radius 0.938 m within 0.4m of the midplane.  This surface bounds the family of +2cm flux surfaces over the required range of flexibility in β-li space.

1.5 Fueling

1.5.1 Gas Injection

The device shall be provided with a programmable gas injection system capable of supplying H, D, and other gases to the outboard midplane with a maximum throughput of 250 Torr-l/s.Three piezoelectric valves shall be provided to control gas injection to the VV. The valves shall be located on the mid plane at three locations separated by 120 degrees. The valves shall be supplied by manifolds which connect to four source tanks in such a way that any valve can be supplied from any source tank.

This is sort of in conflict with the earlier specification of throughtput. (CN)

1.5.2 Pellet Injection

The device and facility shall be capable of accommodating (as an upgrade) a pellet injection system capable of injecting H or D pellets with a maximum steady state fueling rate of 85 Torr-l/s.

1.6 TF Ripple

The peak-to-average TF ripple shall be ≤0.5% over the entire plasma cross section.

1.7 Field Errors

The non-axisymmetric field errors should be less than 0.1% of the vacuum toroidal field.

1.8 Plasma Startup and Equilibrium Control

1.8.1 Radial Plasma Position Control

The device shall include a feedback equilibrium control system which will provide radial plasma position control such that:

• the radial location of the separatrix at the outboard midplane can be maintained within 1 cm (rms displacement) of the nominal location, and

• the radial location of the separatrix at the outboard midplane can be moved by ±2 cm from nominal in 50 msec, in response to needs for RF coupling

for the reference scenarios defined in Section 1.1.

PF coils shall be designed to satisfy radial position control requirements for the full range of equilibrium conditions identified in Section 1.2.

Power systems shall be designed to satisfy radial position control requirements attendant with the reference scenarios per Sections 1.1 and 1.2.

1.8.2 Vertical Plasma Position Control

A conducting structure shall be provided which makes the growth rate of the vertical instability, less than 0.2/ts, where ts is the L/R time constant of the lowest order, up-down anti-symmetric mode in the conducting structure (approximately 40 msec) for all the equilibrium conditions required per Section 1.2.

The device shall include a feedback equilibrium control system which will provide vertical position control such that:

• the plasma can be vertically positioned to any location within ±3cm of the midplane with a response time of 0.1s using one pair of external PF coils and

• the plasma vertical position can be controlled within 1cm (rms displacement) of the nominal location on the time scale of the vertical instability growth time.

The PF coils shall satisfy the vertical position control requirements for the full range of equilibrium conditions identified in Section 1.1 and 1.2.

Power systems shall satisfy vertical position control requirements attendant with the reference scenarios per Section 1.1 and 1.2.

1.8.3 Separatrix Position Control

The device shall include a feedback equilibrium control system which will provide outboard strike point position control such that:

• the location of the outboard  strike point can be moved ±5cm along the divertor surface with a response time of 0.5s and

• the location of the outboard strike point can be maintained within ±1 cm of nominal

for the reference scenarios defined in Section 1.1 and 1.2.

PF coils shall satisfy the outboard strike point control requirements for the full range of equilibrium conditions identified in Section 1.1 and 1.2.

Power systems shall satisfy the outboard strike point control control requirements attendant with the reference scenarios per Section 1.1 and 1.2.

1.8.4 Plasma Startup

The stabilizing shell shall include resistive breaks to provide sufficient toroidal resistance for penetration of the startup voltage.

The device and facility must be capable of initiating the reference inductive discharge scenarios with a common, preprogrammed startup sequence meeting the following specifications:

• ECH pre-ionization.

• The applied loop voltage shall rise to at least 6 V in the absence of plasma.

• Starting from the time of breakdown, the plasma current shall increase to 500kA within 0.1s (can we relax this to 0.2 s? - let's see how long a flattop TSC says)

• Feedback control of the plasma current and radial equilibrium shall be established during the current ramp up phase.

1.8.5 External Kink Mode Control

Conducting structure shall be provided for the purpose of passively stabilizing the ideal external kink mode for all the equilibrium conditions required per Section 1.2.

The conducting structure shall be compatible with the bake out and the initial complements of heating and current drive systems per Section 1.4.

Flexibility to reconfigure the structure by removing and re-installing its elements, including the ability to make it compatible with the upgrade complement of heating and current drive systems per Section 1.4, shall be provided.

1.9 Vacuum and Wall Conditioning

1.9.1 Base Pressure

The device and facility shall produce high vacuum conditions with a base partial pressure of ≤1x10-7 torr for H and D, and a base partial pressure of ≤1x10-9 torr for impurity gases (Z ≥ 2) at 65°C.

The impurity gas load during pumpdown to base pressure shall not exceed 1x10-5 torr-l/s.

1.9.2 Bakeout Temperature

The vacuum vessel shell and all components internal to the vacuum vessel shall be bakeable at 350°C.

Vacuum pumping ducts and large horizontal ports shall have nominal bakeout temperatures of 350°C at the interface to the vacuum vessel and 150°C at the interface to the torus vacuum isolation valve.

1.9.3 Glow Discharge Cleaning

The device shall provide a glow discharge cleaning (GDC) capability with DC glow in 10-20 mtorr of H, D, or He at 400 V and 20 A for indefinitely long periods with the vacuum vessel at 350°C and a throughput 44 torr-l/s (He).

The device design shall include the possibilty for GDC between shots to facilitate its implementation during the operation.

1.9.4 Boronization

The device shall provide the capability for boronization for all surfaces with line-of-sight to the plasma by a glow discharge technique with the interior surfaces at 350°C.

1.9.5 Pre-shot Temperature

In-vessel components shall be maintained at temperatures not less than 50°C between discharges.  The vacuum vessel and internal components shall be maintained at the same temperatures during the interval between discharges.

1.10 Disruption Handling

The device shall be designed to accommodate major disruptions characterized by:

• possible vertical displacement

• a thermal quench occurring in 0.1-1 ms

• a current decay with a time-varying decay rate, such that the peak rate is ≤0.5 MA/ms• a peak poloidal halo current up to 400 kA, 40% of the maximum plasma current prior to the disruption, with a toroidal peaking factor of 2:1; that is, the toroidal dependence of the halo current is [1 + 1.0*cos (φ - φ0)], for all toroidal phase angles φ0  from 0 to 2π.

• a frequency of occurrence of 10% in any phase or mode of operation.

1.11 Diagnostics

The NSTX machine shall incorporate a system of diagnostics to provide the measurements of the plasma behavior that are necessary to fulfill the physics mission of the NSTX project.  In addition, certain measurements shall be provided for plasma control and operational purposes.  The principal categories of the measurements are as follows:

•  Plasma Control  An array of magnetic diagnostics shall be provided to measure the plasma and conducting shell currents, the radial and vertical plasma position, the outboard divertor strike point, and the plasma shape with time resolution and accuracy sufficient to satisfy the associated control and operational requirements in Section 1.8.  Profiles of the plasma current density and the plasma pressure shall be provided to support the NSTX advanced tokamak mission through active control of the plasma profiles.  Magnetic measurements of MHD and locked mode activity shall be provided in support of disruption handling requirements in Section 1.10.  The line-averaged density (<n> ≤ 1.0x1020 m-3) shall be measured to provide feedback control of the plasma density.

•  Confinement  Core profiles of the electron density (neo ≤ 2x1020 m-3), electron temperature (Teo ≤ 6 keV), ion temperature (Tio ≤ 6 keV), average impurity concentration (Zeff ~ 1-5), and the radiated power shall be measured.  Measurements of the plasma poloidal beta (0.01<βp<5.0) and the low-Z and high-Z impurity concentrations shall be provided.  Internal and external fluctuations shall be measured over a range of frequencies adequate to encompass MHD activity and plasma instabilities.

•  Divertor/Edge Plasma  The divertor and limiter surface temperatures shall be measured from ~ 300 oC to 3000 oC with maximum coverage consistent with viewing access to provide operational information needed for wall conditioning, impurity control, and protection of plasma facing components.  Measurements of the electron density, ion/electron temperatures, neutral pressure, and the Hα and impurity line radiation in the SOL and divertor plasma region shall be provided.

•  Fusion Yield  The total neutron source rate and the radial profile of the neutron source emission shall be provided, as well as measurements of the charged fusion products.

All diagnostics shall be implemented consistent with achievable viewing access and state-of-the-art measurement techniques.

Diagnostics shall be designed for H and D operation only.

1.11.1 Baseline diagnostics

The device and facility shall include the following diagnostics:

Table 1: Day One Diagnostics Summary

| Machine Diagnostics | Physics Diagnostics | Divertor/Edge Diagnostics |
| --- | --- | --- |

| Rogowski coils | Mirnov Coils | Thermocouples |
| --- | --- | --- |

| Flux Loops |  |  |
| --- | --- | --- |

| Visible Camera |  |  |
| --- | --- | --- |

| IR Camera |  |  |
| --- | --- | --- |

Table 2: Baseline Diagnostics Summary

| Machine Diagnostics | Physics Diagnostics | Divertor/Edge Diagnostics |
| --- | --- | --- |

| 1 mm microwave interferometer | Multichannel TVTS | Multichannel bolometer |
| --- | --- | --- |

| Flux loops | CHERS | Hα detectors |
| --- | --- | --- |

| Rogowski Coils | MSE | Langmuir probes |
| --- | --- | --- |

| Visible Camera | Visible continuum array |  |
| --- | --- | --- |

| IR Camera | Survey spectrometer (SPRED) |  |
| --- | --- | --- |

| Residual gas analyzers | Soft X-ray imaging system |  |
| --- | --- | --- |

|  | Mirnov Coils |  |
| --- | --- | --- |

|  | Flux loops (for fluctuations) |  |
| --- | --- | --- |

1.11.2 Upgrade diagnostics

The device and facility shall be capable of accommodating diagnostic upgrades as follows:

Table 3: Upgrade Diagnostics Summary

| Machine Diagnostics | Physics Diagnostics | Divertor/Edge Diagnostics |
| --- | --- | --- |

| Diamagnetic loop | “DensPak” Mirnov coil array | Fast reciprocating edge probe |
| --- | --- | --- |

|  | Soft X-ray tomography | Divertor Langmuir probes |
| --- | --- | --- |

|  | AM reflectometry | Rogowski coils (on backing plate supports) |
| --- | --- | --- |

|  | Beam emission spectroscopy |  |
| --- | --- | --- |

|  | Multichannel radiometry |  |
| --- | --- | --- |

|  | Multichannel charge exchange analyzer |  |
| --- | --- | --- |

|  | Gas neutron counters |  |
| --- | --- | --- |

1.12 Instrumentation, Control, and Data Acquisition

The NSTX facility shall have an instrumentation, control, and data acquisition (I&C) system that allows for coordination of operations and the acquisition, archiving, and display of all pertinent information.

The I&C system will be designed for operation of NSTX, and it will provide supervision, remote control, monitoring, data acquisition, and data handling for subsystems.

The national perspective of NSTX requires that researchers located at the NSTX site and at other laboratories and universities have access to the facility in support of the physics planning, analysis, coordination of experimental objectives, and carrying out the experimental program.

1.13 Material Selection

The materials to be used in the tokamak and peripheral equipment shall be limited based on vacuum, bakability, impurity control, magnetic permeability, and activation considerations.

All materials to be used in the tokamak and peripheral equipment must have a relative magnetic permeability less than 1.02 unless otherwise authorized by the Project.

All materials to be used within the plasma vacuum boundary must be approved by the Project.

2 OPERATIONAL REQUIREMENTS

Operational limits for D operation shall be established such that the environmental, safety, and health effects determined for PBX-M, as presented in the Environmental Assessment for PBX-M, shall not be exceeded during normal operation of NSTX.

2.1 Pulse Length

The NSTX facility shall initially be configured to provide 5 s pulses with H and D plasmas for the reference scenarios per Section 1.1 with initial neutron source rates per Section 2.5 and pulse repetition rates per Section 2.2.

The tokamak shall be designed for and the facility be capable of being upgraded for 60 sec operation with H and D plasmas for the full range of flexibility specified in Section 1.2 with the ultimate neutron source rates specified in Section 2.5.

2.2 Pulse Repetition Rate

In its initial configuration, the NSTX facility shall be capable of providing 96 - 5 sec. pulses at 300 sec. intervals in a 24 hour period when operated per the reference scenarios.

2.3 Annual Operating Limits

The design shall allow for up to

• 13,000 pulses

• 6.5x104s of operation

2.4 Lifetime Operating Limits

All NSTX systems, with the exception of the divertor modules, shall be designed for the number of cycles and operating time attendant with a 10 year operating life.  New building and utilities shall be designed for a 25 year operating life from the start of plasma operations.

The design shall allow for up to

• 130,000 pulses

• 6.5x105s of D operation

when operated per the reference scenarios based on the factors for fatigue life specified in the NSTX Structural Design Criteria Document.

The design shall allow for greater than 130,000 charge-discharge cycles of the toroidal field coils (which includes cycling required for GDC).

2.5 Neutron Source Rates, Yields, and Dose-equivalents

The NSTX facility shall be designed to accommodate heat loads attendant with the peak neutron source rate initially envisioned for D operation at the pulse repetition rates specified in Section 2.2.

In addition, the NSTX tokamak systems and facility shall be designed for and the facility be capable of being upgraded for heat loads attendant with the peak neutron source rates ultimately envisioned for 5 sec and 60 sec, long-pulse D operations.

| Heating Operation With Present Shielding | Estimated D-D neutron rate | Total D-D neutrons per yr | Total D-D discharge sec per yr | Total D-D 5 sec discharges per yr | Total D-D 60 sec demo discharges per yr |
| --- | --- | --- | --- | --- | --- |

| RF-only | 4 x10+12 | 2 x10+16 | 5000 | 1000 | 83 |
| --- | --- | --- | --- | --- | --- |

| RF + NBI | 1.5x10+14 | 2 x10+16 | 133 | 27 | 2 |
| --- | --- | --- | --- | --- | --- |

| Heating Operation With x12 Shielding Upgrade |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| RF-only | 4 x10+12 | 3 x10+17 | 60,000 | 12,000 | 1000 |
| --- | --- | --- | --- | --- | --- |

| RF + NBI | 1.5x10+14 | 3 x10+17 | 1600 | 320 | 27 |
| --- | --- | --- | --- | --- | --- |

Table 2.5.1 NSTX discharges per calendar year, at the estimated maximum neutron rate with present radiation shielding, and with a feasible x12 shielding upgrade.

The NSTX facility includes the tokamak, ancillary systems, and facilities.

Per December 11, 1992 memo from Davidson to Johnson

(Ψ∗-Ψs)/(Ψs−Ψmag) = 0.0 to 0.2, where Ψ∗ is the vacuum flux surface, Ψs is the separatrix flux surface value, and Ψmag is the flux value at the magnetic axis.

Unless otherwise qualified, the pulse length refers to the flattop time. i.e. the period of constant plasma current.