National Spherical Tokamak Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

PLASMA FACING COMPONENTS

NSTX-SRD-11-013

Revision 0

June 28, 1996

________________________________

John Spitzer

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 6/28/96 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	1

Plasma Facing Components Requirements	1

1.0 Functions	1

2.0 Configuration Requirements & Essential Features	1

2.1 Passive Stabilizers and Limiters	1

2.2 Inner Wall Armor	2

2.3 Inboard and Outboard Divertor Area Strike Plates	2

3.0 Performance & Operational Requirements	2

3.1 Baseline Operation	2

3.2 Upgrades	3

List of Tables

PFC-3.1-1	PFC Heat Loads for Baseline NSTX Duty Cycle	2

PFC-3.2-1	PFC Heat Loads for Upgrade NSTX Duty Cycle	3

APPENDIX

LIST OF REFERENCES

Introduction

This System Requirements Document (SRD) defines the requirements and design of the Plasma Facing Components (WBS 11) of the National Spherical Tokamak Experiment (NSTX).

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix."

WBS 11 -  Plasma Facing Components Requirements

1.0 Functions

The Plasma Facing Components (PFC) consist of plasma facing graphite tiles and mounting hardware, passive stabilizers, inner wall protection, divertor area strike plates and local Instrumentation and Control.

The functions of the Plasma Facing Components are to:

a.  Provide sufficient MHD wall stabilization for the high performance plasmas.

b.  Provide protection for the vacuum vessel diagnostic and heating components from the anticipated heat loads.

2.0 Configuration Requirements & Essential Features

The plasma facing components include the outer passive stabilizers, inner wall armor, divertor area strike plates and the plasma facing tiles.  The PFC designs shall conform to the space requirements defined in drawings:

TBD.

2.1 Passive Stabilizers and Limiters

a.  The outer passive stabilizers consist of one upper and one lower conically shaped copper stabilizer.

b.  Each stabilizer shall be made up of a series of copper plates which are electrically connected in the poloidal direction.  Each stabilizer shall have a minimum of one toroidal electrical break (R1).

c.  The passive stabilizers shall be electrically connected to the vessel via their mounting structures (R2).  The connections shall contain provisions to allow for alignment adjustments and facilitate removal of individual plates for modification to accommodate access for future diagnostic upgrades.

d.  The overall size of the individual plates making up the stabilizers must be such that they can be passed through the large horizontal vacuum vessel ports ( largest dimension not greater than 20.-in.) and easily manipulated by no more than two persons ( < 100 lb.)

e.  Four outboard poloidal limiters shall be provided as part of the outer passive structure to protect the High Harmonic Fast Wave launchers. (R3)

f.  The plasma facing surfaces of the stabilizers shall be protected with graphite tiles. (Union Carbide - ATJ or equivalent)

2.2 Inner Wall Armor

a.  The inner wall armor shall be composed of carbon-carbon tiles attached to the center stack casing.

b.  The poloidal extent of the inner wall armor/inboard divertor strike plates (see below) shall be such that it eliminates all line of sight between the upper and lower ceramic ring/bellows assemblies and the plasma.

2.3 Inboard and Outboard Divertor Area Strike Plates

a.  The divertor area strike plates shall consist of upper and lower toroidal rings composed of carbon-carbon tiles.

b.  The outboard divertor rings shall be attached to the upper and lower domes of the vacuum vessel.

c.  The inboard divertor rings are attached to the centerstack casing.

3.0 Performance & Operational Requirements

All materials for the PFCs, their connections and mountings must :

a.  Be approved for high vacuum compatibility. (R4)

b.  Be bakeable to 350°C (R5).

c.  Have a relative magnetic permeability of < 1.02 (R6).  Exceptions must have prior NSTX project approval.

d.  Be compatible with Glow Discharge Cleaning (R7)

3.1 Baseline Operation

a.  For baseline operation there will be no active cooling of the plasma facing components during operation.  The plasma facing tiles of the outer passive stabilizer, inner wall armor and divertor strike plates must therefore accommodate a heat flux for the baseline duty cycle as described in Table PFC-3.1-1.

Table PFC-3.1-1: PFC Heat Loads for Baseline NSTX Duty Cycle

| Component | Value | Units | Ref |
| --- | --- | --- | --- |

| Passive Stabilizers | TBD | MW/m2 | TBD |
| --- | --- | --- | --- |

| Limiters | TBD | MW/m2 | TBD |
| --- | --- | --- | --- |

| Inner Wall Armor | TBD | MW/m2 | TBD |
| --- | --- | --- | --- |

| Divertor Area Strike Plates | 6 | MW/m2 | TBD |
| --- | --- | --- | --- |

b.  The peak and average temperature limits for the graphite tiles shall be TBD°C and TBD°C, respectively.

c.  The peak and average temperature limits for the carbon-carbon tiles shall be TBD°C and TBD°C, respectively.

3.2 Upgrades

a.  For the NSTX upgrades, actively cooled PFCs will replace the baseline components.  The plasma facing tiles of the outer passive stabilizer, inner wall armor and divertor strike plates shall be capable of removing heat generated during the pulse for the long pulse upgrade duty cycle as described in Table PFC-3.2-1.

b.  For the neutral beam upgrade, armor shall be provided to protect the vessel and center stack casing due to neutral beam shine-through attendant with a line-averaged plasma density of TBD, and due to inadvertent unattenuated neutral beam loads for TBD ms at full power.

Table PFC-3.1-1: PFC Heat Loads for Upgrade NSTX Duty Cycle

| Component | Long Pulse Upgrade Value | NB Upgrade Value | Units | Ref |
| --- | --- | --- | --- | --- |

| Passive Stabilizers | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Limiters | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Inner Wall Armor | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Divertor Area Strike Plates | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

c.  The actively cooled PFCs will be cooled by deionized water supplied by WBS 32.

d.  The peak and average temperature limits for the graphite tiles shall be TBD°C and TBD°C, respectively.

e.  The peak and average temperature limits for the carbon-carbon tiles shall be TBD°C and TBD°C, respectively.

APPENDIX

| # | Description | Value | Units | Reference |
| --- | --- | --- | --- | --- |

| 1 | PFC GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.1.c |
| --- | --- | --- | --- | --- |

| 2 | PFC GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.1.g |
| --- | --- | --- | --- | --- |

| 3 | Limiter Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 2.5.2.1.b |
| --- | --- | --- | --- | --- |

| 4 | High Vacuum Compatibility Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 2.3.1.b and .c |
| --- | --- | --- | --- | --- |

| 5 | Bakeout Temperature | 350 | °C | NSTX-RQMTS-GRD r0, sec. 2.3.2.a |
| --- | --- | --- | --- | --- |

| 6 | Relative Magnetic Permeability | <1.02 | n.a. | NSTX-RQMTS-GRD r0, sec. 1.1.4.a |
| --- | --- | --- | --- | --- |

| 7 | Glow Discharge Cleaning | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 2.3.1.e |
| --- | --- | --- | --- | --- |