National Spherical Tokamak Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

VACUUM VESSEL AND TOKAMAK SUPPORT STRUCTURE

NSTX-SRD-12-014

Revision 0

May 6, 1996

________________________________

John Spitzer

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 5/6/96 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	3

Vacuum Vessel and Tokamak Support Structure Requirements	3

1.0 Functions	3

2.0 Configuration Requirements & Essential Features	3

2.1 Vacuum Vessel	3

2.2  Tokamak Support Structure	4

3.0 Performance & Operational Requirements	4

3.1 Baseline Operation	5

3.1.1 Vacuum Vessel	5

3.1.2 Tokamak Support Structure	6

3.2 Upgrades	6

List of Tables

VV-3.0-1	design loads	4

VV-3.2-1	upgrade nstx duty cycle	6

APPENDIX

LIST OF REFERENCES

Introduction

This System Requirements Document (SRD) defines the requirements and design of the Vacuum Vessel and Tokamak Support Structure (WBS 12) of the National Spherical Tokamak Experiment (NSTX).

Figures and Tables are numbered in format X-Y-Z where X=section (i.e. section abbreviation, e.g. S for Sample System), Y=paragraph number, and Z= sequence number within that paragraph.

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix."

WBS 12 -  Vacuum Vessel and Tokamak Support Structure Requirements

1.0 Functions

The vacuum vessel and tokamak support structure consist of the vessel chamber, all ports and vacuum boundary closures, and the tokamak support structure.

The functions of the vacuum vessel is to:

a.  Provide a vacuum boundary around the plasma suitable for high vacuum conditions.

b.  Provide structural support for plasma facing components  (PFC).

c.  Provide access and structural support for vacuum pumping ducts, plasma heating and current drive, and diagnostics.

d.  Provide a structural support for the coils systems including the centerstack assembly, poloidal field coils number PF #2 a & b, PF# 3a &b, and PF#4 plus the outer TF coil legs.

e.  Provide an electrical break between the vacuum vessel and the centerstack casing to support helicity injection (CHI) during startup.

The function of the tokamak support structure is to:

a.  Provide the overall support mechanism for the vacuum vessel and all tokamak components which are attached to it.

2.0 Configuration Requirements & Essential Features

The vacuum vessel and tokamak support structure designs shall conform to the space requirements defined in drawings:

TBD.

2.1 Vacuum Vessel

a.  The vacuum vessel is a single walled enclosure consisting of an upper and lower domes and a straight cylindrical center section.  The two halves of the S-1 spherical vessel will be used for the dome sections.

b.  All sections of the vessel shall be constructed of 304 stainless steel and have a nominal wall thickness of 0.5-in.

c.  The upper and lower domes shall each have 11 ports.  The cylindrical section of the vessel shall have a sufficient number of large (24-inch) ports and smaller ports to accomodate plasma heating, diagnostic and pumping configuration requirements. (R1)

d.  The vacuum vessel shall incorporate upper and lower ceramic rings to electrically isolate the vessel from the centerstack.

e.  The vacuum vessel assembly shall be electrically isolated from the TF and PF coil systems.

f.  The vacuum vessel assembly shall be grounded with a single point ground through an insulated cable connected to "pit" ground..  The capability to isolate from ground and demonstrate a TBD kV voltage standoff shall be provided. (R2)

2.2  Tokamak Support Structure

a.  The tokamak support structure shall support vacuum vessel and all components attached to the vessel including the plasma facing components and outer PF coils

b.  The supports will be attached to the existing PLT base plate.

3.0 Performance & Operational Requirements

The vacuum vessel and tokamak support structure shall be designed to sustain the loads described in Table VV-3.0-1.

Table VV-3.0-1 Design Loads

| Load | Description |
| --- | --- |

| Dead Load (D) | The dead load includes the weight of the vacuum vessel, ports and their attachments, centerstack (during assembly), PFCs and external PF and TF coils. |
| --- | --- |

| Pressure Load (Pr) | The pressure within the vacuum vessel will varies from atmospheric, about 14.7 psi, to 1x10-7 torr. (R3) |
| --- | --- |

| Bakeout Thermal Load (To) | The vessel wall temperature will reach a maximum temperature of 350 °C. (R4) |
| --- | --- |

| Electromagnetic Loads (EM) | The electromagnetic loads acting on the vessel include those induced during normal pulsed operation of the device as well as the electromagnetic effects of discharge cleaning. |
| --- | --- |

| Plasma Disruptions (PD) | The vacuum vessel assembly shall be designed to accommodate plasma disruptions as defined in the GRD. (R5) |
| --- | --- |

| Seismic Loads (EQ) | The vacuum vessel shall be designed to satisfy the seismic loading for a Performance Category 1 (PC-1) structure per the Department of Energy's (DOE) standard for natural phenomena hazard (NPH) events. (R6) |
| --- | --- |

To assess the design adequacy, the design loads specified in Table VV-3.1-1 shall be combined using the following combinations

•	Case #1	Combination of D + Pr + EM

•	Case #2	Combination of D + Pr + EM + PD

•	Case #3	Combination of D + Pr + EM + EQ

•	Case #4	Combination of D + Pr  + To

In addition to the design loads the following performance and operational requirements apply.

3.1 Baseline Operation

3.1.1 Vacuum Vessel

a.  The vacuum vessel assembly shall be capable of operation in high vacuum conditions with a base partial pressure of ≤1x10-7 torr for fuel gases (Z ≤ 2) and a base partial pressure of ≤1x10-9 torr for impurity gases (Z > 2) at 65°C. (R3)

b.  All vacuum vessel flanges shall provide a vacuum seal compatible with high vacuum conditions.

c.  All materials exposed to the high vacuum within the plasma chamber must be approved for high vacuum compatibility.

d.  Materials containing mercury, sulfur, chlorine or other halogens (e.g. halons and freons) shall not be used within the vessel.

e.  The vessel shall be capable of handling the heat load resulting from the 350°C bakeout of the plasma facing components. (R4)

f.  The vessel shall be compatible with glow discharge cleaning. (R7)

g.  The vacuum vessel material shall have a relative magnetic permeability of < 1.02 (R8).  Exceptions must have prior NSTX project approval.

h.  The toroidal resistance of the vessel shall be greater than TBD.

3.1.2 Tokamak Support Structure

a.  All materials used for the tokamak support structure shall have a relative magnetic permeability of < 1.02 (R8).  Exceptions must have prior NSTX project approval.

3.2 Upgrades

For the NSTX upgrades, the vacuum vessel and support structure must satisfy all of the baseline requirements.  In addition, some active cooling may be required to remove the heat  generated during the pulse for the long pulse upgrade duty cycle as described in Table VV-3.2-1.

Table VV-3.2-1: Upgrade NSTX Duty Cycle

| Parameter | Value | Units | Ref |
| --- | --- | --- | --- |

| Baseline maximum plasma duration (t) | 60.0 | sec | 9 |
| --- | --- | --- | --- |

| Baseline minimum repetition period (T) | TBD | sec | 10 |
| --- | --- | --- | --- |

APPENDIX

| # | Description | Value | Units | Reference |
| --- | --- | --- | --- | --- |

| 1 | VV GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.2.c and .d |
| --- | --- | --- | --- | --- |

| 2 | VV GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.2.e |
| --- | --- | --- | --- | --- |

| 3 | Base Partial Pressure Requirement | 1x10-7 | torr | NSTX-RQMTS-GRD r0, sec. 2.3.1.a |
| --- | --- | --- | --- | --- |

| 4 | Bakeout Temperature | 350 | °C | NSTX-RQMTS-GRD r0, sec. 2.3.2.a |
| --- | --- | --- | --- | --- |

| 5 | Plasma Disruptions | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 2.7.a and .b |
| --- | --- | --- | --- | --- |

| 6 | Seismic Loads | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 4.2 |
| --- | --- | --- | --- | --- |

| 7 | Glow Discharge Cleaning | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 2.1.3.e |
| --- | --- | --- | --- | --- |

| 8 | Relative Magnetic Permeability | <1.02 |  | NSTX-RQMTS-GRD r0, sec. 1.1.4.a |
| --- | --- | --- | --- | --- |

| 9 | Long Pulse Maximum Plasma Duration | 60 | sec | NSTX-RQMTS-GRD r0, sec. 2.1.3.2.a |
| --- | --- | --- | --- | --- |

| 10 | Long Pulse Minimum Repetition Rate | TBD | sec | TBD |
| --- | --- | --- | --- | --- |