National Spherical Tokamak Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

MAGNET SYSTEMS

NSTX-SRD-13-015

Revision 0

June 28, 1996

________________________________

John Spitzer

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 6/28/96 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	1

WBS 13 - Magnet Systems Requirements	1

1.0 Functions	1

2.0 Configuration Requirements & Essential Features	1

3.0 Performance & Operational Requirements	1

WBS 131 - Outer Poloidal Field Coils Requirements	2

1.0 Functions	2

2.0 Configuration Requirements & Essential Features	2

3.0 Performance & Operational Requirements	3

3.1 Baseline Operation	3

3.2 Upgrades	3

WBS 132 - Outer Toroidal Field Coils Requirements	3

1.0 Functions	3

2.0 Configuration Requirements & Essential Features	4

3.0 Performance & Operational Requirements	4

3.1 Baseline Operation	4

3.2 Upgrades	4

WBS 133 - Centerstack Subsystem Requirements	5

1.0 Functions	5

2.0 Configuration Requirements & Essential Features	5

2.1  TF Inner Legs	5

2.2  Ohmic Heating Coils	5

2.3  Shaping Coils	6

2.4   Centerstack Casing	6

3.0 Performance & Operational Requirements	6

3.1 Baseline Operation	6

3.1.1  TF Inner Legs	6

3.1.2  Ohmic Heating Coils	7

3.1.3  Shaping Coils	7

3.1.4   Centerstack Casing	7

3.2 Upgrades	8

3.2.1  TF Inner Legs	8

3.2.2  Ohmic Heating Solenoid	8

3.2.3  Shaping Coils	8

3.2.4   Centerstack Casing	8

List of Tables

MS-3.0-1	Design Loads	1

PF-3.1-1	Outer PF Coil Currents	3

TF-3.1-1	TF Outer Leg Performance	4

CS-3.1-1	TF Inner Leg Baseline Performance	6

List of FIGURES

PF-2.0-1	NSTX Coordinate system	3

APPENDIX

LIST OF REFERENCES

Introduction

This System Requirements Document (SRD) defines the requirements and design of the Magnet Systems (WBS 13) of the National Spherical Tokamak Experiment (NSTX).

Figures and Tables are numbered in format X-Y-Z where X=section (i.e. section abbreviation, e.g. S for Sample System PF for Poloidal Field, TF for Toroidal Field, CS for Center Stack), Y=paragraph number, and Z= sequence number within that paragraph.

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix."

WBS 13 - Magnet Systems Requirements

1.0 Functions

Refer to the sections for the individual subsystems.

2.0 Configuration Requirements & Essential Features

Refer to the sections for the individual subsystems.

3.0 Performance & Operational Requirements

a.  The magnet systems shall be designed to sustain the loads described in Table MS-3.0-1.

Table MS-3.0-1 Design Loads

| Load | Description |
| --- | --- |

| Dead Load (D) | The weights of the centerstack (during assembly), and outer PF and TF coils. |
| --- | --- |

| Thermal Load (To) | The conductors will reach a maximum temperature of 120°C during operation. |
| --- | --- |

| Electromagnetic Loads (EM) | The electromagnetic loads acting on the magnets during normal pulsed operation. |
| --- | --- |

| Seismic Loads (EQ) | The magnet systems shall be designed to satisfy the seismic loading for a Performance Category 1 (PC-1) structure per the Department of Energy's (DOE) standard for natural phenomena hazard (NPH) events. (R12) |
| --- | --- |

b.  To assess the design adequacy, the design loads specified in Table MS-3.0-1 shall be combined using the following combinations

•	Case #1	Combination of D + EM

•	Case #2	Combination of D + EM + EQ

•	Case #3	Combination of D + To

c.  In addition to these design loads, any performance and operational requirements identified in the requirements for the subsystems apply.

WBS 131 - Outer Poloidal Field Coils Requirements

1.0 Functions

The function of the poloidal field (PF) coil system is to:

a.  Support the plasma against radially expanding current and pressure forces.

b.  Provide plasma shaping and divertor/scrape-off control.

c.  Provide vertical/radial position stability control using feedback.

d.  Help compensate for the error fields of the ohmic heating solenoids.

e.  Provide additional flux to the plasma.

2.0 Configuration Requirements & Essential Features

a. The outer PF magnets shall consist of three pairs of water-cooled copper coils designated as PF 2a/2b, 3a/3b, and 4a/4b/4c.  The pairs shall be symmetric about the same, horizontal mid-plane of the NSTX device. (R2)  PF 1a/1b are part of the centerstack subsystem, WBS 133.

b.  Coils identified on the S-1 spheromak as EF coils #1, #2 and #3. shall be used for the outer PF coils on NSTX.

c.  The outer PF coils shall be supported off of the vacuum vessel structure.  The support mechanism shall be compatible with the vacuum vessel thermal expansion during bakeout.

d.  Each PF coil pair (upper and lower) shall be connected in series groups with independent current control as defined in the GRD. (R3)

e.  All series coil connections shall result in current flow which is equal in magnitude, and in the same φ direction, in the upper and lower coils. (R4)  The coordinate system used for NSTX is illustrated in Figure PF-2.0-1.

NSTX Coordinate System

Figure PF-2.0-1

f.  The outer PF coils shall be water cooled.  The cooling water will be supplied by WBS 32

3.0 Performance & Operational Requirements

3.1 Baseline Operation

a. The PF coils shall be capable of supplying the current scenarios established by the NSTX Project Physics Group, consistent with the requirements for start-up field null quality and loop voltage generation, as well as plasma equilibrium control. (R5)

b.  The maximum current and maximum equivalent square wave for the outer PF magnets are given in Table PF-3.1-1.

Table PF-3.1-1 Outer PF Coil Currents

| Coil | Max Current (kA) | Max ESW (sec) |
| --- | --- | --- |

| PF 2a/2b | 20.0 | 5.0 |
| --- | --- | --- |

| PF 3a/3b | 10.020.0 | 5.0 |
| --- | --- | --- |

| PF 4a/4b/4c | 20.0 | 5.0 |
| --- | --- | --- |

c. The PF coils shall be compatible with NSTX operation with plasma current in either  φ direction.  (R6)

d. The maximum voltage which will be impressed on the PF coils 1200V. Although the grounding system should limit the voltage to ground to 1/2 of this value, the coil design should anticipate the full value. Therefore the PF coil system shall be designed to accommodate a DC hipot level of at least 2*1200+1000 = 2.5kV to ground (R12).

3.2 Upgrades

a.  There are no additional upgrade requirements for the outer PF coils.

WBS 132 - Outer Toroidal Field Coils Requirements

1.0 Functions

The function of the toroidal field (TF) coil system is to provide the toroidal magnetic field which is necessary to magnetically confine the NSTX plasma.

2.0 Configuration Requirements & Essential Features

a.  The outer TF coils consist of an outer array of twelve coil bundles  The inner legs of the TF coils are part of the centerstack subsystem WBS 133.

b.  The outer legs of the TF coils shall be copper.

c.  Detachable joints shall be provided in the TF outer legs to allow removal/replacement of the centerstack assembly.

d.  The outer legs of the TF coils shall be water cooled.  Cooling water shall be supplied by WBS 32.

3.0 Performance & Operational Requirements

3.1 Baseline Operation

a. The baseline design of the outer legs of the TF coils shall be capable of supplying the toroidal fields as described in Table TF-3.1-1.

Table TF-3.1-1  TF Outer Leg Performance

| Pulse Duration (sec) | ITF (Max) (kA) | BTF (R=8085.4cm) (kG) |
| --- | --- | --- |

| 0.6 | 34.5 | 6.46.0 |
| --- | --- | --- |

| 5.0 | 17.7 | 3.23.0 |
| --- | --- | --- |

| 60.0 | 17.7 | 3.23.0 |
| --- | --- | --- |

b.  For baseline operations, the outer TF conductors will be inertially cooled.  The initial temperature of the coils shall be 20°C.

c.  The maximum temperature of the TF conductor insulation shall be < 120°C during pulsing.

d.  The coils shall be capable of being recooled to the initial temperature in < 5 minutes using the existing cooling water systems (WBS 32), such that an indefinite number of pulses can be performed with a repetition period of 5 minutes.

e. The TF coils shall be compatible with NSTX operation with plasma current in either φ direction, with toroidal field in either φ direction.  (R7)

f. The peak coil terminal voltage anticipated in the TF system is 1784 volts. Although the grounding system should limit the voltage to ground to 1/2 of this value, the coil design should anticipate the full value. The TF coil system shall be designed to accommodate a DC hipot level of at least 2*1784+1000 = 4.6kV to ground (R10).

3.2 Upgrades

a.  For the long pulse upgrade the outer legs of the TF coils shall be actively cooled using cooling water supplied by WBS 32.

WBS 133 - Centerstack Subsystem Requirements

The centerstack subsystem includes the inner legs of the TF coils, the Ohmic Heating solenoid, shaping coils - PF coils 1a/1b, and the centerstack casing.

1.0 Functions

1.1 TF Inner Legs

a.  The function of the TF coils is described in WBS 132.

1.2 Ohmic Heating Coils

a.  The Ohmic Heating solenoid's primary function is to initiate and increase the discharge current and permit heating within the plasma by ohmic effects .

1.3  Shaping Coils

a.  The function of the PF coils is described in WBS 131.

1.4  Centerstack Casing

a.  The centerstack casing functions as the internal vacuum and thermal barrier and provides support for the centerstack coils and inner wall armor.

2.0 Configuration Requirements & Essential Features

a.  The centerstack radial build shall be comprised of the TF inner legs located in the core of the centerstack surounded surrounded by the OH-1 solenoid.  The upper and lower turns of OH-1 are surrounded by three pairs of coils designated as OH-2, and PF 1a/1b  These pairs shall be symmetric about the horizontal mid-plane of the NSTX device.

2.1  TF Inner Legs

a.  The inner legs of the TF coils shall consist of tightly-nested fully-bonded copper coils.

b.  The inner legs of the TF coils shall be water cooled.  Cooling water shall be supplied by WBS 32.

2.2  Ohmic Heating Coils

a.  Three OH coils, located in the centerstack assembly shall provide the Ohmic heating (OH) fields.

b. The OH #1 solenoid shall consist of layered copper conductors surrounding the TF inner legs.

c.  The OH #2 solenoid shall consist of one coil pair.  Each coil shall be comprisd comprised of independently wound, copper conductors.

d.  The OH #2 solenoid shall be fitted over the outside of the OH #1 solenoid coil in the centerstack assembly.

e.  The OH coils shall be water cooled.  Cooling water will be supplied by WBS 32.

2.3  Shaping Coils

a.  The shaping coils shall consist of upper and lower coil pairs; PF 1a and 1b.

b.  Each coil shall be comprisd comprised of independently wound, copper conductors

c.  PF1s, upper and lower, will be fitted over the OH-2 solenoid coil in the centerstack assembly.  PF1b, upper and lower, will be fitted over PF1a.

d.  PF-1a/1b shall be water cooled.  Cooling water will be supplied by WBS 32.

2.4   Centerstack Casing

a.  The centerstack casing shall be electrically isolated from the outer vacuum vessel.

b.  The centerstack casing assembly shall be compatible with operation in high vacuum conditions. (R11)

c.  All centerstack casing flanges shall provide a vacuum seal compatible with high vacuum conditions.

d.  An electrical break shall be provided between the vacuum vessel and the centerstack casing to support helicity injection (CHI) during startup.

3.0 Performance & Operational Requirements

3.1 Baseline Operation

3.1.1  TF Inner Legs

a. The baseline design of the inner legs of the TF coils shall be capable of supplying the toroidal fields as described in Table CS-3.1-1.

Table CS-3.1-1  TF Inner Leg Baseline Performance

| Pulse Duration (sec) | ITF (Max) (kA) | BTF (R=8085.4cm) (kG) |
| --- | --- | --- |

| 0.6 | 34.5 | 6.46.0 |
| --- | --- | --- |

| 5.0 | 17.7 | 3.23.0 |
| --- | --- | --- |

b.  For baseline operations the inner TF conductors will be inertially cooled.  The initial temperature of the coils shall be 20°C.

c.  The maximum temperature of the TF conductor insulation shall be < 120°C during pulsing.

d.  The coils shall be capable of being recooled to the initial temperature in < 5 minutes using the existing cooling water systems (WBS 32).

e. The TF coils shall be compatible with NSTX operation with plasma current in either φ direction, with toroidal field in either φ direction.

f. The peak coil terminal voltage anticipated in the TF system is 1784 volts. Although the grounding system should limit the voltage to ground to 1/2 of this value, the coil design should anticipate the full value. The TF coil system shall be designed to accommodate a DC hipot level of at least 2*1784+1000 = 4.6kV to ground (R10).

g. The maximum voltage which will be impressed on the PF1 coils is 1200V. Although the grounding system should limit the voltage to ground to 1/2 of this value, the coil design should anticipate the full value. Therefore the PF1 coil system shall be designed to accommodate a DC hipot level of at least 2*1200+1000 = 2.5kV to ground. Depending on the ground plane potential in the center stack of NSTX (which may be influenced by CHI operation), the ground insulation in the center stack may have to be designed for a voltage higher than that due simply to the power supply system (R12).

3.1.2  Ohmic Heating Coils

a.  The ohmic heating solenoid shall be capable of operation with an IOH2∆t of 200 kA2-sec

at 20 kA.

b.  The OH coils will be cooled with water at an inlet temperature of 20°C.

c.  The maximum temperature of the OH conductor insulation shall be < 120°C during pulsing.

d.  The coils shall be capable of being recooled to 20°C in < 10 minutes, such that an indefinite number of pulses can be performed with a repetition period of 10 minutes.

e. The maximum voltage which will be impressed on the OH circuit is 4kV. Although the grounding system should limit the voltage to ground to 1/2 of this value, the coil design should anticipate the full value. Therefore the OH coil system shall be designed to accommodate a DC hipot level of at least 2*4000+1000 = 9kV to ground. Depending on the ground plane potential in the center stack of NSTX (which may be influenced by CHI operation), the ground insulation in the center stack may have to be designed for a voltage higher than that due simply to the OH power supply system (R11).

3.1.3  Shaping Coils

a. The PF-1a/1b coils shall be capable of supplying the current scenarios established by the NSTX Project Physics Group, consistent with the requirements for start up field null quality and loop voltage generation, as well as plasma equilibrium control.

b.  The PF-1a/1b coils shall be capable of operation with a maximum current of 5 10.0kA for a maximum equivalent square wave of 5 seconds.

c. The PF-1a/1bcoils shall be compatible with NSTX operation with plasma current in either φ direction, with toroidal field in either φ direction.

d.  The maximum temperature of the PF-1a/1b conductor insulation shall be < 120°C during pulsing.

3.1.4   Centerstack Casing

a.  The centerstack casing shall provide the load path to the coil support structure to react the EM loads of the OH-2 and shaping coils.

b.  The centerstack casing shall be capable of handling the heat load resulting from the 350°C bakeout of the plasma facing components. (R9)

c.  The centerstack casing shall provide a thermal barrier sufficient to limit the maximum temperature of the coil insulation to < 160°C.

3.2 Upgrades

For the NSTX long pulse upgrade the entire centerstack assembly will be replaced.

3.2.1  TF Inner Legs

a.  For the long pulse upgrade the baseline TF inner legs will be replaced.  The upgrade TF inner legs shall be actively using cooling water supplied by WBS 32.

3.2.2  Ohmic Heating Solenoid

a.  The ohmic heating solenoid is not required for the long pulse upgrade.

3.2.3  Shaping Coils

a.  Long pulse upgrade requirements for PF coils 1a/1b shall be satisfied in the baseline design.

3.2.4   Centerstack Casing

a.  There are no additional upgrade requirements for the centerstack casing.

APPENDIX

| # | Description | Value | Units | Reference |
| --- | --- | --- | --- | --- |

| 1 | Seismic Loads | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 4.2 |
| --- | --- | --- | --- | --- |

| 2 | PF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.1.a and .b |
| --- | --- | --- | --- | --- |

| 3 | PF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.1.c |
| --- | --- | --- | --- | --- |

| 4 | PF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.1.d |
| --- | --- | --- | --- | --- |

| 5 | PF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.1.f |
| --- | --- | --- | --- | --- |

| 6 | PF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.1.e |
| --- | --- | --- | --- | --- |

| 7 | TF GRD Requirement | n.a. | n.a. | NSTX-RQMTS-GRD r0, sec. 1.2.3.2.1.b |
| --- | --- | --- | --- | --- |

| 8 | Base Partial Pressure Requirement | 1x10-7 | torr | NSTX-RQMTS-GRD r0, sec. 2.3.1.a |
| --- | --- | --- | --- | --- |

| 9 | Bakeout Temperature | 350 | °C | NSTX-RQMTS-GRD r0, sec. 2.3.2.a |
| --- | --- | --- | --- | --- |

| 10 | TF Coil Insulation Hipot Req't | 4.6 | kVDC | NSTX-SDD-5X sec. TF-3.1.c |
| --- | --- | --- | --- | --- |

| 11 | OH Coil Insulation Hipot Req't | 9.0 | kVDC | NSTX-SDD-5X sec. PF-3.1.c |
| --- | --- | --- | --- | --- |

| 12 | PF Coil Insulation Hipot Req't | 2.5 | kVDC | NSTX-SDD-5X sec. PF-3.1.d |
| --- | --- | --- | --- | --- |