National Spherical Tokamak Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

HEATING & CURRENT DRIVE SYSTEMS

NSTX-SRD-2X-020

Revision 0

May 13, 1996

________________________________

J. Randall Wilson

WBS 2 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

INTRODUCTION	1

HEATING & CURRENT DRIVE SYSTEMS	1

1.0 Functions	1

1.1 Basic functions of Heating&Current Drive (H&CD) Systems	1

2.0 Configuration Requirements & Essential Features	1

2.1 General Configurational Requirements	1

3.0 Performance & Operational Requirements	1

3.1 Baseline Operation	2

3.2 Upgrades	2

ECH Subsystem	4

1.0 Functions	4

2.0 Configuration Requirements & Essential Features	4

3.0 Performance & Operational Requirements	4

3.1 Baseline Performance Requirements	4

3.1.1 In Vessel Components	4

3.1.2 Vacuum Windows	5

3.1.3 Transmission System	5

3.1.4 RF source	5

3.1.5 DC power supply	5

3.1.6 Design Loads	5

3.1.7 Load Combinations	6

3.2 Baseline Operational Requirements	6

3.2.1 Pulse Length	6

3.2.2 Pulse Repetition Length	6

3.2.3 Annual Operating limits	6

3.2.4 Lifetime Operating Limits	6

3.2.5 Neutron Source Rates/Yields	6

3.3 Reliability Requirements	6

3.3.1 Mission reliability	6

3.3.2 Operational Availability	6

3.3.3 Shot Reliability	7

3.3.4 Maintainability	7

3.4.5 Scheduled Maintenance	7

3.5 Upgrades	7

HHFW Subsystem	7

1.0 Functions	7

2.0 Configuration Requirements and Essential Features	7

3.0 Performance and Operational Requirements	8

3.1 Baseline Performance Requirements	8

3.1.1 Vacuum qualification and conditioning.	8

3.1.2 Safety Requirements.	8

3.1.3 Electrical Requirements.	8

3.1.4 Material requirements.	9

3.1.5 Magnetic Permeability.	9

3.2 Baseline Operational Requirements	9

3.2.1 RF conditioning.	9

3.2.2 Plasma operation.	9

3.2.3 Diagnostic and control requirements	10

3.2.4 Design Loads.	10

3.3 Reliabilty Requirements	11

3.3.1 Mission reliability	11

3.3.2 Operational availability	11

3.3.3 Shot reliability	11

3.3.4 Maintainability	11

3.4 Upgrades	11

CHI Subsystem	11

1.0 Function	11

2.0 Configuration Requirements & Essential Features	12

3.0 Performance &Operational Requirements	12

3.1 Baseline Performance Requirements	12

3.1.1 In Vessel Components	12

3.1.2 Bus requirements	12

3.2 Baseline Operational Requirements	12

3.3 Reliability Requirements	12

3.4 Upgrades	12

NBI Subsystem	13

1.0 Function	13

2.0 Configuration Requirements & Essential Features	13

3.0 Performance & Operational Requirements	13

3.1 Baseline Requirements	13

3.1.1 Test cell floor space	13

3.1.2 Port allocation	13

3.2 Upgrade Requirements	13

List of Tables

HCD-3.1-1	Baseline NSTX Parameters	2

HCD-3.2-1	Upgrade NSTX Partameters	2

ECH-3.2-1	ECH Requirements	6

HHFW-3.2-1	HHFW Requirements	9

CHI-3.2-1	CHI Requirements	12

NBI-3.2-1	NBI Requirements	13

List of FIGURES

APPENDIX

LIST OF REFERENCES

INTRODUCTION

This System Requirements Document (SRD) defines the requirements and design of the Heating and Current Drive Systems (WBS 02) of the National Spherical Tokamak Experiment (NSTX). These systems are presently envisioned to consist of High Harmonic Fast Wave Heating (HHRW), Electron Cyclotron Heating (ECH), Neutral Beam Injection (NBI) and Coaxial Helicity Injection (CHI). The baseline systems include HHFW for heating and current drive (CD), ECH for plasma breakdown and densification and CHI for start-up assist. The NBI systems will not be in place for baseline operations but provisions for future NBI installation will be made (space, utilities etc.)

WBS 02 -  HEATING & CURRENT DRIVE SYSTEMS

1.0 Functions

1.1 Basic functions of Heating&Current Drive (H&CD) Systems

a.  Heating:  All four of the H&CD systems provide energy input into the plasma either directly or as a consequence of providing momentum  Thsi energy input ios required to increase and sustain the thermal energy of the plasma.

b. Current Drive: All four of the systems may be called on to provide current drive in the NSTX plasmas. This current drive may be usefull for ramping up the current at the beginning of a discharge, maintaining the flat-top current, or providing radilly localized currents for current profile control. These systems are in addition to the Ohmic current (OH) system described in WBS 13.

c.  Breakdown: These systems may also be used for plasma initiation, breakdown of the prefill gas and preheating to make the OH system more efficient.

2.0 Configuration Requirements & Essential Features

2.1 General Configurational Requirements

a. All four systems require space inside the vacuum vessel, at the vacuum vessel interface on the floor of the torus hall and in auxiliary rooms.

b. The systems consist of power supplies, a device that transforms the power into either rf, dc or neutral particles, a transmission system and a launching device for coupling the power to the plasmas.

c. The power supplies are located in separate rooms and power yards, the transforming devices may be either in separate rooms or the torus hall, the transmission system connects from the tranforming unit to the vacuum vessel and the launching element is either at or just inside the vacuum vessel.

3.0 Performance & Operational Requirements

a. The H&CD Systems will satisfy the perfromance requirements specified in the NSTX General Requiremnets Document (GRD), and as specifically addressed below.

b. Performance requirements would be, for example, maximum current, voltage, power. etc.

c. Operational requirements are similar to performance requirements except refer mainly to things like duty cycle, number of pulses, etc.

3.1 Baseline Operation

a. Baseline operation includes ECH for plasma breakdown and densification, HHFW for plasma heating and CHI for current ramp-up assist.

Table HCD-3.1-1: Baseline NSTX Parameters

3.2 Upgrades

a. In the upgrade phase all four systems will be excersized to full capabilities.

.c.Table HCD-3.2-1 Upgrade NSTX Parameters

| Parameter | Value | Units | Ref |
| --- | --- | --- | --- |

| Upgrade Maximum Plasma Duration | 60 | sec |  |
| --- | --- | --- | --- |

| Upgrade Minimum Repition Rate | ? |  |  |
| --- | --- | --- | --- |

| Duty Factor | ? |  |  |
| --- | --- | --- | --- |

| Maximum ECH pulse length | 100 | msec |  |
| --- | --- | --- | --- |

| Maximum HHFW pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum CHI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum NBI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum ECH power | 200 | kW |  |
| --- | --- | --- | --- |

| Maximum HHFW power | 6 | MW |  |
| --- | --- | --- | --- |

| Maximum CHI Voltage | tbd |  |  |
| --- | --- | --- | --- |

| Maximum CHI current | tbd |  |  |
| --- | --- | --- | --- |

| Maximum NBI power | 5 | MW |  |
| --- | --- | --- | --- |

WBS 21 -  ECH Subsystem

1.0 Functions

The electron cyclotron heating (ECH) system consists of a microwave source and power supply, a transmission system, a vacuum interface and a launching structure along with suitable diagnostics and controls. The functions of the ECH system are to :

• provide pre ionization for plasma breakdown,

• provide power for ionization and heating during start-up (~100 ms)

• provide possible plasma heating and/or current drive during discharge flat-top.

2.0 Configuration Requirements & Essential Features

The ECH system will require one or more in vessel launchers which direct the microwave radiation toward the desired absorption location. The launchers must be compatible with in vessel component requirements. Each launcher will have a vacuum window connecting it to the external transmission system. This window must have low electrical loss at microwave frequencies and satisfy the thermal and mechanical requirements for vacuum interfaces. In addition, this window may require cooling for long pulse operation. The transmission system must efficiently transport the microwave power from the rf sources to the vacuum window. It will consist of a waveguide and special components  for power measurement. The rf sources will be located in a separate room and will provide the microwave power. They will include local controls and interlocks. The DC power for the rf sources will provided from the power yard.

3.0 Performance & Operational Requirements

3.1 Baseline Performance Requirements:

The ECH system design shall satisfy the performance requirements specified in the NSTX General Requirements Document (GRD) and as specifically addressed below.

3.1.1 In Vessel Components:

Materials Requirements

The materials to be used for the in-vessel components shall be limited based on vacuum, impurity control, magnetic permeabilty and activation considerations.

- High Vacuum qualification

All materials exposed to the high vacuum within the plasma chamber must be approved by the project for high vacuum compatibility

- Magnetic Permeability

Materials used for the in-vessel components shall be non-magnetic with a relative permeability at or below 1.02. Exceptions must have project approval.

- Bakeout

Materials used for the in-vessel components must be compatible with the bakeout scenarios described in the GRD.

- Conditioning

All materials used for the in-vessel components must be compatible with glow discharge cleaning and boronization as described in the GRD

3.1.2 Vacuum Windows

The ECH vacuum windows shall satisfy all requirements of NSTX vacuum windows as spelled out in the GRD

In addition the windows shall be composed of materials with low absorption at microwave frequencies and be capable of long pulse operation

3.1.3 Transmission System

The transmission system will be electrically isolated from the vessel as specified in the GRD.

The system will have low transmission loss TBD

The system will be designed to satisfy the design loads

3.1.4 RF source

The rf source shall capable of delivering TBD kW for TBD seconds.

It shall be interlocked for test cell entry per the GRD.

It shall be provided with source protection controls, local controls and an interface to the general machine controls.

3.1.5 DC power supply

The DC power supply shall provide the high voltage (V TBD I TBD) required by the rf source.

3.1.6 Design Loads

• Dead Load

The dead load includes the weight of the various transmission components.

• Pressure Loads

The vacuum window shall be able to withstand a pressure load of TBD psi.

• Thermal Loads

The launcher assemble shall be designed to withstand a heat load of TBD associated with full power plasma operation.

• Electromagnetic Loads

Electromagnetic loads induced during normal operation of the device.

• Electromagnetic Loads During Faults

The electromagnetic loads during abnormal operations including control failures, power supply failures, bus faults, or magnetic faults (plasma disruptions).

• Seismic Loads

The transmission system shall be designed to retain its structural integrity during a TBD

3.1.7 Load Combinations

TBD

3.2 Baseline Operational Requirements

The ECH system design shall satisfy the operational requirements specified in the NSTX General Requirements Document (GRD) and as specifically addressed below:

3.2.1 Pulse Length

• The ECH system shall be designed to deliver  TBD kW for TBD s.

3.2.2 Pulse Repetition Length

The ECH system shall be capable of delivering pulses with a TBD duty factor.

3.2.3 Annual Operating limits

The ECH system design shall allow for the following operational limits in any year

- TBD pulses

- TBD s of total operation

3.2.4 Lifetime Operating Limits

The ECH system shall be designed for a TBD year operating life with the above yearly limits.

3.2.5 Neutron Source Rates/Yields

The test cell ECH system components shall be designed for the neutron source rates/yields as specified in the GRD.

Table ECH 3.2-1 ECH Requirements

| parameter | value | units | reference |
| --- | --- | --- | --- |

| primary power required |  |  |  |
| --- | --- | --- | --- |

| aux. power required |  |  |  |
| --- | --- | --- | --- |

| water required |  |  |  |
| --- | --- | --- | --- |

3.3 Reliability Requirements

3.3.1 Mission reliability

TBD

3.3.2 Operational Availability

TBD

3.3.3 Shot Reliability

TBD

3.3.4 Maintainability

TBD

3.4.5 Scheduled Maintenance

TBD

3.5 Upgrades

The ECH system is identical in the upgrade period

WBS 22 -  HHFW Subsystem

1.0 Functions

The High Harmonic Fast Wave Heating and Current Drive system consists of the rf generators, transmission lines, tuning and matching systems, rf feedthroughs and internal transmission lines, antennas with Faraday shields and protective limiters, and the associated diagnostic and control systems. The functions of the High Harmonic Fast Wave (HHFW) system are to:

• Provide electron heating power to the NSTX plasma, via the fast magnetosonic wave.

• Provide noninductive current drive.

• Provide control over the heating / current drive deposition profile, through the antenna phasing.

2.0 Configuration Requirements and Essential Features

The HHFW system wil utilize twelve stripline antennas mounted on the vacuum vessel wall in the gap between the upper and lower passive stabilizing plates. The antennas will be toroidally located between the NSTX vacuum vessel midplane ports. Each antenna will consist of a current strap, a backplane, a Faraday shield, and a local (bumper) limiter

structure. The antennas will be bakeable. The antenna current straps will be actively cooled. Limiter and Faraday shield cooling TBD.

Each antenna will be fed by a single coaxial rf vacuum feedthrough, mounted on either the upper or lower ports on the lateral wall of the vacuum vessel. Short internal runs of coaxial or shielded stripline transmission line will connect the feedthroughs to the antennas. The feedthrough and internal transmission line will be bakeable. Short runs of transmission line external to the vacuum vessel will be supported from the NSTX structure; the remainder of the transmission line runs will be mechanically isolated from the feedthrough  through bellows assemblies. Each antenna strap will be excited in parallel with a second strap.  Each pair of antenna straps will be impedance matched using a line stratcher and stub, and fed by one of the six rf generators.

The six (existing) generators will operate at no more than three frequencies (minimum of four adjacent straps at each frequency). Operating frequencies will be in the range of 30 - 50 MHz. All rf generators operating at any single frequency will be phase locked to provide control over the wavenumber spectrum excited in the plasma at that frequency. Amplitude modulation control and feedback is required for generator operation. The existing phase, amplitude modulation, and feedback controls  are adequate for the NSTX requirements. The generator controls are and shall remain local to the generators. DC power for the rf sources exists, and is supplied from the power yard.

3.0 Performance and Operational Requirements

The HHFW system design shall satisfy the performance requirements specified in the NSTX General Requirements Document (GRD), and as specifically addressed below.

3.1 Baseline Performance Requirements

3.1.1 Vacuum qualification and conditioning.

• The antenna,  internal transmission line, and feedthrough construction shall be consistent with good vacuum practices.

• The antenna, internal transmission line, and feedthrough assemblies shall tolerate glow discharge cleaning of the NSTX vessel and limiters.

• The current straps,  backplanes,  Faraday shields, internal transmission lines and rf feedthroughs will be bakeable to the same temperature as the outboard vacuum vessel wall  -oC.

• The bumper limiters will be bakeable to 300oC (or whatever bakeout temperature is specified for the carbon tiles interior to the vacuum vessel, as per the GRD).

• The antenna, transmission line, and feedthrough assemblies will be compatible with boronization.

3.1.2 Safety Requirements.

• DC shorting stubs will be incorporated in the transmission line prior to the point where the line exits the test cell.

• A suitable kirk key lockout system, as specified in the GRD,  will be installed to prevent rf power from being applied to components in the test cell when the test cell is accessible.

3.1.3 Electrical Requirements.

• The antenna, feedthrough, and internal transmission line construction shall be consistent with good electrical practices at radio frequencies and high RF voltages.

• The antenna backplane with attached Faraday shield shall be electrically connected to the vacuum vessel wall via a low inductance ground.

• The transmission line will be electrically isolated from the vacuum vessel as specified in the GRD.

3.1.4 Material requirements.

The in-vessel materials used will be consistent with the vacuum, impurity, magnetic permeability, and activation requirements for NSTX as detailed in the GRD. In particular, the antenna and bumper limiters are plasma facing components. The plated Faraday shield assembly will be overcoated with a suitable material.

3.1.5 Magnetic Permeability.

Materials used in the antenna and feedthrough assemblies will have permeability at or below the specified limit in the GRD.

3.2 Baseline Operational Requirements

3.2.1 RF conditioning.

• The HHFW system shall permit vacuum conditioning of the antenna at pulse lengths of up to 5 sec.

• Cooling will be sufficient to maintain strap, Faraday shield, and limiter temperature at or below TBDoC  during conditioning.

• RF power supplied to the antennas during vacuum conditioning will be sufficient to bring the antennas to a voltage of 40 kV.

3.2.2 Plasma operation.

• The HHFW system shall deliver at least 6 MW of power to the NSTX plasma, under operating conditions TBD.

• The HHFW system shall provide 500 kA (?TBD?) of noninductive current drive in the NSTX plasma, under operating conditions TBD.

• Strap cooling will be sufficient to maintain strap temperature at or below TBDoC during operation.

• Faraday shield and limiter temperature limits during operation TBD.

• Pulse capability of the HHFW system will be 5 seconds in the baseline program.

• Pulse repetition rate is as specified in the GRD.

• Annual and lifetime operating limits are as specified in the GRD.

Table HHFW- 3.2-1Baseline HHFW Requirements

| parameter | value | units | ref |
| --- | --- | --- | --- |

| primary power required |  |  |  |
| --- | --- | --- | --- |

| aux. power required |  |  |  |
| --- | --- | --- | --- |

| water required |  |  |  |
| --- | --- | --- | --- |

3.2.3 Diagnostic and control requirements

• Diagnostics, signal detection, and data recording for antenna strap current, voltage, and relative phase will be included in the HHFW system.

• Diagnostics, signal detection and data recording for forward power, reverse power, and phase will be included in the HHFW system.

• Real-time (preprogrammed) modification of the relative antenna current strap phasing will be accomodated.

• Preprogrammed and feedback modification of the rf power will be accomodated.

3.2.4 Design Loads.

• Dead load.

The dead load internal to the vacuum vessel includes the weight of the antenna assembly and bumper limiters.  The dead load external to the vacuum vessel, including those sections of transmission line connected to the feedthrough and isolated from the remainder of the line by the mechanical isolation bellows, shall be adequately taken up by external supports from the NSTX structure. The dead load for the remainder of the transmission  line run includes tthe ransmission line itself with associated components including tees, decoupling loops,  stubs, and line stretchers.

• Pressure loads.

The rf vacuum feedthrough will withstand a pressure of TBD psi.

•  Thermal loads.

The bumper limiter assembly will accomodate the heat load specified for plasma facing components in the GRD. The Faraday shield will accomodate the heat load associated with line-of-site radiation and neutral particle flux from the plasma, along with rf losses at a level TBD.

• Electromagnetic loads.

Electromagnetic loads induced during normal operation of the device.

• Electromagnetic loads during faults.

The electromagnetic loads induced during abnormal operating events such as plasma control failures, power supply failures, bus opens or shorts, or magnetic faults shall be included in the design and associated disruption loads.

• Disruption loads.

The antenna assembly and bumper limiters shall accomodate disruptive loading of the character and frequency defined in the GRD.

• Seismic loads.

The antenna, limiter, tuning and matching, and transmission line system shall be designed to retain its structural integrity during the Most Intense Earthquake (MIE).

3.3 Reliabilty Requirements

3.3.1 Mission reliability

- TBD.

3.3.2 Operational availability

- TBD.

3.3.3 Shot reliability

- TBD.

3.3.4 Maintainability

• The availability of in-vessel maintenance for the in-vessel components of the HHFW system is presumed.

• Scheduled maintenance requirements shall not exceed one week per three weeks operating period (?)

3.4 Upgrades

The pulse length shall be increased to 60 secs for the upgrade period

WBS 22 -  CHI Subsystem

1.0 Function

The CHI system is designed to allow for non-inductive start-up, a feature that would be attractive in a ULART reactor and to provide edge current drive for long pulse operation if it proves needed.

2.0 Configuration Requirements & Essential Features

The CHI system relies on toroidal breaks that electrically isolate the center stack from the outer vacuum vessel. This break is provided by two ceramic spool pies incorporated into the vacuum vessel design. Bus connections to the two sides of the breaks are needed to bring the energy from the CHI power supply which will consist of a capacitor bank for start-up assist (baseline program) and a rectifier for long pulse edge current sustainment (upgrade).

3.0 Performance &Operational Requirements

3.1 Baseline Performance Requirements

3.1.1 In Vessel Components

The ceramic break material used must be compatible with the NSTX vacuum requirements and the bakeout requirements. Integration of the break will take place in WBS 12.

The break must be able to stand of a voltage of tbd kV.

3.1.2 Bus requirements

The external bus and connections must be capable of handling the voltage and current requirements of the system for the pulse lengths and duty factors required for baseline operation. Consideration should be given to possible simple allowances for upgrade operation such as bus cooling.

3.2 Baseline Operational Requirements

The operational requiremnts are summarized in the table below

Table CHI-3.2-1 CHI Baseline Requirements

| Parameter | value | units | reference |
| --- | --- | --- | --- |

| CHI max voltage |  |  |  |
| --- | --- | --- | --- |

| CHI max current |  |  |  |
| --- | --- | --- | --- |

| CHI pulse length |  |  |  |
| --- | --- | --- | --- |

| CHI duty factor |  |  |  |
| --- | --- | --- | --- |

3.3 Reliability Requirements

3.4 Upgrades

The CHI upgrade system will include an additional steady state power supply that will allow long pulse edge current drive

WBS 24 -  NBI Subsystem

1.0 Function

The NBI system is required for plasma heating and current drive as well as plasma rotation.

2.0 Configuration Requirements & Essential Features

The NBI sysytem will consist of up to four NBI injectors installed in the corners of the test cell and firing through ports B, F, H, and L. In vessel armor will be needed for the beam shine through strike points. Exact beam aiming angles will be determined by physics considerations and strike point compromises. The beam systems will require substantiual auxiliary services, electrical power, cryogenics, water, and vacuum.

3.0 Performance & Operational Requirements

3.1 Baseline Requirements

NBI is not part of the baseline program, however some requirements are imposed to ensure that NBI can be incorporated easily as an  upgrade

3.1.1 Test cell floor space

Adequate floor space to accomodate the NBI boxes shall be laid out in the four corners of the room.

3.1.2 Port allocation

Four ports are required for the neutral beam injection. In order to fit the injectors in the test cell these ports will be B,F H and L.

3.2 Upgrade Requirements

Table NBI-3.2-1 NBI Requirements

| parameter | value | units | ref |
| --- | --- | --- | --- |

| max beam voltage |  |  |  |
| --- | --- | --- | --- |

| max beam current |  |  |  |
| --- | --- | --- | --- |

| water requirements |  |  |  |
| --- | --- | --- | --- |

| aux electrical requirements |  |  |  |
| --- | --- | --- | --- |

| vacuum requirements |  |  |  |
| --- | --- | --- | --- |

| primary electrical reqs. |  |  |  |
| --- | --- | --- | --- |

APPENDIX