National Spherical Tokamak Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

HEATING & CURRENT DRIVE SYSTEMS

NSTX-SRD-2X-020

Revision 0

July 11, 1996

________________________________

J. Randall Wilson

WBS 2 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

INTRODUCTION	1

HEATING & CURRENT DRIVE SYSTEMS	1

1.0 Functions	1

1.1 Basic functions of Heating&Current Drive (H&CD) Systems	1

2.0 Configuration Requirements & Essential Features	1

2.1 General Configurational Requirements	1

3.0 Performance & Operational Requirements	1

3.1 Baseline Operation	1

3.2 Upgrades	2

ECH Subsystem	3

1.0 Functions	3

2.0 Configuration Requirements & Essential Features	3

2.1.1 General requirements	3

2.1.2 In-Vessel Components	3

2.1.3 Vacuum Windows	4

2.1.4 Transmission System	4

2.1.5 RF source	4

2.1.6 DC power supply	4

3.0 Performance & Operational Requirements	4

3.1 Baseline Performance Requirements	4

3.1.1 Mechanical Design Loads	4

3.1.2 Load Combinations	5

3.2 Baseline Operational Requirements	5

3.3 Upgrades	5

HHFW Subsystem	5

1.0 Functions	5

2.0 Configuration Requirements and Essential Features	5

3.0 Performance and Operational Requirements	6

3.1 Baseline Performance Requirements	6

3.1.1 Vacuum qualification and conditioning.	6

3.1.2 Safety Requirements.	6

3.1.3 Electrical Requirements.	6

3.1.4 Material requirements.	7

3.2 Baseline Operational Requirements	7

3.2.1 RF conditioning.	7

3.2.2 Plasma operation.	7

3.2.3 Diagnostic and control requirements	7

3.2.4 Design Loads.	7

3.3 Upgrades	8

CHI Subsystem	8

1.0 Function	8

2.0 Configuration Requirements & Essential Features	9

3.0 Performance &Operational Requirements	9

3.1 Baseline Performance Requirements	9

3.1.1 In Vessel Components	9

3.1.2 Bus requirements	9

3.2 Baseline Operational Requirements	9

3.4 Upgrades	9

NBI Subsystem	9

1.0 Function	9

2.0 Configuration Requirements & Essential Features	9

3.0 Performance & Operational Requirements	10

3.1 Baseline Requirements	10

3.1.1 Test cell floor space	10

3.1.2 Port allocation	10

3.2 Upgrade Requirements	10

List of Tables

HCD-3.1-1	Baseline NSTX Parameters	2

HCD-3.2-1	Upgrade NSTX Partameters	2

List of FIGURES

APPENDIX

LIST OF REFERENCES

INTRODUCTION

This System Requirements Document (SRD) defines the requirements and design of the Heating and Current Drive Systems (WBS 02) of the National Spherical Tokamak Experiment (NSTX). These systems are presently envisioned to consist of High Harmonic Fast Wave Heating (HHRW), Electron Cyclotron Heating (ECH), Neutral Beam Injection (NBI) and Coaxial Helicity Injection (CHI). The baseline systems include HHFW for heating and current drive (CD), ECH for plasma breakdown and densification and CHI for start-up assist. The NBI systems will not be in place for baseline operations but provisions for future NBI installation will be made (space, utilities etc.)

WBS 02 -  HEATING & CURRENT DRIVE SYSTEMS

1.0 Functions

1.1 Basic functions of Heating&Current Drive (H&CD) Systems

a. Heating:  All four of the H&CD systems provide energy input into the plasma either directly or as a consequence of providing momentum  This is energy input is required to increase and sustain the thermal energy of the plasma.

b. Current Drive: All four of the systems may be called on to provide current drive in the NSTX plasmas. This current drive may be usefull for ramping up the current at the beginning of a discharge, maintaining the flat-top current, or providing radilly localized currents for current profile control. These systems are in addition to the Ohmic current (OH) system described in WBS 13.

c. Breakdown: These systems may also be used for plasma initiation, breakdown of the prefill gas and preheating to make the OH system more efficient.

2.0 Configuration Requirements & Essential Features

2.1 General Configurational Requirements

a. All four systems require space inside the vacuum vessel, at the vacuum vessel interface on the floor of the torus hall and in auxiliary rooms.

b. Each system consists of power supplies, devices that transform the power into either rf, dc or neutral particles, transmission systems, and launching devices for coupling the power to the plasma.

c. The power supplies are located in separate rooms and power yards, the transforming devices may be either in separate rooms or the torus hall, the transmission system connects from the tranforming unit to the vacuum vessel and the launching element is either at or just inside the vacuum vessel.

3.0 Performance & Operational Requirements

3.1 Baseline Operation

a. Baseline operation includes ECH for plasma breakdown and densification, HHFW for plasma heating and CHI for current ramp-up assist.

Table HCD-3.1-1: Baseline NSTX Parameters

b. The Heating Systems shall be designed for a pulse repetition period of 300 seconds and a maximum of 100 pulses per 24 hour period [GRD 2.1.2.3].

c. The Heating Systems shall be designed to operate for a period of 10 years [GRD 2.1.2.3].

d. The Heating Systems operation shall be assumed to consist of 100 pulses per day, 5 days per week, 3 weeks per month, 9 months per year, for 10 years (total of 135,000 pulses). Non-operating times are available for maintenance [GRD 2.1.2.3].

3.2 Upgrades

a. Steady state CHI by adding a sustained power supply

b. Adding NBI injection.

c. increaing the pulse duration to 60 seconds while maintaining the 1/300 duty factor.

.c.Table HCD-3.2-1 Upgrade NSTX Parameters

| Parameter | Value | Units | Ref |
| --- | --- | --- | --- |

| Upgrade Maximum Plasma Duration | 60 | sec |  |
| --- | --- | --- | --- |

| Upgrade Minimum Repition Rate | 3600 | sec |  |
| --- | --- | --- | --- |

| Maximum ECH pulse length | 100 | msec |  |
| --- | --- | --- | --- |

| Maximum HHFW pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum CHI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum NBI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum ECH power | 200 | kW |  |
| --- | --- | --- | --- |

| Maximum HHFW power | 6 | MW |  |
| --- | --- | --- | --- |

| Maximum CHI current (external) | 20 | kA |  |
| --- | --- | --- | --- |

| Maximum CHI current (plasma) | 200 | kA |  |
| --- | --- | --- | --- |

| Maximum NBI power | 5 | MW |  |
| --- | --- | --- | --- |

WBS 21 -  ECH Subsystem

1.0 Functions

The electron cyclotron heating (ECH) system consists of a microwave source and power supply, a transmission system, a vacuum interface and a launching structure along with suitable diagnostics and controls. The functions of the ECH system are to :

• provide pre ionization for plasma breakdown,

• provide power for ionization and heating during start-up (~100 ms)

• provide possible plasma heating and/or current drive during discharge flat-top.

2.0 Configuration Requirements & Essential Features

2.1.1 General requirements

a. The ECH system shall utilize one in-vessel launcher which direct the microwave radiation toward the desired absorption location.

b. The ECH launchers shall be compatible with in-vessel component requirements, including glow discharge cleaning and bakeout.

c. Each launcher shall have a vacuum window connecting it to the external transmission system. This window shall have low electrical loss at microwave frequencies and satisfy the thermal and mechanical requirements for vacuum interfaces. In addition, this window may require cooling for long pulse operation.

d. The transmission system shall efficiently transport the microwave power from the rf sources to the vacuum window. It shall consist of a waveguide and special components  for power measurement.

e. The rf source shall be located outside of the test cell and will provide the microwave power. It shall include local controls and interlocks.

f. Switches shall be provided to isolate ECH power from the NSTX test cell. These switches shall be interlocked with the test cell access control system.

2.1.2 In-Vessel Components

a. All materials to be used in the tokamak and peripheral equipment (R ≤ 3.0 m, |Z| ≤ 3.0m) must have a relative magnetic permeability ≤ 1.02 unless otherwise authorized by the Project [GRD 1.1.4].

b. Materials used for the in-vessel components must be compatible with bakeout. The surfaces of the PFCs shall be heated during bakeout to 350°C, and the temperature of the inner VV surfaces shall be heated to 150°C [GRD 2.3.2]

c. All materials used for the in-vessel components must be compatible with glow discharge cleaning.

2.1.3 Vacuum Windows

a. The Vacuum Windows shall have low electrical loss at microwave frequencies and satisfy the thermal and mechanical requirements for vacuum interfaces [GRD 1.1.3].

2.1.4 Transmission System

a. The transmission system shall be electrically isolated from the vessel. Isolation shall be adequate for TBD kV.

2.1.5 RF source

a. The RF Source shall be interlocked to allow test cell entry

b.The RF Source shall be provided with source protection controls, local controls and an interface to the general machine controls.

2.1.6 DC power supply

a. The DC power supply shall provide the high voltage (V TBD I TBD) required by the rf source.

3.0 Performance & Operational Requirements

3.1 Baseline Performance Requirements:

3.1.1 Mechanical Design Loads

a. Mechanical structures shall be designed with due consideration of the following loads:

• Dead Load

The dead load includes the weight of the various transmission components.

• Pressure Loads

The vacuum window shall be able to withstand a pressure load of TBD psi.

• Thermal Loads

The launcher assemble shall be designed to withstand a heat load of TBD associated with full power plasma operation.

• Electromagnetic Loads

Electromagnetic loads induced during normal operation of the device.

• Electromagnetic Loads During Faults

The electromagnetic loads during abnormal operations including control failures, power supply failures, bus faults, or magnetic faults (plasma disruptions).

• Seismic Loads

The transmission system shall be designed to retain its structural integrity during a TBD

3.1.2 Load Combinations

TBD

3.2 Baseline Operational Requirements

a. The ECH system shall be designed to deliver  200 kW for 0.1 s.

3.3 Upgrades

The ECH system is identical in the upgrade period

WBS 22 -  HHFW Subsystem

1.0 Functions

The High Harmonic Fast Wave Heating and Current Drive system consists of the rf generators, transmission lines, tuning and matching systems, rf feedthroughs and internal transmission lines, antennas with Faraday shields and protective limiters, and the associated diagnostic and control systems. The functions of the High Harmonic Fast Wave (HHFW) system are to:

• Provide electron heating power to the NSTX plasma, via the fast magnetosonic wave.

• Provide noninductive current drive.

• Provide control over the heating / current drive deposition profile, through the antenna phasing.

2.0 Configuration Requirements and Essential Features

a. The HHFW system shall utilize RF power produced by the six existing Tokamak Fusion Test Reactor (TFTR) Ion Cyclotron Range of Frequencies (ICRF) sources. The sources and associated controls and interlocks shall be designed to accommodate shared usage between NSTX, and the Poloidal Beta Experiment- Modified (PBX-M).

b. The HHFW system shall utilize twelve antennae mounted on the vacuum vessel wall in the gap between the upper and lower passive stabilizing plates. The antennae shall be arranged toroidally between the NSTX vacuum vessel mid plane ports. Each antenna shall consist of a current strap, a back plane, a Faraday shield, and a local (bumper) limiter

structure. The antennae shall be compatible with bakeout and glow discharge modes of operation.

c. The set of six sources (and twelve antennae) shall be capable of operation in subsets with up to three different frequencies during an NSTX discharge. Operating frequencies shall be in the range of 30 - 50 MHz. All RF generators operating at any single frequency shall be phase locked to provide control over the wave number spectrum excited in the plasma at that frequency.

d. Switches shall be provided to isolate HHFW power from the NSTX test cell. These switches shall be interlocked with the test cell access control system.

3.0 Performance and Operational Requirements

The HHFW system design shall satisfy the performance requirements specified in the NSTX General Requirements Document (GRD), and as specifically addressed below.

3.1 Baseline Performance Requirements

3.1.1 Vacuum qualification and conditioning.

a. The antenna, internal transmission line, and feedthrough construction shall be consistent with good vacuum practices.

This really needs to be spelled out in the GRD. I and any vacuum enginner know what this means  in general ( no trapped volumes , relieved bolts and welds, proper materials, no lubricants (ideally)  but NSTX needs a formal ddefinition of what is acceptable and how exceptions will be made and this should be defined at a higher level since it affects several WBS's

b. The antenna, internal transmission line, and feedthrough assemblies shall be compatible with glow discharge cleaning of the NSTX vessel and limiters.

c. The current straps, backplanes,  Faraday shields, internal transmission lines and rf feedthroughs will be bakeable to the same temperature as the outboard vacuum vessel wall, namely 150oC.

d. The antenna protection limiters shall be bakeable to 350C

3.1.2 Safety Requirements.

a. DC shorting stubs shall be incorporated in the transmission line prior to the point where the line exits the test cell.

b. A suitable kirk key lockout system, shall be installed to prevent rf power from being applied to components in the test cell when the test cell is accessible.

3.1.3 Electrical Requirements.

a. The antenna, feedthrough, and internal transmission line construction shall be consistent with good electrical practices at radio frequencies and high RF voltages.

b. The antenna backplane with attached Faraday shield shall be electrically connected to the vacuum vessel wall via a low inductance ground.

c. The transmission line  shall be electrically isolated from the vacuum vessel with a voltage rating of TBD kV.

3.1.4 Material requirements.

a. The in-vessel materials used shall be consistent with the vacuum, impurity, magnetic permeability, and activation requirements for NSTX. In particular, the antenna and antenna protective limiters are plasma facing components. The Faraday shield assembly shall be overcoated with a suitable material.

3.2 Baseline Operational Requirements

3.2.1 RF conditioning.

a. The HHFW system shall permit vacuum conditioning of the antenna at pulse lengths of up to 5 sec.

b. Cooling shall be sufficient to maintain strap, Faraday shield, and limiter temperature at or below TBDoC  during conditioning.

c. RF power supplied to the antennas during vacuum conditioning shall be sufficient to bring the antennas to a voltage of 40 kV.

3.2.2 Plasma operation.

a. The HHFW system shall deliver at least 6 MW of power to the NSTX plasma, under operating conditions TBD.

b. Strap cooling shall be sufficient to maintain strap temperature at or below TBDoC during operation.

c. Faraday shield and limiter temperature limits during operation shall be limited to a maximum temperature of TBD oC

3.2.3 Diagnostic and control requirements

a. Diagnostics, signal detection, and data recording for antenna strap current, voltage, and relative phase  shall be included in the HHFW system.

b. Diagnostics, signal detection and data recording for forward power, reverse power, and phase shall be included in the HHFW system.

c. Real-time (preprogrammed) modification of the relative antenna current strap phasing  shall be accomodated.

d. Preprogrammed and feedback modification of the rf power shall be accomodated.

3.2.4 Design Loads.

a. Dead load.

The dead load internal to the vacuum vessel includes the weight of the antenna assembly and bumper limiters.  The dead load external to the vacuum vessel, including those sections of transmission line connected to the feedthrough and isolated from the remainder of the line by the mechanical isolation bellows, shall be adequately taken up by external supports from the NSTX structure. The dead load for the remainder of the transmission  line run includes tthe ransmission line itself with associated components including tees, decoupling loops,  stubs, and line stretchers.

b. Pressure loads.

The rf vacuum feedthrough shall  withstand a pressure of 30 psi. (from S. Raftopoulos)

Can you get a number from R Parsells or S Raftopoulos?

c.  Thermal loads.

The bumper limiter assembly shall accomodate a heat load of 1 MW/m^2. (per Peng) The Faraday shield will accomodate the heat load associated with line-of-site radiation and neutral particle flux from the plasma, along with rf losses at a level TBD.

I think that you need to do some legwork to find out the numbers (talk to Kaye/Peng) at least on a preliminary basis.

d. Electromagnetic loads.

Electromagnetic loads induced during normal operation of the device.

e. Electromagnetic loads during faults.

The electromagnetic loads induced during abnormal operating events such as plasma control failures, power supply failures, bus opens or shorts, or magnetic faults shall be included in the design and associated disruption loads.

f. Disruption loads.

The antenna assembly and bumper limiters shall accomodate disruptive loading of the character and frequency defined in the GRD.

g. Seismic loads.

The antenna, limiter, and transmission line system shall be designed in accordancewith the specification given in the GRD section 3.2. These components would be in class PC-1.

3.3 Upgrades

a. The pulse length shall be increased to 60 secs for the upgrade period

WBS 22 -  CHI Subsystem

1.0 Function

a.. The CHI system is designed to allow for non-inductive start-up, a feature that would be attractive in a ULART reactor and to provide edge current drive for long pulse operation if it proves needed.

2.0 Configuration Requirements & Essential Features

a. The CHI system relies on toroidal breaks that electrically isolate the center stack from the outer vacuum vessel. This break is provided by two ceramic spool pieces incorporated into the vacuum vessel design. Bus connections to the two sides of the breaks are needed to bring the energy from the CHI power supply which will consist of a capacitor bank for start-up assist (baseline program) and a rectifier for long pulse edge current sustainment (upgrade).

3.0 Performance &Operational Requirements

3.1 Baseline Performance Requirements

3.1.1 In Vessel Components

a. The ceramic break material used must be compatible with the NSTX vacuum requirements and the bakeout requirements. Integration of the break will take place in WBS 12.

3.1.2 Bus requirements

a.. The external bus and connections must be capable of handling the voltage and current requirements of the system for the pulse lengths and duty factors required for baseline operation. Consideration should be given to possible simple allowances for upgrade operation such as bus cooling.

3.2 Baseline Operational Requirements

The baseline CHI will be a one-quarter sine wave discharge from a capacitor bank 5mS to peak, 500kA (plasma) /50kA (external circuit), once every 300 sec, Irms = 160 amp.

3.4 Upgrades

The CHI upgrade system will include an additional steady state power supply that will allow long pulse edge current drive.

An interim upgrade is  to 5.0 sec, 200kA (plasma) /20kA (external circuit), once every 300 sec, Irms = 2.6Kamp.You may wish to see the memo I wrote on this (54-960520-CLN-01).

WBS 24 -  NBI Subsystem

1.0 Function

a. The NBI system is required for plasma heating and current drive as well as plasma rotation.

2.0 Configuration Requirements & Essential Features

a.The NBI sysytem will consist of up to four NBI injectors installed in the corners of the test cell and firing through ports B, F, H, and L.

b. In vessel armor will be needed for the beam shine through strike points. Exact beam aiming angles will be determined by physics considerations and strike point compromises.

c. The beam systems will require substantiual auxiliary services, electrical power, cryogenics, water, and vacuum.

3.0 Performance & Operational Requirements

3.1 Baseline Requirements

NBI is not part of the baseline program, however some requirements are imposed to ensure that NBI can be incorporated easily as an  upgrade

3.1.1 Test cell floor space

a.. Adequate floor space to accomodate the NBI boxes shall be laid out in the four corners of the room.

3.1.2 Port allocation

a.Four ports are required for the neutral beam injection. In order to fit the injectors in the test cell these ports will be B,F H and L.

b. Inner armour will not be provided in the baseline design

3.2 Upgrade Requirements

I need to put something here but I will get to it later.

APPENDIX