National Spherical Torus Experiment

NSTX

SYSTEM DESIGN DESCRIPTION

PLASMA FACING COMPONENTS

NSTX-SDD-11-028

Revision 0

June 12, 1998

________________________________

James Chrzanowski

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

NSTX SYSTEM DESIGN DESCRIPTION

RECORD OF CHANGES

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 6/12/98 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	2

Plasma Facing Components Design Description	2

1.0 Design Description	2

1.1  System Design Features	2

1.2  Layout		3

1.3  Component Design Features	6

1.3.1 Center Stack Casing (CSC) PFCs (sh. 001-100)	6

1.3.2 Inboard Divertor (IBD) (sh. 007, 401-458)	13

1.3.3 Outboard Divertor (OBD) (sh. 140-186)	17

1.3.4 Passive Plates (sh. 200-400)	22

1.3.5 Heating/Cooling Fluid Distribution System (HCFDS)	29

2.0 System Performance	30

2.1 Thermal	30

2.2 Electromechanical	31

3.0 Interfaces	32

3.1 Interface Points	32

3.2 Interface Requirements	32

4.0 Design Basis	33

4.1 Standards and Codes	33

4.2 Supporting Analysis	34

5.0 Design Implementation	35

5.1 Prototypes	35

5.2 Mock-ups	35

5.2.1  Pull Out Tests	35

5.2.2 Bakeout Tests	35

5.3 Component Procurement and Fabrication	35

5.3.1 CFC Material	35

5.3.2 Other Parts and Materials	35

5.4  System Assembly, Installation, and Testing	35

Introduction

This System Design Description (SDD) describes the design of the Plasma Facing Components (WBS 11) of the National Spherical Torus Experiment (NSTX).

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in Appendix A.

The design described herein is based on the requirements given in the System Requirements Document (SRD) SRD-11-013-1 [R1].

WBS 11 -  Plasma Facing Components Design Description

1.0 Design Description

List of drawings is given in Appendix II. Sheet numbers of drawings related to component descriptions in following sections are indicated.

1.1  System Design Features

The Plasma Facing Components (PFCs) consist of the following components:

•  Center Stack Casing (CSC) PFCs;

• Inboard Divertor (IBD);

• Outboard Divertor (OBD);

• Secondary Passive Plates (SPP);

• Primary Passive Plates (PPP);

• Heating/Cooling Fluid Distribution System.

For the tiles which directly face the plasma, two materials are utilized:

• Carbon Fiber Composite (CFC) material manufactured by Allied Signal Corporation [R2];

• Graphite material manufactured by Union Carbide Corporation (type ATJ).

Component features are summarized in Table 1.1-1.

Table 1.1-1: PFC Component Features

| Component | Tile Mat'l | Tile Thickness* (inch) | Backplate Thickness (inch) | Mounting Method | Surface Contour, Plasma Side | Surface Contour, Back Side | Tile Overlap, Toroidal Direction | Tile Overlap, Poloidal Direction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| CSC Rail Tiles | CFC | 0.55 | n.a. | rail | cylinder | cylinder | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| CSC Basic Tiles | Graphite | 0.55 | n.a. | rail | cylinder | cylinder | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Conical Section | Graphite | 0.75 | n.a. | bar | conical | conical | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Vertical Section | Graphite | 1.0 | n.a. | T-bar | cylinder | cylinder | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| IBD Horizontal Section | Graphite | 2.0 | n.a. | T-bar | flat | flat | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| OBD | Graphite | 1.0 | 1.0 | T-bar | flat | flat | yes | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| SPP | Graphite | 1.0 | 0.5 | T-bar | flat | flat | no | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PPP | Graphite | 1.0 | 0.5 | T-bar | flat | flat | no | yes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

* Refer to fabrication drawings (listed in Appendix II of this SDD) for exact dimensions and geometries

1.2  Layout

The PFC designs (in particular the location of the front plasma facing surfaces) conform to the geometries defined on Figure 1.2-1 and given in Table 1.2-1, which describes the 3-d surfaces via 2-d line segments assumed swept through toroidal angle 0 to 360 degrees. Data given in table corresponds to PFCs above midplane; corresponding mirror image PFCs are located below midplane [R1].

Figure 1.2-1  PFC Geometry

Table 1.2-1  PFC Geometry (Inches)

|  |  | # | R (in) | Z (in) | # | R (in) | Z (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| CSC | Struct | 1 | 6.738 | 0.000 | 2 | 6.738 | 40.235 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 3 | 7.289 | 0.000 | 4 | 7.289 | 39.690 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Lower Conical Section | Struct | 5 | 6.738 | 40.235 | 6 | 10.000 | 45.885 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 7 | 7.289 | 39.690 | 8 | 11.000 | 46.117 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Vertical Section | Struct | 9 | 10.000 | 45.885 | 10 | 10.000 | 62.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 11 | 11.000 | 46.117 | 12 | 11.000 | 63.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Upper Conical Section | Struct | 13 | 10.000 | 62.125 | 14 | 11.732 | 65.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 15 | 11.000 | 63.125 | 16 | 11.000 | 63.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Horizontal Section | Struct | 17 | 11.732 | 65.125 | 18 | 22.500 | 65.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 19 | 11.000 | 63.125 | 20 | 22.500 | 63.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OBD | Struct | 21 | 24.660 | 65.025 | 22 | 47.393 | 56.071 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 23 | 24.294 | 64.095 | 24 | 47.027 | 55.141 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Slot | Struct | 25 | 43.823 | 54.143 | 26 | 46.256 | 53.184 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 27 | 41.074 | 56.300 | 28 | 46.623 | 54.115 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| SPP | Struct | 29 | 43.823 | 54.143 | 30 | 52.752 | 41.509 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 31 | 41.074 | 56.300 | 32 | 51.935 | 40.932 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PPP | Struct | 33 | 53.540 | 39.594 | 34 | 59.416 | 21.775 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 35 | 52.592 | 39.276 | 36 | 58.468 | 21.457 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Table 1.2-1  PFC Geometry (Centimeters)

|  |  | # | R(cm) | Z(cm) | # | R(cm) | Z(cm) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| CSC | Struct | 1 | 17.115 | 0.000 | 2 | 17.115 | 102.197 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 3 | 18.515 | 0.000 | 4 | 18.515 | 100.812 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Lower Conical Section | Struct | 5 | 17.115 | 102.197 | 6 | 25.400 | 116.548 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 7 | 18.515 | 100.812 | 8 | 27.940 | 117.137 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Vertical Section | Struct | 9 | 25.400 | 116.548 | 10 | 25.400 | 157.798 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 11 | 27.940 | 117.137 | 12 | 27.940 | 160.338 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Upper Conical Section | Struct | 13 | 25.400 | 157.798 | 14 | 29.799 | 165.418 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 15 | 27.940 | 160.338 | 16 | 27.940 | 160.338 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| IBD/Horizontal Section | Struct | 17 | 29.799 | 165.418 | 18 | 57.150 | 165.418 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 19 | 27.940 | 160.338 | 20 | 57.150 | 160.338 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OBD | Struct | 21 | 62.636 | 165.164 | 22 | 120.378 | 142.420 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 23 | 61.706 | 162.800 | 24 | 119.447 | 140.057 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Slot | Struct | 25 | 111.311 | 137.523 | 26 | 117.490 | 135.089 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 27 | 104.327 | 143.003 | 28 | 118.421 | 137.452 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| SPP | Struct | 29 | 111.311 | 137.523 | 30 | 133.989 | 105.433 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 31 | 104.327 | 143.003 | 32 | 131.915 | 103.967 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PPP | Struct | 33 | 135.992 | 100.569 | 34 | 150.917 | 55.309 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 35 | 133.584 | 99.761 | 36 | 148.509 | 54.501 |
| --- | --- | --- | --- | --- | --- | --- | --- |

1.3  Component Design Features

1.3.1 Center Stack Casing (CSC) PFCs (sh. 001-100)

a. Configuration of CFC tiles and mounting accessories is given in Figure 1.3.1-1, -2, and -3. There are two assemblies, namely the Rail Tile Assembly (168 pieces)  and the  Vertical Tile Assembly (336 pieces). The Rail Assemblies are mounted on to the Center Stack Casing and mechanically capture the Vertical Tile Assemblies.

Figure 1.3.1-1: Rail Tile Assembly

Figure 1.3.1-2: Vertical Tile Assembly

Figure 1.3.1-3: Section View, Center Stack Tiles

Rail Tile Assembly (sh. 038-061) parts consist of:

• Inconel studs welded on to center stack casing;

• Inconel rails (channels);

• CFC rail tiles, which fit in the channel of the Inconel rails;

• Inconel drift pins which secure the CFC tiles to the Inconel rails;

• A-286 allen head nut, self-locking, which secures the Inconel rails to the studs and center stack casing.

Vertical Tile Assembly (sh. 010-036, 063-064) parts consist of:

• Graphite vertical tiles;

• Inconel Radiation shield to minimize heat flow inwards to the center stack casing;

• Molybdenum wave spring;

• Inconel spacer, which forms a channel for routing of diagnostic leads.

b. Three variants of the basic tiles include provision for diagnostics. The first two (Figure 1.3.1-4 is typical) utilizes a screw-in plug, with a 0.34 inch diameter, 0.29 inch deep recess, to accommodate diagnostics (Langmuir probes, or thermocouples) as required. The second (Figure 1.3.1-5) includes a cut-out which accommodates a 1" x 1" x 0.1" Mirnov (Br, or Br/Bz) coil.

c. Heat deposited on the CSC tiles during plasma operations is removed primarily by radiation to the other PFC components; there is no active cooling [R3].

d. Heat is input to the CSC tiles during bakeout by radiation received from the other PFCs. In addition the center stack casing is ohmically heated by a DC current to maintain the required temperature, to make up for heat flow inward to the water cooled OH and TF coils located within the center stack casing [R4,R5].

Figure 1.3.1-4: Langmuir/Thermocouple CS Tile Variant

Figure 1.3.1-5: Mirnov Coil CS Tile Variant

1.3.2 Inboard Divertor (IBD) (sh. 007, 401-458)

a. As shown in Figure 1.3.2-1, IBD consists of three tile sections, namely Horizontal Section (tile rows 1-2), Vertical Section (tile rows 3-6), and Conical Section (tile row 7).

b. Tiles in the Vertical and Horizontal Sections are mounted to the center stack casing via Inconel "T-bars". The T-bars (Figure 1.3.2-2) are first attached to the Graphite tiles by sliding into T-shaped channels cut in the back face of the tiles, then tightening A-286 stainless steel fasteners which fit into countersunk holes in the back face of the tiles. Then, the T-bar/tile assemblies are attached to the center stack casing using A-286 stainless steel allen head fasteners/flat washers which are captured within the T-bars and accessible through holes in the tile front face.

c. Tiles in the Conical Section are mounted to the center stack casing via a Rail Assembly (Figure 1.3.2-3) consisting of the following parts:

• Graphite tiles, with square channels cut on the back side;

• Inconel rails, which fit into the channels in the back side of the Graphite tiles;

• Inconel drift pins which secure the Graphite tiles to the Inconel rails;

• Captured A-286 stainless steel allen head fasteners/flat washers, which secure the Inconel rails to the center stack casing (accessible via allen wrench through small holes in the tile front face).

d. The Horizontal Section includes provision for installation of a Rogowski coil to measure halo current flow in the center stack casing as shown in Figure 1.3.2-4.

e. Heat deposited on the IBD tiles during plasma operations is removed by active cooling on the inside of the center stack casing (Vertical Section) and within the flange at the ends (Horizontal Section). This is accomplished by WBS 33, via Dowtherm fluid circulated by the Heating/Cooling system [R6].

f. Heat is input to the IBD tiles during bakeout by radiation received from the other PFCs, as well as from the circulation of Dowtherm liquid on the inside of the center stack casing (Vertical Section) and within the flange at the ends (Horizontal Section).

Figure 1.3.2-1: Inboard Divertor Horizontal, Vertical, and Conical Sections

Figure 1.3.2-2: T-Bar Assembly (typ. IBD Vertical & Horizontal Sections)

Figure 1.3.2-3: Rail Assembly (typ. IBD Conical Section)

1.3.3 Outboard Divertor (OBD) (sh. 140-186)

a. OBD (Figure 1.3.3-1) consists of 48 toroidally segmented tile/backplate assemblies in each half plane (total 96) mounted on  Three Ring Support Assemblies (Figure 1.3.3-2), which in turn mounted to the outer vacuum vessel. There are two variants of the tile assemblies, namely the Rail Tile Assembly and the T-bar Tile Assembly.

b. Rail Tile Assembly consisting of the following parts:

• Graphite tile, with square channels cut on the back side;

• Inconel bar, which fits into the channel in the back side of the Graphite tile;

• Inconel drift pins which secure the Graphite tile to the Inconel bar;

• Captured Stainless Steel allen head fasteners/flat washers, which secure the Inconel bar to the Helicoil inserts in the Copper back plate.

c. T-bar Tile Assembly consisting of the following parts:

• Graphite tile, with T-channel cut on the back face;

• Inconel T-bar, which fits into the channel in the back face of the Graphite tile;

• Inconel Fasteners which attach the T-bar to the back face of the Graphite tiles via countersunk holes;

• Captured Stainless Steel allen head fasteners/flat washers, which secure the Inconel T-bar to Helicoil inserts in the Copper backplate (accessible via allen wrench through small holes in the tile front face).

d. Backplate Assembly (Figure 1.3.3-3) consists of the following parts:

• 0.062" thick Grafoil thermal conductive gasket;

• 1" thick Copper backplate, with channel cut out in back face;

• Stainless Steel cooling tube which fits into Copper backplate;

• Hardware for mounting to Three Ring Support Assembly.

The mounting to the Three Ring Support Assembly is depicted in Figures 1.3.3-4 and 1.3.3-5. The Stainless Steel center bolts, belleville, and flat washers are installed via Stainless Steel inserts which are threaded into the Copper backplates. The Stainless Steel end bolts (shoulder bolts), belleville, and flat washers are mounted via Stainless Steel located bushing which fit into countersunk holes in the backplates. A clearance is provided between the shoulder bolts and the backplates. This scheme fixes the location of the backplates at the center (via the center bolts) but allows for  thermal expansion of the backplates via the clearance holes through which the end (shoulder) bolts pass.

e. Three Ring Support Assembly is mounted to the Vacuum Vessel Domes via a clevis/pin scheme. This scheme allows for differential thermal expansion of the parts, as well as the dimensional tolerances, which can be large (≈ 1/4") on the domes.

f. Heat deposited on the OBD tiles during plasma operations is removed by active cooling of the backplates via the circulation of Dowtherm fluid by the Heating/Cooling system (WBS 33).

g. Heat is input to the OBD tiles during bakeout by the circulation of Dowtherm liquid through the backplates by the Heating/Cooling system (WBS 33).

Figure 1.3.3-1: Outboard Divertor Assembly

Figure 1.3.3-2: Three Ring Support Assembly

Figure 1.3.3-3: OBD Backplate Mounting, Center Bolts

Figure 1.3.3-4: OBD Backplate Mounting, End Bolts

1.3.4 Passive Plates (sh. 200-400)

The Passive Plates (PP) consist of 12 toroidal segments, and 2 poloidal segments in each half plane (total 48 plates), which are electrically connected in the toroidal direction except at one location which contains a toroidal electrical break (Figure 1.3.4-1).  At this location there is a saddle connection between the plates in the upper and lower half planes such that current flowing, e.g., clockwise in the upper set of plates is constrained to flow counterclockwise in the lower set of plates.

The Primary Passive Plates (PPP, those closest to the midplane) and the Secondary Passive Plates (SPP, those furthest from the midplane, sh. 200-300) are of identical construction (Figure 1.3.4-2). They consist of the following elements:

• Plate Assemblies;

• Tile Assemblies ;

• Toroidal Electrical Jumper Assemblies;

• Poloidal Electrical Jumper Assemblies [R7];

• Saddle Connector;

• Supports.

a. The Plate Assemblies consist of the following parts (Figure 1.3.4-3):

• Copper/Chromium/Zirconium plates, with silver plated contact surfaces;

• Grafoil thermally conductive gaskets between the tile back faces and the plates;

• Helicoil inserts for tile attachment;

• Stainless Steel fasteners/flat washers which attach the plates to the Plate Support Assemblies, via countersunk holes in the plates.

b. The Tile Assemblies consist of the following parts:

• Graphite tile, with T-channel cut on the back face;

• Inconel T-bar, which fits into the channel in the back face of the Graphite tile;

• Inconel Fasteners which attach the T-bar to the back face of the Graphite tiles via countersunk holes;

• Captured Stainless Steel allen head fasteners/flat washers, which secure the Inconel T-bar to Helicoil inserts in the Copper backplate (accessible via allen wrench through small holes in the tile front face).

c. The Toroidal Electrical Jumper Assemblies consist of the following parts:

• 3/8" Copper jumpers, with silver plated contact surfaces;

• 1/2" OD x 0.49" wall Stainless Steel cooling lines, brazed to perimeter of jumpers.

Figure 1.3.4-1: Passive Plate System

Figure 1.3.4-2: Passive Plate Assembly (typ. Primary/Secondary Plates)

Figure 1.3.4-3: Passive Plate Assembly Detail (typ. Primary/Secondary Plates)

d. The Poloidal Electrical Jumper Assemblies consist of 1/8" Copper straps, with silver plated contact surfaces.

e. Saddle Jumper Assembly  (Figure 1.3.4-4) consists of copper conductors arranged with two (+) conductors on either side of one (-) conductor. In this configuration there is no net moment on the structure, and field errors are reduced.

f. The Support Assemblies (Figure 1.3.4-5) consist of the following parts:

• Stainless Steel radial support ribs;

• Stainless Steel channels which are welded to the outer vacuum vessel;

• Stainless Steel pins which join the radial support ribs to the channels;

• Stainless Steel shim plates which provide adjustable spacers between the Toroidal Jumper Assembles and the radial support ribs.

This design is noted to allow for the relative thermal expansion of the various parts via the gaps between the radial support ribs and the channels, and the pins. In addition, the design allows for dimensional tolerances in the parts (≈ 1/4" on the outer vacuum vessel) since the position of the holes in the radial support ribs, which receive the pins, can be determined during installation after the channels are welded to the outer vacuum vessel.

g. Heat deposited on the PP tiles during plasma operations is removed by active cooling of the Toroidal Jumper Assemblies via the circulation of Dowtherm fluid by the Heating/Cooling system (WBS 33). The plates themselves, and the tiles mounted to them, will be conductively cooled to the jumpers.  In this way, the plates can be demounted for access and modification without disturbing the cooling circuit.

h. Heat is input to the PP tiles during bakeout by the circulation of Dowtherm liquid through the Toroidal Jumper Assemblies by the Heating/Cooling system (WBS 33).

Figure 1.3.4-4: Saddle Jumper

Figure 1.3.4-5: Passive Plate Supports

1.3.5 Heating/Cooling Fluid Distribution System (HCFDS)

a. HCFDS is depicted on drawings TBD "Vacuum Vessel Cooling Manifold", TBD "Modified Flange Assembly", and TBD "Manifold Assembly and Details". Dowtherm fluid (supplied by the Heating/Cooling System WBS 33) is delivered to manifold inlets located outside the vacuum vessel in the upper and lower half planes, and returned via three manifold outlets located outside the vacuum vessel at the midplane. The manifolds at each of the three elevations (upper, lower and midplane) each provide 120 degree toroidal coverage. Each input manifold connects to subcircuits which deliver fluid, in series, to four OBD backplates, one SPP Toroidal Jumper Assembly, one PPP Toroidal Jumper Assembly. There are 11 circuits in which the cooling line crosses the gap between the segmented passive plates, electrically in parallel with the copper plate to plate jumpers. To avoid crossing the gap at the toroidal gap location, circuits 12 ane 13 cool the ends of the plates at the gap but do not cross the gap. General arrangement is depicted in Figure 1.3.5-1.

Figure 1.3.5-1: Heating/Cooling Fluid Distribution System

b. Flow instrumentation is not provided; thermocouples within the PFC system will (indirectly) indicate the adequacy of the fluid flow.

2.0 System Performance

2.1 Thermal

a.  PFCs are designed in accordance with the baseline requirements given in the NSTX GRD [R8, R9] and PFC SRD. Heat loads are given in Table 2.0-1 and 2.0-2.

Table PFC-3.1-1: Max PFC Heat Loads for Baseline NSTX Duty Cycle

| Component | Max Value | Units |
| --- | --- | --- |

| Passive Stabilizers | 0.1 | MW/m2 |
| --- | --- | --- |

| Inner Wall Armor | 1.9 | MW/m2 |
| --- | --- | --- |

| Inboard Divertor Area Strike Plates | 7 | MW/m2 |
| --- | --- | --- |

| Outboard Divertor Area Strike Plates | 17.1 | MW/m2 |
| --- | --- | --- |

Table 3.1-2: PFC Heat Loads for Baseline NSTX Operation Scenarios

| Scenario | Parameter | Inner Div | Outer Div | Inner Wall | Total |  |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | Upper | Lower | Upper | Lower |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Natural Divertor | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 0.0 | 1.1 | 1.1 | 2.1 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.000 | 0.094 | 0.094 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 0.0 | 4.3 | 4.3 | 1.9 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Double Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.4 | 0.4 | 1.7 | 1.7 | 0.0 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.038 | 0.038 | 0.038 | 0.038 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 5.1 | 5.1 | 17.1 | 17.1 | 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Single Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 1.4 | 0.0 | 2.8 | 0.0 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.092 | 0.000 | 0.100 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 7.0 | 0.0 | 10.8 | 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

b.  Peak surface temperature of the tiles at the end of a pulse will depend on the backplate initial temperature, heat flux spatial and temporal profiles, overall pulse length, tile material, and tile thickness. At heat flux levels ≤ 600 watt/cm2 (6MW/m2) all of the tile configurations can

withstand 5 second pulses once every 300 seconds, even with conservative heat flux spatial profiles, with peak temperatures < 1200oC. Beyond this level (e.g. on the OBD, for Double Null and Single Null plasma configurations), some operational reduction will be necessary (reduce repetition rate, utilize divertor sweeping, etc.). Monitoring of divertor diagnostics (e.g. thermocouples, Infrared TV, etc.) will be required to establish actual limits during machine operations. References for thermal analysis is given in section 4.0.

2.2 Electromechanical

a. PFCs and associated supports have been designed to withstand forces due to normal plasma operations, as well as disruptions. The electromechanical design assumes forces arising from disruptions of 1MA plasmas, at rates ≤ 1MA/6mS, with background toroidal field due to Bt at R0 ≤ 6kG, with background poloidal due to worst case combinations of PF coil currents at their peak rated values, and with poloidal halo currents up to 10% of Ip, with toroidal peaking factor up to 2. References for electromechanical analysis are given in section 4.0.

3.0 Interfaces

3.1 Interface Points

Table 3.1-1: Interfaces with the Plasma Facing Components

| WBS | Interface Point |
| --- | --- |

| 12 | The OBD and PP PFCs interface with the vacuum vessel at the interior surfaces of the wall of the outer vacuum vessel, and with vessel feedthrough port flanges for heating/cooling lines. |
| --- | --- |

| 13 | The CSC and IBD PFCs interfaces with the center stack at the interior surface of the casing. |
| --- | --- |

| 33 | The OBD and PP PFCs interface with the Heating/Cooling system outside the vacuum vessel at the six Dowtherm distribution manifold inlet/outlet connection points. |
| --- | --- |

| 4x | Diagnostics at various locations along and through the PFCs. This includes Thermocouples, Langmuir Probes, and Mirnov coils. |
| --- | --- |

3.2 Interface Requirements

The PFCs impose interface requirements on WBS 33, the Heating/Cooling System. Dowtherm thermal/hydraulic requirements are summarized in Table 3.2-1 [R10].

Table 3.2-1: Dowtherm Thermal/Hydraulic Requirements

| Parameter | Value | Units |
| --- | --- | --- |

| #Circuits | 6 | n.a. |
| --- | --- | --- |

| #Sub-circuits | 26 | n.a. |
| --- | --- | --- |

| Flow/sub-circuit | 4 | GPM |
| --- | --- | --- |

| Sub-circuit tube OD | 0.5 | in |
| --- | --- | --- |

| Sub-circuit tube ID | 0.402 | in |
| --- | --- | --- |

| Sub-circuit tube length | 25 | ft |
| --- | --- | --- |

| ∆P | 15 | psi |
| --- | --- | --- |

| ∑Heat during Plasma Operations | 100.0 | kW |
| --- | --- | --- |

| ∆T during Operations | 10.0 | deg C |
| --- | --- | --- |

| ∑Heat during Bakeout | 69.0 | kW |
| --- | --- | --- |

| ∆T during Bakeout | 10.0 | deg C |
| --- | --- | --- |

4.0 Design Basis

4.1 Standards and Codes

a. 	The plasma facing components will be designed in accordance with the general design guidelines set forth in the General Requirements Document (GRD).  [R8]

b. 	All loads in the plasma facing components will be reacted through the vacuum vessel, with the exception of the loads on the inner wall armor which will be reacted through the center stack casing.

c. 	In addition to the heat loads from the plasma, the plasma facing components will be designed to sustain the design loads from normal operation, plasma disruption and bakeout to which the vessel is subjected.

d.	The stress acceptance criteria are primarily based on the ASME Boiler and Pressure Vessel Code, Section III, Division I.For some one-dimensional types of elements such as column and bracing, it is more appropriate to apply the allowable stresses specified in the AISC Manual of Steel Construction.

e. 	Based on the linear finite element analysis, the ASME code requires that the Tresca stress of the 2-D or 3-D element shall meet the following stress limits:

- General primary membrane stress shall not exceed 1.0 K Sm

- Local primary membrane stress shall not exceed 1.5 K Sm

- Primary membrane plus bending stresses shall not exceed 1.5 K Sm

- Total primary plus secondary stresses shall not exceed 3.0 K Sm

(Where the K factor is 1.0 for normal operating conditions or 1.1 if case seismic force is involved.  Sm is the design stress intensity value defined in the ASME Boiler and Pressure Vessel code.)

f.	Since the von Mises stress is generally more accurate than the Tresca stress in the representation of the equivalent one-dimensional stress and the difference between the two is small, it will be used instead of the Tresca stress in determining the stress acceptance criteria.

g. 	The design stress intensity value is set at 2/3 of the minimum specified yield at temperature.

h.  For bolting material, the average tensile stress shall not exceed 1.0 Sm and maximum direct tensile plus bending stress shall not exceed 1.5 Sm.

4.2 Supporting Analysis

Design basis calculations were performed as indicated in Table 4.2-1.

| Calc # | Calc Title | Summary |
| --- | --- | --- |

| CALC-11-01 | SPARK Plasma Disruption Calculation | Calculation of toroidal currents due to stationary plasma disruption using SPARK, checked using LRSIM. |
| --- | --- | --- |

| CALC-11-02 | Internal Hardware Force Calculation | Calculation of forces and moments on internal hardware. |
| --- | --- | --- |

| CALC-11-03 | Outboard Divertor Load/Stress Analysis | Calculation of loads and stresses in OBD PFC components |
| --- | --- | --- |

| CALC-11-04 | PFC Load/Stress Analysis | Calculation of loads and stresses in CSC, IBD, and PP PFCs. |
| --- | --- | --- |

| CALC-11-05 | Thermal Analysis of NSTX Center Stack and Outboard Divertor Tiles | Tile heat load/temperature rise calculations. |
| --- | --- | --- |

| CALC-11-06 | Load/Stress Analysis of Passive Plate Support Structures | Calculation of loads and stresses in passive plates and related supports. |
| --- | --- | --- |

| CALC-11-07 | Dowtherm Heating/Cooling System Thermal/Hydraulic Analysis | Calculation of flow, pressure drop, and temperature rise in Dowtherm heating/cooling system. |
| --- | --- | --- |

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

5.2.1  Pull Out Tests

Samples of CFC materials were subjected to mechanical pull-out tests [R11].

5.2.2 Bakeout Tests

Samples of CFC materials were baked out to determine outgassing products [R12].

5.3 Component Procurement and Fabrication

5.3.1 CFC Material

Specification SPEC-11-045 was issued on 2/2/98 for the procurement of CFC material. The contract was awarded to Allied Signal Corporation.

5.3.2 Other Parts and Materials

Procurement packages will be issues as required to obtain machined graphite tiles and the various mechanical hardware.

5.4  System Assembly, Installation, and Testing

Later

APPENDIX I

LIST OF REFERENCES

| R | Description | Reference |
| --- | --- | --- |

| 1 | PFC System Requirements Document | SRD-11-13-1 |
| --- | --- | --- |

| 2 | CFC Procurement Specification | SPEC-11-045 |
| --- | --- | --- |

| 3 | Thermal Analysis of Normal Operation | 13-970214-AWB-02, "Thermal Analysis VV/CS During Normal Operation" |
| --- | --- | --- |

| 4 | Thermal Analysis of Bakeout | 13-970214-AWB-01, "Bakeout of Vacuum Vessel and Internals" |
| --- | --- | --- |

| 5 | Thermal Analysis of Bakeout, Revised | 13-980511-AWB-01, "Update of VV Bakeout Heating Requirements" |
| --- | --- | --- |

| 6 | Heating/Cooling System Design Description | SDD-3X |
| --- | --- | --- |

| 7 | Description of Passive Plate Electrical Connections | 11-971029-CLN-01, "Passive Plate Connections" |
| --- | --- | --- |

| 8 | NSTX General Requirements Document | GRD-018-01 |
| --- | --- | --- |

| 9 | Heat Flux Estimates | "NSTX Divertor Heat Flux Estimates", S Kaye, 10/24/96 |
| --- | --- | --- |

| 10 | Dowtherm Thermal/Hydraulic Requirements | "PFC Heating/Cooling Thermal/Hydraulic Design", CALC-33-01 |
| --- | --- | --- |

| 11 | Pull Out Tests | 11-980501-LLMES-PLG-02, "NSTX Center Stack Tile Analysis and Testing" |
| --- | --- | --- |

| 12 | CFC Bakeout Tests | 11-9806012-JHC-01, "Allied Signal Bakeout Tests" |
| --- | --- | --- |

| 13 | PFC FDR Results | 1X-970807-PB-01, "NSTX Design Review" |
| --- | --- | --- |

| 14 | PFC Peer Review Results | 11-980129-CLN-01, "Peer Review on NSTX Plasma Facing Components" |
| --- | --- | --- |

| 15 |  |  |
| --- | --- | --- |

| 16 |  |  |
| --- | --- | --- |

| 17 |  |  |
| --- | --- | --- |

| 18 |  |  |
| --- | --- | --- |

| 19 |  |  |
| --- | --- | --- |

| TITLE (DESCRIPTION) | ORNL# | PPPL# |
| --- | --- | --- |

| CENTERSTACK CASING TILE STUD LAYOUT ASSEMBLY | 002 | 0180 |
| --- | --- | --- |

| CENTERSTACK UPPER CASING STUD LAYOUT SHEET 1 | 003 | 0177 |
| --- | --- | --- |

| CENTERSTACK UPPER CASING STUD LAYOUT SHEET 2 | 003 | 0178 |
| --- | --- | --- |

| CENTERSTACK UPPER CASING STUD LAYOUT SHEET 3 | 003 | 0179 |
| --- | --- | --- |

| CENTERSTACK LOWER CASING STUD LOCATION LAYOUT SHEET 1 | 004 | 0174 |
| --- | --- | --- |

| CENTERSTACK LOWER CASING STUD LOCATION LAYOUT SHEET 1 | 004 | 0175 |
| --- | --- | --- |

| CENTERSTACK LOWER CASING STUD LOCATION LAYOUT SHEET 1 | 004 | 0176 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE/FOIL, STYLE-1 PROBE ASSEMBLY | 010 | 0173 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 PROBED DETAIL SHEET1 | 011 | 0171 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 PROBED DETAIL SHEET 2 | 011 | 0172 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 BLANK DETAIL SHEET 1 | 012 | 0169 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 BLANK DETAIL SHEET 2 | 012 | 0170 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 PROBED ASSEMBLY | 013 | 0168 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 PROBED DETAIL SHEET 1 | 014 | 0166 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 PROBED DETAIL SHEET 2 | 014 | 0167 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 BLANK DETAIL SHEET 1 | 015 | 0164 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 BLANK DETAIL SHEET 2 | 015 | 0165 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 T/C DETAIL SHEET 1 | 016 | 0162 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 T/C DETAIL SHEET 2 | 016 | 0163 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 T/C DETAIL SHEET 1 | 017 | 0160 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 T/C DETAIL SHEET 2 | 017 | 0161 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE PROBE ASSEMBLY | 018 | 0159 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE PROBE DETAILS | 019 | 0158 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 PROBED FOIL DETAIL | 020 | 0157 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 BLANK FOIL DETAIL | 021 | 0156 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 PROBED FOIL DETAIL | 022 | 0155 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 BLANK FOIL DETAIL | 023 | 0154 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 T/C FOIL DETAIL | 024 | 0153 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 T/C FOIL DETAIL | 025 | 0152 |
| --- | --- | --- |

| CENTERSTACK SHIELDING WAVEWASHER/RETAINING BAR DETAILS | 026 | 0151 |
| --- | --- | --- |

| CENTERSTACK SHIELDING STYLE-1 PROBED FOIL/WAVEWASHER WELDMENT | 027 | 0150 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 BLANK FOIL/WAVEWASHER WELDMENT | 028 | 0148 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 PROBED FOIL/WAVEWASHER WELDMENT | 029 | 0149 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 BLANK FOIL/WAVEWASHER WELDMENT | 030 | 0147 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 T/C FOIL/WAVEWASHER WELDMENT | 031 | 0146 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 T/C FOIL/WAVEWASHER WELDMENT | 032 | 0145 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE /FOIL STYLE-1 SUB-ASSEMBLY | 033 | 0144 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE /FOIL STYLE-2 BLANK ASSEMBLY | 034 | 0143 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 T/C ASSEMBLY | 035 | 0142 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-2 T/C ASSEMBLY | 036 | 0141 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-1 ASSEMBLY | 038 | 0140 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-1 DETAIL SHEET 1 | 040 | 0138 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-1 DETAIL SHEET 2 | 040 | 0139 |
| --- | --- | --- |

| CENTERSTACK SHIELDING DETAILS | 041 | 0137 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-2 ASSEMBLY | 042 | 0136 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-2 DETAIL SHEET1 | 043 | 0134 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL RAIL TILE STYLE-2 DETAIL SHEET2 | 043 | 0135 |
| --- | --- | --- |

| CENTERSTACK SHIELDERING MACHINED VERTICAL RAIL DETAIL SHEET 1 | 048 | 0001 |
| --- | --- | --- |

| CENTERSTACK SHIELDERING MACHINED VERTICAL RAIL DETAIL SHEET 2 | 048 | 0002 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 DIAGNOSTIC | 051 | 0133 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 MAGNETIC DIAGNOSTIC FOIL DETAIL | 053 | 0132 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE STYLE-1 MAGNETIC DIAGNOSTIC ASSMBLY | 055 | 0131 |
| --- | --- | --- |

| CENTERSTACK VERTICAL RAIL TILE ROW 28 STYLE 3 ASSEMBLY | 060 | 0130 |
| --- | --- | --- |

| CENTERSTACK VERTICAL RAIL TILE ROW 28 STYLE 3 DETAIL SHEET 1 | 061 | 0128 |
| --- | --- | --- |

| CENTERSTACK VERTICAL RAIL TILE ROW 28 STYLE 3 DETAIL SHEET2 | 061 | 0129 |
| --- | --- | --- |

| CENTERSTACK VERTICAL TILE ROW 28 STYLE 2 ASSEMBLY | 063 | 0127 |
| --- | --- | --- |

| CENTERSTACK VERTICAL RAIL TILE ROW 28 STYLE 3 DETAIL SHEET 1 | 064 | 0125 |
| --- | --- | --- |

| CENTERSTACK VERTICAL RAIL TILE ROW 28 STYLE 3 DETAIL SHEET 2 | 064 | 0126 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSY COLUMN 0 DEG. THRU 330 DEG. | 070 | 0124 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN A | 071 | 0123 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN B | 072 | 0122 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN C | 073 | 0121 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN D | 074 | 0120 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN E | 075 | 0119 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN F | 076 | 0118 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN G | 077 | 0117 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN H | 078 | 0116 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN I | 079 | 0115 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN J | 080 | 0114 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN K | 081 | 0113 |
| --- | --- | --- |

| CENTERSTACK SHIELDING VERTICAL TILE SUB-ASSEMBLY COLUMN L | 082 | 0112 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE T-BAR RAIL | 141 | 0111 |
| --- | --- | --- |

| OUTBOARD DIVERTOR TILE RAIL RECTANGULAR | 142 | 0110 |
| --- | --- | --- |

| OUTBOARD DIVERTOR TILE STANDARD GASKET DETAIL | 146 | 0109 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 1A, 1C, & 1G | 151 | 0108 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 2A, 2C, 2E & 2G | 152 | 0107 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 3A, 3C, 3E & 3G | 153 | 0106 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 4A, 4C & 4E | 154 | 0105 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 5A, 5C &5E | 155 | 0104 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE LOCATIONS 1B, 1D, 1F & 1H | 156 | 0103 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE LOCATIONS 2B, 2D, 2F & 2H | 157 | 0102 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE LOCATIONS 3B, 3D, 3F & 3H | 158 | 0101 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE LOCATIONS 4B, 4D, 4F & 4H | 159 | 0100 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE LOCATIONS 5B, 5D, 5F & 5H | 160 | 0099 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE T-BAR RAIL ASSEMBLY | 170 | 0098 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE PIN RAIL ASSEMBLY | 171 | 0097 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE NOTCHED GASKET | 174 | 0096 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE OUTBOARD TILE ASSEMBLY 360 DEG. | 175 | 0095 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE TILE ASSEMBLY 30 DEG. | 176 | 0094 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE BASE PLATE ASSY 30 DEG. WITH COOLING TUBE | 179 | 0093 |
| --- | --- | --- |

| OUTBOARD DIVERTOR STANDARD PLATE | 182 | 0092 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE COOLING TUBE | 183 | 0091 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE NOTCHED | 185 | 0090 |
| --- | --- | --- |

| OUTBOARD DIVERTOR PLATE ASSEMBLY LOCATING BUSHING | 186 | 0089 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 2A | 205 | 0088 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 3A | 206 | 0087 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 4A | 207 | 0086 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 5A | 208 | 0085 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 1B, 1D, &1F | 214 | 0084 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 2B, 2D & 2F | 215 | 0083 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 3B, 3D & 3F | 216 | 0082 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 4B, 4D & 4F | 217 | 0081 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 5B, 5D & 5F | 218 | 0080 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 1C, 1E & 1G | 224 | 0079 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 2C, 2E & 2G | 225 | 0078 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 3C, 3E & 3G | 226 | 0077 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 4C, 4E & 4G | 227 | 0076 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 5C, 5E & 5G | 228 | 0075 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 2H | 235 | 0074 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 3H | 236 | 0073 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 4H | 237 | 0072 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TILE LOCATIONS 5H | 238 | 0071 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TOP RAIL T-BAR ASSEMBLY | 241 | 0070 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE STANDARD T-BAR | 244 | 0069 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TOP RAIL T-BAR | 245 | 0068 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE MAIN FACE GASKET | 246 | 0067 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TOP SURFACE GASKET | 247 | 0066 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE MAIN FACE GASKET | 248 | 0065 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE TOP SURFACE GASKET | 249 | 0064 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE JUMPER TUBE DETAIL | 282 | 0063 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE DETAIL | 289 | 0062 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE RIGHT BRACKET WELDMENT | 293 | 0003 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 1A DETAIL | 301 | 0061 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 2A DETAIL | 302 | 0060 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 3A DETAIL | 303 | 0059 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 4A DETAIL | 304 | 0058 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 1B, 1D & 1F DETAILS | 307 | 0057 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 2B, 2D & 2F DETAILS | 308 | 0056 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 3B, 3D & 3F DETAILS | 309 | 0055 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 4B, 4D & 4F DETAILS | 310 | 0054 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 1C, 1E & 1G DETAILS | 311 | 0053 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 2C, 2E & 2G DETAILS | 312 | 0052 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 3C, 3E & 3G DETAILS | 313 | 0051 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 4C, 4E & 4G DETAILS | 314 | 0050 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 1H DETAILS | 317 | 0049 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 2H DETAILS | 318 | 0048 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 3H DETAILS | 319 | 0047 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE TILE 4H DETAILS | 320 | 0045 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE STANDARD T-BAR | 327 | 0046 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE GASKET DETAIL | 329 | 0044 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE GASKET #2 DETAIL | 330 | 0043 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE DETAIL | 389 | 0042 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 1-STYLE 2 DETAIL SHEET 1 | 401 | 0040 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 1-STYLE 2 DETAIL SHEET 2 | 401 | 0041 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 2-STYLE 2 DETAIL SHEET 1 | 402 | 0038 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 2-STYLE 2 DETAIL SHEET 2 | 402 | 0039 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 3-STYLE 2 DETAIL SHEET 1 | 403 | 0036 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 3-STYLE 2 DETAIL SHEET 2 | 403 | 0037 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 4-STYLE 2 DETAIL SHEET 1 | 404 | 0034 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 4-STYLE 2 DETAIL SHEET 2 | 404 | 0035 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 5-STYLE 2 DETAIL SHEET 1 | 405 | 0032 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 5-STYLE 2 DETAIL SHEET 2 | 405 | 0033 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 6-STYLE 2 DETAIL SHEET 1 | 406 | 0030 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 6-STYLE 2 DETAIL SHEET 2 | 406 | 0031 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 2 DETAIL SHEET1 | 407 | 0027 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 2 DETAIL SHEET2 | 407 | 0028 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 2 DETAIL SHEET3 | 407 | 0029 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE DETAIL SHEET1 | 408 | 0025 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE DETAIL SHEET2 | 408 | 0026 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 1-STYLE 1 DETAIL SHEET1 | 411 | 0023 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 1-STYLE 1 DETAIL SHEET2 | 411 | 0024 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 2-STYLE 1 UPPER DETAIL SHEET 1 | 412 | 0181 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 3-STYLE 1 DETAIL SHEET1 | 413 | 0021 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 3-STYLE 1 DETAIL SHEET2 | 413 | 0022 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 4-STYLE 1 DETAIL SHEET1 | 414 | 0019 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 4-STYLE 1 DETAIL SHEET2 | 414 | 0020 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 5-STYLE 1 DETAIL SHEET1 SHEET 2 MISSING | 415 | 0018 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 1 DETAIL SHEET1 | 417 | 0015 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 1 DETAIL SHEET 2 | 417 | 0016 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 7-STYLE 1 DETAIL SHEET 3 | 417 | 0017 |
| --- | --- | --- |

| INBOARD DIVERTOR TILE ROW 4-STYLE 2 ASSEMBLY | 444 | 0014 |
| --- | --- | --- |

| INBOARD DIVERTOR TILES GASKET ROW 2 LOWER | 486 | 0013 |
| --- | --- | --- |

| PASSIVE PLATE SUPPORT BRACKET WITH WELD EARS ASSEMBLY SHEET 1 | 540 | 0004 |
| --- | --- | --- |

| PASSIVE PLATE SUPPORT BRACKET WITH WELD EARS ASSEMBLY SHEET 2 | 540 | 0005 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE LEFT BRACKET WELDMENT | 541 | 0006 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE RIGHT SUPPORT BRACKET WELDMENT | 542 | 0007 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE LEFT SUPPORT BRACKET WELDMENT | 543 | 0008 |
| --- | --- | --- |

| SECONDARY PASSIVE PLATE FLEX SUPPORT BRACKET VERTICAL WELD EAR | 544 | 0009 |
| --- | --- | --- |

| PRIMARY PASSIVE PLATE RIGID SUPPORT BRACKET VERTICAL WELD EAR | 545 | 0010 |
| --- | --- | --- |

| PRIMARY-SECONDARY PASSIVE PLATE SUPPORT BRACKET RIGHT HORIZONTAL WELD EAR | 546 | 0011 |
| --- | --- | --- |

| PRIMARY-SECONDARY PASSIVE PLATE SUPPORT BRACKET LEFT HORIZONTAL WELD EAR | 547 | 0012 |
| --- | --- | --- |