National Spherical Torus Experiment

NSTX

SYSTEM DESIGN DESCRIPTION

MAGNET SYSTEMS

NSTX-SDD-13-030

Revision 0

February 14, 1997

________________________________

John Spitzer

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 2/14/97 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	1

General Magnet Design Description	1

1.0 Design Description	1

1.1 System Design Features	1

1.1.1 PF/OH Coil Design Features	1

1.1.2 TF Coil System Design Features	5

1.2 Layout	5

1.3 Component Design Features	5

2.0 System Performance	5

a.  Refer to the sections on the individual components.	6

3.0 Interfaces	6

3.1 Interface Points	6

3.2 Interface Requirements	6

4.0 Design Basis	7

4.1 Standards and Codes	7

4.2 Supporting Analysis	8

5.0 Design Implementation	8

5.1 Prototypes	8

5.2 Mock-ups	8

5.3 Component Procurement and Fabrication	9

5.4 System Assembly, Installation and Testing	9

WBS 131 Outer Poloidal Field Coil Design Description	10

1.0 Design Description	10

1.1 System Design Features	10

1.2 Layout	10

1.3 Component Design Features	10

2.0 System Performance	11

3.0 Interfaces	12

3.1 Interface Points	12

3.2 Interface Requirements	13

4.0 Design Basis	13

5.0 Design Implementation	13

5.1 Prototypes	13

5.2 Mock-ups	13

5.3 Component Procurement and Fabrication	13

5.4 System Assembly, Installation and Testing	13

WBS 132 Outer Toroidal Field Coil Design Description	14

1.0 Design Description	14

1.1 System Design Features	14

1.2 Layout	14

1.3 Component Design Features	15

2.0 System Performance	16

3.0 Interfaces	17

3.1 Interface Points	17

3.2 Interface Requirements	17

4.0 Design Basis	17

5.0 Design Implementation	17

5.1 Prototypes	17

5.2 Mock-ups	17

5.3 Component Procurement and Fabrication	17

5.4 System Assembly, Installation and Testing	18

WBS 133 Center Stack Subsystem Design Description	19

1.0 Design Description	19

1.1 System Design Features	19

1.2 Layout	20

1.3 Component Design Features	21

1.3.1 TF Inner Legs	21

1.3.2 OH Coil	23

1.3.3 Inner PF Coils	25

1.3.4 Center Stack Casing	26

2.0 System Performance	27

2.1 TF Coil Inner Leg Performance	27

2.2 OH Coil Performance	28

2.3 Inner PF Coil Performance	29

2.4 Center Stack Casing Performance	29

3.0 Interfaces	31

3.1 Interface Points	31

3.2 Interface Requirements	31

4.0 Design Basis	31

4.1 Standards and Codes	31

4.2 Supporting Analysis	31

5.0 Design Implementation	31

5.1 Prototypes	31

5.2 Mock-ups	31

5.3 Component Procurement and Fabrication	32

5.4 System Assembly, Installation and Testing	33

List of Tables

MS-1.1.1-1	PF/OH Coils	2

MS-1.1.1-2	PF/OH Circuits	2

MS-1.1.1-3	PF/OH Coil Geometry	3

MS-1.1.1-4	PF/OH Circuit 20C Resistances	3

MS-1.1.1-5	PF/OH Mutual Inductance Matrix	3

MS-1.1.1-6	PF/OH Coil Current Ratings	4

MS-1.1.2-1	TF Coil System Electrical Parameters	5

MS-1.1.2-2	TF Coil System Current Ratings	5

MS-3.2-1	Cooling Water Flow Requirements	6

MS-3.2-2	Cooling Water Heat Loads	6

MS-4.2-1	Design Basis Calculations	8

PF-1.3-1	Outer PF Coil Drawings	10

PF-2.0-2	Outer PF Coil Maximum EM Loads	12

PF-2.0-3	Outer PF Coil Mechanical Performance	12

PF-3.1-1	Interfaces with Outer PF Coils	12

TF-1.2-1	TF Coil Outer Leg Layout Drawings	14

TF-1.3-1	TF Coil Outer Leg Drawings	15

TF-1.3-2	TF Coil Outer Leg Dimensional Details	15

TF-1.3-3	TF Coil Outer Leg Conductors	15

TF-1.3-4	TF Coil Outer Leg Insulation Features	16

TF-2.0-1	TF Coil Outer Leg Electrical/Thermal Performance	16

TF-2.0-1	TF Coil Outer Leg Maximum Loads Per Turn	16

TF-2.0-1	TF Coil Outer Leg Mechanical Performance	17

TF-3.1-1	Interfaces with TF Coil Outer Legs	17

CS-1.2-1	Center Stack Layout Drawings	20

CS-1.3.1-1	TF Coil Inner Leg Drawings	21

CS-1.3.1-2	TF Coil Inner Leg Dimensional Details	21

CS-1.3.1-3	TF Coil Inner Leg Conductors	21

CS-1.3.1-4	TF Coil Inner Leg Insulation Features	22

CS-1.3.1-5	TF Bolted Joint Features	22

CS-1.3.2-1	OH Coil Drawings	23

CS-1.3.2-2	OH Coil Dimensional Details	23

CS-1.3.2-3	OH Coil Conductors	24

CS-1.3.2-4	OH Coil Turn Insulation Features	24

CS-1.3.2-5	OH Coil Ground Insulation Features	25

CS-1.3.3-1	Inner PF Coil Drawings	25

CS-1.3.3-2	Inner PF Coil Dimensional Details	25

CS-1.3.3-3	Inner PF Coil Conductors	26

CS-1.3.3-4	Inner PF Coil Turn Insulation Features	26

CS-1.3.3-5	Inner PF Coil Ground Insulation Features	26

CS-2.1-1	TF Coil Inner Leg Electrical/Thermal Performance	27

CS-2.1-2	TF Coil Inner Leg Mechanical Performance	28

CS-2.2-1	OH Coil Electrical/Thermal Performance	28

CS-2.2-2	OH Coil Mechanical Performance	29

CS-2.3-1	Inner PF Coil Electrical/Thermal Performance	29

CS-2.3-2	Inner PF Coil Mechanical Performance	29

CS-3.1-1	Interfaces with the Center Stack Assembly	31

List of figures

MS-1.0-1	Elevation View of NSTX Showing Magnet Systems	1

CS-1.1-1	Center stack Cross-section at the Mid Plane	20

APPENDIX

LIST OF REFERENCES

Introduction

This System Design Description (SDD) describes the design of the Magnet Systems (WBS 13) of the National Spherical Torus Experiment (NSTX).

Figures and Tables are numbered in format X-Y-Z where X=section (i.e. section abbreviation, e.g. PF for Poloidal Field, TF for Toroidal Field, CS for Center Stack), Y=paragraph number, and Z= sequence number within that paragraph.

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix.

Primary reference for this SDD is NSTX-SRD-13-015 [R1].

WBS 13 -  General Magnet Design Description

1.0 Design Description

The magnet systems consist of the outer poloidal field (PF) coils (WBS 131), outer legs of the toroidal field (TF) coils (WBS 132) and the center stack assembly (WBS 133). The center stack assembly includes the inner legs of the TF coils, two additional PF coils (PF1a and 1b), and the OH coil, as well as the center stack casing. See Fig. MS-1.0-1.

Figure MS-1.0-1: Elevation View of NSTX Showing Magnet Systems

1.1 System Design Features

1.1.1 PF/OH Coil Design Features

a. The OH and PF coils consist of three new types, and seven types which were formerly in service on the Spheromak-1 (S-1) machine.

Table MS-1.1.1-1: PF/OH Coils

| Designation | WBS Grouping | Source |
| --- | --- | --- |

| OH | 133 - Center Stack | New |
| --- | --- | --- |

| PF1a | 133 - Center Stack | New |
| --- | --- | --- |

| PF1b | 133 - Center Stack | New |
| --- | --- | --- |

| PF2a | 131 - Outer PF | S-1, EF-1a |
| --- | --- | --- |

| PF2b | 131 - Outer PF | S-1, EF-1b |
| --- | --- | --- |

| PF3a | 131 - Outer PF | S-1, EF-2a |
| --- | --- | --- |

| PF3b | 131 - Outer PF | S-1, EF-2b |
| --- | --- | --- |

| PF4a | 131 - Outer PF | S-1, EF-3a |
| --- | --- | --- |

| PF4b | 131 - Outer PF | S-1, EF-3b |
| --- | --- | --- |

| PF4c | 131 - Outer PF | S-1, EF-3c |
| --- | --- | --- |

b. The OH coil is a continuous solenoid winding extending above and below the mid plane. All of the other coils consist of symmetric upper and lower pairs, with the exception of PF1b which consists of a lower coil only.

c. The coils are connected in independently controllable series circuits as follows:

Table MS-1.1.1-2: PF/OH Circuits

| PF Circuit | Coil Grouping |
| --- | --- |

| 1 | OH |
| --- | --- |

| 2 | PF1a, upper |
| --- | --- |

| 3 | PF1a, lower |
| --- | --- |

| 4 | PF1b, lower |
| --- | --- |

| 5 | PF2a, PF2b, upper |
| --- | --- |

| 6 | PF2a, PF2b, lower |
| --- | --- |

| 7 | PF3a, PF3b, upper |
| --- | --- |

| 8 | PF3a, PF3b, lower |
| --- | --- |

| 9 | PF4a, PF4b, PF4c, upper & lower |
| --- | --- |

d. The OH coil is used for plasma initiation and current drive. The remaining coils are used for plasma equilibrium control and shaping.

e. Single null plasma configurations as well as vertical position control are enabled via the provision for independent control of the upper and lower PF1a, PF2, and PF3 coils.

f. PF1b is a special purpose coil used to form the stringent single null X-point configuration which is required for Coaxial Helicity Injection (CHI)

g. Coil Geometries, circuit resistances, and circuit inductances are summarized in the following Tables MS-1.1.1-1, -2, and -3 [R2].

Table MS-1.1.1-3: PF/OH Coil Geometry

| Coil | R (center) | ∆R | Z (center) | ∆Z | Turns | Turn CSA | Fill |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (cm) | (cm) | (cm) | (cm) |  | (cm^2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH | 12.90 | 4.49 | 106.57 | 213.13 | 482.00 | 1.4139 | 0.7120 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1a | 18.03 | 4.16 | 144.83 | 53.88 | 48.00 | 3.3513 | 0.7177 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1b | 30.48 | 8.48 | 181.88 | 17.13 | 28.00 | 3.3513 | 0.6457 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF2a | 79.92 | 16.27 | 193.35 | 6.80 | 14.00 | 5.8528 | 0.7409 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF2b | 79.92 | 16.27 | 185.26 | 6.80 | 14.00 | 5.8528 | 0.7409 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF3a | 149.45 | 18.64 | 163.35 | 6.80 | 15.00 | 5.8528 | 0.6928 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF3b | 149.45 | 18.64 | 155.26 | 6.80 | 15.00 | 5.8528 | 0.6928 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4a | 178.27 | 6.78 | 72.75 | 6.80 | 5.00 | 5.8528 | 0.6349 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4b | 179.46 | 9.15 | 64.66 | 6.80 | 8.00 | 5.8528 | 0.7525 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4c | 180.65 | 11.53 | 56.57 | 6.80 | 9.00 | 5.8528 | 0.6723 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Note: #Turns in above table are the number above and/or below the mid plane (Z=0)

Table MS-1.1.1-4: PF/OH Circuit 20C Resistances

| Circuit | Resistance |
| --- | --- |

|  | (Ω) |
| --- | --- |

| OH | 9.528E-02 |
| --- | --- |

| PF1au | 2.798E-03 |
| --- | --- |

| PF1al | 2.798E-03 |
| --- | --- |

| PF1b | 2.76E-03 |
| --- | --- |

| PF2u | 4.14E-03 |
| --- | --- |

| PF2l | 4.14E-03 |
| --- | --- |

| PF3u | 8.298E-03 |
| --- | --- |

| PF3l | 8.298E-03 |
| --- | --- |

| PF4 | 1.463E-02 |
| --- | --- |

Table MS-1.1.1-5: PF/OH Mutual Inductance Matrix (mH)

|  | OH | PF1au | PF1al | PF1b | PF2u | PF2l | PF3u | PF3l | PF4 | Plasma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| OH | 1.24E+1 | 6.99E-1 | 6.99E-1 | 3.58E-1 | 2.64E-1 | 2.64E-1 | 2.82E-1 | 2.82E-1 | 4.84E-1 | 1.26E-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1au | 6.99E-1 | 3.76E-1 | 1.95E-4 | 2.26E-4 | 7.23E-2 | 1.35E-3 | 6.04E-2 | 5.29E-3 | 3.90E-2 | 5.22E-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1al | 6.99E-1 | 1.95E-4 | 3.76E-1 | 9.01E-2 | 1.35E-3 | 7.23E-2 | 5.29E-3 | 6.04E-2 | 3.90E-2 | 5.22E-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF1b | 3.58E-1 | 2.26E-4 | 9.01E-2 | 5.36E-1 | 1.66E-3 | 1.86E-1 | 6.61E-3 | 1.01E-1 | 4.93E-2 | 4.96E-4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF2u | 2.64E-1 | 7.23E-2 | 1.35E-3 | 1.66E-3 | 1.98E+0 | 1.03E-2 | 7.31E-1 | 4.11E-2 | 3.08E-1 | 2.68E-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF2l | 2.64E-1 | 1.35E-3 | 7.23E-2 | 1.86E-1 | 1.03E-2 | 1.98E+0 | 4.11E-2 | 7.31E-1 | 3.08E-1 | 2.68E-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF3u | 2.82E-1 | 6.04E-2 | 5.29E-3 | 6.61E-3 | 7.31E-1 | 4.11E-2 | 5.18E+0 | 1.66E-1 | 1.31E+0 | 9.32E-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF3l | 2.82E-1 | 5.29E-3 | 6.04E-2 | 1.01E-1 | 4.11E-2 | 7.31E-1 | 1.66E-1 | 5.18E+0 | 1.31E+0 | 9.32E-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| PF4 | 4.84E-1 | 3.90E-2 | 3.90E-2 | 4.93E-2 | 3.08E-1 | 3.08E-1 | 1.31E+0 | 1.31E+0 | 8.56E+0 | 3.45E-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Plasma | 1.26E-2 | 5.22E-4 | 5.22E-4 | 4.96E-4 | 2.68E-3 | 2.68E-3 | 9.32E-3 | 9.32E-3 | 3.45E-2 | 9.77E-5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

h. In order to satisfy the NSTX operating scenario requirements and range of plasma equilibria, the current ratings of the coil circuits are given in the following Table 3.2-4 [R2, R3].

Table MS-1.1.1-6: PF/OH Coil Current Ratings

| Circuit | Max Current | Max ESW | Max ∫i^2(t)dt | Min Trep |
| --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (A^2-sec) | (sec) |
| --- | --- | --- | --- | --- |

| OH (full inductive) | 24.0 | 0.525 | 3.02E+08 | 600.0 |
| --- | --- | --- | --- | --- |

| OH (partial inductive) | 24.0 | 0.2375 | 1.37E+08 | 300.0 |
| --- | --- | --- | --- | --- |

| PF1au | 15.0 | 5.0 | 1.13E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF1al | 15.0 | 5.0 | 1.13E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF1b | 20.0 | 1.0 | 4.00E+08 | 300.0 |
| --- | --- | --- | --- | --- |

| PF2u | 20.0 | 5.0 | 2.00E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF2l | 20.0 | 5.0 | 2.00E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF3u | 20.0 | 5.0 | 2.00E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF3l | 20.0 | 5.0 | 2.00E+09 | 300.0 |
| --- | --- | --- | --- | --- |

| PF4 | 20.0 | 5.0 | 2.00E+09 | 300.0 |
| --- | --- | --- | --- | --- |

i. The repetition period for the full OH (full bipolar current swing) is 600 seconds, whereas the other coils are rated for a 300 second repetition period. The 600 second period is the requirement for the fully inductive NSTX operation, and the 300 second period for partial and non-inductive operation.  In the partial inductive mode, the OH will be operated with a unipolar, single swing of current.

j. The Equivalent Square Wave (ESW) for the OH reflects the 0.5 second flat top requirement for the inductively driven NSTX scenario. The 1.0 second ESW for the PF1b reflects the short duration of the special x-point required to initiate the CHI. The ESW of the other coils reflect the 4.5 second flat top requirement for partial inductive and non-inductive operation of NSTX, including an allowance for the current rise and fall times.

k. All of the coils will be driven by series/parallel connected phase controlled thyristor converters at D-site. Individual units are rated 1kV (1012.85V) no-load.

l. Nominally, the OH coil will be driven by an anti-parallel set of four series connected converter units, for a total voltage of 4kV. To ensure the availability of adequate loop voltage for plasma initiation under all conditions, an additional 2kV will be available if required. Therefore the coil is rated for 6kV operation [R3].

m. All coils other than the OH will be driven at a maximum voltage of 2kV [R3].

n. All circuits will be grounded via resistors from each terminal to ground, so that (nominally) the maximum voltage to ground will be equal to 1/2 of the voltage applied across the coil terminals. The resistor values (≈ 10kΩ) will be chosen to limit the ground fault current due to a single ground fault. Therefore the terminal voltages to ground may deviate somewhat from their nominal values, and the coils must be insulated to ground based on the maximum applied voltage across the terminals [R4].

o. All of the coils are water cooled. The inlet water temperature is 10 C for the OH, and 20 C for all other coils. Due to the relatively long length of the conductor and hence the cooling path in each of the windings which comprise the OH coil, a 400 psi pressure drop is required to obtain the required flow. A 100 psi pressure drop provide sufficient flow in the other PF coils.

1.1.2 TF Coil System Design Features

a. The TF coil system consists of 36 turns. Each turn consists of a straight inner leg section contained within the central bundle of the center stack, and a bow shaped outer leg. The inner legs are connected to the outer legs via radial flags and demountable connectors.

b. The outer legs are arranged toroidally in 12 groups of 3 turns each.

c. TF coil system electrical parameters are given in Table MS-1.1.2-1 [R5].

Table MS-1.1.2-1: TF Coil System Electrical Parameters

| Parameter | Value |
| --- | --- |

| Resistance @ 20C | 6.52 mΩ |
| --- | --- |

| Inductance | 4.176 mH |
| --- | --- |

d. TF coil system current ratings are given in Table MS-1.1.2-2 [R5].

Table MS-1.1.2-2: TF Coil System Current Ratings

| Field @ R=0.854m | Current/ Turn | Flat Top Time | ESW | Min Trep |
| --- | --- | --- | --- | --- |

| (T) | (kA) | (sec) | (sec) | (sec) |
| --- | --- | --- | --- | --- |

| 0.3 | 35.583 | 5.0 | 6.0 | 300.0 |
| --- | --- | --- | --- | --- |

| 0.6 | 71.166 | 0.6 | 1.5 | 300.0 |
| --- | --- | --- | --- | --- |

e. The TF coil system will be driven by one series layer by four parallel connected phase controlled thyristor converters at D-site. Individual units are rated 1kV (1012.85V) no-load [R4].

f. The TF coil system will be grounded via resistors from each terminal to ground, so that (nominally) the maximum voltage to ground will be equal to 1/2 of the voltage applied across the coil terminals. The resistor values (≈ 10kΩ) will be chosen to limit the ground fault current due to a single ground fault. Therefore the terminal voltages to ground may deviate somewhat from their nominal values, and the coils must be insulated to ground based on the maximum applied voltage across the terminals [R4].

f. The TF coils are water cooled. The inlet water temperature is 20 C. The water enters the bottom of the inner legs, is connected via hoses to the outer legs at the top, and returns through the outer legs.

1.2 Layout

a. Refer to Fig. MS-1.0-1 and Table MS-1.1.1-3.

1.3 Component Design Features

a.  Refer to the sections on the individual components.

2.0 System Performance

a.  Refer to the sections on the individual components.

3.0 Interfaces

3.1 Interface Points

a.  WBS 3, Cooling Water Systems, shall supply all cooling water for the magnets.

b. WBS 3, Cooling Water Systems, shall monitor the flow and temperature of each magnet cooling path.

c. WBS 5, Power Systems, shall supply all electric power to the magnets.

d. WBS 5, Power Systems, shall monitor the current and voltage of each magnet.

e. WBS 3, Auxiliary Systems, shall supply electric power to the center stack casing for bakeout ohmic heating.

3.2 Interface Requirements

a.  Cooling water flow requirements are given in Table MS-3.2-1. OH numbers refer to layer numbers 1-4. OH X and Y terminology refers to the two windings in hand. TF reflects each inner leg turn in series with each outer leg turn [R2,R5].

Table MS-3.2-1: Cooling Water Flow Requirements

| Coil | Max Inlet Temp | Max Outlet Temp | Flow/Ckt | Velocity | # Ckts | ∑Flow | ∆P |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (deg C) | (deg C) | (GPM) | (m/s) |  | (GPM) | (PSI) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH1X | 10.0 | 94.5 | 0.64 | 2.3 | 1.0 | 0.6 | 207.3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH1Y | 10.0 | 94.5 | 0.64 | 2.3 | 1.0 | 0.6 | 207.8 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH2X | 10.0 | 94.5 | 0.71 | 2.5 | 1.0 | 0.7 | 266.3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH2Y | 10.0 | 94.5 | 0.71 | 2.5 | 1.0 | 0.7 | 266.3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH3X | 10.0 | 94.5 | 0.76 | 2.7 | 1.0 | 0.8 | 329.7 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH3Y | 10.0 | 94.5 | 0.76 | 2.7 | 1.0 | 0.8 | 331.8 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH4X | 10.0 | 94.5 | 0.82 | 2.9 | 1.0 | 0.8 | 400.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| OH4Y | 10.0 | 94.5 | 0.82 | 2.9 | 1.0 | 0.8 | 400.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1a | 20.0 | 75.2 | 6.15 | 3.0 | 2.0 | 12.3 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1b | 20.0 | 38.3 | 3.10 | 3.1 | 1.0 | 3.1 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF2a | 20.0 | 50.8 | 5.38 | 2.6 | 2.0 | 10.8 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF2b | 20.0 | 50.8 | 5.38 | 2.6 | 2.0 | 10.8 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF3a | 20.0 | 50.8 | 3.62 | 1.8 | 2.0 | 7.2 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF3b | 20.0 | 50.8 | 3.62 | 1.8 | 2.0 | 7.2 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4a | 20.0 | 50.8 | 6.12 | 3.0 | 2.0 | 12.2 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4b | 20.0 | 50.8 | 4.66 | 2.3 | 2.0 | 9.3 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4c | 20.0 | 50.8 | 4.34 | 2.1 | 2.0 | 8.7 | 100.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| TF | 20.0 | 100.0 | 3.20 | 1.0 | 36.0 | 115.2 | 94.8 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  |  |  | Totals | 61.0 | 202.7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Note: ∆P is the nominal pressure drop required to achieve the required flow rate.

b.  Cooling water flow heat loads are given in Table MS-3.2-2 for the three modes of NSTX operation. TF losses are the same in all three modes (BTF = 0.6T assumed in short flat top, inductive mode) [R2,R5].

Table MS-3.2-2: Cooling Water Heat Loads

| Mode | OH ESW | PF ESW | TF ESW | Pulsed Loss | Rep Period | Avg Loss |
| --- | --- | --- | --- | --- | --- | --- |

|  | (sec) | (sec) | (sec) | (MJ) | (sec) | (kW) |
| --- | --- | --- | --- | --- | --- | --- |

| Inductive | 0.525 | 1.0 | 1.5 | 75.1 | 600.0 | 125.2 |
| --- | --- | --- | --- | --- | --- | --- |

| Partial Inductive | 0.238 | 5.0 | 6.0 | 104.5 | 300.0 | 348.2 |
| --- | --- | --- | --- | --- | --- | --- |

| Non-Inductive | 0.000 | 5.0 | 6.0 | 96.3 | 300.0 | 321.1 |
| --- | --- | --- | --- | --- | --- | --- |

c. WBS 5, Power Systems, shall limit the current, voltage, and ∫i2(t)dt, to ≤ 105% of the values given herein.

d. WBS 3, Auxiliary Systems, shall provide a power supply for ohmic heating of the center stack casing with a maximum input power of 5.3kW. The range of resistance of the center stack casing over the range of temperature during bakeout operations is 1.0 mΩ to TBDmΩ [R6].

4.0 Design Basis

4.1 Standards and Codes

a.  The magnet systems will be designed in accordance with the general design guidelines set forth in the General Requirements Document (GRD) [R7].

b.  The magnet systems will be designed to sustain the design loads as specified in the WBS 13 System Requirements Document (SRD) [R1].

c.  The stress acceptance criteria are primarily based on the ASME Boiler and Pressure Vessel Code, Section III, Division I [R8].  Based on the linear finite element analysis, the ASME code requires that the Tresca stress of the 2-D or 3-D element shall meet the following stress limits:

- General primary membrane stress shall not exceed 1.0 K Sm

- Local primary membrane stress shall not exceed 1.5 K Sm

- Primary membrane plus bending stresses shall not exceed 1.5 K Sm

- Total primary plus secondary stresses shall not exceed 3.0 K Sm

(Where the K factor is 1.0 for normal operating conditions or 1.1 in case of the seismic force is involved.  Sm is the design stress intensity value defined in the ASME Boiler and Pressure Vessel code.)

d.  Since the von Mises stress is generally more accurate than the Tresca stress in the representation of the equivalent one-dimensional stress and the difference between the two is small, it will be used instead of the Tresca stress in determining the stress acceptance criteria.

e.  The design stress intensity value is set at 2/3 of the minimum specified yield at temperature.

f.  For bolting material, the average tensile stress shall not exceed 1.0 Sm and maximum direct tensile plus bending stress shall not exceed 1.5 Sm.

4.2 Supporting Analysis

a. Calculations listed in Table MS-4.2-1 provide the design basis for WBS 13.

Table MS-4.2-1: Design Basis Calculations

| Calculation Title | File Number | Ref. |
| --- | --- | --- |

| Plasma Initiation | NSTX-CALC-72-1 | [R9] |
| --- | --- | --- |

| Plasma Disruption Analysis | NSTX-CALC-13-3 | [R10] |
| --- | --- | --- |

| Center Stack Halo Current Analysis | NSTX-CALC-13-4 | [R11] |
| --- | --- | --- |

| PF/OH Coil Electrical Parameters | NSTX-CALC-13-1 | [R2] |
| --- | --- | --- |

| TF Coil Electrical Parameters | NSTX-CALC-13-2 | [R5] |
| --- | --- | --- |

| OH Electrical Performance Analysis | NSTX-CALC-53-1 | [R12] |
| --- | --- | --- |

| TF Electrical Performance Analysis | NSTX-CALC-52-1 | [R13] |
| --- | --- | --- |

| TF Electrical Joint Analysis | NSTX-CALC-13-5 | [R14] |
| --- | --- | --- |

| OH Coil Thermal/Hydraulic Analysis | NSTX-CALC-13-6 | [R15] |
| --- | --- | --- |

| TF Coil Thermal/Hydraulic Analysis | NSTX-CALC-13-7 | [R16] |
| --- | --- | --- |

| Center Stack PFC Thermal Analysis | NSTX-CALC-13-8 | [R17] |
| --- | --- | --- |

| Integrated Thermal Analysis, Normal Operation | NSTX-CALC-13-9 | [R18] |
| --- | --- | --- |

| Integrated Thermal Analysis, Bakeout | NSTX-CALC-13-10 | [R19] |
| --- | --- | --- |

| OH Coil Stress Analysis | NSTX-CALC-13-11 | [R20] |
| --- | --- | --- |

| TF Coil Stress Analysis | NSTX-CALC-13-12 | [R21] |
| --- | --- | --- |

| Center Stack Stress Analysis | NSTX-CALC-13-13 | [R22] |
| --- | --- | --- |

| S-1 Spheromak Project Engineering Handbook, R1 |  | [R23] |
| --- | --- | --- |

b. A preliminary Failure Modes and Effects Analysis (FMEA) has been performed [R27]. This will be revised as the designs of the ancillary systems progress.

5.0 Design Implementation

5.1 Prototypes

Refer to the sections describing the individual subsystems

5.2 Mock-ups

Refer to the sections describing the individual subsystems

5.3 Component Procurement and Fabrication

Refer to the sections describing the individual subsystems

5.4 System Assembly, Installation and Testing

Refer to the sections describing the individual subsystems

WBS 131 Outer Poloidal Field Coil Design Description

1.0 Design Description

1.1 System Design Features

a. The EF coils from the S-1 Spheromak are used for the outer PF coils.

1.2 Layout

a.  The outer PF coils are located symmetrically about the horizontal centerline of the device.

b.  PF-2 and PF-3 are mounted to the vessel domes.  PF-2 is mounted beneath the vessel umbrella structure.

c.  PF-4 is supported from the cylindrical portion of the vessel.

1.3 Component Design Features

a. Refer to the drawings listed in Table PF-1.3-1.

Table PF-1.3-1: Outer PF Coil Drawings

| Coil | Dwg. Title | Dwg. # |
| --- | --- | --- |

| PF2a & PF2b | EF Coil 1A & 1B 14 Turn Coil Assembly | 6003-E-30000 |
| --- | --- | --- |

| PF3a & PF3b | EF Coil 2A & 2B - 15 Turn Coil Assembly | 6003-E-30015 |
| --- | --- | --- |

| PF4a | EF Coil #3C 5 Turn Coil Assembly | 6003-E-30061 |
| --- | --- | --- |

| PF4b | EF Coil #3B 8 Turn Coil Assembly | 6003-E-30056 |
| --- | --- | --- |

| PF4c | EF Coil #3A - 9 Turn Coil Assembly | 6003-E-30035 |
| --- | --- | --- |

b. Coil dimensional details are given in Table PF-1.3-2. Refer also to Table MS-1.1.1-3.

Table PF-1.3-2: Outer PF Coil Dimensional Details

| Coil | Cond Width | Cond Height | Hole Dia | Turn-Turn | Turn-Ground |
| --- | --- | --- | --- | --- | --- |

|  | (in) | (in) | (in) | (in) | (in) |
| --- | --- | --- | --- | --- | --- |

| PF2a/b | 0.802 | 1.272 | 0.356 | 0.132 | 0.188 |
| --- | --- | --- | --- | --- | --- |

| PF3a/b | 0.802 | 1.272 | 0.356 | 0.132 | 0.188 |
| --- | --- | --- | --- | --- | --- |

| PF4a/b/c | 0.802 | 1.272 | 0.356 | 0.132 | 0.188 |
| --- | --- | --- | --- | --- | --- |

c. Refer to Tables MS-1.1.1-4 and -5 for electrical characteristics.

d. Prior to winding the conductors were stretched to raise their yield strength to ~12ksi.

e. Eight layers of 0.00325-in. Mylar tape followed by four layers of 0.010-in. thick epoxy-glass tape are used for the turn insulation.

f. Ground wall insulation consists of 0.188-in. thick wrap made using epoxy-glass tape.

g.  Each outer PF coil is supported from the vacuum vessel using a sliding joint mechanism to prevent loading of the coils during vacuum vessel bakeout .

2.0 System Performance

a.  Electrical/thermal performance is given in Table PF-2.0-1 [R2].

Table PF-2.0-1: Outer PF Coil Electrical/Thermal Performance

| Coil | Imax | ESW | Tfinal | Tallowable | Vmax | Insulation Emax | Insulation Allowable E |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (deg C) | (deg C) | (V) | (volts/mil) | (volts/mil) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF2a/b | 20.0 | 5.0 | 50.8 | 120.0 | 2025.7 | 15.3 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF3a/b | 20.0 | 5.0 | 50.8 | 120.0 | 2025.7 | 15.3 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF4a/b/c | 20.0 | 5.0 | 50.8 | 120.0 | 2025.7 | 15.3 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

b. Temperatures given in Table PF-2.0-1 are based on adiabatic conditions.

c. PF2a and PF2b coils are identical and always carry the same series current. The same is true of PF3a and PF3b. PF4a, b, and c are each of different dimensions but always carry the same series current. Maximum electromagnetic and dead weight loads on the coils/bundles are summarized in Table PF-2.0-2.

Table PF-2.0-2: Outer PF Coil Maximum EM Loads

| Coil/Bundle | Hoop Tension on Coil/Bundle | Axial EM Load on Bundle, Toward Mid plane | Axial EM Load on Bundle, Away From Mid plane | Dead Weight Load |
| --- | --- | --- | --- | --- |

|  | (lbs) | (lbs) | (lbs) | (lbs) |
| --- | --- | --- | --- | --- |

| PF2a/b bundle | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF3a/b bundle | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF4a/b/c bundle | n.a. | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF4a coil | TBD | n.a. | n.a. | n.a. |
| --- | --- | --- | --- | --- |

| PF4b coil | TBD | n.a. | n.a. | n.a. |
| --- | --- | --- | --- | --- |

| PF4c coil | TBD | n.a. | n.a. | n.a. |
| --- | --- | --- | --- | --- |

d. Mechanical performance is summarized in Table PF-2.0-3.

Table PF-2.0-3: Outer PF Coil Mechanical Performance

| Coil/Bundle | Cond vonMises Stress max | Cond Allowable vonMises Stress | Insulation Shear Stress max | Insulation Shear Allowable @Tallowable |
| --- | --- | --- | --- | --- |

|  | (ksi) | (ksi) | (ksi) | (ksi) |
| --- | --- | --- | --- | --- |

| PF2a/b | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF3a/b | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF4a | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF4b | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

| PF4c | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- |

3.0 Interfaces

3.1 Interface Points

Table PF-3.1-1: Interfaces with the Outer PF Coils

| WBS | Interface Point |
| --- | --- |

| 11 | None. |
| --- | --- |

| 12 | The PF coils interface with the vacuum vessel at the PF supports. |
| --- | --- |

| 13 | Not Applicable |
| --- | --- |

| 2x | None |
| --- | --- |

| 3x | The outer PF coils will be cooled by water supplied by the cooling water system. Interface points are at fitting on coil conductors. |
| --- | --- |

| 4x | None |
| --- | --- |

| 5x | The outer PF coils receive power from the PF/OH power system. Interface points are at terminals of series connected coil groups. |
| --- | --- |

| 6x | None. |
| --- | --- |

3.2 Interface Requirements

a. See section MS-3.2.

4.0 Design Basis

a.  The design basis and supporting analyses for the outer PF coils are given in the S-1 Spheromak Project Engineering Handbook [R23].

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

None

5.3 Component Procurement and Fabrication

The outer PF coils already exist.

5.4 System Assembly, Installation and Testing

The outer PF coils will be removed from the Spheromak domes so that their holders can be modified.  Final installation on the vacuum vessel is the responsibility of WBS 8.

WBS 132 Outer Toroidal Field Coil Design Description

1.0 Design Description

1.1 System Design Features

a.  The outer TF coils (a.k.a. the "outer legs") consist of twelve bundles of three turns each.

b.  The deadweight of the outer legs is supported through the tokamak support structure to the floor.

c.  Each of the twelve legs is mechanically connected to the adjacent legs via an upper and lower "hoop".  The hoop is connected to the vessel through cables.  The cables and hoop form a triangular arrangement which will react the TF EM loads and seismic loads into the vessel.  The cables will also allow expansion of the vessel during bakeout without loading the outer TF legs.

d.  Connection to the inner TF conductors is made through demountable, flexible connectors, and flags which extend from the connectors to the inner legs.  The use of the flags allows the TF inner leg bundle outside diameter to be smaller than the inside diameter of the OH tension tube, allowing the former to be inserted into the latter. The use of the connectors facilitates the removal of the ceramic insulator, if required, without disturbing the inner or outer TF legs.

e.   The connectors are made of laminated copper strips.  The resultant flexibility provides mechanical isolation between the connectors and the radial loads resulting from the magnetic force on the outer legs.  Turn to turn transitions are also accomplished by these links.  Some of the transitions are made on the upper level, and some on the lower. At each level, some of the transitions proceed clockwise in a toroidal direction and others anti-clockwise. By careful adjustment of the angle of the central bundle with respect to the outer legs the net toroidal current component, and resultant field error, is nullified.

f. The flags will be machined and formed from CDA-104 copper plate.  The flags and connectors are 1” x 5” in cross-section (effective cross section is some what less due to bolt holes through the flags and gaps between the connector laminations).  Current density and heating is low;  consequently, direct water cooling is unnecessary.  They are cooled primarily by conduction to the adjacent water-cooled inner and outer legs.

1.2 Layout

a.  Refer to the layout drawings listed in Table TF-1.2-1.

Table TF-1.2-1: TF Coil Outer Leg Layout Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

1.3 Component Design Features

a. Refer to the drawings listed in Table TF-1.3-1.

Table TF-1.3-1: TF Coil Outer Leg Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

| TF System General Arrangement | 13.E0105.0 |
| --- | --- |

b. Coil dimensional details are given in Table TF-1.3-2.

Table TF-1.3-2: TF Coil Outer Leg Dimensional Details

| Element | Cond Width | Cond Height | Hole Dia | Cond CSA | Turn-Turn | Turn-Ground |
| --- | --- | --- | --- | --- | --- | --- |

|  | (in) | (in) | (in) | (in^2) | (in) | (in) |
| --- | --- | --- | --- | --- | --- | --- |

| TF Outer Leg | 2.000 | 3.000 | 0.500 | 5.790 | 0.064 | 0.250 |
| --- | --- | --- | --- | --- | --- | --- |

| Flag | 1.000 | 5.000 | 4 @ 0.75 | 4.555 | n.a. | n.a. |
| --- | --- | --- | --- | --- | --- | --- |

| Connector | 1.000 | 5.000 | n.a. | 4.997 | n.a. | n.a. |
| --- | --- | --- | --- | --- | --- | --- |

c. The outer leg conductors are roll-formed from copper bus conductor. Features are listed in Table TF-1.3-3.  Copper tubing soldered into a machined groove provides cooling.  Note that the outer leg conductors, with their very low current density,  would be adequately cooled by air.  Water cooling is provided since the cost is low and it provides for the long pulse upgrade possibility.

Table TF-1.3-3: TF Coil Outer Leg Conductors

| Feature | Description |
| --- | --- |

| Alloy | CDA 104 (silver bearing oxygen free copper) |
| --- | --- |

| Yield Strength | 30 ksi min |
| --- | --- |

| Ultimate Strength | 38 ksi min |
| --- | --- |

| Dimensions | 2" x 3" |
| --- | --- |

| Net Cross Sectional Area | 5.79 in2 |
| --- | --- |

| Cooling Tube | 1/2" ID |
| --- | --- |

d. The TF outer leg insulation consists of a B-Stage epoxy glass wrap turn and ground insulation. Main features are given in Table TF-1.3-4.

Table 1.3-4: TF Coil Outer Leg Insulation Features

| Feature | Description |
| --- | --- |

| Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (turn material, 10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers (turn ) | 3 half-lapped |
| --- | --- |

| Nominal Thickness (ground material, 10% compression) | TBD |
| --- | --- |

| Number of layers (ground) | TBD half-lapped |
| --- | --- |

| Copper Surface Preparation | Grit blast, solvent wipe, Ciba Geigy DZ80-1 primer |
| --- | --- |

| Turn Insulation Build | 0.032" (0.064" turn-turn) |
| --- | --- |

| Ground Insulation Build | 0.250" (0.282" turn-ground) |
| --- | --- |

2.0 System Performance

a.  Electrical/thermal performance is given in Table TF-2.0-1 [R5,R13].

Table TF-2.0-1: TF Coil Outer Leg Electrical/Thermal Performance

| Element | Imax | ESW | Tfinal | Tallowable | Vmax | Insulation Emax | Insulation Allowable E |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (deg C) | (deg C) | (V) | (volts/mil) | (volts/mil) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| TF Outer Leg | 71.2 | 1.5 | 22.6 | 120.0 | 1012.9 | 15.8 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Flag | 71.2 | 1.5 | 24.3 | 120.0 | 1012.9 | n.a. | n.a. |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Connector | 71.2 | 1.5 | 23.5 | 120.0 | 1012.9 | n.a. | n.a. |
| --- | --- | --- | --- | --- | --- | --- | --- |

b. Temperatures given in Table TF-2.0-1 are based on adiabatic conditions.

c. Maximum per turn electromagnetic and dead weight loads on the outer legs are summarized in Table TF-2.0-2 [R21].

Table TF-2.0-2: TF Coil Outer Leg Maximum Loads per Turn

| Peak Torsional Load | Net Torsional Load Above/ Below Mid plane | Tensile Load | Dead Weight Load |
| --- | --- | --- | --- |

| (lbs/in) | (lbs) | (lbs) | (lbs) |
| --- | --- | --- | --- |

| TBD | TBD | TBD | TBD |
| --- | --- | --- | --- |

d. Mechanical performance is summarized in Table TF-2.0-3 [R21].

Table TF-2.0-3: TF Coil Outer Leg Mechanical Performance

| Cond vonMises Stress max | Cond Allowable vonMises Stress | Insulation Shear Stress max | Insulation Shear Allowable @Tallowable |
| --- | --- | --- | --- |

| (ksi) | (ksi) | (ksi) | (ksi) |
| --- | --- | --- | --- |

| TBD | TBD | TBD | TBD |
| --- | --- | --- | --- |

3.0 Interfaces

3.1 Interface Points

Table TF-3.1-1: Interfaces with TF Coil Outer Legs

| WBS | Interface Point |
| --- | --- |

| 11 | None. |
| --- | --- |

| 12 | The vessel and tokamak support structure must support all loads on the outer TF legs. |
| --- | --- |

| 13 | Not Applicable |
| --- | --- |

| 2x | None |
| --- | --- |

| 3x | The outer TF coils will be cooled by water supplied by the cooling water system. Interface points are at fitting on coil conductors. |
| --- | --- |

| 4x | None |
| --- | --- |

| 5x | The outer TF coils receive power from the TF power system. Interface points are at terminals of series connected turns. |
| --- | --- |

| 6x | None. |
| --- | --- |

3.2 Interface Requirements

a. See section MS-3.2.

4.0 Design Basis

a. See section MS-4.0.

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

None

5.3 Component Procurement and Fabrication

a. Outer TF coil legs will be procured via a stand alone specification.

5.4 System Assembly, Installation and Testing

a. Installation of the outer TF legs is the responsibility of WBS 8.

WBS 133 Center Stack Subsystem Design Description

1.0 Design Description

1.1 System Design Features

See Figure CS-1.1-1.

a.  The center stack assembly consists of the inner legs of the TF coils, the ohmic heating solenoid, the inner PF coils (PF1a and PF1b), and the center stack casing.

b. The TF inner legs consist of an inner layer of 12 turns and an outer layer of 24 turns. All turns have approximately equal cross sectional area.

c. Surrounding the TF inner leg bundle is the Ohmic Heating (OH) coil.  This coil generates the loop voltage to initiate, and flux swing to drive, the plasma current.

d.	The OH coil is wound over a Tension Cylinder which reacts axial electromagnetic loads on the OH coil but is free to move with respect to the TF bundle.

e. 	Surrounding the OH coil just inboard of its ends are the PF1a Upper and Lower coils, whose function is shape control.

f. 	PF1b,  located at the lower end of the CS assembly, forms the magnetic geometry required for Coaxial Helicity Injection (CHI).

g. 	Magnetic Diagnostics including flux loops and rogowski coils are installed on the surface of the OH solenoid, along with Thermocouples to measure the surface temperature of the insulation (to be accommodated by WBS 133, but not part of the scope of work of WBS 133).

h. 	The OH solenoid, magnetic diagnostics and thermocouples are over-wrapped with a  Thermal Insulation Jacket to provide shielding from heat loads emanating inward from the vessel due to heating of the inner wall during plasma operations and due to heating during bakeout.

i. 	A Center Stack Casing encloses the entire assembly described above, and forms the inner vacuum vessel wall.

j. Ceramic Insulators are provided at both ends of the CS casing for electrical isolation from the outer vacuum vessel. During CHI operations the CS casing potential may reach 2kV [R4].

Figure CS-1.1-1:  Center Stack Cross-section at Mid Plane

1.2 Layout

a.  Refer to the layout drawings listed in Table CS-1.2-1.

Table CS-1.2-1: Center Stack Layout Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

| Center Stack Top @ 0-90 Layout | 13.E0101.0 |
| --- | --- |

| Center Stack Bottom @ 0-90 Layout | 13.E0102.0 |
| --- | --- |

| Radial Build Bottom TF/OH/PF Coils Layout | 13.E0103.0 |
| --- | --- |

| Center Stack Torque Restraint Structure General Arrangement | 13.E0106.0 |
| --- | --- |

| Center Stack Torque Restraint Structure Lower Flag Layout | 13.E0108.0 |
| --- | --- |

| Center Stack Upper Flag Support Layout | 13.E0109.0 |
| --- | --- |

| TF-OH-PF1a Center Stack Assembly - Bottom | 13.E0017.0 |
| --- | --- |

| TF-OH-PF1a Center Stack Assembly - Top | 13.E0018.0 |
| --- | --- |

1.3 Component Design Features

1.3.1 TF Inner Legs

a. Refer to the drawings listed in Table CS-1.3.1-1.

Table CS-1.3.1-1: TF Coil Inner Leg Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

| TF Inner Core Conductor/Insulation Arrangement | 13.E0100.0 |
| --- | --- |

| TF Core Inner Conductor Details | 13.E0025.0 |
| --- | --- |

| TF Core Outer Conductor Details | 13.E0026.0 |
| --- | --- |

| TF Inner Core Assembly | 13.E0015.0 |
| --- | --- |

| TF Inner Core Quadrant Assembly | 13.E0016.0 |
| --- | --- |

b. Coil dimensional details are given in Table CS-1.3.1-2.

Table CS-1.3.1-2: TF Coil Inner Leg Dimensional Details

| Element | Hole Dia | Effective Cond CSA | Turn-Turn | Turn-Ground |
| --- | --- | --- | --- | --- |

|  | (in) | (in^2) | (in) | (in) |
| --- | --- | --- | --- | --- |

| Inner Leg, Inner Layer | 0.25 | 1.054 | 0.064 | 0.064 |
| --- | --- | --- | --- | --- |

| Inner Leg, Outer Layer | 0.25 | 1.045 | 0.064 | 0.064 |
| --- | --- | --- | --- | --- |

c. The inner leg conductors are machined from rolled bus conductor, with properties as listed in Table CS-1.3.1-3.  The conductors of the inner layer are cut in profile with slight “flags” on both ends.   This “radial expansion” of the turns results in the  bolting surfaces  for both tiers of flags at the same diameter.  The outer layer conductors are simply of constant cross-section along their entire lengths.

Table CS-1.3.1-3: TF Coil Inner Leg Conductors

| Specification | Requirement |
| --- | --- |

| Alloy | CDA 104 (silver bearing oxygen free copper) |
| --- | --- |

| Yield Strength | 30 ksi |
| --- | --- |

| Ultimate Strength | 38 ksi |
| --- | --- |

| Net Cross Sectional Area, Inner Layer | 1.054 sq. in. |
| --- | --- |

| Net Cross Sectional Area, Outer Layer | 1.045 sq. in. |
| --- | --- |

| Cooling Tube | 5/16" OD tubing, 1/4" ID |
| --- | --- |

d. Cooling will be provided by a soldered in place copper tubing.

e. The TF inner leg insulation consists of a B-Stage epoxy glass wrap turn and ground insulation. Main features are given in Table CS-1.3.1-4.

Table CS-1.3.1-4: TF Coil Inner Leg Insulation Features

| Feature | Description |
| --- | --- |

| Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers | 3 half-lapped |
| --- | --- |

| Copper Surface Preparation | Grit blast, solvent wipe, Ciba Geigy DZ80-1 primer |
| --- | --- |

| Turn Insulation Build | 0.032" (0.064" turn-turn) |
| --- | --- |

| Ground Insulation Build | 0.032" |
| --- | --- |

f. Bolted joints connect the inner legs to the radial flags.  Four 5/16” type A286 stainless steel bolts are used, with their length extending the full length of the flag to provide elastic preload on the joint which will track thermal expansions and variations in axial loads.  These bolts are “body bound” (i.e., tight fitting) at the joint interface so they react shear loads without slippage.  They are torqued to provide a preload of 2000 psi.   Stainless steel “Keensert” thread inserts are used to provide a hardened bolt interface with improved pull-out strength.

g. Main features of the TF inner leg/flag bolted joint are given in Table 1.3.1-5.

Table CS-1.3.1-5: TF Bolted Joint Features

| Feature | Description |
| --- | --- |

| Size & Number of Bolts | (4) 5/16" bolts |
| --- | --- |

| Bolt Material | A286 |
| --- | --- |

| Bolt Length | 11 in. |
| --- | --- |

| Nominal Joint Pressure | 2000 psi |
| --- | --- |

| Allowable Contact Resistance | 2µΩ/in2 |
| --- | --- |

| Average Current Density @ 71.6kA | 14.3kA/in2 |
| --- | --- |

| Load per Bolt | 2500 lb |
| --- | --- |

| Axial Stress in Bolts | 43000 psi |
| --- | --- |

| Shear Stress in Bolts | 2300 psi |
| --- | --- |

| Pullout Yield Load for Insert | 5000 lb |
| --- | --- |

| Factor of Safety for Pullout | 2 |
| --- | --- |

h.	A stainless steel torque reaction collar is bonded to the OD of the TF inner leg bundle just above the ends of the OH tension cylinder via glass epoxy which acts in shear.  A bolted flange connection  transmits the torque from the collar to the hub portion of the restraint structure.

i. 	Both “tiers” of flags are tightly shimmed in the vertical direction  between the two discs in the hub portion of the restraint structure. In this manner, the entire circular array of radial flags clamped between upper and lower discs together form two tiers of thick discs.

j. 	Vertical loads on the disc result in simple shearing loads across bolts of the joints.  The bolts are shoulder bolts designed to act in shear and resist slippage at the joint. The shearing load is absorbed by a total of 36*4, or 144 bolts.

k.	G-10 wedges are installed between the flags so the entire array of flags behave as a solid disc.  The resulting torque is reacted through the restraint structure to the umbrella structure which is welded to the vacuum vessel.  The connection between the CS restraint structure is free to move vertically (to permit free thermal expansion of the central bundle) but fixed in torsion.

l.	The TF bundle is supported from the Test Cell floor via support pedestals.  This support arrangement, in conjunction with the flexible links between the outer legs and the bundle,  permits the bundle to freely expand upwards.

1.3.2 OH Coil

a. Refer to the drawings listed in Table CS-1.3.2-1.

Table CS-1.3.2-1: OH Coil Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

| OH Coil Conductor Detail | 13.A0010.0 |
| --- | --- |

| Tension Tube Machining/Assembly | 13.D0005.0 |
| --- | --- |

| OH Coil/Tension Tube Final Assembly Section @ 0-180 | 13.E0001.0 |
| --- | --- |

| OH Coil/Tension Tube Final Assembly Section @ 90 & 270 | 13.E0002.0 |
| --- | --- |

| OH Coil/Tension Tube Final Assembly End View Sections | 13.E0003.0 |
| --- | --- |

b. The OH coil consists of four layers, wound two-in-hand, for a total of eight windings. There are 962 turns total, and the coil produces a flux swing of 0.62 volt-seconds with a current swing from +24 to -24kA [R2].

c. Each winding consists of a continuous length (360' max.) of extruded copper conductor with circular central cooling hole, grit blasted and coated with a primer, then insulated with two half-lapped layers of Kapton tape followed by two half-lapped layers of a B-staged epoxy glass tape.

d. The conductor cross section is extruded with a slightly non-rectangular (keystoned) shape such that after winding the desired rectangular shape and is obtained.

e. The leads of each winding exit the winding pack at various elevations and toroidal angles in order to facilitate their series connection and connection to the incoming/outgoing bus bars. Thus the heights of the layers vary, with the inner most layer the tallest and the outer most the shortest. The net number of turns above and below the mid plane is balanced.

f. As a result of the winding pattern and the series connection the maximum voltages which appears between turns in the same layer and between layers are 25% and 75% of the total terminal voltage, respectively.

g. OH dimensional details are given in Table CS-1.3.2-2.

Table CS-1.3.2-2: OH Coil Dimensional Details

| Cond Width | Cond Height | Hole Dia | Turn-Turn | Turn-Ground |
| --- | --- | --- | --- | --- |

| (in) | (in) | (in) | (in) | (in) |
| --- | --- | --- | --- | --- |

| 0.394 | 0.630 | 0.188 | 0.064 | 0.064 |
| --- | --- | --- | --- | --- |

h. OH conductor details are listed in Table CS-1.3.2-3.

Table CS-1.3.2-3: OH Coil Conductors

| Specification | Requirement |
| --- | --- |

| Alloy | CDA 104 (silver bearing oxygen free copper) |
| --- | --- |

| Yield Strength | 36 ksi |
| --- | --- |

| Ultimate Strength | 42 ksi |
| --- | --- |

| Net Cross Sectional Area | 0.2192 sq. in. |
| --- | --- |

i. The winding is formed over an inconel tube which serves as a tension cylinder to react axial loads.  Therefore the ground wall insulation on the bore of the coil, consisting of three half-lapped layers of B-staged epoxy glass tape, is applied first over the tube prior to winding.

j. The conductor will be grit blasted and coated with a primer, then insulated with two half-lapped layers of Kapton tape followed by two half-lapped layers of a B-staged epoxy glass tape.

k. The ground wall over the outside of the coil is obtained via taping in the same manner as applied over the tension cylinder. The ground wall over the ends of the coil is obtained using glass reinforced epoxy end caps. The insulation at the ends is carefully tailored to ensure a homogeneous ground wall insulation.  After cure the ground wall is painted with a conducting varnish to form an electrostatic shield (ground plane).

l. Main features of the electrical turn insulation are given in Table CS-1.3.2-4, and the ground insulation in Table CS-1.3.2-5.

Table CS-1.3.2-4: OH Coil Turn Insulation Features

| Feature | Description |
| --- | --- |

| Kapton Material | Kapton w/silicone adhesive |
| --- | --- |

| Nominal Thickness | 0.0025" (0.0635 mm) |
| --- | --- |

| Number of Layers | 2 half-lapped |
| --- | --- |

| B-Stage Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers | 2 half-lapped |
| --- | --- |

| Copper Surface Preparation | Grit blast, solvent wipe, Ciba Geigy DZ80-1 primer |
| --- | --- |

| Turn Insulation Build | 0.032" (0.064" turn-turn) |
| --- | --- |

Table CS-1.3.2-5: OH Coil Ground Insulation Features

| Feature | Description |
| --- | --- |

| B-Stage Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers | 3 half-lapped |
| --- | --- |

| Ground Insulation Build | 0.032" (0.064" turn-ground) |
| --- | --- |

m. The OH coil is supported from the Test Cell floor via support pedestals.  It is free to expand vertically upwards independent from the TF bundle, tension cylinder and center stack casing.

n. The OH coil is independently supported so that TF/OH/PF1a/PF1b differential thermal excursions are all de coupled.

o. The OH solenoid is vertically constrained and preloaded via the tension cylinder so that net axial loads do not result in tension between the solenoid turns or rebounding.

1.3.3 Inner PF Coils

a. Refer to the drawings listed in Table CS-1.3.3-1.

Table CS-1.3.3-1: Inner PF Coil Drawings

| Dwg. Title | Dwg. # |
| --- | --- |

| PF1a & PF1b Conductor Detail | 13.A0011.0 |
| --- | --- |

| PF1a Coil Section @ 90 & 270 Final Assembly | 13.E0012.0 |
| --- | --- |

| PF1a Coil Sections Showing Transition & Leads | 13.E0013.0 |
| --- | --- |

| PF1b Coil Final Assembly | 13.E0006.0 |
| --- | --- |

| PF1b Final Assembly Winding Detail | 13.E0007.0 |
| --- | --- |

a. The PF1a and PF1b coils are 2 layer,  48 turn and 4 layer, 28 turn coils, respectively.

b. The PF1a coils are located symmetrically above and below the mid plane; the PF1b coil is a single (lower only) coil.

c. Inner PF Coil dimensional details are given in Table CS-1.3.3-2.

Table CS-1.3.3-2: Inner PF Coil Dimensional Details

| Cond Width | Cond Height | Hole Dia | Turn-Turn | Turn-Ground |
| --- | --- | --- | --- | --- |

| (in) | (in) | (in) | (in) | (in) |
| --- | --- | --- | --- | --- |

| 0.787 | 0.787 | 0.354 | 0.064 | 0.064 |
| --- | --- | --- | --- | --- |

d. Inner PF Coil conductor details are listed in Table CS-1.3.3-3.

Table CS-1.3.3-3: Inner PF Coil Conductors

| Specification | Requirement |
| --- | --- |

| Alloy | CDA 104 (silver bearing oxygen free copper) |
| --- | --- |

| Yield Strength | 36 ksi |
| --- | --- |

| Ultimate Strength | 42 ksi |
| --- | --- |

| Net Cross Sectional Area | 0.5195 sq. in. |
| --- | --- |

e. Main features of the electrical turn insulation are given in Table CS-1.3.3-4, and the ground insulation in Table CS-1.3.3-5.

Table CS-1.3.3-4: Inner PF Coil Turn Insulation Features

| Feature | Description |
| --- | --- |

| Kapton Material | Kapton w/silicone adhesive |
| --- | --- |

| Nominal Thickness | 0.0025" (0.0635 mm) |
| --- | --- |

| Number of Layers | 2 half-lapped |
| --- | --- |

| B-Stage Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers | 2 half-lapped |
| --- | --- |

| Copper Surface Preparation | Grit blast, solvent wipe, Ciba Geigy DZ80-1 primer |
| --- | --- |

| Turn Insulation Build | 0.032" (0.064" turn-turn) |
| --- | --- |

Table CS-1.3.3-5: Inner PF Coil Ground Insulation Features

| Feature | Description |
| --- | --- |

| B-Stage Material | Cryogenic Materials Inc. CTD-112 P |
| --- | --- |

| Nominal Thickness (10% compression) | 0.0054” (0.137 mm) |
| --- | --- |

| Number of layers | 3 half-lapped |
| --- | --- |

| Ground Insulation Build | 0.032" (0.064" turn-ground) |
| --- | --- |

f. An array of gussets attach the inner PF coils to the end flange of the OH solenoid tension tube.

g. The inner PF coils are independently supported so that TF/OH/PF1a/PF1b differential thermal excursions are all de coupled.

1.3.4 Center Stack Casing

a.  The center stack casing has an overall height of 3.75-m. and is fabricated from 0.4-cm. thick Inconel.

b. A thermal insulation jacket is required to protect the OH and PF1a coils during bakeout and from heat loads during operation, and to provide electrical insulation (the center stack casing can go to ≤ 2kV during CHI operations).  A thermal insulating material known as Microtherm [R24] was chosen.  Microtherm insulation uses a finely divided silica particle network to create a microporous construction.  Its ability to operate continuously at

temperatures well above 350°C, its very low thermal conductivity, similar to still air (~0.025 W/m-°K), and its good dielectric strength (3.5kV breakdown voltage @ 400C for 2mm thickness, minimum 6mm thickness will be used), provides the required thermal and electrical insulation.

c.  Adequate clearances are maintained between the various center stack components to facilitate insertion of the individual coil assemblies into the center stack case.  These gaps are left unfilled after assembly to allow uninhibited differential thermal growth.

d.  The center stack casing is electrically isolated from the vacuum vessel by upper and lower high strength porcelain rings (WBS 12) installed between the vacuum vessel flanges and the center stack casing at the upper and lower 1.3-m. diameter ports.

e.  The final vacuum seal between the vacuum vessel and the center stack is made through flange assemblies equipped with Helicoflex metal seals.

f. The inner wall Plasma Facing Components (PFCs, WBS 11) are attached to the center stack casing.

2.0 System Performance

2.1 TF Coil Inner Leg Performance

a.  Electrical/thermal performance is given in Table CS-2.1-1 [R5, R25].

Table CS-2.1-1: TF Coil Inner Leg Electrical/Thermal Performance

| Element | Imax | ESW | Tfinal | Tallowable | Vmax | Insulation Emax | Insulation Allowable E |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (deg C) | (deg C) | (V) | (volts/mil) | (volts/mil) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inner Turn | 71.2 | 1.5 | 116.2 | 120.0 | 1012.9 | 15.8 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Outer Turn | 71.2 | 1.5 | 118.2 | 120.0 | 1012.9 | 15.8 | ≈100 |
| --- | --- | --- | --- | --- | --- | --- | --- |

b. Temperatures given in Table PF-2.1-1 are based on adiabatic conditions, and a 6.0 second ESW. During normal operations with a 5.5 second ESW and cooling water flowing in the coils the nominal final temperature is < 100C [R13].

c. Local temperatures near the TF coil inner leg/flag joint (on the outer layer) are expected to reach ≈ 140C [R14]. Although this exceeds the allowable in Table CS-2.1-1, it is in an area of low mechanical loading, and is therefore considered acceptable.

d. Resistive (∫i2(t)Rdt) heating of the TF coil inner legs results in an overall growth in their length of ≈ 3/8”.

e. Torsional loads are imposed on the TF coil inner legs due primarily to the OH solenoid fields crossing over the inner leg conductors; primarily the outer layer.  This results in a torque  of ~48,000 N-m (BTF= 0.6T) on the TF bundle.

f. Additional torsional forces are imposed on related radial elements of the TF system (i.e., the flags, connectors, and outer legs) due to the poloidal fields crossing over these elements.

g. Vertical forces are generated by gravity loads, as well as the TF fields crossing over the flags, connectors, and outer legs.

h. Radial loads are generated radially inwards on the TF coil inner leg bundle due to the magnetic force ITF x BTF on the central TF bundle.

i. Mechanical performance (BTF = 0.6T, Ioh = 24kA, Ipf1a = 10kA) is summarized in Table CS-2.1-2 [R21, R25].

Table CS-2.1-2: TF Coil Inner Leg

Mechanical Performance (BTF = 0.6T)

| Cond vonMises Stress max | Cond Allowable vonMises Stress | Insulation Shear Stress max | Insulation Shear Allowable @Tallowable |
| --- | --- | --- | --- |

| (ksi) | (ksi) | (ksi) | (ksi) |
| --- | --- | --- | --- |

| < 10 | 20 | 3.4 | 6.1 |
| --- | --- | --- | --- |

j. Shear stress in the insulation due to different thermal expansion coefficients of the copper and insulation is small (< 1ksi).

2.2 OH Coil Performance

a. The OH coil produces a flux swing of 0.62 volt-seconds with a current swing between +24kA and -24kA [R2].

b.  Electrical/thermal performance is given in Table CS-2.2-1 for two cases, namely the bipolar (fully inductive NSTX mode) and the unipolar (partial inductive NSTX mode) cases [R2, R25].

Table CS-2.2-1: OH Coil Electrical/Thermal Performance

| Case | Imax | ESW | Tfinal | Tallowable | Vmax | Insulation Emax | Insulation Allowable E |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (deg C) | (deg C) | (V) | (volts/mil) | (volts/mil) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Bipolar | 24.0 | 0.5 | 94.5 | 120.0 | 6077.1 | 95.0 | ≈500 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Unipolar | 24.0 | 0.2375 | 50.9 | 120.0 | 6077.1 | 95.0 | ≈500 |
| --- | --- | --- | --- | --- | --- | --- | --- |

c. The OH coil cooling is such that minimal ratcheting of temperature is expected. However, some of the dissipation of heat due to plasma operations (auxiliary heating power) results in a temperature rise of the center stack casing, and a flow of heat inward to the OH coil winding. This occurs primarily in the partial and non-inductive modes (5 second pulse length), when the OH current is unipolar, and ∆T during a pulse is 40 C or less. In practice, the allowable OH ∫i^2(t)dt will be limited as a function of the initial temperature of the coil (at the water outlet) and the center stack casing.

d. Radial in-plane EM forces are imposed on the OH coil due to the high toroidal magnetic flux within the center stack. The forces peak at the inner layer of the solenoid.

e. Axial EM forces are imposed on the OH coil due to the interaction of the OH current with the fields from nearby PF coils.  PF1b has the greatest effect since it has no upper counterpart.  The axial load it creates, which is always downward, is reacted through the common structure which supports both the OH and PF1b at the bottom of the center stack.

f. Vertical loads result from the dead weight.

g. Additional loads are imposed as the coolant wave progresses through the winding.  Cooling water at 10°C enters each conductor at the bottom of the solenoid and exits at the top at nearly 100°C.

g. Mechanical performance (Ioh = 24kA, Ipf1a = 15kA, Ipf1b=20kA) is summarized in Table CS-2.2-2 [R20, R25].

Table CS-2.2-2: OH Coil

Mechanical Performance

| Cond vonMises Stress max | Cond Allowable vonMises Stress | Insulation Shear Stress max | Insulation Shear Allowable @Tallowable |
| --- | --- | --- | --- |

| (ksi) | (ksi) | (ksi) | (ksi) |
| --- | --- | --- | --- |

| 21 | 21 | 0.96 | 1.7 |
| --- | --- | --- | --- |

i. The results given in Table CS-2.2-2 include thermal effects.

2.3 Inner PF Coil Performance

a.  Electrical/thermal performance is given in Table CS-2.3-1 [R2, R25].

Table CS-2.3-1: Inner PF Coil Electrical/Thermal Performance

| Coil | Imax | ESW | Tfinal | Tallowable | Vmax | Insulation Emax | Insulation Allowable E |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | (kA) | (sec) | (deg C) | (deg C) | (V) | (volts/mil) | (volts/mil) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1a | 15.0 | 5.0 | 75.2 | 120.0 | 2025.7 | 31.7 | ≈500 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| PF1b | 20.0 | 1.0 | 38.3 | 120.0 | 2025.7 | 31.7 | ≈500 |
| --- | --- | --- | --- | --- | --- | --- | --- |

b. Temperatures given in Table CS-2.3-1 are based on adiabatic conditions.

c. Mechanical performance is summarized in Table CS-2.3-2 [R25].

Table CS-2.3-2: Inner PF Coil Mechanical Performance

| Coil | Cond vonMises Stress max | Cond Allowable vonMises Stress | Insulation Shear Stress max | Insulation Shear Allowable @Tallowable |
| --- | --- | --- | --- | --- |

|  | (ksi) | (ksi) | (ksi) | (ksi) |
| --- | --- | --- | --- | --- |

| PF1a | <21 | 21 | <1.7 | 1.7 |
| --- | --- | --- | --- | --- |

| PF1b | <21 | 21 | <1.7 | 1.7 |
| --- | --- | --- | --- | --- |

2.4 Center Stack Casing Performance

a. After a full day (8 hours) of normal operations (full rated auxiliary heating power and 300 sec repetition period, partial or non-inductive operation, OH copper temperature ≤ 50C), the center stack casing maximum bulk average temperature is anticipated to be ≤ 460C. Local hot spots are anticipated to vary between 549 and 568C before and after pulses [R18].

b. During normal operations, when the center stack casing temperature is at a maximum temperature of 568C, the maximum temperature on the surface of the OH coil insulation is anticipated to be ≤ 61C, subject to the condition that the OH coil copper temperature is ≤ 50C (corresponding to the partial inductive, unipolar OH operations) [R18].

c. After a full day (8 hours) of normal operations (full rated auxiliary heating power and 600 sec repetition period, fully inductive operation), when the OH coil copper temperature is at a maximum of 95C, the maximum anticipated OH coil insulation temperature is ≤ 102C, and the maximum center stack casing temperature is 424C [R18].

d. During bakeout operations the required center stack casing maximum temperature is 390C [R19].

e. During bakeout the center stack casing will be ohmically heated with a maximum input power of 5.3 kW. The range of resistance of the center stack casing over the range of temperature during bakeout operations is 1.0 mΩ (50C) Ω to TBDΩ [R19].

f. During bakeout, when the center stack casing temperature is at a maximum temperature of 390C, the maximum temperature on the surface of the OH coil insulation is anticipated to be ≤ 20C, subject to the condition that the OH coil copper temperature is ≤ 14C and that it is cooled with rated inlet water temperature (10C) and flow in each coolant path [R19].

g. During bakeout , in the event of complete loss of cooling to the OH and TF coils when the center stack casing is at its maximum allowable temperature, but assuming the removal of all bakeout heating sources immediately after the event, the OH coil insulation would be anticipated to reach its maximum allowable temperature of 180C after 10 hours [R19].

h. During plasma disruptions a maximum (halo) current of 400kA is anticipated to flow in the center stack casing. The current can be decomposed into axial (poloidal) and circumferential (toroidal) components [R10, R11].

i. The cross product of axial (poloidal) components of the disruption current with the toroidal magnetic field result in radial forces on the center stack casing. To the extent that the poloidal components are not equally distributed in the toroidal direction, a net radial force may act on the center stack casing. To the extent that the poloidal components are equally distributed a hoop tension may be imposed on the cylinder or a compressive force may be imposed, depending on the polarity.

j. The cross product of circumferential (toroidal) components of the disruption current with vertical components of the poloidal magnetic field result in a hoop tension or a buckling force on the center stack casing, depending on the polarity.

k. Compared to the strength and stiffness of the center stack casing, none of the forces due to plasma disruption are anticipated to result in any significant stresses or deflections. The maximum anticipated net radial deflection of the center stack casing due to the cross product of poloidal halo current and the toroidal field is ≤ 1mm (BTF = 0.6T case). Since the total radial space allowed for the Microtherm insulating jacket and gap to the OH coil is ≈ 10

times this dimension, it is anticipated that the Microtherm will be able to withstand any disruption induced compressive loads without permanent deformation [R22].

3.0 Interfaces

3.1 Interface Points

Table CS-3.1-1: Interfaces with the Center Stack Assembly

| WBS | Interface Point |
| --- | --- |

| 11 | Center stack casing interfaces with the inner wall armor and inboard divertor plate |
| --- | --- |

| 12 | The center stack interfaces with the vacuum vessel at the center bore flanges of the upper and lower domes |
| --- | --- |

| 13 | The center stack interfaces with the outer TF legs. |
| --- | --- |

| 2x | None. |
| --- | --- |

| 3x | Inner TF, OH and PF coils interface with the Coolant Systems. Interface point is at the coil conductor coolant fittings. Bakeout (ohmic heating) power supply connects to center stack casing at terminals located on the center stack casing. |
| --- | --- |

| 4x | Magnetics diagnostics are located on the OD of the OH coil. |
| --- | --- |

| 5x | All coils receive power from the power systems. Interface point is at the coil terminals, after all turns have been connected in series. Center stack casing connects to CHI power supply system at terminals located on the center stack casing. |
| --- | --- |

| 6x | None. |
| --- | --- |

3.2 Interface Requirements

a. See section MS-3.2.

4.0 Design Basis

4.1 Standards and Codes

Refer to Section 4.1 in the General Vacuum Vessel and Tokamak Support Structure Design Description.

4.2 Supporting Analysis

a. See section MS-4.2.

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

a. Mock-ups of OH coil sections have been fabricated to investigate winding tension requirements, conductor keystoning, and insulation shear.

b. Mock-ups of the TF inner leg/flag joint have been fabricated to confirm contact resistance parameters [R26].

c. Insulation samples, both B-Staged epoxy glass, as well as Kapton/B-Staged epoxy glass composite, have been fabricated to test shear properties [R25].

d. Insulation samples, both B-Staged epoxy glass, as well as Kapton/B-Staged epoxy glass composite, will be fabricated to test dielectric strength [R25].

5.3 Component Procurement and Fabrication

a. TF Coil Inner Leg Conductors will be procured using a stand alone specification.

b. TF Coil Outer Leg Conductors will be procured using a stand alone specification.

c. PF/OH Coil Conductors will be procured using a stand alone specification.

d. TF Coil Inner Leg Assembly will be procured via a stand alone specification.

e. TF Coil Outer Legs will be procured via a stand alone specification.

f. OH and Inner PF Coil Assemblies will be procured via a stand alone specification.

c. Center Stack Casing will be procured via a stand alone specification.

5.4 System Assembly, Installation and Testing

a.  All assembly of the center stack coils and thermal insulation within the casing, transitions to the connector assembly and assembly of the final center stack system will be performed at PPPL.

b.  Installation of the center stack within the vacuum vessel is the responsibility of WBS 8.

APPENDIX

| # | Description | Value | Units | Reference |
| --- | --- | --- | --- | --- |

| 1 | Magnet Systems System Requirements Document | n.a. | n.a. | NSTX-SRD-13-015-0 |
| --- | --- | --- | --- | --- |

| 2 | PF/OH Coil Electrical Parameters | n.a. | n.a. | NSTX-CALC-13-1-0 |
| --- | --- | --- | --- | --- |

| 3 | PF/OH Current & Voltage Ratings | n.a. | n.a. | 13-970203-CLN-02, "PF/OH Current and Voltage Ratings" |
| --- | --- | --- | --- | --- |

| 4 | Power Systems SDD | n.a. | n.a. | NSTX-SDD-5X-YYY, 1/31/97 |
| --- | --- | --- | --- | --- |

| 5 | TF Coil Electrical Parameters | n.a. | n.a. | NSTX-CALC-13-2-0 |
| --- | --- | --- | --- | --- |

| 6 | Bakeout Ohmic Heating Electrical Requirements | n.a. | n.a. | NSTX-CALC-13-10-0 |
| --- | --- | --- | --- | --- |

| 7 | NSTX General Requirements Document | n.a. | n.a. | NSTX-RQMTS-GRD-018 |
| --- | --- | --- | --- | --- |

| 8 | ASME Boiler and Pressure Vessel Code | n.a. | n.a. | ASME, New York, NY |
| --- | --- | --- | --- | --- |

| 9 | Plasma Initiation |  |  | NSTX-CALC-72-1 |
| --- | --- | --- | --- | --- |

| 10 | Plasma Disruption Analysis |  |  | NSTX-CALC-13-3 |
| --- | --- | --- | --- | --- |

| 11 | Center Stack Halo Current Analysis |  |  | NSTX-CALC-13-4 |
| --- | --- | --- | --- | --- |

| 12 | OH Electrical Performance Analysis |  |  | NSTX-CALC-53-1 |
| --- | --- | --- | --- | --- |

| 13 | TF Electrical Performance Analysis |  |  | NSTX-CALC-52-1 |
| --- | --- | --- | --- | --- |

| 14 | TF Electrical Joint Analysis |  |  | NSTX-CALC-13-5 |
| --- | --- | --- | --- | --- |

| 15 | OH Coil Thermal/Hydraulic Analysis |  |  | NSTX-CALC-13-6 |
| --- | --- | --- | --- | --- |

| 16 | TF Coil Thermal/Hydraulic Analysis |  |  | NSTX-CALC-13-7 |
| --- | --- | --- | --- | --- |

| 17 | Center Stack PFC Thermal Analysis |  |  | NSTX-CALC-13-8 |
| --- | --- | --- | --- | --- |

| 18 | Integrated Thermal Analysis, Normal Operation |  |  | NSTX-CALC-13-9 |
| --- | --- | --- | --- | --- |

| 19 | Integrated Thermal Analysis, Bakeout |  |  | NSTX-CALC-13-10 |
| --- | --- | --- | --- | --- |

| 20 | OH Coil Stress Analysis |  |  | NSTX-CALC-13-11 |
| --- | --- | --- | --- | --- |

| 21 | TF Coil Stress Analysis |  |  | NSTX-CALC-13-12 |
| --- | --- | --- | --- | --- |

| 22 | Center Stack Stress Analysis |  |  | NSTX-CALC-13-13 |
| --- | --- | --- | --- | --- |

| 23 | S-1 Spheromak Project Engineering Handbook Rev. 1 | n.a. | n.a. | PPPL 4/81 |
| --- | --- | --- | --- | --- |

| 24 | Microtherm Application Data | n.a. | n.a. | Microtherm Inc., 1227 McArthur Rd, Maryville TN 37804 (423) 681-0016 |
| --- | --- | --- | --- | --- |

| 25 | Mechanical and Electrical Allowables for NSTX Electrical Insulation | n.a. | n.a. | TBD |
| --- | --- | --- | --- | --- |

| 26 | Tests on Mock-Up of TF Joint | n.a. | n.a. | TBD |
| --- | --- | --- | --- | --- |

| 27 | Preliminary FMEA | n.a. | n.a. | 13-970203-CLN-02, "Preliminary FMEA of Center Stack" |
| --- | --- | --- | --- | --- |