National Spherical Tokamak Experiment

NSTX

SYSTEM DESIGN DESCRIPTION

HEATING & CURRENT DRIVE SYSTEMS

NSTX-SDD-2X-026

Revision 0

July 16, 1996

________________________________

J. Randall Wilson

WBS 2 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

Introduction	1

General Heating & Current Drive System Design Description	2

1.0 Design Description	2

1.1 System Design Features	2

1.2 Layout	2

1.3 Component Design Features	2

2.0 System Performance [R1]	2

2.1 Baseline Operation	2

2.2 Upgrades	3

2.2.1 ECH upgrades	3

2.2.2 HHFW Upgrade	3

2.2.3 CHI Upgrades	3

2.2.4 NBI Upgrade	3

3.0 Interfaces	4

3.1 Interface Points	4

3.1 Interface Requirements	4

4.0 Design Basis	4

4.1 Standards & Codes	4

4.2 Supporting Analysis	5

5.0 Design Implementation	5

5.1 Prototypes	5

5.2 Mock-ups	5

Refer to sections describing individual WBS 2 subelements	5

5.3 Component Procurement and Fabrication	5

5.4 System Assembly, Installation, and Testing	5

ECH Baseline System Design Description	6

ECH Upgrade System Design Description	7

1.0 Design Description	7

1.1 System Design Features	7

1.2 Layout	7

1.3 Component Design Features	9

2.0 System Performance	15

3.0 Interfaces	15

3.1 Interface Points	15

3.2 Interface Requirements	15

4.0 Design Basis	16

4.1 Standards & Codes	16

4.2 Supporting Analysis	16

5.0 Design Implementation	16

5.1 Prototypes	16

5.2 Mock-ups	16

5.3 Component Procurement and Fabrication	16

5.4 System Assembly, Installation, and Testing	16

HHFW System Description	17

1.0 Design Description	17

1.1 System Design Features	17

1.2 Layout	17

2.0 System Performance	20

2.1 Baseline Operation	20

2.2 Upgrades	20

2.2.1 Neutral Beam Injection (NBI) Upgrade	20

2.2.2 Long pulse upgrade	20

2.3 Pulse Repetition Period and Operating Life	20

3.0 Interfaces	21

3.1 Interface requirements	21

3.2 Interface Requirements	21

4.0 Design Basis	22

4.1 Standards and Codes	22

4.2 Supporting Analysis	22

5.0 Design Implementation	22

5.1 Prototypes	22

5.2 Mockups	22

5.3 Component Procurement and Fabrication	22

5.4 System assembly, installation, and testing	23

CHI System Description	23

1.0 Design Description	23

1.1 System Design Features	23

1.2 Layout	23

1.3 Component Design Features	23

2.0 System Performance	24

3.0 Interfaces	24

3.1 Interface Points	24

3.1 Interface Requirements	24

4.0 Design Basis	24

4.1 Standards and Codes	24

4.2 Supporting Analysis	24

5.0 Design Implementation	24

5.1 Prototypes	24

5.2 Mock-ups	24

5.3 Component Procurement and Fabrication	24

5.4 System Assembly, Installation, and Testing	24

NBI System Description	24

1.0 Design Description	24

1.1 System Design Features	24

1.2 Layout	25

1.3 Component Design Features	25

2.0 System Performance	25

3.0 Interfaces	25

3.1 Interface Points	25

3.1 Interface Requirements	25

4.0 Design Basis	26

4.1 Standards and Codes	26

4.2 Supporting Analysis	26

5.0 Design Implementation	26

5.1 Prototypes	26

5.2 Mock-ups	26

5.3 Component Procurement and Fabrication	26

5.4 System Assembly, Installation, and Testing	26

List of Tables

HCD-2.1-1	Baseline Requirements	3

HCD-2.2-1	Upgrade Requirements	4

ECH-3.1-1	Interfaces with the ECHU System	15

ECH-3.2-1	ECHU Cooling Water Requirements	15

ECH-3.2-2	ECHU I/O Requirements	15

HHFW-3.1-1	Interfaces with the HHFW System	21

HHFW-3.2-1	HHFW I/O Requirements	22

List of FIGURES

ECHU-1.2-1	NSTX ECHU System Layout	8

ECHU-1.3-1	NSTX ECHU Gyrotron Tank configuration	9

ECHU-1.3-2	NSTX ECHU Power Supply Configuration	11

ECHU-1.3-3	NSTX ECHU Transmission Line Block Diagram	13

ECHU-1.3-4	NSTX ECHU Launcher Layout	14

HHFW-1.2-1	NSTX HHFW System Layout	18

HHFW-1.2-2	NSTX HHFW Tranmission Line Block Diagram	19

HHFW-1.2-3	NSTX HHFW Antenna Strap Layout	19

NBI-1.2-1	NSTX NBI Source Layout in Test Cell	25

APPENDIX

LIST OF REFERENCES

Introduction

This System Design Description (SDD) document describes the design of the Heating & Current drive Systems (WBS 2X) of the National Spherical Tokamak Experiment (NSTX), in response to the requirements given in NSTX-SRD-2X.

A general section covers information which is relevant to the entire WBS element. Individual subelements of the WBS work scope are covered in separate sections, namely:

• WBS 21 Electron Cyclotron Heating (ECH) subsystem

• WBS 22 High Harmonic Fast Wave (HHFW) subsystem

• WBS 23 Coaxial Helicity Injection  (CHI) subsystem

• WBS 24 Neutral Beam Injection (NBI) subsystem

Figures and Tables are numbered in format X-Y-Z where X=section (i. e. section abbreviation, e.g. ECH for Electron Cyclotron heating), Y= paragraph number, and Z= sequence number within that paragraph.

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix

WBS 2X -  General Heating & Current Drive System Design Description

1.0 Design Description

The Heating & Current Drive WBS Element covers the supply and delivery of auxiliary (non-ohmic) power to the NSTX plasma.

1.1 System Design Features

a. The Heating and Current Drive systems consist of DC power supplies, energy transformation systems (e.g. rf amplifiers), transmission systems, and launching systems.

1.2 Layout

Refer to sections describing individual WBS 2 subelements

1.3 Component Design Features

Refer to sections describing individual WBS 2 subelements

2.0 System Performance [R1]

a. The baseline design of the NSTX Heating and current Drive systems will satisfy the baseline performance requirements, and will anticipate the upgrade requirements but not necessarily satisfy them.

b. Anticipated upgrades include the following:

• ECH start-up upgrade (increased power and duration)

• long pulse CHI

• addition of neutral beams

• long pulse HHFW

2.1 Baseline Operation

a. Baseline operation requirements are given in table HCD-2.1-1.

Table HCD-2.1-1 Baseline Requirements

| Parameter | Value | Units | Ref |
| --- | --- | --- | --- |

| Baseline maximum plasma duration (t) | 5.0 | sec | 1 |
| --- | --- | --- | --- |

| Baseline minimum repetition period (T) | 300.0 | sec | 2 |
| --- | --- | --- | --- |

| RMS factor (sqrt(t/T)) | 0.129 |  |  |
| --- | --- | --- | --- |

| Baseline ECH duration | 5.0 | msec |  |
| --- | --- | --- | --- |

| Baseline HHFW duration | 5.0 | sec |  |
| --- | --- | --- | --- |

| Baseline CHI duration (1/4 sine wave) | 10.0 | msec |  |
| --- | --- | --- | --- |

| Baseline NBI | N/A |  |  |
| --- | --- | --- | --- |

| Baseline ECH power | 20.0 | kW |  |
| --- | --- | --- | --- |

| Baseline HHFW power | 6.0 | MW |  |
| --- | --- | --- | --- |

| Baseline CHI current (plasma) | 500.0 | kA |  |
| --- | --- | --- | --- |

| Baseline CHI current (external) | 50.0 | kA |  |
| --- | --- | --- | --- |

2.2 Upgrades

Upgrade parameters are given in Table HCD-2.2-1 below.

2.2.1 ECH upgrades

a. The ECH power will be increased from 20kW to 200kW, and the pulse length extended from 5.0mS to 100.0 mS.

2.2.2 HHFW Upgrade

a. HHFW will have its pulse length extended from 5 secs. to 60 secs.

2.2.3 CHI Upgrades

a. A first stage upgrade of CHI will allow 5 second pulses by adding a steady state power supply.

b. A second stage upgrade would accomodate 60 sec operation.

2.2.4 NBI Upgrade

a. The NBI upgrade would place the four existing PBX-M beamlines onto NSTX. Their pulse length would be increased to 60 seconds.

Table HCD-2.2-1 Upgrade Parameters

| Parameter | Value | Units | Ref |
| --- | --- | --- | --- |

| Upgrade Maximum Plasma Duration | 60 | sec |  |
| --- | --- | --- | --- |

| Upgrade Minimum Repition Rate | 3600 | sec |  |
| --- | --- | --- | --- |

| Maximum ECH pulse length | 100 | msec |  |
| --- | --- | --- | --- |

| Maximum HHFW pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum CHI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum NBI pulse length | 60 | sec |  |
| --- | --- | --- | --- |

| Maximum ECH power | 200 | kW |  |
| --- | --- | --- | --- |

| Maximum HHFW power | 6 | MW |  |
| --- | --- | --- | --- |

| Maximum CHI current (external) | 20 | kA |  |
| --- | --- | --- | --- |

| Maximum CHI current (plasma) | 200 | kA |  |
| --- | --- | --- | --- |

| Maximum NBI power | 5 | MW |  |
| --- | --- | --- | --- |

3.0 Interfaces

3.1 Interface Points

Refer to sections describing individual WBS 2 subelements.

3.1 Interface Requirements

Refer to sections describing individual WBS 2 subelements

4.0 Design Basis

4.1 Standards & Codes

a. The NSTX Heating and Current Drive Systems will be designed, constructed, and installed in compliance with the applicable provisions of the following:

1) PPPL Health & Safety Manual, ES&H -5008

2). Others as listed in DoE Order 5480.4, ES&H Protection Standards, including:

-Title 40 CFR 110,112 EPA Regulations on Discharge of Oil and Oil Pollution 	Prevention

-Title 42 U.S.C. 4901, Et. Seq., "Noise Control Act of 1972"

-National Fire Codes

-Product Directories of Underwriters Laboratories

-Title 29 CFR 1910, Occupational Safety and Health Standards

-Title 29 CFR 1926, Safety and Health Regulations for Construction

-ANSI/NFPA 70-(latest edition) National Electrical Code

-ANSI C20-(latest edition) National Electric Safety Code

4.2 Supporting Analysis

Refer to sections describing individual WBS 2 subelements

5.0 Design Implementation

5.1 Prototypes

Refer to sections describing individual WBS 2 subelements

5.2 Mock-ups

Refer to sections describing individual WBS 2 subelements

5.3 Component Procurement and Fabrication

Refer to sections describing individual WBS 2 subelements

5.4 System Assembly, Installation, and Testing

Refer to sections describing individual WBS 2 subelements

WBS 21 -  ECH Baseline System Design Description

LATER

WBS 21 -  ECH Upgrade System Design Description

1.0 Design Description

1.1 System Design Features

a. The ECH upgrade system is composed of the following components:

• Existing 28 GHz gyrotron, socket and oil tank provided by ORNL

• Existing series-pass voltage regulator provided by ORNL

• Existing magnet power supplies provided by ORNL

• Existing 100 kV 60 μF capacitor bank provided by PPPL

• New charging supply provided by ORNL

• Waveguide run from gyrotron to NSTX utilizing existing components provided 		by ORNL

•New waveguide launcher provided by ORNL.

1.2 Layout

a. The capacitor bank, power supplies and gyrotron will be located on the third floor of the rf building in rooms (TBD)

b. The waveguide run will be from the gyrotron tank in room (TBD) to the NSTX machine in the test cell.

c. The launcher will be mounted on the NSTX machine.

d. The controls will be integrated through the existing ICRF PLC on the second floor of the rf building

e. The overall layout is shown in Figure ECHU-1.2-1

Figure ECHU-1.2-1

1.3 Component Design Features

a. The 28 GHz, 200kW, 100 ms consists of a TE02 mode gyrotron tube (Varian VGA-8050M) and associated magnet, socket and waveguide hardware. This equipment is available at ORNL. For full power operation a -80 kV, 8A well regulated beam power supply is required. In addition the gyrotron magnet requires four 30V, 500A (15kW) magnet power supplies. The gyrotron tank and associated hardware is shown in figure ECHU-1.3-1

FIGURE ECHU-1.3-1

b. The gyrotron beam power supply will utilize a series pass regulator capable of supplying -80 kV at 8A. The regulator utilizes an Eimac X2062 Tetrode and contains a crowbar. The regulator is available from ORNL

c. The regulator is supplied from a 100 kV, 60 μF capacitor bank. This will be provide by PPPL utilizing the existing bank of 20 kV, 60 μF capacitors in room C41.

d. The capacitor bank will be charged from a -120kV, 0.1 A charging supply. This will be purchased and provided by ORNL.

An outline of the gyrotron beam power supply configuration is shown in figure ECHU-1.3-2

Figure ECHU-1.3-2

e. The gyrotron magnet requires four 30V, 500 A power supplies which exist and will be provided by ORNL.

f. A waveguide transmission and launching system is used to transport the power from the source to the tokamak and launch the power into the vacuum vessel. For this system, a high mode purity, overmoded waveguide system is proposed which will maximize the power coupling efficiency and minimize leakage and arcing at the vacuum vessel.  The transmission line length from the proposed gyrotron location to the NSTX vessel is approximately 30 m long and will require three 90° bends. Circular waveguide with a diameter of 63.5 mm will be utilized for most of the transmission line.  The transmission mode will be TE02 in some sections and TE01 for the majority of the length.  A two-period ripple wall mode converter will convert the TE02 gyrotron output power to the TE01 mode.  Corrugated radius bends with optimized curvature will be used for bends.  Several options are available for generating a gaussian-like launch beam from the TE01 transmission mode near the NSTX end of the waveguide.  The overall transmission efficiency is expected to be 85% or better which includes 7% mode conversion loss and 7% resistive loss. The transmission system is shown in outline in figure ECH-1.3-3

g. At the launcher, a polarized directional launch beam will be generated using a HE11 mode waveguide antenna and a single focusing reflector to launch power from the low field-side of the resonance layer through a mid-plane 300 mm diameter port of the NSTX vacuum vessel.  The launch beam will be linearly polarized and the plane of polarization can be adjusted to be optimized for O or X mode launch.  The approximate beam width at the resonance zone will be 24 cm. One of the beam launchers from the ATF experiment at ORNL can be utilized for this application.  A waveguide vacuum window consisting of an edge cooled alumina disc is located just above the launcher. The launcher configuration is shown in figure ECHU-1.3-4.

Figure ECHU-1.3-3

Figure ECHU-1.3-4

2.0 System Performance

3.0 Interfaces

3.1 Interface Points

Table ECH-3.1-1: Interfaces with the  ECHU System

| WBS | Interface Point |
| --- | --- |

| 11 | None. |
| --- | --- |

| 12 | Attachment of ECH launcher with Vacuum Vessel |
| --- | --- |

| 13 | None |
| --- | --- |

| 3x | Chilled, deionized cooling water supply |
| --- | --- |

| 4x | None |
| --- | --- |

| 5x | AC Input Power |
| --- | --- |

| 6x | Various I/O points, and Safety Interlocks |
| --- | --- |

3.2 Interface Requirements

a. Cooling water requirements are listed in Table ECHU-3.2-1.

Table ECHU-3.2-1: ECHU Cooling Water Requirements

| Flow (GPM) | ∆P (PSI) | Heat Load (Joules/pulse) | Heat Load (kW avg.) |
| --- | --- | --- | --- |

| TBD | TBD | TBD | TBD |
| --- | --- | --- | --- |

b. Central I&C interface points (input/output sense w/respect to Central I&C).are listed in Table ECHU-3.2-2.

Table ECH-3.2-2: ECHU I/O Requirements

| Anlg Out | Anlg In | Dig In | Dig Out | Lo Speed | Mod Speed | Hi Speed |
| --- | --- | --- | --- | --- | --- | --- |

| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- |

Note:

Low (Lo) Speed ==> Data Acquisition signals requiring sample rates >100 Hz and ≤2 KHz.

Moderate  (Mod) Speed ==> Data Acquisition signals  requiring sample rates >2 KHz and ≤100 KHz.

High (Hi) Speed ==> Data Acquisition signals  requiring sample rates >100 KHz and ≤1 MHz.

c. 480 AC input power shall be supplied to the ECH system. The power level is TBD kVA, and the power factor is TBD.

4.0 Design Basis

4.1 Standards & Codes

See Heating and Current drive General section

4.2 Supporting Analysis

Supporting analysis will be performed as required at a later stage in the design process

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

None

5.3 Component Procurement and Fabrication

a. Charging power supply -120 kV, 0.1 A  from 480 V AC to be provided by ORNL.

b. Fabrication of waveguide launcher

c. Fabrication of waveguide supports

5.4 System Assembly, Installation, and Testing

a. Gyrotron system will be tested at ORNL prior to shipment to PPPL.

b. Standard PPPL installation and integrated systems test procedures will be utilized.

WBS22 -  HHFW System Description

1.0 Design Description

The HHFW WBS element covers the generation, transmission, and coupling of radio-frequency power for heating and current drive to the NSTX plasma.

1.1 System Design Features

a. Generation of radio frequency power will be accomplished by six existing rf generators and their associated power supplies and cooling systems. These generators are located in the C-site RF building.

b. Transmission of radio frequency power will be accommodated by six runs of 9 inch transmission line from the (existing) switching/dummy load facilities in the C-site RF building to the NSTX test cell.

c. Existing switch facilities which permit the routing of radio frequency power to the PBX-M experimental facility will be maintained.

d. Transmission line matching and distribution of radio frequency power to the launching antennas internal to NSTX shall be accomplished with six sets of tuning stubs and line stretchers, along with associated decoupling and transmission line elements. Transmission lines, stubs and stretchers, and decoupling elements shall be gas tight to TBD and capable of pressurization to TBD psi of sulfur hexaflouride.

e. Transmission of radio frequency power through the NSTX vacuum vessel shall be accomplished with 12 vacuum feedthroughs.

f. Coupling of radio frequency power to the NSTX plasma shall be accomplished with two antenna sets internal to the NSTX vacuum vessel. Each antenna set will consist of local protection limiters, Faraday shields, current straps, radial/toroidal feeders, and a backplane assembly.

g. The limiters, Faraday shield and current straps will not be actively cooled. Cooling at the backplane/vessel wall may be required to remove heat between pulses.

h. The final assembly of the antennas will be accomplished in-vessel.

i. The transmission line external to the vacuum vessel will be mechanically isolated from the vessel by means of transmission line bellows assemblies, and electrically isolated from the vessel by means of DC breaks.

j. A DC shorting stub will be incorporated into the transmission line run prior to the point where the line exits the test cell.

1.2 Layout

The general layout of the HHFW system up to the vacuum vessel interface is shown in Figure HHFW-1.2-1

Figure HHFW-1.2-1

a. The existing layout of the rf generators and associated power supplies and cooling systems will remain.

b. The existing switching and dummy load facilities will remain. The existing transmission line runs to TFTR (D site) will be disconnected in the C-site switching/dummy load room.

c. The routing of the 9 inch transmission line runs from the switching/dummy load room to the NSTX is TBD.

d. Tuners (tuning stubs and line stretches) are to be located in the NSTX test cell. Locations TBD. Layout of the transmission lines and decouplers distributing radio frequency power to the two antenna sets is TBD.

e. The twelve vacuum  feedthroughs will occupy 12 of 16 off-midplane six-inch ID flanges located about ports C, D, I, and J. See Figures HHFW-1.2-2 and HHFW 1.2-3 for port locations.

Figure HHFW-1.2-2

f. One antenna set will occupy ports C, D. The second antenna set will occupy  I and J. The layout of the antenna set within each pair of ports is shown in Figure 3. The plasma facing surface of the antenna set is defined by the local limiters. The leading edge of the limiters shall conform to the surface of a torus with major radius 20.32 inches (56.7 cm) and minor radius 40 inches (1.016 m) within ± TBD in.

Figure HHFW-1.2-3

2.0 System Performance

a. The baseline design of the HHFW system will satisfy the baseline performance requirements.

b. Anticipated upgrades:

• addition of neutral beam injection

• long pulse operation.

The configuration of the antenna sets will accomodate neutral beam injection, with possible minor modifications. Neither the rf generators nor the baseline antenna sets will accomodate long pulse operation.

2.1 Baseline Operation

a. Baseline operation of NSTX will be pulsed. The HHFW system will deliver 6 MW to the plasma with a 5 second maximum pulse length and a 300 second minimum repetition period  for all three baseline non inductive discharge scenarios:

• Natural divertor

• Double null X-point divertor

• Single null X-point divertor.

.

b. The shape of the leading edge of the antenna limiter set as defined in 1.2 (f) is designed to accomodate a minimum 5 cm gap between the last closed flux surface for all three baseline non inductive discharge scenarios.

2.2 Upgrades

2.2.1 Neutral Beam Injection (NBI) Upgrade

a. The layout of the antenna set as defined in 1.2 (f) and shown in Figure HHFW-1.2-1 will accomodate the planned NBI upgrade. The antenna set will not impinge upon the beam footprint.

2.2.2 Long pulse upgrade

a. The NSTX long pulse upgrade will have an expected maximum pulse length of 60 seconds, and a minimum repetition period of 3600 seconds. The baseline HHFW system will not accommodate long pulse operation.

b. Upgrade paths to the rf generators, transmission lines, tuning and matching systems, vacuum feedthroughs, and antenna sets will be identified.

2.3 Pulse Repetition Period and Operating Life

a. The baseline HHFW system will accomodate the target maximum of 100 pulses per 24 hour period.

b. The rf vacuum feedthroughs and all components of the HHFW system external to the vacuum vessel will be designed for 10 year operation, with periodic maintenance and

replacement of transmission line insulators, etc. The antenna sets will be designed for TBD year operation. Frequent (?TBD) modification of the antenna set will be acommodated.

c. The HHFW system will accomodate the following schedule of NSTX operation - 100 pulses per day, 5 days per week, 3 weeks per month, 9 months per year, for 10 years (total of 135,000 pulses). Non-operating times are available for maintenance on components external to the vacuum vessel. In addition, TBD vacuum breaks per year are assumed to permit maintenance of in-vessel components.

3.0 Interfaces

3.1 Interface requirements

Table HHFW-3.1-1: Interfaces with the  HHFW System

| WBS | Interface Point |
| --- | --- |

| 11 | None. |
| --- | --- |

| 12 | Attachment of the antennas to Vacuum Vessel |
| --- | --- |

| 13 | None |
| --- | --- |

| 3x | Chilled, deionized cooling water supply |
| --- | --- |

| 4x | None |
| --- | --- |

| 5x | AC Input Power |
| --- | --- |

| 6x | Various I/O points, and Safety Interlocks |
| --- | --- |

3.2 Interface Requirements

a. Existing cooling for the rf generators is adequate for baseline operation. Existing cooling for the dummy loads is adequate. No cooling of the transmission lines, tuning and matching systems, decoupling systems, in-test-cell rf distribution, or the rf vacuum feedthroughs is required. Water cooling may be required for the in-vessel areas of the backplane interfacing to the antenna limiters, current straps, and Faraday shield. Flow requirements are TBD.

The HHFW system will require additional cooling water for the long pulse upgrade.

b. Central I & C requirements:

The HHFW system will require an arm permissive, an enable permissive, a timing (T=0) pulse, and a plasma permissive.

c. Interlock requirements:

The existing rf generator interlock system will be retained. In addition, a kirk key system which locks out the generator system in the event of test cell access will be required.

d. Central I&C interface points (input/output sense w/respect to Central I&C).are listed in Table HHFW-3.2-1.

Table HHFW-3.2-1: HHFW I/O Requirements

| Anlg Out | Anlg In | Dig In | Dig Out | Lo Speed | Mod Speed | Hi Speed |
| --- | --- | --- | --- | --- | --- | --- |

| TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- |

Note:

Low (Lo) Speed ==> Data Acquisition signals requiring sample rates >100 Hz and ≤2 KHz.

Moderate  (Mod) Speed ==> Data Acquisition signals  requiring sample rates >2 KHz and ≤100 KHz.

High (Hi) Speed ==> Data Acquisition signals  requiring sample rates >100 KHz and ≤1 MHz.

e. 480 AC input power shall be supplied to the HHFW system. With 6MW delivery to the plasma at an efficiency of 50%, the input power level is 13.3MVA and the power factor is 0.9.

f. Vacuum vessel interface:

The HHFW system interface to the vacuum vessel requires 6 inch ID flangement about ports C, D, I, and J, as indicated in Figures 2 and 3.

4.0 Design Basis

4.1 Standards and Codes

See Heating and Current Drive Systems General section

4.2 Supporting Analysis

Supporting analysis will be performed as required at a later stage of the design process.

5.0 Design Implementation

5.1 Prototypes

None.

5.2 Mockups

None planned at present; partial mockups of the antenna set may be required.

5.3 Component Procurement and Fabrication

a. The tuners presently located at the D site RFE in the mockup area will be demounted and disassembled as necessary and moved to the NSTX test cell.

b. Nine inch transmission line elbows, directional couplers, and other assorted transmission line components located at or near the RFE will be removed and utilized in the transmission line run from the rf generators to the NSTX test cell.

c. Adequate stock of 9 inch transmission line is available to assemble the transmission line run from the generators to the NSTX test cell, except for specialized components as stated in 5.3 (b). Disassembly of the main C-site to D-site transmission line run presently feeding rf power to TFTR is not expected to be required.

d. The decoupling system will be fabricated using 6 inch transmission line components. Requirements over and above existing stock of 6 inch components TBD.

e. Twelve 6 inch rf vacuum feedthroughs (from the existing PPPL design) will be procured.

f. Twelve 6 inch transmission line DC breaks will be fabricated.

g. Transmission line bellows for mechanical isolation of the transmission line from the vacuum vessel will be fabricated, or transferred from TFTR if the activation level is sufficiently low.

h. The antenna sets will be fabricated.

i. RF voltage and phase detectors presently in service on TFTR will be removed from the test cell basement and utilized in the HHFW system.

5.4 System assembly, installation, and testing

a. The rf antenna sets, vacuum feedthroughs, tuning stubs and line stretchers will be installed after the  magnet systems, main tokamak structure, and passive stabilizing plates are installed. The transmission line runs external to the test cell may be installed at any time after TFTR ceases operations.

b. Procedures will be written to cover system commissioning.

WBS 23 - CHI System Description

1.0 Design Description

The CHI System is composed of a CHI power supply (WBS 53), a bus connection to the machine, and a ceramic break in the vacuum vessel (WBS1).

1.1 System Design Features

1.2 Layout

1.3 Component Design Features

2.0 System Performance

3.0 Interfaces

3.1 Interface Points

3.1 Interface Requirements

4.0 Design Basis

4.1 Standards and Codes

See Heating and Current Drive Systems General section

4.2 Supporting Analysis

Supporting analysis will be performed as required at a later stage of the design process.

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

None

5.3 Component Procurement and Fabrication

a. CHI power supply in WBS (53)

b. Bus work (to be fabricated?)

c. Ceramic break in Vacuum vessel to be fabricated in (WBS 1)

5.4 System Assembly, Installation, and Testing

WBS 24 - NBI System Description

The NBI system of up to four PBX-M neutral beam injectors is an upgrade

1.0 Design Description

1.1 System Design Features

The injectors will provide 5 MW for 60 seconds

1.2 Layout

a. The  layout of the NBI sources is shown in figure NBI-1.2-1

Figure NBI-1.2-1

b. The ports allocated for NBI are B,F,H and L.

1.3 Component Design Features

2.0 System Performance

3.0 Interfaces

3.1 Interface Points

3.1 Interface Requirements

4.0 Design Basis

4.1 Standards and Codes

See Heating and Current Drive Systems General section

4.2 Supporting Analysis

Supporting analysis will be performed as required at a later stage of the design process.

5.0 Design Implementation

5.1 Prototypes

None

5.2 Mock-ups

None

5.3 Component Procurement and Fabrication

5.4 System Assembly, Installation, and Testing