National Spherical Torus Experiment

NSTX

SYSTEM REQUIREMENTS DOCUMENT

PLASMA FACING COMPONENTS

NSTX-SRD-11-013

Revision 0

August 13, 1997

________________________________

John Spitzer

WBS 1 Manager

________________________________

Charles Neumeyer

NSTX Project Engineering Manager

________________________________

Masa Ono

NSTX Project Manager

| Revision | Date | TRB | Description of Change |
| --- | --- | --- | --- |

| 0 | 8/13/97 |  | Initial Release |
| --- | --- | --- | --- |

Introduction	1

Plasma Facing Components Requirements	1

1.0 Functions	1

2.0 Configuration Requirements & Essential Features	1

2.1 Overall Geometry	1

2.2 Passive Stabilizers	2

2.3 Inner Wall Armor	2

2.4 Inboard Divertor Area Strike Plates	3

2.5 Outboard Divertor Area Strike Plates	3

3.0 Performance & Operational Requirements	3

3.1 Baseline Operation	4

3.2 Upgrades	5

List of Tables

PFC-2.1-1	PFC Geometry	1

PFC-3.1-1	Max PFC Heat Loads for Baseline NSTX Duty Cycle	4

PFC-3.1-2	PFC Heat Loads for Baseline NSTX Operation

Scenarios	4

PFC-3.1-3	Max PFC Heat Loads for Upgrade NSTX Duty Cycle	5

APPENDIX

LIST OF REFERENCES

Introduction

This System Requirements Document (SRD) defines the requirements for the Plasma Facing Components (WBS 11) of the National Spherical Torus Experiment (NSTX).

References to the requirements identified herein are cited in the body of the document using serial numbers preceded by an "R" and are listed in the Appendix.

WBS 11 -  Plasma Facing Components Requirements

1.0 Functions

The Plasma Facing Components (PFC) consist of plasma facing graphite tiles and mounting hardware, passive stabilizers, inner wall protection, divertor area strike plate assemblies and local Instrumentation and Control.

The functions of the Plasma Facing Components are to:

a.  Provide sufficient MHD wall stabilization for the high performance plasmas.

b.  Provide protection for the vacuum vessel and diagnostics from the anticipated heat loads.

2.0 Configuration Requirements & Essential Features

2.1 Overall Geometry

a. The plasma facing components include the inner wall armor, divertor area strike plates, passive stabilizers,  and associated plasma facing tiles.  The PFC designs (in particular the location of the front plasma facing surfaces) shall conform to the geometries given in Table PFC-2.1-1, which describes the 3-d surfaces via 2-d line segments assumed swept through toroidal angle 0 to 360 degrees. Data given in table corresponds to PFCs above midplane; corresponding mirror image PFCs are located below midplane.

Table PFC-2.1-1  PFC Geometry

| PFC | Tile Thickness | R1 | Z1 | R2 | Z2 |
| --- | --- | --- | --- | --- | --- |

|  | (in) | (in) | (in) | (in) | (in) |
| --- | --- | --- | --- | --- | --- |

| Inner Wall | 0.551 | 7.289 | 0.000 | 7.289 | 44.484 |
| --- | --- | --- | --- | --- | --- |

| Inboard Divertor, Vertical Section | 1.000 | 9.967 | 44.484 | 9.967 | 65.055 |
| --- | --- | --- | --- | --- | --- |

| Inboard Divertor, Horizontal Section | 1.000 | 8.967 | 64.055 | 20.000 | 64.055 |
| --- | --- | --- | --- | --- | --- |

| Outboard Divertor | 1.000 | 23.000 | 64.490 | 45.686 | 56.970 |
| --- | --- | --- | --- | --- | --- |

| Primary Passive Stabilizer | 1.000 | 47.753 | 43.341 | 55.350 | 20.550 |
| --- | --- | --- | --- | --- | --- |

| Secondary Passive Stabilizer | 1.000 | 38.515 | 58.537 | 47.847 | 43.139 |
| --- | --- | --- | --- | --- | --- |

b. Surfaces of plasma facing tiles located on convex structures shall be machined to a convex shape to conform to the surfaces given in Table PFC-2.1-1.

c. For surfaces of plasma facing tiles located on concave structures, if discrete flat faced tiles are used to approximate the required curved surfaces, the center of the flat faces shall be tangent to the surfaces given in Table PFC-2.1-1.

2.2 Passive Stabilizers

a.  The passive stabilizers shall consist of primary (closest  to midplane) and secondary (furthest from midplane) pieces, one set above the midplane and one set below the midplane of the NSTX device.

b.  Each stabilizer shall be made up of a series of conically shaped copper plates which are electrically connected in the toroidal and poloidal directions.  Each stabilizer shall have a minimum of one toroidal electrical break  [R1].

c.  The passive stabilizers shall be mounted to brackets extending from the outer vacuum vessel cylinder, and will be located from machined surfaces on these brackets

d.  The passive stabilizers shall be electrically connected to the vessel via their mounting structures [R2].  The connections shall contain provisions to allow for alignment adjustments and facilitate removal of individual plates for modification to accommodate access for future diagnostic upgrades.

e.  The overall size of the individual plates making up the stabilizers must be such that they can be passed through the large horizontal vacuum vessel ports ( largest dimension not greater than 20 in.) and easily manipulated by no more than two persons ( < 125 lb.) [R3].

f. The stabilizers shall be connected poloidally with a saddle connection at the same location as the toroidal electrical break [R1].

g.  The plasma facing surfaces of the stabilizers shall be protected with graphite tiles. (Union Carbide - ATJ or equivalent)

h.  The plasma facing surface of the attached tiles shall be installed such that they are aligned relative to adjacent tile surfaces within +/-0.5mm.

i.  Toroidal gaps shall be provided between adjacent passive stabilizer plates at a minimum of 4 locations spaced 90 degrees apart and at a maximum of 12 locations spaced 30 degrees apart.   The gaps shall provide a 2 inch clear path for diagnositic viewing of the plasma.

2.3 Inner Wall Armor

a.  The inner wall armor shall be composed of carbon-carbon tiles attached to the center stack casing.

b. The tiles shall be designed and fabricated such that the plasma facing surfaces of the tiles will be aligned relative to adjacent tile surfaces within +/-0.5mm without individual adjustment.

c. The tiles shall be mounted in such a way that heat transfer from the tiles to the center stack casing is minimized.

d.  Slots shall be provided in the backside of at least half the tiles such that diagnostic wires and magnetic diagnostics can be positioned between the center stack casing and the tiles.

2.4 Inboard Divertor Area Strike Plates

a.  The inboard divertor area strike plates shall consist of carbon-carbon tiles attached to the center stack casing.

b.  The tiles shall be designed and fabricated such that the plasma facing surfaces of the tiles will be aligned relative to adjacent tile surfaces within +/-0.5mm.

c.  The poloidal extent of the inboard divertor strike plates shall be such that it eliminates all line of sight between the upper and lower ceramic ring/bellows assemblies and the plasma.

d.  The inboard divertor plates shall not be traced for active temperature control, but will be thermally connected to the upper and lower flanges of the center stack casing.  The center stack casing flanges will have active temperature control.

2.5 Outboard Divertor Area Strike Plates

a.  The outboard divertor area strike plates shall consist of segmented upper and lower toroidal rings composed of copper plates covered with carbon-carbon tiles.

b.  The outboard divertor plates shall be attached to the upper and lower domes of the vacuum vessel, and will be located relative to machined features on these domes to within +/- 0.2 mm

c.  The tiles shall be designed and fabricated such that the plasma facing surfaces of the tiles will be aligned relative to adjacent tile surfaces within +/-0.5mm.

d.  The outboard divertor plates shall be traced with stainless steel tubing for active temperature control during bakeout and operation.

3.0 Performance & Operational Requirements

All materials for the PFCs, their connections and mountings must :

a.  Be approved for high vacuum compatibility [R4].

b.  Be bakeable to 350°C [R5].

c.  Have a relative magnetic permeability of < 1.02 [R6].  Exceptions must have prior NSTX project approval.

d.  Be compatible with Glow Discharge Cleaning  [R7].

3.1 Baseline Operation

a.  For baseline operation there will be no active cooling of the plasma facing components during a plasma discharge.  However, the plasma facing tiles on the outer passive stabilizer and in the divertor regions will be thermally attached to actively cooled plates, such that the tiles will cool to the plate temperature between discharges.  The inner wall armor will be thermally isolated from the center stack casing and must rely on radiation cooling to the other plasma facing tiles between discharges.  The tiles must accommodate a heat flux for the baseline duty cycle as described in Table PFC-3.1-1 [R8].

Table PFC-3.1-1: Max PFC Heat Loads for Baseline NSTX Duty Cycle

| Component | Max Value | Units | Ref |
| --- | --- | --- | --- |

| Passive Stabilizers | 0.1 | MW/m2 | R3 |
| --- | --- | --- | --- |

| Inner Wall Armor | 1.9 | MW/m2 | R3 |
| --- | --- | --- | --- |

| Inboard Divertor Area Strike Plates | 7 | MW/m2 | R3 |
| --- | --- | --- | --- |

| Outboard Divertor Area Strike Plates | 17.1 | MW/m2 | R3 |
| --- | --- | --- | --- |

Table 3.1-2: PFC Heat Loads for Baseline NSTX Operation Scenarios

| Scenario | Parameter | Inner Div | Outer Div | Inner Wall | Total |  |
| --- | --- | --- | --- | --- | --- | --- |

|  |  | Upper | Lower | Upper | Lower |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Natural Divertor | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 0.0 | 1.1 | 1.1 | 2.1 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.000 | 0.094 | 0.094 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 0.0 | 4.3 | 4.3 | 1.9 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Double Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.4 | 0.4 | 1.7 | 1.7 | 0.0 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.038 | 0.038 | 0.038 | 0.038 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 5.1 | 5.1 | 17.1 | 17.1 | 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Single Null | Total Power |  |  |  |  |  | 6.0 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Radiated Power |  |  |  |  |  | 1.8 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Non-Radiated Power | 0.0 | 1.4 | 0.0 | 2.8 | 0.0 | 4.2 | MW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Power Flux Width | 0.000 | 0.092 | 0.000 | 0.100 | 2.000 |  | m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Peak Flux | 0.0 | 7.0 | 0.0 | 10.8 | 0.0 |  | MW/m^2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

b.  The peak temperature limit for the graphite tiles shall be 1200°C.

c.  The peak temperature limits for the carbon-carbon tiles shall be 1200°C.

3.2 Upgrades

a.  For the NSTX upgrades, actively cooled PFCs may replace the baseline components.  The plasma facing tiles of the outer passive stabilizer, inner wall armor and divertor strike plates shall be capable of removing heat generated during the pulse for the long pulse upgrade duty cycle and for neutral beam upgrades as described in Table PFC-3.1-3.

Table PFC-3.1-3: Max PFC Heat Loads for Upgrade NSTX Duty Cycle

| Component | Long Pulse Upgrade Value | NB Upgrade Value | Units | Ref |
| --- | --- | --- | --- | --- |

| Passive Stabilizers | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Limiters | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Inner Wall Armor | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

| Divertor Area Strike Plates | TBD | TBD | MW/m2 | TBD |
| --- | --- | --- | --- | --- |

b.  For the neutral beam upgrade, armor may be provided as part of a PFC upgrade to protect the vessel and center stack casing due to neutral beam shine-through and due to inadvertent unattenuated neutral beam loads.  However, if no PFC upgrade is undertaken, then the pulse duration and repetition period shall be limited in accordance with the capability of the baseline PFCs (during combined NBI + RF operations) for any pulse during which the heating power exceeds the baseline.

APPENDIX

| # | Description | Value | Units | Reference |
| --- | --- | --- | --- | --- |

| 1 | Passive Stabilizer Requirements | n.a. | n.a. | GRD, sec. 1.2.1.c |
| --- | --- | --- | --- | --- |

| 2 | Passive Stabilizer Mounting Requirements | n.a. | n.a. | GRD, sec. 1.2.1.g |
| --- | --- | --- | --- | --- |

| 3 | Passive Stabilizer Removal Requirement | n.a. | n.a. | GRD, sec. 1.2.1.h |
| --- | --- | --- | --- | --- |

| 4 | High Vacuum Compatibility Requirement | n.a. | n.a. | GRD, sec. 1.1.4.c |
| --- | --- | --- | --- | --- |

| 5 | Bakeout Temperature | 350 | °C | GRD, sec. 2.3.2.a |
| --- | --- | --- | --- | --- |

| 6 | Relative Magnetic Permeability | <1.02 | n.a. | GRD, sec. 1.1.4.a |
| --- | --- | --- | --- | --- |

| 7 | Glow Discharge Cleaning | n.a. | n.a. | GRD, sec. 2.3.3 |
| --- | --- | --- | --- | --- |

| 8 | Heat Loads | n.a | n.a | GRD, sec. 2.6.2. |
| --- | --- | --- | --- | --- |