13-960418-AWB-01

TO: C Neumeyer

FROM: A Brooks

DATE:  20 March 1996
SUBJECT:  TF Resistance vs I2t

References: 	(1) Memo 52-960306-CLN-01 by C Neumeyer titled"Preliminary Analysis of TF performance"

(2) Memo 13-960320-AWB-01 by A Brooks titled  "TF Center Stack Geometry"

The R vs I2t curve presented in reference 1 above for use in power supply simulations has been updated to reflect the TF CS geometry given in reference 2.  The difference is in the smaller area of the "Inner Leg" ( ie CS ) allowed since we are operating at higher tempeatures. The resistance given includes only the TF winding and does not account for any buss work.

| Number TF Coils | 12 |  |
| --- | --- | --- |

| Turns/Coil | 6 |  |
| --- | --- | --- |

| Inner Leg |  |  |
| --- | --- | --- |

| Total Area | 0.002017 | m^2 |
| --- | --- | --- |

| Turn Area | 0.00033617 | m^2 |
| --- | --- | --- |

| Turn Length | 5.04 | m |
| --- | --- | --- |

| Total Length | 362.88 | m |
| --- | --- | --- |

| Outer Leg |  |  |
| --- | --- | --- |

| Total Area | 0.0232 | m^2 |
| --- | --- | --- |

| Turn Area | 0.00386667 | m^2 |
| --- | --- | --- |

| Turn Length | 7.03 | m |
| --- | --- | --- |

| Total Length | 506.16 | m |
| --- | --- | --- |

| Copper Properties |  |  |
| --- | --- | --- |

| Resistivity | 1.72E-08 | ohm-m |
| --- | --- | --- |

| Reference Temperature | 20 | C |
| --- | --- | --- |

| Resist Temp Coef | 0.0041 | 1/C |
| --- | --- | --- |

| Density | 8940 | kg/m3 |
| --- | --- | --- |

| Specific Heat | 386 | J/kg-C |
| --- | --- | --- |

| Inital Temp | 20 | C |
| --- | --- | --- |

| Initial Resistivity | 1.724E-08 | ohm-m |
| --- | --- | --- |

The figure below shows the Resistance due to the Inner Leg,  the Outer Leg, and the Total. verses ∫ i2(t) dt.  Also shown is the temperature of the inner leg and outer leg.

Using the same form as previoulsy presented, the above curve for the total TF resistance can be fit as follows:

R = 0.020874 + 3.3314e-12* X + 3.6422e-22 * X2

where once again:

R = total coil set resistance

X = ∫ i2(t) dt

cc:

P Heitzenroeder 	M Ono	J Spitzer

NSTX File