13-960320-AWB-01

TO: DISTRIBUTION

FROM: A Brooks

DATE:  20 March 1996
SUBJECT:  TF Center Stack Geometry

Reference:  NSTX Memo 52-960306-CLN-01

The revised TF Coil Center Stack geometry is defined herein. The TF conductor bundle outer radius ( exclusive of overwrap/groundwrap ) has grown from 8.5 cm to 9.78 cm to meet the requirements laid out below.

The TF conductor is sized to limit the maximum insulation temperature to 120 C when the TF is pulsed to 3.2 kG ( at 0.80 m radius ) for a 5 second flat top. The initial temperature is 20 C, the temperature of the available cooling water. No credit is taken for the cooling during a pulse since the instrumentation planned to protect against coolant loss ( blockage, rupture, etc ) is not designed to provide detection and shutdown on the timescale required during a pulse .

The TF Coils will be driven by the the C-Site MG sets, as described in the reference memo. While the initial analysis presented indicates a 5.6 sec ESW may be achievable, a 6.0 sec ESW was used as the design basis to cover the uncertainty of the MG controls and to survive  an L/R decay within the TF.

The TF Coils must be recooled in less than 5 minutes using the existing cooling water system ( 4800 gpm at 147 psi ). It is also necessary to maintain a high exit pressure ( 50 psia ) on the water to prevent boiling. This limits the available pressure drop thru the coil to 75 psi making allowances for pressure drop in the supply and return piping. These requirments can be met by a rather small hole ( 0.10"). The diameter choosen, based more on manufacturing constraint, is 0.25" dia. With an 0.25" hole and the available dP, flow velocities will be greater than 6 m/s. and recool time will be less than 1 minute. The minimum required flow for a 5 minute recool is less than 1 m/s. At this low flow the available pressure can easily cool the outer legs of the TF coil, reducing the total number of  coolant flow paths.

The voltage standoff for the TF turn to turn conductor insulation is modest. The total voltage drop across the TF during ramping is much less than 1 KV, for which a 0.020" wrap ( 0.040" turn to turn ) is adequate.

The layout and shapes of the conductor are chosen to maximize the conductor packing fraction for the 72 Turns. ( Rectangular and Hexagonal arrays of identical cross section were examined but yielded turn packing fractions of only 84% and 83% respectively, requiring further increases to the bundle radius of nearly a centimeter )  There are four conductor types of equal areas arranged in concentric layers. The number of turns per layer (4,12,20&36) was chosen to permit fabrication of four seperate quadrants. The inner radius face of the 3 outer layer conductors is flat to facilitate wrapping of turn insullation. An overrap of 0.25" provides some room for final machining of bundle to meet assembly tolerances.

The attached table and figure summarize the conductor parameters. Note that the minimum required area to satisfy thermal performance reflects 0.005" tolerance on conductor dimensions ( width, thickness and hole ).

cc:

J Citrolo	H M Fan	P Heitzenroeder 	C Neumeyer

M Ono	J Robinson	J Spitzer

NSTX File

T

| NSTX TF Coil Center Stack |  |  |
| --- | --- | --- |

|  |  | Minimum Requirement | Design Value |
| --- | --- | --- | --- |

| Copper Properties |  |  |  |
| --- | --- | --- | --- |

| Density | kg/m3 | 8940 |  |
| --- | --- | --- | --- |

| Specific Heat | J/kg-C | 386 |  |
| --- | --- | --- | --- |

| Resistivity at T0 | ohm-m | 1.724E-08 |  |
| --- | --- | --- | --- |

|  |  | 0.0041 |  |
| --- | --- | --- | --- |

|  |  | 20 |  |
| --- | --- | --- | --- |

| Intial Temperature | C | 20 |  |
| --- | --- | --- | --- |

| Max Temperature | C | 120 |  |
| --- | --- | --- | --- |

| Initial Resistivity | ohm-m | 1.724E-08 |  |
| --- | --- | --- | --- |

| Max Resistivity | ohm-m | 2.43084E-08 |  |
| --- | --- | --- | --- |

| Allowable "G" function | a2/m4-s | 1.67743E+16 |  |
| --- | --- | --- | --- |

| Equivalent Square Wave | s | 6 |  |
| --- | --- | --- | --- |

| Allowable Current Density | a/m2 | 52874503.02 |  |
| --- | --- | --- | --- |

| Turn Current | a | 17778 |  |
| --- | --- | --- | --- |

| Required Copper Area | m2 | 0.00033623 |  |
| --- | --- | --- | --- |

| Coolant Hole Dia | m | 0.00635 | 0.006477 |
| --- | --- | --- | --- |

| Coolant Hole Area | m2 | 3.16692E-05 | 3.29487E-05 |
| --- | --- | --- | --- |

| Conductor Area | m2 | 0.000367899 | 0.000373098 |
| --- | --- | --- | --- |

| Corner Radii | m | 0.0015875 | 0.0015875 |
| --- | --- | --- | --- |

| Corner Area | m2 | 2.16332E-06 | 2.16332E-06 |
| --- | --- | --- | --- |

| Conductor Width to Thickness Ratio |  | 2 | 2 |
| --- | --- | --- | --- |

| Conductor Width | m | 0.027205244 | 0.027332244 |
| --- | --- | --- | --- |

| Conductor Thickness | m | 0.013602622 | 0.013729622 |
| --- | --- | --- | --- |

| Turn Insulation Thickness | m | 0.0005 | 0.0005 |
| --- | --- | --- | --- |

| Turn Insulation Area | m2 | 4.39712E-05 | 4.42252E-05 |
| --- | --- | --- | --- |

| Total Turn Area | m2 | 0.000411871 | 0.000417323 |
| --- | --- | --- | --- |

| Number of Conductors |  | 72 | 72 |
| --- | --- | --- | --- |

| Area of Center Stack | m2 | 0.029654677 | 0.030047273 |
| --- | --- | --- | --- |

| Radius of Center Stack | m | 0.09715646 | 0.09779747 |
| --- | --- | --- | --- |

| Cu Fraction |  | 0.816 | 0.815 |
| --- | --- | --- | --- |

| Hole Fraction |  | 0.077 | 0.079 |
| --- | --- | --- | --- |

| Insulation Fractio |  | 0.107 | 0.106 |
| --- | --- | --- | --- |

| Ground Wrap Thickness | m | 0.00635 | 0.00635 |
| --- | --- | --- | --- |

| Radius after Ground Wrap | m | 0.10350646 | 0.10414747 |
| --- | --- | --- | --- |