13-970214-AWB-02

TO: DISTRIBUTION

FROM: A Brooks

DATE:  14 February 1997
SUBJECT:  Thermal Analysis VV/CS during Normal Operation

Ref: 	1) Memo from S. Kaye Dated Oct 24,1996, "NSTX Divertor Heat Flux Estimates"

2) Memo 13-970214-AWB-01 from A Brooks dated Feb 14,1997,

"Bakeout of Vacuum Vessel and Internals"

This memo documents the thermal analysis of Vacuum Vessel , Center Stack and Plasma Facing Components as it affects the Center Stack. As the VV and PFC designs evolve, they will need to be addressed in more detail.

Requirements

Normal operation of the NSTX device will result in heat fluxes from the plasma impinging on the surfaces of PFCs and VV. Reference 1 provides guidance on the magnitude and distribution of these heat fluxes under various modes of operation, specifically Natural Divertor ( ND ), Double Null Divertor ( DN ) and Single Null Divertor ( SN ) Plasma Configurations. For the Center Stack, the most significant of these is the Natural Divertor Configuration during which, for a 6 MW plasma, half of the non-core-radiated power or 2.1 MW is deposited on the center stack giving average heat fluxes of ~ 1 MW/m2 with a peaking of 1.6 MW/m2. The other half is deposited on the outer divertor plates, split between the top and bottom.

The radiative power which accounts for the remaining 1.8 MW or 30 % of the total plasma power is assumed to be distributed uniformly over all plasma view surfaces. Using approximately 35 m2 of viewing surface area, the average radiative flux is 0.051 MW/m2.

The device is being designed for a pulse duration of 5 seconds when operating at 6 MW. The repetition rate is once every 300 seconds ( 5 minutes ) for OH single swing operation where the heating in the OH is minimal ( ~ 50 C ) For OH double swing operation, during which the OH Coil reaches 95 C in a single pulse, the rep rate is reduced to once every 600 seconds ( 10 minutes ) to eliminate thermal ratcheting.

One of the prime concerns is to demonstate adequate protection of the center stack coils by limiting heat flow and resultant temperatures at the OH groundwrap insulation.

Design Overview

The Center Stack Casing and Tiles are radiatively cooled to the outer PFCs and VV. To provide an adequate radiative sink, the outer PFCs ( Inboard Divertor, Outboard Divertor and Passive Plates ) and the Vacuum Vessel will be actively cooled. The system will also provide heating and cooling during Bakeout. ( see reference 2 ).

There is additonal heat leakage to the center stack thru the Microtherm insulation which provides some cooling to the CS Tiles.

Analysis

The 2D Axisymmetric ANSYS Thermal radiation and conduction model used to analyse Bakeout ( reference 2 ) was adapted to provide a global picture of heat flows and resultant temperature distributions. Since the Center Stack is raditively cooled, an understanding of its radiative enviroment is necessary.

This model was run using time averaged loading to permit simulation of the ratcheting during an 8 hour.

A local 1-D ( R in cylindrical cordinates ) model of the CS tiles, casing, Microtherm Insulation, and Cooled OH Coil was run thru the pulse transient using the peak heat fluxes ( 1.6 MW/m2 ) on the CS for the Natural Divertor Operation. Both single swing OH and double swing OH operations were investigated.

Assumptions

The assumptions stated in reference 2 regarding bakeout are also applicable here.

The CS Tiles are assumed to be equivalent to ATJ Graphite with normal thermal conductivity a function of temperature ranging from 101 w/m-C at room temperature down to 59 w/m-C at an elevated temperature of 977 C. The surface emissivity was taken at 0.7 for all the tiles and a value of 0.27 was used for the stainless steel vv and inconel casing.

Approach

Figure 1 shows the model geometry used with the applied loading for the Natural Divertor Configuration at 6 MW.

Figure 2 identifies the location of cooling and the heat transfer coefficients used.

Results

Figure 3 shows the temperature transient using the time averaged loading for an 8 hour operation day followed by an 8 hour cooldown period. The plots represent bulk temperatures of the components identified. The active cooling of the VV, Divertors and PP limits their thermal ratcheting, providing a fairly cool ( ~ 50 C ) radiative sink for the CS tiles. Over 90% of the heat deposited on the CS is reradiated outboard. Less than 10% is leaked to the CS coils

Table I below summarizes the bulk temperature distribution found after fully ratcheting for the Natural Divertor Plasma. The data is based on averaged heat loads ( due to the coarseness of the model ) and does not include peaking.

Table I

|  | Bulk Temperature Distribution following 8 hours of 6MW pulsing every 300 s |
| --- | --- |

|  | Min Temperature, C | Max Temperature, C |
| --- | --- | --- |

| CS Tiles | 211 | 520 |
| --- | --- | --- |

| CS Casing | 35 | 460 |
| --- | --- | --- |

| VV | 25 | 34 |
| --- | --- | --- |

| PP | 33 | 36 |
| --- | --- | --- |

| Outboard Divertor | 60 | 70 |
| --- | --- | --- |

| Inboard Divertor | 39 | 49 |
| --- | --- | --- |

Table II below shows the resulting heat flows to the various components for 6MW of total heating for 5 secs every 300 secs ( average 100 KW )

Table II

|  | Heat Flows, KW |
| --- | --- |

| Heat Loss to CS OH Coil | 2.8 |
| --- | --- |

| Heat Loss to CS PF1a Coils | 0.4 |
| --- | --- |

| Vacuum Vessel | 16.6 |
| --- | --- |

| Protective Plates | 36.3 |
| --- | --- |

| Divertors | 44.3 |
| --- | --- |

| Total | 100.4 |
| --- | --- |

Using a 50 C radiative sink and the peak heat flux of 1.6 MW/m2 in the 1-D model, figures 4 and 5 shows the transient response at the surface of the CS tiles, the Inconel Casing and the OH groundwrap Insulation temperatures. Figure 4 is for the OH single swing operation where the OH conductor is at 50 C and pulsing is every 300 sec. Figure 5 is for OH double swing operation where the OH conductor is at 95 C but pulsing is only every 600 s.

The curves for the tiles show approximately 320 C rise in the surface temperature of the tile at the end of a pulse, equilibrating shortly thereafter for a bulk temperature rise of 200 C.

The Center Stack fully ratchets in less than 2 hours. At that point there is only about a 60-70 C difference on average between the bulk temperature of the tile  and the hot Inconel casing ( peak temperature 568 C).. Earlier analyses using poorer thermal insulation showed much lower casing temperature and slightly lower tile temperatures, with a larger difference between the two, indicative of much larger heat losses to the CS coils. While the casing temperature has gone up significantly, the heat loss and corresponding temperature gradient from the OH conductor to the OH groundwrap has gone down offering better protection to the CS coils.

The Table II below summarizes the fully ratchet temperature variation seen during a pulse cycle.

Table II

|  | OH=50C, 300s RR |  | OH=95C, 600s RR |  |
| --- | --- | --- | --- | --- |

|  | Min Temp, C | Max Temp, C | Min Temp, C | Max Temp, C |
| --- | --- | --- | --- | --- |

| CS Tiles | 525 | 837 | 380 | 702 |
| --- | --- | --- | --- | --- |

| CS Casing | 549 | 568 | 400 | 424 |
| --- | --- | --- | --- | --- |

| OH Groundwrap | 60 | 61 | 101 | 102 |
| --- | --- | --- | --- | --- |

cc:

J Citrolo	H M Fan	P Heitzenroeder 	C Neumeyer

M Ono	J Robinson	J Spitzer		R Parsells

NSTX File