13-970214-HMF-01

TO: 		Distribution

FROM:  	H. M. Fan, J. Spitzer
SUBJECT:  	TF Inner Bundle Stress Analysis

The inner TF bundle in NSTX is subjected to torsional radial and vertical  mechanical loads as well as thermal loads.  The major portion of the torsional load results from the OH solenoid fields crossing over the inner bundle conductors, primarily the outer layer.  This results in a torque of ~24,000 N-m on TF bundle.  In addition torsional loads are created in the flags and connectors of the inner bundle from the poloidal fields crossing over these elements.  The inner bundle is subjected to a radial compression resulting from the magnetic force ITF X BTF.  In addition to gravity loads, vertical loads are generated by the TF fields crossing over the flags, connectors, and outer legs, creating a magnetic force.  Thermal loads arising from the i2(t) heating of the TF bundle leg result in an overall growth in its length of ~3/8".

The inner TF bundle is supported from the Test Cell floor via support pedestals.  At the top the bundle is free to grow vertically but is restrained against translation and rotation.  These restraints are provided by the NSTX CS restraint structure, shown in Fig. 1.

The restraint structure is made up of a stainless steel torque reaction collar which is bonded to the OD of the TF bundle just above the ends of the OH tension cylinder via glass epoxy which acts in shear.  A bolted flange connection  transmits the torque from the collar to the hub portion of the restraint structure.  Both “tiers” of flags are tightly shimmed in the vertical direction between the two discs in the hub portion of the restraint structure.  In this manner, the entire circular array of radial flags clamped between upper and lower discs together form two tiers of  thick discs.

Vertical loads on the disc result in simple shearing loads across bolts of the joints.  The bolts are shoulder bolts designed to act in shear and resist slippage at the joint. The shearing load is absorbed by a total of 36*4, or 144 bolts.  G-10 wedges are installed between the flags so the entire array of flags behave as a solid disc.  The resulting torque is reacted through the restraint structure to the umbrella structure which is welded to the vacuum vessel.  The connection between the CS restraint structure and the umbrella structure allows movement vertically (to permit free thermal expansion of the central bundle) but is fixed in torsion.  This support arrangement, in conjunction with the flexible links between the outer legs and the bundle,  permits the bundle to freely expand upwards.

Fig 1 NSTX CS Restraint Structure

An ANSYS finite element model of the TF coil system was generated to quantify the loads described above and determine the resulting stresses.  The model included one-quadrant of the TF coil system (Figure 2); three outboard legs with three conductors each, nine inner core conductors and their turn insulation (Figure 3), the inner and outer TF flags and the demountable links between the inner conductors and the outer legs.  Only the top half of the conductors were modeled with both the inner and outer conductors fixed at the midplane.

In these analyses the OH current was set to its maximum value of 24kA and PF1a was modeled assuming a current of 10 kA.  The TF current used in the analysis was 35.56 kA which corresponds to the 0.3 T baseline toroidal field (the analyses are linear and the results can be scaled to obtain the responses at a BT=0.6 T.  Figure 4 illustrates the torque, q per unit distance (Nm/m) along the length of the TF inner core with the upper end of the core unconstrained.  This loading creates a net torque of ~24,000. Nm on the core, Figure 5.  This loading results in a von Mises stress of 4.20 ksi (28.9 MPa) in the copper, a maximum shear of 1.51 ksi (10.4 MPa) in the insulation in the vicinity of PF1a, and a torsional displacement of 0.1 in.  These results are presented as Model A in Table 1.  This analysis represents the "upper" bound for yz shear in the magnet insulation.

The model was exercised in three subsequent analyses to quantify the effect of the CS restraint structure as well as determine the "lower" bound of the insulation shear.  Figure 6 illustrates the constraints imposed on these models in the analyses.  Model B provided torsional constraints along the radial length of the flag.  This boundary condition simulates the effect of the G-10 wedges which are installed between the flags.  These wedges prevent bending along the length of the flag and transmit the torsional loads through the hub assembly to the umbrella structure of the vacuum vessel.  For this set of constraints the maximum von Mises stress in the copper is 5.0 ksi while the "maximum" shear stress in the TF insulation is -1.7 ksi at the top of the OH solenoid.  The torsional displacement drops to 0.01 in.

Model C added additional constraints to quantify the effect of the torque reaction collar.  The result of these additional constraints was a slight reduction in the copper and insulation stresses and minimal change in the displacement.  The final analysis, model D, examined the effects of restraints acting only at the torque reaction collar.  This model represents the effect of having the G-10 wedges not properly engaged.  Once again the stresses were comparable to Models C and D but the displacement is larger.

Note that the yz shear stress predicted in Models B, C and D is of approximately the same magnitude as that predicted in Model A but in the opposite direction.  As such these models quantify the "lower" bound of the maximum expected response of the TF inner bundle.

Table 1: TF Inner Bundle Stress Analysis

| FEM* | Min./ | Copper Stress |  | Insulation Stress |  |  |  |  |  | Displacement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Max. | von Mises |  | shear xy |  | shear yz |  | stress y |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  |  | MPa | ksi | MPa | ksi | MPa | ksi | MPa | ksi | mm | in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Model A | Min. |  |  | -4.42 | -0.64 | -5.53 | -0.80 | -7.11 | -1.03 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Max. | 28.93 | 4.20 | 3.46 | 0.50 | 10.44 | 1.51 | 4.86 | 0.70 | 2.572 | 0.101 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Model B | Min. |  |  | -7.26 | -1.05 | -11.77 | -1.71 | -8.06 | -1.17 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Max. | 34.34 | 4.98 | 1.60 | 0.23 | 3.72 | 0.54 | 8.99 | 1.30 | 0.282 | 0.011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Model C | Min. |  |  | -10.94 | -1.59 | -11.52 | -1.67 | -7.02 | -1.02 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Max. | 32.83 | 4.76 | 2.47 | 0.36 | 3.10 | 0.45 | 1.77 | 0.26 | 0.274 | 0.011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Model D | Min. |  |  | -10.80 | -1.57 | -11.60 | -1.68 | -7.15 | -1.04 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | Max. | 32.94 | 4.78 | 4.78 | 0.69 | 3.10 | 0.45 | 1.72 | 0.25 | 0.535 | 0.021 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Notes: | 1. All models are fixed at mid-plane and flag elements are coupled for the vertical displacement |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | 2. Model B has additional torsional constraints at flags |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | 3. Model C has additional torsional constraints at flags and TF collar |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | 4. Model D has additional torsional constraints at TF collar |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As highlighted earlier the TF inner core is supported at its base and is allowed to grow/move freely in the vertical direction.  This support arrangement creates small thermal stresses in the conductor and additional stress in the insulation of because of differences in the coefficients of thermal expansion between the copper and the insulation.  A yz shear stress in the insulation of < 1 ksi occurs.

Fig 2 - TF Inner Bundle FEA Model

Fig 3 - TF Inner Bundle FEA Model Cross-Section

Fig 4 - TF Inner Bundle Torque per Unit Height

Fig 5 - TF Inner Bundle Total Torque

Fig 6 - FEA Model Constraints

The TF inner conductors will be fabricated from quarter hard C104 copper which has a yield strength of 30.0 ksi and an allowable stress at room temperature of 20.0 ksi.  Short Beam Shear tests on the CTD-112P pre-preg insulation system which has been selected for the centerstack magnets indicate a shear strength of 11.0 ksi (77 MPa) at room temperature and 9.2 ksi (63 MPa) at 120°C which gives static allowable of 6.1 ksi (42 MPa) at 120°C.  From these analyses it can be concluded that the stresses in the TF inner core copper and insulation are comfortably below the allowable limits.

Distribution:

J. Citrolo	P. Heitzenroeder		C. Neumeyer	NSTX File