13-970214-HMF-02

TO: 		Distribution

FROM:  	H. M. Fan, J. Spitzer
SUBJECT:  	OH Solenoid Stress Analysis

As depicted in Fig. 1, the OH coil consists of four layers, wound two-in-hand, for a total of eight windings. There are 962 turns total, and the coil produces a flux swing of 0.62 volt-seconds with a current swing from +24 to -24kA.

Each winding consists of a continuous length (360' max.) of extruded copper conductor (0.394" x 0.630") with circular central cooling hole (0.188" dia). The winding is formed over a 304 stainless steel tube which serves as a tension spool piece to react axial loads. The conductor will be CDA alloy 104, silver bearing copper, (8 troy ounces per ton.  Conductor will be quarter hard with a yield strength of 30-38 ksi (0.5% elongation).

The OH solenoid is supported from the Test Cell floor via support pedestals.  It is free to expand vertically upwards independent from the TF bundle, PF1a, tension cylinder and centerstack casing.  The loading on the OH solenoid is comprised of:

•  Radial in-plane EM forces due to the high vertical magnetic flux within the centerstack.  The forces peak at the inner layer of the solenoid;

•  Thermal loads as the coolant wave progresses through the winding.  Cooling water at 10°C enters each conductor at the bottom of the solenoid and exits at the top at nearly 90°C;

•  Tension/axial loads resulting from the interaction of the OH current with the fields from nearby PF coils.  PF1b has the greatest effect since it has no upper counterpart.  The axial load it creates, which is always downward, is reacted through the common structure which supports both the OH and PF1b at the bottom of the centerstack; and

•  Vertical force due to the interaction of the OH current with the radial magnetic field at both ends of the solenoid.

•  Gravity loads, which are small compared to these other effects.

The lower end of the centerstack accommodates both a PF1a coil and PF1b coil whereas the upper end must accommodate only a PF1a coil.  The general support

Figure 1: OH Coil Winding

arrangement, is basically similar.  An array of gussets attach the PF coils to the end flange of the OH solenoid tension tube.

The OH and other poloidal coils located in the centerstack are independently supported so thermal excursions of the TF do not change their vertical positions.  The OH solenoid is also vertically constrained and preloaded via the tension cylinder so that net axial loads do not result in tension between the solenoid turns.

To characterize the axial loads in the OH solenoid a series of finite element analyses were conducted.  The purpose of these analyses was to produce an influence matrix which would describe the effect of each PF coil on the axial load on the OH.  Each term in the matrix was developed by applying a unit current on the OH solenoid and on an individual PF coil bundle.  The effect of the plasma was also included.  The net vertical forces due to these influence coefficients was then calculated.  This matrix is presented in Table 1.

The maximum axial force, caused by PF1b is a downward axial force of ~40000 lb.  The relative current directions of the OH and PF1b are such that this force will always be downward and is reacted through the lower support hub to the floor.

A finite element analysis was conducted using the ANSYS program to determine the stresses and deflections in the OH solenoid.  Figure 2 illustrates the model used in the analyses.  Seven different analyses were conducted.  The first examined the stresses and displacements under the 5000 lb. preload.  The second examined the effects of EM loads only.  The third case had all conductors at 92°C.  This is reflective of the temperatures that would result adiabatically should the OH lose cooling.  Case four applies the maximum temperature gradient which occurs during operation, with the lower turn at 14.15°C and the maximum temperature of 83°C.  Cases 5 and 6 are combinations of the EM with the two thermal cases.  In these analyses the OH, PF1a and PF1b coils were all assumed to be operating at their maximum current.  The results of the analysis are given in Table 2.  The maximum stress in the OH copper is 23 ksi while the maximum shear stress in the insulation is less than 1 ksi.

Distribution:

J. Citrolo	P. Heitzenroeder	C. Neumeyer	J. Spitzer	NSTX File

Fig 2 OH Solenoid FEA Model

Table 1: Launching Load Influence Coefficients

| Circuit ID | Bsum | EM Loads Fz* | EM Loads Fr* | No. of Turns | Imax | Max. EM Loads Fz** |
| --- | --- | --- | --- | --- | --- | --- |

|  | (T/kA^2) | (lb/kA^2) | (lb/kA^2) |  | (kA) | (lb) |
| --- | --- | --- | --- | --- | --- | --- |

| OH | 0.27768 | 0.000 | 22206.9 | 952 | 24 | 0 |
| --- | --- | --- | --- | --- | --- | --- |

| PF1a-u | 0.11917 | 12.489 | 2461.1 | 48 | 10 | 2997 |
| --- | --- | --- | --- | --- | --- | --- |

| PF1a-l | 0.11917 | -12.489 | 2461.1 | 48 | 10 | -2997 |
| --- | --- | --- | --- | --- | --- | --- |

| PF1b-l | 0.08837 | -110.252 | 1507.4 | 36 | 15 | -39691 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2a-u | 0.01133 | 28.871 | 402.4 | 14 | 20 | 13858 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2a-l | 0.01133 | -28.871 | 402.4 | 14 | 20 | -13858 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2b-u | 0.01133 | 27.403 | 438.4 | 14 | 20 | 13153 |
| --- | --- | --- | --- | --- | --- | --- |

| PF2b-l | 0.01133 | -27.403 | 438.4 | 14 | 20 | -13153 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3a-u | 0.00636 | 14.338 | 464.2 | 15 | 20 | 6882 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3a-l | 0.00636 | -14.338 | 464.2 | 15 | 20 | -6882 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3b-u | 0.00636 | 13.633 | 482.0 | 15 | 20 | 6544 |
| --- | --- | --- | --- | --- | --- | --- |

| PF3b-l | 0.00636 | -13.633 | 482.0 | 15 | 20 | -6544 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4a-u | 0.00177 | 1.727 | 187.3 | 5 | 20 | 829 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4a-l | 0.00177 | -1.727 | 187.3 | 5 | 20 | -829 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4b-u | 0.00282 | 2.424 | 302.1 | 8 | 20 | 1164 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4b-l | 0.00282 | -2.424 | 302.1 | 8 | 20 | -1164 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4c-u | 0.00315 | 2.360 | 342.1 | 9 | 20 | 1133 |
| --- | --- | --- | --- | --- | --- | --- |

| PF4c-l | 0.00315 | -2.360 | 342.1 | 9 | 20 | -1133 |
| --- | --- | --- | --- | --- | --- | --- |

| PL-u | 0.00067 | 0.072 | 47.6 | 1 | 1000 | 1727 |
| --- | --- | --- | --- | --- | --- | --- |

| PL-l | 0.00067 | -0.072 | 47.6 | 1 | 1000 | -1727 |
| --- | --- | --- | --- | --- | --- | --- |

| Notes: | * Total forces in OH due to 1 kA in OH and column 1 circuit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

|  | ** Total forces in OH due to max. current in OH and column 1 circuits |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

Table 2: Results of OH Solenoid FEA for OH=24kA, PF1a=15kA, PF1b=20kA

| Load | Load | Copper Stress |  | Insulation Stress |  |  |  | Displacement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Case |  | von Mises (ksi) | Sy (ksi) | Sy (ksi) | Sz (ksi) | Sxy (ksi) | Sxz (ksi) | Dmax (in) | Dz (in) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 1 | Preload | 0.15 | 0.03 | -0.042 | -0.119 | 0.002 | 0.010 | 0.011 | 0.011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 2 | EM | 22.88 | 19.80 | 0.732 | -6.436 | 0.198 | 0.175 | 0.101 | -0.101 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 3 | Maximum Temp. | 2.27 | -2.46 | 1.499 | -1.196 | 0.106 | 0.327 | 0.246 | 0.245 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 4 | Temp. at 15.5 sec. | 8.23 | 8.01 | 2.134 | -5.366 | 0.096 | 0.959 | 0.217 | 0.217 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 5 | EM + Case 3 | 23.20 | 19.91 | 1.978 | -7.248 | 0.290 | 0.416 | 0.148 | 0.147 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 6 | EM + Case 4 | 23.20 | 19.93 | 2.210 | -7.142 | 0.280 | 0.823 | 0.119 | 0.119 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |