13-970214-JMS-01

TO: 		Distribution

FROM:  	J. Spitzer
SUBJECT:  	Centerstack Halo Current Stress Analysis

References:

1) NSTX Memo:  13-961010-JMB-01 "Estimate of Halo Loads for Inner Leg of the Vacuum Vessel"

2) Technical Note: "Aspect Ratio Study for Consistent Resistive Solutions (Halo Force Approximations), J. Bialek, 10/30/96.

3) NSTX Memo:  13-970130-JMB-01 "More Aspect Ratio Work... Estimate of Halo Loads for Inner Leg of the Vacuum Vessel"

To estimate the halo loads on the centerstack casing the IVBRANCH code was used to solve for the resistive distribution of current that resulted from four different halo current events, Reference 1.  In all cases 40% of the 1 megampere plasma current was assumed to flow in the casing with the toroidal peaking factor (TPF) equal to either 1 or 2.  The four cases analyzed were:

Case 1) 400 kA out at z=0.5m and 400kA in at z=-0.5m with a TPF=1

Case 2) 400 kA out at z=0.5m and 400kA in at z=-0.5m with a TPF=2

Case 3) 400 kA out at z=0.25m and 400kA in at z=-0.75m with a TPF=1

Case 4) 400 kA out at z=0.25m and 400kA in at z=-0.75m with a TPF=2

In all cases the toroidal field used was Btf=0.3T at .0.854m.  Figure 1a shows the current distribution for case 2.  For clarity, Figure 1b shows the same distribution with the structure "unwrapped."

The current distribution is nearly uniform and does not exhibit the toroidal peaking that is generally expected.  This effect was found to be due to the 'aspect ratio' ( the circumference of the casing divided by the vertical separation of the current sinks and sources) of the centerstack structure and the presence of additional conducting structure above and below the current sinks and sources.  For these load cases the vertical separation of the sinks and sources was 1.0 m while the circumference is 1.005 m therefore an aspect ratio of 1.  Further discussion on the effects of various aspect ratios will be presented in later sections.

The loads resulting from these analyses were applied to a finite element model of the centerstack casing and analyzed using ANSYS Figure 2.  In these analyses boundary conditions consistent with the casing support scheme were applied to the model; the casing was considered fixed at the base and constrained in the translational directions but free to move vertically at the top.

For Case 1 the loading is essentially a uniform inward pressure loading on the center of casing and the predicted stresses agree with textbook solutions for thin-walled pressure vessels.  Case 3 exhibits the same behavior.

For case 2 several different analyses were performed.  The first of these examines the effect of loading created by the interaction of the current with the toroidal field.  In addition to the effects from the inward pressure this loading results in a bending of the casing.  The maximum von Mises stress occurs at the flange connection to the shell and is 8.6 ksi (59.4 MPa), see Figure 3.  At the midplane the maximum bending stress is 2.6 ksi and the maximum displacement is < 0.4 mm.  The second analysis of the case 2  examined the effect of loading created by the interaction of the current with the poloidal fields.  In this instance the loading produces a slight deflection at the upper end of the casing.  The maximum stresses again occur at the attachment of the cylinder to the flange, Figure 4.  The last of the analyses was the combination of the effects of the combined PF and TF, Figure 5.

From the analyses of Case 2 the effect of the PF loads was seen to be small.  Therefore only the combined loading condition was analyzed for Case 4.  Here, as expected the result is nearly the same as that for the combined loading in Case 2.

As stated above the resistive current distributions in the centerstack analyses demonstrated very little toroidal peaking.  It was believed this effect was due to the overall geometry of the NSTX centerstack, a long slender cylinder.  To verify this belief an aspect ratio study was undertaken, Reference 2.  Here the 'aspect ratio,' AR, is defined as the circumference of the casing divided by the vertical separation of the current sinks and sources.  In this initial study a simple cylinder was examined.  The diameter of the cylinder was changed to reflect aspect ratios of 1, 2 and 3.  The effect of including additional conducting material above and below the current sinks and sources was also examined.  The results indicate that as the aspect ratio increases, a peaking does occur in the resistive distribution, Figure 6.  Figure 7 however, indicates that the presence of additional conducting structure reduces the magnitude of this peaking effect.

With some peaking evident at the higher aspect ratios further analyses were performed to determine whether the centerstack analyses had identified the peak loads on the centerstack, Reference 3.  In these analyses the vertical distance between the current sinks and sources was shortened to create aspect ratios of 4/3, 2 and 4.  Some peaking occurs in the models but not to the degree that was evident in the aspect ratio study.  Table 1 shows the net forces resulting from halo currents at the various aspect ratios.  In all cases the net loading decreases as the aspect ratio goes up.

Table 1: Net Forces for Various Aspect Ratios

| AR | Field | net Fx | net Fy | net Fz | net M |
| --- | --- | --- | --- | --- | --- |

|  |  | (N) | (N) | (N) | (N*m ) |
| --- | --- | --- | --- | --- | --- |

| 1 | TF | 0.008 | 385.546 | 0 | (0,0,0) |
| --- | --- | --- | --- | --- | --- |

| 1 | Poloidal | 118.67 | -0.00003 | -0.0003 | (0,-9055.96,19.86) |
| --- | --- | --- | --- | --- | --- |

| 4/3 | TF | 0.0038 | 178.455 | 0 | (0,0,0) |
| --- | --- | --- | --- | --- | --- |

| 4/3 | Poloidal | 96.48 | -0.00001 | -0.00017 | (0,-7490.9,15.06) |
| --- | --- | --- | --- | --- | --- |

| 2 | TF | 0.0017 | 80.153 | 0 | (0,0,0) |
| --- | --- | --- | --- | --- | --- |

| 2 | Poloidal | 62.023 | 0 | -0.00007 | (0,-5340.8,10.123) |
| --- | --- | --- | --- | --- | --- |

| 4 | TF | 0.0006 | 30.714 | 0 | (0,0,0) |
| --- | --- | --- | --- | --- | --- |

| 4 | Poloidal | 32.284 | 0 | -0.00003 | (0,-2776.7,4.917) |
| --- | --- | --- | --- | --- | --- |

To conclude this aspect ratio study the effect of the loading on the centerstack  at the various aspect ratios was calculated.  For simplicity the casing was examined as if it were simply supported.  Figure 8 shows the load distribution for the net force in the y (horizontal) direction.  The stresses and deflections for each case are given in Table 2.  It is clear from the table that the maximum value for this study was  AR=1.  (Note the FEA model was analyzed with simply supported boundary conditions and the results agree to within 1% with the calculated stresses and deflections.)  One final exercise extrapolated the results of the aspect ratio study to examine the effect of an aspect ratio of 0.5 using the same approach as above.  Here it would be expected that the current distribution would be uniform but there would be some increase in the net translational force.  These results indicate a small increase in the stresses and deflection to this effect σmax = 5.1 ksi, ∂ = 0.053 in.

Table 2: Maximum Stress and Deflection from Net Loads

| AR | Field | net Fy | Max Stress | Max Deflection |
| --- | --- | --- | --- | --- |

|  |  | lb (N) | psi (MPa) | in (m) |
| --- | --- | --- | --- | --- |

| 1 | TF | 86.69 (385.546) | 3550 (24.5e6) | .0356 (.0009) |
| --- | --- | --- | --- | --- |

| 2 | TF | 18.01 (178.455) | 2887 (19.9e6) | .0212 (.00054) |
| --- | --- | --- | --- | --- |

| 4 | TF | 6.91 (30.714) | 1966 (13.6e6) | .00058 (.00001) |
| --- | --- | --- | --- | --- |

Distribution:

J. Bialek	J. Citrolo	P. Heitzenroeder	C. Neumeyer	NSTX File

Fig 1 Current Distribution, Case 2

Fig 2 ANSYS Model of Centerstack

Fig 3 von Mises Stresses for Current Interaction with TF, Case 2

Fig 4 von Mises Stresses for Current Interaction with PF, Case 2

Fig 5 von Mises Stresses for Current Interaction with PF and TF, Case 2

Fig 6 Vertical Current vs Toroidal Angle for Various Aspect Ratios without Additional Structure

Fig 7 Vertical Current vs Toroidal Angle for Various Aspect Ratios

with Additional Structure

Fig 8 Net Load Distribution for Various Aspect Ratios