13-970214-JMS-02

TO: 		Distribution

FROM:  	J. Spitzer
SUBJECT:  	Stress Analysis of the Centerstack Case and Support Structure from 		Bakeout and Operational Thermal Loads

References:

1) NSTX Memo:  13-970214-AWB-01 "Bakeout of Vacuum Vessel and Internals"

2) NSTX Memo:  13-970214-AWB-01 "Thermal Analysis VV/CS during Normal Operation"

NSTX  Drawings

13.E.0101.0 - Centerstack Top @ 0° and 90° Layout.

13.E.0102.0 - Centerstack Bottom @ 0° and 90° Layout.

The Inconel centerstack casing is supported at its base by a support structure which reacts the loads through the TF hub assembly to the hot cell floor.  This support structure is comprised of a series of annular rings separated by eight 0.5 in. thick vertical plates spaced 45° apart  The support structure is bolted to the casing at the top and connected to the TF hub assembly at its base (13.E.0102.0).  At the top, the casing is "guided" by the hub assembly to prevent translational movement but is free to grow vertically (13.E.0101.0)

Reference 1 describes the thermal analysis of the vacuum vessel and PFCs during bakeout.  From this analysis the centerstack casing temperature is predicted reach a maximum temperature of 390°C.    To examine the effect of this thermal load on the centerstack and support structure, the centerstack model created to analyze the halo current loads was modified.  The new model of the centerstack and support structure, Figure 1.  In the analysis for the support structure only the vertical plates were explicitly modeled.  These plates were assumed fixed at their connection to the TF hub assembly.  At the top the centerstack restrained in the translational directions but free to move vertically.

For the bakeout scenario, the model was analyzed using ANSYS with temperatures on the entire centerstack set to 400°C while the base of the support was set to room temperature, 20°C.  Figure 2 illustrates the temperature distribution within the support for this condition.  Using this temperature distribution as body forces the stresses and displacments in the model were

calculated.  During baking the casing grows nearly 0.75 in. vertically and ~.080 in. radially at the supports.  Since the centerstack is free to grow vertically it is this radial growth that creates a bending stress in the support, Figure 3.  The resulting von Mises stresses in the supports are given in Figure 4 and has a maximum value of ~20 ksi.

A similar analysis was conducted for the temperature distribution during operations.  Reference 2 predicts a maximum temperature in the casing of ~ 560°C at the midplane.  During operations the inboard divertor will be cooled so temperatures at the lower and upper centerstack flanges are set at 50°C.  The base of the support structure is again set at 20°C in this analysis although some heating of this structure is likely to occur during operation.  Figure 5 illustrates this temperature distribution.  The Sz and Svm stress distribution is given in Figures 6 and 7.  Since the inboard divertor and thus the base of the casing is cooled during operation the radial growth, and thus the stresses at the support are minimal.

Distribution:

J. Bialek	J. Citrolo	P. Heitzenroeder	C. Neumeyer	NSTX File

Fig 1 ANSYS Model of Centerstack and Support

Fig 2 Temperature Distribution during Bakeout

Fig 3 Axial Bending Stresses from Bakeout Thermal Loads

Fig 4 von Mises Stresses for Bakeout

Fig 5 Temperature Distribution during Bakeout

Fig 6 Axial Bending Stresses from Operating Thermal Loads

Fig 7 von Mises Stresses for Operating Thermal Loads