13-970606-AWB-01

TO: S Kaye

FROM: A Brooks

DATE:  6 June 1997
SUBJECT: Passive Plates Toroidal Time Constants

Reference:  NSTX Memo 13-970217-AWB-02, "Loads on CS from Plasma Disruption"

The question arose as to whether the time constant of the passive plates differs if one assumes the gap resistance is uniformly distributed toroidally ( as I understand is assumed in the startup codes ) verses being lumped at the toroidal gap.

To address this, I've extracted time constants for the passive plate structure by running the existing spark model, previously used to assess disruption loads on the center stack ( ref 1 ). The thickness of the PP supports to the vv were modified to produce a total toroidal resistance of the PP of 40 μΩ. For the distributed resistance, an equivalent uniform resistivity of 22.21e-8 Ωm for the copper plates and SS supports was used to provide the same 40 μΩ toroidal resistance. An initial current distribution in the PP,VV and CS was obtained by allowing the plasma current distribution to instaneously drop to zero ( inductive limit ) and observing the transient decay.

Figures 1 and 2 show the net toroidal currents flowing in the PP, VV and CS for the lumped and the distributed PP resistance. Figure 3 compares the PP currents for the two cases. Figures 4 and 5 provide the curve fits from which the time constants are obtained.

Because of the mutual inductance coupling between components, a single time constant does not characterize the current decay found. For the lumped resistance case, three time constants were needed whereas for the distributed resistance case, two time constants produced comparable agreement. However the most significant values appear to be the longer time constants which are 69 millisecs for the lumped resistance case and 49 millisecs for the distributed.

What is not show here ( the plot files are too large to include ) is the difference in current patterns that develope in the PP as the net toroidal current decays. For the distributed resistance model, the current maintains fairly uniform flow across the plates during the decay. However for the lumped resistance model, large eddy loops develope in the plates within 10's of millisecs which do not cross the gap. The effect, if any, on startup is not clear.

cc: 	C Neumeyer

J Spitzer

P Heitzenroeder

M Ono

NSTX Files

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5