11-971117-CLN-01

TO: DISTRIBUTION

FROM: C NEUMEYER
SUBJECT:  PFC GEOMETRY

This memo serves to document the agreed to geometry of the PFCs. Line segments which describe the front and back surfaces of the PFCs are given in the attached figure and tables.

Additional details not covered by the line segment definition are are follows:

1 - the tip of the secondary passive plate tiles at the slot with the outboard divertor shall be rounded with radius of 1 cm

2 - the toroidally oriented edge of the primary passive plate tiles on the midplane end shall be quarter rounded on the side facing the plasma with a radius of 1 inch

G Barnes

J Chrzanowski

P Heitzenroeder

S Kaye

R Maingi

B Nelson

M Ono

J Robinson

NSTX File

|  |  | # | R (in) | Z (in) | # | R (in) | Z (in) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inner Wall | Struct | 1 | 6.738 | 0.000 | 2 | 6.738 | 42.617 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 3 | 7.289 | 0.000 | 4 | 7.289 | 41.780 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inner Wall/Inboard Divertor Angled Section | Struct | 5 | 6.738 | 42.617 | 6 | 10.000 | 44.500 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 7 | 7.289 | 41.780 | 8 | 11.000 | 43.923 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Vertical Section | Struct | 9 | 10.000 | 44.500 | 10 | 10.000 | 62.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 11 | 11.000 | 43.923 | 12 | 11.000 | 61.857 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Vertical/Horizontal Angled Section | Struct | 13 | 10.000 | 62.125 | 14 | 11.732 | 65.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 15 | 11.000 | 61.857 | 16 | 12.309 | 64.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Horizontal Section | Struct | 17 | 11.732 | 65.125 | 18 | 22.500 | 65.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 19 | 12.309 | 64.125 | 20 | 22.500 | 64.125 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Outboard Divertor | Struct | 21 | 23.618 | 65.438 | 22 | 46.004 | 57.918 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 23 | 23.300 | 64.490 | 24 | 45.686 | 56.970 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Slot | Struct | 25 | 42.498 | 55.823 | 26 | 45.017 | 54.977 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 27 | 40.227 | 57.641 | 28 | 45.335 | 55.925 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Secondary Passive Plate | Struct | 29 | 42.498 | 55.823 | 30 | 49.557 | 44.175 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 31 | 40.227 | 57.641 | 32 | 48.702 | 43.657 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Primary Passive Plate | Struct | 33 | 49.967 | 43.025 | 34 | 57.248 | 21.182 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 35 | 49.018 | 42.708 | 36 | 56.299 | 20.866 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  |  | # | R(cm) | Z(cm) | # | R(cm) | Z(cm) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inner Wall | Struct | 1 | 17.115 | 0.000 | 2 | 17.115 | 108.246 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 3 | 18.515 | 0.000 | 4 | 18.515 | 106.122 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inner Wall/Inboard Divertor Angled Section | Struct | 5 | 17.115 | 108.246 | 6 | 25.400 | 113.030 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 7 | 18.515 | 106.122 | 8 | 27.940 | 111.564 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Vertical Section | Struct | 9 | 25.400 | 113.030 | 10 | 25.400 | 157.798 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 11 | 27.940 | 111.564 | 12 | 27.940 | 157.117 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Vertical/Horizontal Angled Section | Struct | 13 | 25.400 | 157.798 | 14 | 29.799 | 165.418 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 15 | 27.940 | 157.117 | 16 | 31.266 | 162.878 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Inboard Divertor Horizontal Section | Struct | 17 | 29.799 | 165.418 | 18 | 57.150 | 165.418 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 19 | 31.266 | 162.878 | 20 | 57.150 | 162.878 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Outboard Divertor | Struct | 21 | 59.991 | 166.212 | 22 | 116.851 | 147.112 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 23 | 59.182 | 163.805 | 24 | 116.043 | 144.704 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Slot | Struct | 25 | 107.945 | 141.791 | 26 | 114.342 | 139.642 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 27 | 102.177 | 146.408 | 28 | 115.151 | 142.050 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Secondary Passive Plate | Struct | 29 | 107.945 | 141.791 | 30 | 125.875 | 112.205 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 31 | 102.177 | 146.408 | 32 | 123.703 | 110.889 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Primary Passive Plate | Struct | 33 | 126.916 | 109.282 | 34 | 145.409 | 53.803 |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | PFC | 35 | 124.506 | 108.479 | 36 | 142.999 | 53.000 |
| --- | --- | --- | --- | --- | --- | --- | --- |