12-980108-HMF-01

Princeton University (PPPL):	NSTX Engineering

TO:	Distribution	DATE:	Jan. 8, 1998

FROM:	H.M. Fan	SUBJECT:	Normal Operation Cooling of the NSTX

Center Section

The results of NSTX vacuum vessel cooling for normal operation, assuming the center section to be cooled down to 50° C, are shown in Tables 1 and 2.  The thermal analyses were based on the following assumptions:

•	Temperatures of outboard divertor plates are 150°C

•	Temperatures of passive plates are 140°C

•	Temperatures of port flanges on domes are 25°C

•	In-vessel bulk temperature for radiation is 520°C

•	Surface emissivity of 304 SS is 0.059 for surface not behind PFC

•	For surface behind PFC and surface between divertor plate and ceramic break, the effective emissivity is 0.156.  The in-vessel bulk temperature for radiation is 120°C.

•	Out-vessel ambient temperature is 25°C

•	Machine floor temperature is 20°C

•	Temperature of umbrella structure is 25°C

•	Film coefficients for port walls are 5 W/m^2/C

•	Film coefficient for vacuum vessel is 0.83 W/m^2/C

•	Film coefficients for ribs are 1.66 W/m^2/C

•	Film coefficients for leg and PF supports are 10 W/m^2/C

•	Film coefficients for port covers are 3 W/m^2/C

In the Tables, the amounts of heat flow were calculated for each port.  The heat flow on the vessel, however, was calculated based on the 1/4 of the vessel.  Therefore, the total amounts of the heat flow on the center section must be multiplied by a factor of 4.

Table 1 assumes the temperatures of  the center section are constrained at 50° C.  Table 2 has the same assumption except that no cooling was provided on the port covers.  The values on the 'subtotal' columns are the summation of the heat flow calculated from each component.  Since they count twice the heat flow values at the common nodes of the joint of two components, the magnitudes of heat are over-estimated.

Distribution: 	A Brooks

J Chrzanowski

L Dudek

P Heitzenroeder

C Neumeyer

R Parsells

M Viola