13-001206-CLN-02

TO: DISTRIBUTION

FROM: C NEUMEYER

SUBJECT: TF LIMITS vs. TEMPERATURE

References

[1] 13-001206-CLN-01, “TF Operation with Water Cooling Absent from Turn Six”

[2] NSTX-CALC-13-2, “TF Coil Parameters”

[3] NSTX-CALC-13-18, “TF As-Built Circuit Parameters”

In order to quantify operating limits as a function of allowable adiabatic temperature rise (as called out in the ref. [1]) a spreadsheet was developed as described herein.

First, the G function is used per ref. [2] to determine the ∆J2T which will heat the coil to the temperature limit, starting from a given inlet water temperature (nominally 12C). Then, a peak allowable field is chosen (in this case 4.5kG) which establishes the ∆J2T associated with current decay, in case a trip occurs at peak field. The final allowable ∆J2T minus the ∆J2T attributable to current decay sets the RIS I2T trip level. Then, given the trip level, and the I2T associated with current rise and current fall, the allowable flat top duration at different field levels can be set such that the current, with power supply ramp down, goes to zero without raising the I2T to the RIS trip level.

Results are given in the attached tables and figure with 4.5kG and ∆T=20C max. chosen as the governing conditions, along with the constraint that the total pulse length not exceed 1.5 seconds (the ACP overtime setting). It is noted that a flat top time of order 500mS can be achieved at 4.5kG, including current ramp down, without an I2T trip. Should an I2T trip occur at the end of flat top, at any field up to 4.5kG, the maximum allowable temperature rise will not be exceeded.

These limits are enforced in hardware by the RIS and ACP overcurrent settings, the RIS I2T setting, and in software by the PSRTC overcurrent, temperature, and I2T settings.

cc:

E. Baker		M. Bell		W. Blanchard		A. Brooks

J. Chrzanowski	L. Dudek 		H. Fan		R. Hatcher

R. Hawyrluk		P. Heitzenroeder	M. Kalish		R. Marsala

M. Ono		G. Pearson		S. Ramakrishnan	A. Von Halle

M. Williams		NSTX File

| Allowable Adiabatic ∆T | 20 | deg C |
| --- | --- | --- |
| T0 | 12 | deg C |
| G0 | 2.53E+15 | (A/m^2)^2-sec |
| Tmax | 32 | deg C |
| Gmax | 6.47E+15 | (A/m^2)^2-sec |
| L | 4.37E-03 | Henry |
| Rcircuit (cold) | 7.81E-03 | Ohm |
| Rcircuit (hot) | 8.19E-03 | Ohm |
| Rpsequiv | 4.03E-03 | Ohm |
| ∑R (cold) | 1.18E-02 | Ohm |
| ∑R (hot) | 1.22E-02 | Ohm |
| CSA inner leg | 6.74E-04 | m^2 |
| Bt | 4.5 | kG |
| Iflat | 5.34E+04 | Amp |
| J2T L/R decay | 1.75E+15 | (A/m^2)^2-sec |
| J2T Trip (RIS) | 4.72E+15 | (A/m^2)^2-sec |
| I2T Trip (RIS) | 1.91E+09 | A^2-sec |
| Trise | 0.360 | sec |
| J2T Rise | 9.45E+14 | (A/m^2)^2-sec |
| Tfall | 0.243 | sec |
| J2T Fall | 5.08E+14 | (A/m^2)^2-sec |
| J2T Flat | 3.26E+15 | (A/m^2)^2-sec |
| Tflat | 0.521 | sec |

| Bt | Iflat | Trise | J2T Rise | Tfall | J2T Fall | J2T Flat Possible | J2T Flat | ∑J2T | Tflat | ∑T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (kG) | (Amp) | (sec) | ((A/m^2) ^2-sec) | (sec) | ((A/m^2) ^2-sec) | ((A/m^2) ^2-sec) | ((A/m^2) ^2-sec) | ((A/m^2) ^2-sec) | (sec) | (sec) |
| 3.0 | 3.56E+04 | 0.198 | 2.09E+14 | 0.179 | 1.66E+14 | 4.34E+15 | 3.13E+15 | 3.50E+15 | 1.123 | 1.500 |
| 3.1 | 3.68E+04 | 0.207 | 2.35E+14 | 0.184 | 1.82E+14 | 4.30E+15 | 3.30E+15 | 3.72E+15 | 1.109 | 1.500 |
| 3.2 | 3.80E+04 | 0.216 | 2.63E+14 | 0.188 | 1.99E+14 | 4.25E+15 | 3.47E+15 | 3.93E+15 | 1.096 | 1.500 |
| 3.3 | 3.91E+04 | 0.225 | 2.93E+14 | 0.193 | 2.17E+14 | 4.21E+15 | 3.65E+15 | 4.16E+15 | 1.082 | 1.500 |
| 3.4 | 4.03E+04 | 0.235 | 3.26E+14 | 0.197 | 2.35E+14 | 4.15E+15 | 3.82E+15 | 4.38E+15 | 1.068 | 1.500 |
| 3.5 | 4.15E+04 | 0.245 | 3.62E+14 | 0.202 | 2.55E+14 | 4.10E+15 | 3.99E+15 | 4.61E+15 | 1.053 | 1.500 |
| 3.6 | 4.27E+04 | 0.255 | 4.02E+14 | 0.206 | 2.76E+14 | 4.04E+15 | 4.04E+15 | 4.72E+15 | 1.007 | 1.468 |
| 3.7 | 4.39E+04 | 0.265 | 4.44E+14 | 0.210 | 2.97E+14 | 3.97E+15 | 3.97E+15 | 4.72E+15 | 0.938 | 1.414 |
| 3.8 | 4.51E+04 | 0.276 | 4.91E+14 | 0.215 | 3.20E+14 | 3.90E+15 | 3.90E+15 | 4.72E+15 | 0.874 | 1.364 |
| 3.9 | 4.63E+04 | 0.287 | 5.41E+14 | 0.219 | 3.44E+14 | 3.83E+15 | 3.83E+15 | 4.72E+15 | 0.814 | 1.320 |
| 4.0 | 4.74E+04 | 0.298 | 5.95E+14 | 0.223 | 3.68E+14 | 3.75E+15 | 3.75E+15 | 4.72E+15 | 0.758 | 1.279 |
| 4.1 | 4.86E+04 | 0.310 | 6.55E+14 | 0.227 | 3.94E+14 | 3.67E+15 | 3.67E+15 | 4.72E+15 | 0.705 | 1.242 |
| 4.2 | 4.98E+04 | 0.322 | 7.19E+14 | 0.231 | 4.21E+14 | 3.58E+15 | 3.58E+15 | 4.72E+15 | 0.655 | 1.208 |
| 4.3 | 5.10E+04 | 0.334 | 7.88E+14 | 0.235 | 4.49E+14 | 3.48E+15 | 3.48E+15 | 4.72E+15 | 0.608 | 1.177 |
| 4.4 | 5.22E+04 | 0.347 | 8.64E+14 | 0.239 | 4.78E+14 | 3.37E+15 | 3.37E+15 | 4.72E+15 | 0.563 | 1.150 |
| 4.5 | 5.34E+04 | 0.360 | 9.45E+14 | 0.243 | 5.08E+14 | 3.26E+15 | 3.26E+15 | 4.72E+15 | 0.521 | 1.124 |