22-970115-HK-01

TO: S. KAYE

FROM: H. W. KUGEL

SUBJECT: DOUBLE CERAMIC BREAKS, CHIT 2.2.2.6a

cc: M Ono, M Peng

NSTX Engineering Design Review Issues.

2.0 Engineering Issues

2.2 Vacuum Vessel

2.2.2. Ceramic Break

1. Perform thermal expansion analysis of bellows/ceramic interface.

2. Perform complete thermal and mechanical analysis.

3. Provide for rapid replacement.

4. Perform worst case motion analysis.

5. Assess lower risk alternatives.

6. Assess performance during strong VDE. Why are two breaks required?

6a. Consider alternatives to a ceramic break for CHI. Investigate the 	possibility of floating the divertor strike plates, and passive plates if 	necessary, to avoid the need for a ceramic break.

The present NSTX vacuum vessel design employs double ceramic breaks for Coaxial Helicity Injection (CHI). These ceramic breaks have raised issues related to performance reliability (CHITS 2.2.2. 1-6).  This section considers alternatives to the vessel ceramic breaks in an effort to eliminate all of these issues, but concludes that the present design is preferable for the initial NSTX experiments.

Origins of the Present Design.

The Helicity Injected Tokamak (HIT) has demonstrated the formation and sustainment of a 150 kA toroidal current using coaxial helicity injection (CHI) with a double insulator ceramic break design.[1]  The present NSTX ceramic break design (Fig.1) is an embodiment of the HIT configuration for CHI. In HIT, the double insulators (top and bottom) are necessary to maintain the flux distribution against relaxation. One insulator, the emitting insulator, has E x B  away from it; the other insulator, the absorbing insulator, has E x B  towards it. This

configuration facilitates the flow of power, toroidal flux, and plasma along the E x B  direction to maintain constant toroidal flux for steady state conditions.[2]  The embodiment of HIT concepts into the NSTX CHI design allows testing the remarkable HIT results in a higher power, more reactor-like regime.

Alternate Designs.

Fig.s 2 and 3 show alternative CHI electrode configurations. The single insulator design (Fig. 2) allows relaxation processes to change the initial CHI flux configuration and is not suitable for steady state operation.[2]  Fig. 3 shows the floating biased divertor configuration used on PBX-M. This configuration does not include the vacuum vessel in the CHI electrical circuit, and hence, does not require vessel ceramic breaks. However, the adoption of this configuration for initial NSTX CHI experiments encounters several uncertainties that only can be resolved experimentally. In PBX-M, for example, the plasma edge was far from the vessel walls. In NSTX, however, under some conditions, the inner plasma edge could wet the inner wall, and bring the vacuum vessel into the circuit. This could create a changing electrode geometry, thereby reducing the effectiveness of the double insulator concepts in maintaining the stability of the CHI configuration.[2] In addition, possible parasitic currents, halo currents, and arcing during disruptions could result in closed current paths through the vessel leading to possibly large magnetic forces on divertor and insulator support structures. In PBX-M, special insulator designs were employed and the backsides of the biased divertor plates were clad with mica sheets to reduce possible arc paths. Similar design techniques might be applied in an NSTX embodiment of Fig. 3, at a later stage, after experience is obtained with the present double ceramic break design.  The remarkable initial current drive efficiencies achieved by HIT, the prospect of future experimentation improving on these efficiencies, and the reactor relevant advantages of eliminating the vessel ceramic breaks, suggest the merits of eventually pursuing some embodiment of floating divertor electrodes for NSTX CHI.

Proposed Approach.

1. Perform all suggested thermal, electrical, and mechanical analysis (CHITS 2.2.2.1-6) to develop a robust design and appropriate operating procedures to maximize the reliability of the double ceramic break system.

2. Develop a design that provides for rapid replacement of ceramic breaks (CHITS 2.2.2.3).

3. Proceed to perform initial NSTX CHI experiments using the double ceramic break design to overlap with the HIT regime. Then, using appropriate operating procedures, extend the resulting NSTX/HIT  database into higher CHI powers and  characterize the new regimes.

4. In parallel with the above, investigate the possibility of performing floating divertor CHI experiments on HIT.[3]  This would allow investigation of the issues discussed above using the same systems used for the initial HIT CHI double ceramic break experiments. The resulting comparisons of CHI efficiency and behavior, and possible innovations, could provide valuable guidance for subsequent NSTX experiments.

References.

[1] B. A. Nelson et al., "Formation and Sustainment of a 150 kA Tokamak by Coaxial Helicity Injection", Phys. Rev. Lett., 72 (23), 3666 (1994).

[2] T. R. Jarboe, " Formation and Steady-State Sustainment of a Tokamak by Coaxial Helicity Injection", Fus. Technol., 15, 7 (1989).

[3] Private communication, T. R. Jarboe and H. W. Kugel, Denver, APS DPP Meeting, Nov. 14, 1996.