4X-971017-RK-01

TO: DISTRIBUTION

FROM: R KAITA
SUBJECT:  BASELINE DIAGNOSTICS  INSTALLATION  PLAN

With the start of the new fiscal year, we are expected to begin serious consideration of baseline diagnostics for NSTX. The list of baseline diagnostics is given below. It is based on discussions at February Research Forum, and it has been endorsed by the NSTX Program Advisory Committee.

Diagnostics like magnetics that are to be installed as NSTX is assembled will have to be built from scratch, and a preliminary design review was held on September 9, 1997. The charge-exchange recombination spectroscopy diagnostic, with its background sightlines serving double duty as an array for visible bremsstrahlung measurements, will also require significant new components, and a preliminary design review is planned for early 1998.

Due to severe budgetary constraints, the remaining diagnostics will rely heavily on existing equipment. This may admittedly result in systems that are less than optimal, and we hope to improve them in future upgrades. For now, however, we want to do our best in providing basic plasma measurements as early as we can, and at the lowest possible cost.

With this in mind, the goal will be to prepare a list of tasks required to install each diagnostic in the last category by the end of January, 1998. We will then have what will be officially called the Final Design Review (FDR) of General Baseline Diagnostics, which is the label given to this set. However, it will not be at the level of detail required for the construction of new diagnostics. As project engineer, C. Neumeyer will explain what will be required. I have also appended some suggestions for issues to be addressed under each system.

Note that the earliest any of the General Baseline Diagnostics can be moved to the NSTX Test Cell will be next October. In the absence of any serious conceptual difficulties, therefore, FY98 tasks are expected to involve primarily “off-line” activities such as collecting, testing, and storing equipment.

Please feel free to contact me if you have any questions. Thank you.

Distribution:

L Dudek		K Hill		D Johnson	H Kugel	E Mazzucato

S Medley		C Neumeyer	M Ono	S Paul	E Perry

A Ramsey		L Roquemore	D Stutman	C Such	NSTX File

Baseline Diagnostics Summary

| System | Function |
| --- | --- |

| Plasma current Rogowski coils1 | Total plasma current |
| --- | --- |

| Eddy current Rogowski coils1 | Halo current monitoring |
| --- | --- |

| Flux loops1 | Poloidal flux for plasma control |
| --- | --- |

| Bθ, Bρ coils1 | Plasma control/magnetic fluctuations |
| --- | --- |

| Mirnov coils1 | Magnetic fluctuations |
| --- | --- |

| Visible TV camera | External shape for plasma control |
| --- | --- |

| IR camera | Heat loads |
| --- | --- |

| “Slow” diamagnetic loop (using toroidal field coil) | Stored energy |
| --- | --- |

| Multichannel bolometer | Radiated power profile |
| --- | --- |

| 170 GHz microwave interferometer | Line-integrated plasma density |
| --- | --- |

| Survey spectrometer (SPRED) | Plasma impurities |
| --- | --- |

| Soft X-ray imaging system | Plasma instabilities and fluctuations |
| --- | --- |

| Hα detectors | Edge recycling |
| --- | --- |

| CHERS | Ion temperature & toroidal rotation |
| --- | --- |

| High-throughput CHERS background array2 | Background subtraction for CHERS and Zeff(r) from visible continuum |
| --- | --- |

| X-ray pulse height analysis2 | Core electron temperature |
| --- | --- |

| Neutral particle analyzer2 | Core ion temperature and fast ions |
| --- | --- |

| Visible spectrometer2 | Edge/divertor spectroscopy |
| --- | --- |

| Ultra-soft X-ray array3 | Start-up and impurity studies |
| --- | --- |

| Langmuir probes/thermocouples1 | Divertor parameters |
| --- | --- |

1Diagnostics to be installed as part of NSTX assembly procedure

2Diagnostics added since Research Forum (February, 1997)

3Diagnostic recommended by Program Advisory Committee

Diagnostic: Visible TV and IR TV

Cognizant Physicist: H. Kugel

Cognizant Engineer: L. Roquemore

Lead Technician: L. Guttadora

1) Overview of Diagnostic

a) Description (including physical characteristics):

tangential views of main plasma chamber and divertor regions.

b) Parameters measured (T, q, n, etc.):

Visible camera: plasma shape, edge recycling.

IR camera: temperature waveform, incident power densities, plasma-wall interactions.

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

Visible camera: imaging of edge region.

IR camera: top and bottom divertor regions, all passive stabilizer surfaces.

d) Resolution (spatial, time, etc.):

20 msec temporal resolution, 5 cm spatial resolution (IR camera: 14o x 18o field-of-view)

e) Detector voltage control conducted remotely through CAMAC interface.

f) Measurements limits (such as density range):

IR camera: -20oC to 1500oC temperature range, 8 to 12 micron spectral range, requires ZnSe (or equivalent) vacuum windows.

2) Installation Information

a) Status of hardware (including replacement materials).

• Need inventory of available cameras and requirements

for refurbishing (in case of IR cameras).

b) Machine interface requirements and utilities.

• Port requirements

- Specify desired window size and material (i. e., for IR).

- Use planned tangential flange(s) or install tangential

flange(s) on large 24" ports?

• Platform space

- Minimal?

• Electricity, water, and gases (air, He, N2, etc.)

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• PBX-M system puts analog signal straight into triggered VCR.

- Is there an inexpensive CAMAC-based digitizer alternative?

Diagnostic: Tangential multichannel bolometer and divertor bolometers

Cognizant Physicist: S. Paul

Cognizant Engineer: L. Roquemore

Lead Technician: L. Guttadora

1) Overview of Diagnostic

a) Description (including physical characteristics): tangential array in horizontal midplane, two divertor views.

b) Parameters measured (T, q, n, etc.):

Radiated power.

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

edge and core, 15 channels in tangential array.

d) Resolution (spatial, time, etc.):

4 cm spatial resolution, 5 msec time resolution.

2) Installation Information

a) Status of hardware (including replacement materials).

• Are there detectors and amplifiers in storage that are available?

• What is the condition of the TFTR tangential bolometer array?

• Is it too difficult and expensive to build another box?

• Should we borrow the PBX-M tangential array instead?

b) Machine interface requirements and utilities.

• Port requirements

- Use planned tangential flange(s) or install tangential

flange(s) on large 24" ports?

- Find gate valve: control manually or remotely with TFTR DVC?

- Is any vacuum equipment required beyond pumpout port?

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Anything needed beyond existing CAMAC digitizer(s)?

Diagnostic: 170 GHz microwave interferometer

Cognizant Physicist: E. Mazzucato

Cognizant Technician: L. Guttadora

Lead Technician: R. Cutler

1) Overview of Diagnostic

a) Description (including physical characteristics):

horizontal  chord through center of plasma.

b) Parameters measured (T, q, n, etc.):

Line-integrated plasma density.

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

single channel.

d) Resolution (spatial, time, etc.):

1 µsec time resolution.

e) Measurements limits (such as density range):

optimized for central density of 5 x 10-13 cm-3

2) Installation Information

a) Status of hardware (including replacement materials).

• Has it been disassembled and set aside for NSTX?

- It is always preferable not to take apart existing systems,

but given our very severe limits in money and personnel,

we may not be able to build new diagnostics from scratch.

b) Machine interface requirements and utilities.

• Port requirements.

- Can get drawings from John Robinson.

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Anything needed beyond existing CAMAC digitizer(s)?

Diagnostic: Survey Spectrometer (SPRED)

Cognizant Physicist: A. Ramsey

Cognizant Engineer: L. Roquemore

Lead Technician: L. Guttadora

1) Overview of Diagnostic

a) Description (including physical characteristics): vacuum ultraviolet spectrometer with intensified self-scanning photodiode array detector.

b) Parameters measured (T, q, n, etc.):

impurity distributions.

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

core or edge region (depending on presence or absence of view across neutral beam), 1024 pixels.

d) Resolution (spatial, time, etc.):

5 cm spatial resolution, 1 msec time resolution, ≈2Å spectral resolution.

e) Measurements limits (such as density range):

100Å-1200Å.

2) Installation Information

a) Status of hardware (including replacement materials).

• Has it been removed from TFTR and set aside for NSTX?

• Does anything have to be repaired or replaced?

b) Machine interface requirements and utilities.

• Port requirements.

- Find gate valve: control manually or remotely with TFTR DVC?

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A for electronics and 208VAC/20A for pump?

- Water and air needed for pump, valves and detector?

c) Computer control and data acquisition requirements.

• Do CAMAC modules for voltage control and digitizer(s) exist?

Diagnostic: Soft X-ray imaging system

Cognizant Physicist: K. Hill

Cognizant Engineer: L. Roquemore

Lead Technician: J. Gorman

1) Overview of Diagnostic

a) Description (including physical characteristics):

detect soft X-ray emission with vertically-viewing and horizontally-viewing diode arrays.

b) Parameters measured (T, q, n, etc.):

MHD fluctuations

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

edge and core, 32 channels along each view.

d) Special features:

fluctuation diagnostic, can be used in conjunction with plasma shape diagnostics for tomographic reconstructions

e) Resolution (spatial, time, etc.):

2-3 cm spatial resolution, 0.001 msec temporal resolution.

f) Measurements limits (such as density range):

maximum resolvable mode number is m = 2, modes up to m = 6 distinguishable but not resolvable with 32 channels.

2) Installation Information

a) Status of hardware (including replacement materials).

• Are there detectors and amplifiers available?

• Is the TFTR box usable?

b) Machine interface requirements and utilities.

• Port requirements

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Do CAMAC modules for voltage control and digitizer(s) exist?

Diagnostic: Hα detectors

Cognizant Physicist: A. Ramsey

Cognizant Engineer: L. Roquemore

Lead Technician: L. Guttadora

1) Overview of Diagnostic

a) Description (including physical characteristics):

Diode detectors with interference filters at various toroidal locations for edge Hα and Dα radiation measurements as a function of space and time.

b) Parameters measured (T, q, n, etc.):

edge Hα and Dα densities

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

edge

d) Resolution (spatial, time, etc.):

1 msec time resolution.

e) Measurements limits (such as density range):

edge region of plasma.

2) Installation Information

a) Status of hardware (including replacement materials).

• Have detectors been located?

b) Machine interface requirements and utilities.

• Port requirements.

- Hand valve and pumpout port only?

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Anything needed beyond existing CAMAC digitizer(s)?

Diagnostic: X-ray pulse height analysis

Cognizant Physicist: K. Hill

Cognizant Engineer: L. Roquemore

Lead Technician: J. Gorman

1) Overview of Diagnostic

a) Description (including physical characteristics):

Perpendicular view in horizontal midplane

b) Parameters measured (T, q, n, etc.):

electron temperature from pulse height spectrum

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

core electron temperature

d) Resolution (spatial, time, etc.):

5 ms temporal resolution(?)

e) Measurements limits (such as density range):

useful primarily in Ohmic and rf-heated plasmas (requires extensive shielding during high-power neutral beam injection in D-D plasmas).

2) Installation Information

a) Status of hardware (including replacement materials).

• Have the detector, amplifier, and power supplies been located?

b) Machine interface requirements and utilities.

• Port requirements.

- Find gate valve: control manually or remotely with TFTR DVC?	• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Anything needed beyond existing CAMAC digitizer(s)?

Diagnostic: Ultra-soft X-ray array

Cognizant Physicist: D. Stutman (Johns Hopkins University)

Cognizant Engineer: L. Roquemore

Lead Technician: J. Gorman

1) Overview of Diagnostic

a) Description (including physical characteristics):

detect ultra-soft X-ray emission with horizontally-viewing midplane and divertor diode arrays.

b) Parameters measured (T, q, n, etc.):

MHD fluctuations during start-up (“Phase I”)

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

32 channels in poloidal array, 16 channels in divertor array(?).

d) Special features:

filtered diodes with provision for multilayer mirrors

e) Resolution (spatial, time, etc.):

0.5 - 3 Å spectral resolution, 0.5-2.5 cm spatial resolution, 0.01 ms temporal resolution.

2) Installation Information

a) Status of hardware (including replacement materials).

• What can be moved from CDX-U and what needs to be added (e. g.,

new housings and mounts).

b) Machine interface requirements and utilities.

• Port requirements.

- Show “Phase I” configurations on midplane and dome(?) flanges.

• Platform space.

- Just standard electronics cabinet?

• Electricity, water, and gases (air, He, N2, etc.).

- Only 120VAC/20A?

c) Computer control and data acquisition requirements.

• Anything needed beyond CAMAC digitizer(s)?

• What does NSTX need to supply?

Diagnostic: Neutral particle analyzer

Cognizant Physicist: S. Medley

Cognizant Engineer: L. Roquemore

Lead Technician: T. Holoman

1) Overview of Diagnostic

a) Description (including physical characteristics):

mass-resolving radial view in horizontal midplane

b) Parameters measured (T, q, n, etc.):

ion temperature, fast ion distribution

c) Plasma coverage (edge, edge to middle, # of channels, etc.):

edge and middle (depending on plasma conditions)

d) Resolution (spatial, time, etc.):

5 ms time resolution

e) Measurements limits (such as density range):

0.5 < A(amu)E(keV) < 600 keV

2) Installation Information

a) Status of hardware (including replacement materials).

• Use analyzer components presently in PLT area and TFTR basement.

• Look into parts for scanning capability (baseplate is under neutron

detectors on PBX-M platform and easily removable).

b) Machine interface requirements and utilities.

• Port requirements.

- Find valve and operate from existing DVC.

• Platform space.

- Standard electronics cabinet.

- Area to permit scanning of analyzer

• Electricity, water, and gases (air, He, N2, etc.).

- 120VAC/60A to electronics cabinet.

- 120VAC/15A to cabinet on cart.

- 208VAC/10A for mechanical pump.

- 480VAC/80A for magnet power supply.

- Water for pumps and magnet and air for valves.

c) Computer control and data acquisition requirements.

• Reuse existing CAMAC-based power supplies, digitizers, etc.