Princeton Plasma Physics Laboratory

To:  C. Neumeyer

From:  P. Heitzenroeder

Subject: NSTX Multi Point Thompson Scattering (MPTS) Vacuum Interface and Window Final Design Reviews

Date:  4/26/1999

Review Committee:  P. Heitzenroeder (chairman); L. Dudek; D. Johnson; M. Viola (part 2 only)

Attendees:

Part 1-D. Johnson; J. Levine; F. Dahlgren; D. Simon; B. LeBlanc; G. Steill; N. Bretz; R. Kaita; P. Heitzenroeder

Part 2- Attendees: R. Parsells, D. Johnson; M. Viola; M. Kalish; F. Dahlgren; D. Simon; B. LeBlanc; G. Steill; N. Bretz; R. Kaita; P. Heitzenroeder

Summary:  This FDR was held in two parts.  The first part (held on March 12) was initially meant to include both the vacuum interface and window.  However, the  window portion was withdrawn by general agreement of the review committee and re-reviewed on April 13 when it became obvious that the window portion required more work due to new information on window safety factors which were not originally taken into consideration.   With the completion of this additional work and review of the window analyses and design,  the Committee now considers both the Vacuum Interface and Window Final Design Reviews satisfactory.  Please note that this assessment is based on the shutter being closed during bakeout to limit the peak temperature of the window.

The discussions on window engineering requirements disclosed an inconsistency in the application of safety factors on vacuum windows on NSTX and at PPPL in general.  It is important that this be addressed at both the Project and laboratory level.  HSD 5008, Sect. 9 specifies a safety factor of 10 be used for vacuum windows. As a second point of reference, Appendix I  (which is sect. 2.1, "Vacum Equipment" of the ORNL Safety Manual) of an an ORNL technical report ("Design of Viewing Windows for Controlled-Atmosphere Chambers" by James N. Robinson) also suggests that  a factor of safety of 10 for windows involving personnel safety.

Many of the review committee were involved with TFTR; they mentioned that  TFTR typically used a minimium factor of safety of 2.  This is somewhat explained by the fact that the  HSD 5008 requirement is dated 5/94…much after TFTR was built.  This previous TFTR practice has apparently clouded the requirements for windows;  there is some carry over of the past practice of using a factor of safety of 2, and only sporadic (if at all) application of a factor of safety of 10.  The other windows on NSTX probably do not meet the factor of safety of 10 criteria currently specified in HSD 5008.  It must be remembered that the heat fluxes on NSTX will be much higher than on TFTR, and therefore its successful window operating experience in itself it is not sufficient to as justification of continued use of this low factor of safety.  The necessity of going as high as 10 might be questioned , but if a change is to be made it must be done formally.  It is recommended that a project/laboratory task force is assigned to this task.  This should be done as soon as possible, but certainly before high heat fluxes are experienced.

Chit Summary

From Part 1 of the FDR:

| Chit No. | Written by | Paraphrased Summary | Review Board Rec. |
| --- | --- | --- | --- |

| 1 | D. Johnson | Threaded holes should be added to both air and vacuum sides of window plates to accommodate a cover plate on vacuum side; protective metal sleeve and Lexan cover on air side. | Concur. The Project must develop administrative procedures to assure they are used. |
| --- | --- | --- | --- |

| 2 | D. Johnson | Consider use of "cold mirror " coatings on the outside an added in-vacuum quartz cover plate to reduce radiation load on vac. Window. | Concur. Should be analyzed and considered. |
| --- | --- | --- | --- |

| 3 | D. Johnson | Other 'inspection windows' should be added for future general diagnostics use. | Concur. R. Parsells to evaluate adding additional ports. |
| --- | --- | --- | --- |

| 4 | N. Bretz | Do an analysis with an interior quartz cover plate which will absorb some of the thermal load on the window& reduce stresses. | Concur. |
| --- | --- | --- | --- |

| 5 | N. Bretz | What testing can be done on the window to assure that it will not break under combined vacuum/thermal loading? | The Project should evaluate safety issues and set policy/criteria for vac. Windows. |
| --- | --- | --- | --- |

| 6 | R. Kaita | Thermal motions of the window relative to external diagnostics during bakeout might be a concern for alignment. | Out of scope for this review, but vessel motion in general should be noted by the Project. |
| --- | --- | --- | --- |

| 7 | B. LeBlanc | Look into active cooling (convective or water) of the window. | Disagree with direct water cooling (too much risk of thermal stress). convection cooling was mentioned as an option in Dahlgren's presentation; should be considered. |
| --- | --- | --- | --- |

| 8 | B. LeBlanc | Investigate thick cover plate for relief of thermal problem. Investigate reflective coatings. | Concur. |
| --- | --- | --- | --- |

From Part 2 of the FDR:

| Chit No. | Written by | Paraphrased Summary | Review Board Rec. |
| --- | --- | --- | --- |

| 9 | D. Johnson | Recommend installation of thermocuples near air side of seal o-ring to monitor its condition during bakeout. | Concur. |
| --- | --- | --- | --- |

| 10 | W. Blanchard | Configure flanges such that protective plexiglass windows can be put over the window when the detector box is not in place. | Concur. |
| --- | --- | --- | --- |

| 11 | W. Blanchard | Ensure that there is a gap between the inner window and the TVTS assembly such that there cannot be a trapped volume. | Concur. |
| --- | --- | --- | --- |

| 12 | W. Blanchard | Ensure that there is a pressure relief between the window an the outside flange/clamp such that it does not make a trapped volume na dthe volume can easily be leak checked. | Concur. |
| --- | --- | --- | --- |

| 13 | M. Viola | Calculate expansion of windows. 1. Ensure clearance fit ti pins; 2. Does the sliding o-ring cause any problems? | (1) Concur. (2) Parsells stated that the sliding o-ring is not a problem. |
| --- | --- | --- | --- |

| 14 | P. Heitzenroeder | A limiter must be added to assure that the flange cannot contact the window (ie, window is suspended on o-rings). | Parsells stated this is being done. |
| --- | --- | --- | --- |

| 15 | L. Dudek | Consider using temperature stickers permanently installed on quarts to monitor temperature. | Concur. |
| --- | --- | --- | --- |

Attachments:

1.	Original chits.

2.	Presentation materials

Cc: (without attachments)

Review committee

M. Williams

M. Ono