53-960809-CLN-01

TO: DISTRIBUTION

FROM: C NEUMEYER
SUBJECT:  ALTERNATE OH COIL DESIGN

REFERENCES:

1) 53-960808-CLN-01, "Preliminary Design of Monolithic OH Coil"

This memo compares two possible OH coil designs. The first was reported in ref. 1, and utilized a 1.5 cm conductor height, 3.56 m total coil height. The second utilizes a 1.6 cm conductor height, and a 4.05 m total coil height. Both coils meet the criteria that they can be driven to a prechage peak current of 24kA with grid voltage 7.5% below nominal , and both coils can deliver 0.6 volt-sec. The second coil end of flat top current is greater than that of the first coil, and there is consequently less margin at the end of flat top for other (PF and Heating) loads. However, if necessary, the flat top duration could be reduced to accomodate the other loads.

A comparison of the coil parameters is as follows:

| Parameter | Coil 1 | Coil 2 | Units |
| --- | --- | --- | --- |

| Conductor Width | 1.0 | 1.0 | cm |
| --- | --- | --- | --- |

| Conductor Height | 1.5 | 1.6 | cm |
| --- | --- | --- | --- |

| Conductor Hole Diameter* | 3/16 | 3/16 | in |
| --- | --- | --- | --- |

| Conductor Corner Radius | 0.1 | 0.1 | cm |
| --- | --- | --- | --- |

| Conductor Hole Elongation | 0.0 | 0.0 | cm |
| --- | --- | --- | --- |

| Turn-Turn & Layer-Layer Spacing | 0.04 | 0.04 | inch |
| --- | --- | --- | --- |

| #Layers | 4 | 4 |  |
| --- | --- | --- | --- |

| #Turn Spaces/Layer | 224 | 240 |  |
| --- | --- | --- | --- |

| #Turns/Layer | 222 | 238 |  |
| --- | --- | --- | --- |

| #Turns | 888 | 952 |  |
| --- | --- | --- | --- |

| R(center) | 12.654 | 12.654 | cm |
| --- | --- | --- | --- |

| Height (over turn insulation) | 355.6 | 405.0 | cm |
| --- | --- | --- | --- |

| Height above midplane (over turn insulation) | 177.77 | 202.5 | cm |
| --- | --- | --- | --- |

| Cooling path length (outer layer)* | 199.5 | 213.9 | m |
| --- | --- | --- | --- |

| Inductance | 12.4 | 12.6 | mH |
| --- | --- | --- | --- |

| Conductor Initial Temperature | 20 | 20 | deg C |
| --- | --- | --- | --- |

| 20C Coil Resistance | 92.9 | 92.6 | mΩ |
| --- | --- | --- | --- |

A comparison of the simulation results is as follows:

| Parameter | Coil 1 | Coil 2 | Units |
| --- | --- | --- | --- |

| Precharge Current | 24.0 | 24.0 | kA |
| --- | --- | --- | --- |

| Central B | 7.5 | 7.1 | Tesla |
| --- | --- | --- | --- |

| End of Flat Top Current | -18.65 | -21.3 | kA |
| --- | --- | --- | --- |

| Flux Swing | 0.6 | 0.6 | Volt-Sec |
| --- | --- | --- | --- |

| Initiation Loop Voltage | 5.9 | 5.5 | Volts/Turn |
| --- | --- | --- | --- |

| Initiation Interval | 20.0 | 20.0 | mS |
| --- | --- | --- | --- |

| Plasma Ramp Time | 400.0 | 400.0 | mS |
| --- | --- | --- | --- |

| Plasma Flat Top Time | 500.0 | 500.0 | mS |
| --- | --- | --- | --- |

| ESW @ Iprecharge | 0.30 | 0.34 | sec |
| --- | --- | --- | --- |

| Tmax | 73.0 | 73.2 | deg C |
| --- | --- | --- | --- |

| Pmax | 73.1 | 73.1 | MW |
| --- | --- | --- | --- |

| Qmax | 68.3 | 68.3 | MVAR |
| --- | --- | --- | --- |

| Smax | 94.3 | 94.3 | MVA |
| --- | --- | --- | --- |

| ∆Vmax (PPPL) | 3.0 | 3.0 | % |
| --- | --- | --- | --- |

| ∆Vmax (Brunswick) | 0.98 | 0.98 | % |
| --- | --- | --- | --- |

| ∆Vmax (Trenton) | 0.65 | 0.65 | % |
| --- | --- | --- | --- |

| Peoft | 50.4 | 65.7 | MW |
| --- | --- | --- | --- |

| Qeoft | 53.4 | 52.4 | MVAR |
| --- | --- | --- | --- |

| Seoft | 73.4 | 84.0 | MVA |
| --- | --- | --- | --- |

A comparison of the cooling results is as follows:

| Parameter | Coil 1 | Coil 2 | Units |
| --- | --- | --- | --- |

| ∆P w/20C inlet for τ/3 = 200 sec | 650 | 890 | psi |
| --- | --- | --- | --- |

| Flow w/20C inlet for τ/3 = 200 sec | 0.76 | 0.87 | GPM |
| --- | --- | --- | --- |

| ∆P w/10C inlet for ∆tcool = 600 sec | 284 | 385 | psi |
| --- | --- | --- | --- |

| Flow w/10C inlet for ∆tcool = 600 sec | 0.47 | 0.54 | GPM |
| --- | --- | --- | --- |

Since concern was expressed over the shortening of the solenoid in the first design, compared to the earlier OH coil, perhaps the second design is superior.

cc:

A Brooks	J Citrolo	T Egebo	R Goldston	P Heitzenroeder

M Kalish	S Kaye	M Ono	R Parsells	M Peng

S Raftopoulos	S Ramakrishnan	W Reiersen	J Robinson	J SchmidtJ Sinnis	J Spitzer 	NSTX File