72-960531-HK-01

TO: S. KAYE

FROM: H. W. KUGEL

SUBJECT: UPDATE ON NSTX NEUTRON  BUDGETS

2.5 Neutron Source Rates, Yields, and Dose-equivalents

The NSTX facility shall be designed to accommodate heat loads attendant with the peak neutron source rate initially envisioned for D operation at the pulse repetition rates specified in Section 2.2.

In addition, the NSTX tokamak systems and facility shall be designed for and the facility be capable of being upgraded for heat loads attendant with the peak neutron source rates ultimately envisioned for 5 sec and 60 sec, long-pulse D operations.

2.5.1 NSTX Radiation Shielding

The NSTX Radiation Shield Wall is shared with PBX-M, and both have similar Test Cell configurations. Typical PBX-M Control Room neutron dose-equivalents are < 2 x 10-14 mrem per PBX-M DD-neutron, and gamma dose-equivalents are < 1 x 10-15 mrem per PBX-M DD-neutron. Therefore, assuming similar radiation conditions for the NSTX Control Room (i.e., < 2.1 x10-14 mrem per NSTX DD-neutron), and assuming an NSTX Control Room radiation limit of 500 mrem per calendar-year as measured by stationary wall detectors (actual worker dose-equivalents will be less), then the total yield limit will be about 2 x 10+16 NSTX DD-neutrons per calendar-year with the present shielding. In addition, for an upgrade, it appears feasible to increase the shielding by a factor of at least 12, and thus permit a total yield limit of about 3 x 10+17 NSTX DD-neutrons per calendar-year.

2.5.2 NSTX Neutron Rates

Using the TRANSP code to simulate various NSTX plasma conditions gives estimated neutron production rates of about  4 x 10+12 D-D neutrons/sec with RF heating, and 1.5 x 10+14 D-D neutrons/sec with Neutral Beam and RF heating.

2.5.3 NSTX Discharges Per Calendar Year

Table 2.5.1 shows the estimated NSTX discharges per calendar year for various plasma discharge lengths with the present shielding and with a x12 upgrade in shielding.

Table 2.5.1 NSTX discharges per calendar year, at the estimated maximum neutron rate with present radiation shielding, and with a feasible x12 shielding upgrade. Refer to text.

Heating Operation

With Present Shielding

| Estimated D-D neutron rate | Total D-D neutrons per yr | Total D-D discharge sec per yr | Total D-D 5 sec discharges per yr | Total D-D 60 sec demo discharges per yr |
| --- | --- | --- | --- | --- |

RF-only

| 4 x10+12 | 2 x10+16 | 5000 | 1000 | 83 |
| --- | --- | --- | --- | --- |

RF + NBI

| 1.5x10+14 | 2 x10+16 | 133 | 27 | 2 |
| --- | --- | --- | --- | --- |

Heating Operation

With  x12

Shielding Upgrade

RF-only

| 4 x10+12 | 3 x10+17 | 60,000 | 12,000 | 1000 |
| --- | --- | --- | --- | --- |

RF + NBI

| 1.5x10+14 | 3 x10+17 | 1600 | 320 | 27 |
| --- | --- | --- | --- | --- |

cc:

J. Gilbert

J. Levine

C. Neumeyer

M. Ono

M. Peng

NSTX File