71-961217-JDL-01

JL-892

TO:		M. Ono					DATE:	December 17, 1996

FROM:	J. Levine					SUBJECT:	Estimated Offsite Doses

from NSTX Operations

REFERENCES:	1.	NSTX-RQMT-PDS, Rev-0, NSTX Project Definition 	Statement, 11/27/96.

2.	NSTX-RQMT-GRD, Draft G, NSTX General Requirements 	Document, 7/16/96.

3.	NSTX-RQMT-PRD-025, Draft C, NSTX Project Requirements 	Document, 7/10/96.

4.	DOE/EA-0813, Environmental Assessment, The Tokamak 	Fusion Test Reactor Decontamination and 	Decommissioning Project and the Tokamak Physics 	Experiment at the Princeton Plasma Physics Laboratory, 	5/94.

5.	PPPL Memorandum, J. Levine to J.W. Anderson, "Offsite 	Doses from TFTR Facility, JL-457, 7/2/92.

6.	DTSD-FSAR-17, Final Safety Analysis Report, Tokamak 	Fusion Test Reactor, Amendment No. 5, 9/20/96.

7.	White Paper, "Tritium Removal and Retention in TFTR; 	Tenaciously Held Tritium during a Vent," 1/16/96.

Estimates have been performed of the maximum expected individual offsite doses (at the PPPL site boundary) from routine NSTX operations and a postulated leak of tritium from the NSTX machine.  This is being done as part of the reexamination of the information in the NSTX Environmental Assessment (DOE/EA-1108, 12/95) relevant to the planned installation of NSTX in the D-Site Hot Cell.

The following assumptions have been used in these estimates:

1. NSTX will be located in the D-Site Hot Cell.  Negative pressure will not be maintained in the Hot Cell, therefore airborne radionuclide releases from this room are assumed to be ground level releases (see #7 below).  Exhaust gases from the NSTX machine and neutral beam are assumed to be released to the D-Site facility stack.

2. As indicated in Reference 1, Section 1.3, NSTX short pulse (5 sec) capability is 96 pulses/day, 5 days/week, 3 weeks/month, 9 months/year.  Repetition rate is 300 sec.

3. As indicated in Reference 2, Section 2.1.3.4, the long pulse upgrade will allow up to 60 sec pulses at a repetition rate of 3600 sec.  Scaling from the short pulse scenario, there would be a maximum of 8 60-sec pulses per day (for 5 days/week, 3 weeks/month, 9 months/year).

4. NSTX D-D neutron generation rate is 2 x 1014 neutrons/sec, per Reference 3 (for RF + NBI).

5. Since NSTX D-D reactions will produce some tritium, some D-T reactions are assumed to take place.  The D-T neutron generation rate in NSTX is assumed to scale in the same manner as documented in Reference 4 (Appendix A) for TPX.  The NSTX D-T neutron generation rate is thus assumed to be 4 x 1012 neutrons/sec.

6. Production of Ar41 from Hot Cell air activation by NSTX neutrons is based on TPX calculations for the Test Cell, scaled as the ratio of the Hot Cell-to-Test Cell volumes (0.38).  No accounting is made for the effects of the non-borated Hot Cell floor (boration of the Test Cell center support plug and the first one-foot of the Test Cell floor may reduce Ar41 production in that room by absorption of thermalized neutrons; not accounting for the absence of this effect in the Hot Cell should not significantly alter the NSTX offsite dose projection because Ar41 contributes <10% of the total offsite dose).

7. Maximum offsite dose/Ci for tritium and Ar41 is as noted in Reference 5, Table 1.  Offsite dose/Ci from routine ground level releases (i.e., due to Ar41 releases from the Hot Cell) are assumed to scale in the same manner as indicated in Reference 5, Table 1 for accidental releases (i.e., dose/Ci from ground level releases are three times higher than dose/Ci from stack releases).

A. Offsite Dose from Routine NSTX Operations

(1) Tritium

The annual quantity of tritium produced by NSTX is dependent on the maximum annual NSTX production of D-D neutrons.  Using Assumptions nos. 2-4 above, D-D neutron generation would be 2 x 1014 n/sec x 5 sec/pulse x 96 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr = 1.3 x 1019 n/yr for the short pulse experiments; and 2 x 1014 n/sec x 60 sec/pulse x 8 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr = 1.3 x 1019 n/yr for the long pulse upgrade.  The amount of tritium produced by n neutrons is n x (1 Ci/3.7 x 1010 disintegrations per sec) x (ln 2/(12.33 years x 3.15 x 107 sec per yr)).  Thus, the amount of tritium produced during short and long pulses with NB would be 0.63 Ci/yr.  Assuming that this tritium is all vented via the vacuum system to the stack as tritiated water vapor (HTO or T2O), the maximum annual offsite individual dose at the site boundary from this pathway would be 0.63 Ci/yr x 0.0026 mrem/Ci = 0.0016 mrem/yr.

(2) Ar41

Using Table A-2 of Reference 4 (attached), the quantities of Ar41 generated by NSTX per pulse is projected to be (Bq = Becquerel):

Short pulse-	[(8.6 x 10-10 Bq/DD neutron x 2.7 x 10-11 Ci/Bq x 2 x 1014 DD neutron/sec) + (8.8 x 10-10 Bq/DT neutron x 2.7 x 10-11 Ci/Bq x 4 x 1012 DT neutron/sec)] x 5 sec/pulse x 0.38

= 9.00 x 10-6 Ci/pulse;

Long pulse-	9.00 x 10-6 Ci/pulse x (60 sec per pulse/5 sec per pulse)

= 1.08 x 10-4 Ci/pulse.

Assuming that all the Ar41 generated is vented to the environment at ground level, the maximum annual releases of this isotope during NSTX operations would be:

Short pulses-	9.00 x 10-6 Ci/pulse x 96 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr

= 0.117 Ci/yr;

Long pulses-	1.08 x 10-4 Ci/pulse x 8 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr

= 0.117 Ci/yr.

The maximum annual offsite individual dose at the site boundary from this pathway would be 0.117 Ci/yr x 0.004 mrem/Ci x 3 = 0.0014 mrem/yr.

(3) Direct/Scattered Neutron and Gamma Radiation

NSTX's location in the D-Site Hot Cell will put it about 150 m from the nearest PPPL site boundary.  Extrapolating from information in Section 4.9.4.1 and Figure 4.9.2-22 of Reference 6, I obtain:

- NSTX neutron + gamma dose per DD neutron is about 1 x 10-21 mrem/DD neutron (5 x 10-22 mrem/DD neutron from neutron radiation + 5 x 10-22 mrem/DT neutron from gamma radiation, using Figure 4.9.2-22);

- NSTX neutron + gamma dose per DT neutron is 8.2 x 10-21 mrem/DT neutron (using DT-to-DD dose equivalent ratios in Section 4.9.4.1).

The maximum annual offsite individual dose at the site boundary from this pathway would be:

Short pulse:	[(1 x 10-21 mrem/DD neutron x 2 x 1014 DD neutron/sec x 5 sec/pulse x 96 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr) + (8.2 x 10-21 mrem/DT neutron x 4 x 1012 DT neutron/sec x 5 sec/pulse x 96 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr)]

= 0.0151 mrem/yr;

Long pulse:	[(1 x 10-21 mrem/DD neutron x 2 x 1014 DD neutron/sec x 60 sec/pulse x 8 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr) + (8.2 x 10-21 mrem/DT neutron x 4 x 1012 DT neutron/sec x 60 sec/pulse x 8 pulses/day x 5 days/wk x 3 wks/mo x 9 mos/yr)]

= 0.0151 mrem/yr.

DOSE PROJECTIONS FROM ROUTINE NSTX OPERATIONS

| PATHWAY | ROUTINE ANNUAL RELEASE OR GENERATION | MAXIMUM SITE BOUNDARY DOSE |
| --- | --- | --- |

| Tritium (HTO or T2O) | 0.63 Ci/yr | 0.0016 mrem/yr |
| --- | --- | --- |

| Activated Air (Ar41) | 0.117 Ci/yr | 0.0014 mrem/yr |
| --- | --- | --- |

| Direct/Scattered Neutron and Gamma Radiation | 1.3 x 1019 DD neutrons/yr + 2.6 x 1017 DT neutrons/yr | 0.0151 mrem/yr |
| --- | --- | --- |

|  |  | TOTAL = 0.0181 mrem/yr |
| --- | --- | --- |

The total estimated maximum site boundary dose from routine NSTX operations is 0.0181 mrem/yr, which is much less than the design objective of 10 mrem/yr.

B. Offsite Dose from Postulated Tritium Leak from NSTX Machine

Entrainment inside the NSTX torus vacuum vessel of tritium produced by D-D reactions may cause a buildup of releasable tritium in the machine over time.  Most of the tritium produced would be pumped by the NB cryopanels (and subsequently vented to the stack), but as much as 45% may be trapped in the torus (note that the routine dose calculation in Item A1 above conservatively assumed that all tritium produced is vented to the stack).  Assuming that this trapped tritium is not deliberately cleaned out of the torus, the maximum amount of tritium that could be held up in the torus after 10 years of NSTX operation could be (10 x 0.63 Ci/yr) x 0.45 = 2.84 Ci.  Based on the results of deliberate air venting of the TFTR torus in 1995 as documented in Reference 7, about 15% of this inventory or 0.43 Ci could be released due to an NSTX torus leak.  If such a leak was too large to be pumped on to maintain negative pressure in the torus, this quantity of tritium could be released to the room and subsequently to the environment.  The maximum individual site boundary dose from the release of 0.43 Ci of tritiated water vapor would be 0.43 Ci x 0.0157 mrem/Ci = 0.007 mrem, much less than the design objective of 50 mrem for this event (which based on information in the TFTR FSAR, would have an estimated probability of 3.7 x 10-2/yr).

cc:	S.	Citrolo

J.	DeLooper				Calculations Checked By:

J.	Gilbert

H.	Kugel

C.	Neumeyer					Jerry D. Gilbert

M.	Peng

J.	Schmidt