72-970522-SMK-01

TO: J.R. Wilson

FROM: S. M. Kaye

SUBJECT:  Beam Aiming Angles

This memo summarizes the TRANSP/neutral beam Monte-Carlo code modeling of NSTX discharges for the purposes of determining the beam aiming angle. The results suggest that Rtan,0=70 cm is a reasonable design choice.

The NSTX discharge that was modeled was one of high-βt (βt~35% thermal + beam), and had R=0.85 m, a=0.68 m, Ip=1.0 MA, BT=0.32 T, and κ=2. The beam was modeled with 5 MW of 80 keV D0 injected into a D+ plasma at varying tangency radii and plasma densities (the ion and electron temperatures were adjusted for the different densities so that the total (thermal + beam) βt was the same. The beams were modeled as TFTR beams; three sources with angular separations of 4° were injected. The central source was injected at different tangency radii; Rtan,0=61, 70, 85, 100, and 115 cm. For the separation angle of 4°, this means that the sources on either side of the central source had tangency radii approximately equal to Rtan,0 ± 10 cm (the exact tangency radii were used in the code). Associated with the varying tangency radii from source-to-source and case-to-case was a varying XLBTNA, which is the distance from the beam source to the tangency point in the plasma. XLBTNA is given by

XLBTNA = XLBAPA + SQRT(Rves2-Rtan2)

where XLBAPA=510 cm=distance from the ion source to the beam aperture into the vacuum vessel, and Rves~170 cm=radius of the vacuum vessel. Other modeling parameters are:

REDGE=11.0 cm=half-width of the aperture

XZEDGE=30.25 cm=half-height of the aperture

NBSHAP=1=ion source is rectangular

BMWIDR=6 cm=half-width of the ion source

BMWIDZ=21.5 cm=half-height of the ion source

DIVR=4.94e-3 radians=horizontal divergence of the beam

DIVZ=1.36e-2 radians=vertical divergence of the beam

FOCLR=FOCLZ=1e6 cm=horizontal and vertical focal lengths of the beam

The following table summarizes the key plasma parameters for the different cases studied

| Line-averaged ne | ne(0) | Te(0) | Ti(0) |
| --- | --- | --- | --- |

| 5.0x1019 m-3 | 6.6x1019 m-3 | 1.8 keV | 3.15 keV |
| --- | --- | --- | --- |

| 4.2 | 5.6 | 2.35 | 4.2 |
| --- | --- | --- | --- |

| 3.3 | 4.5 | 2.8 | 5.0 |
| --- | --- | --- | --- |

The beams were injected in the co-direction in all cases. The passive plates and vessel elements, which acted as limiters, were modeled in the code as straight line segments.

The beam power losses and heating and current drive characteristics were tabulated, and they are presented graphically in Figs. 1 to 8. The beam power losses for the plasma and beam parameters used in the model are essentially negligible. Even at the lowest density, the total shine-thru loss is ≤ 100 kW for Rtan,0 ≤ 100 cm (FIg. 1), rising to 250 kW at the higher Rtan,0. Bad orbit losses (Fig. 2) peak at the lowest Rtan,0 (most perpendicular injection), but account for ≤ 200 kW of power lost. Charge-exchange losses (Fig. 3) increase with decreasing density and Rtan,0, reaching 400 kW, but varies only by about 25% as a function of Rtan,0 at the lowest density. Summed up, these beam power loss channels account for only ~10% of the input power in the worst case (low density, small Rtan,0).

The beam driven current (Fig. 4) is a strong function of density, increasing from ~150 kA to 300 kA as density decreases over the full range, but it is a weak function of Rtan,0 (increasing only slightly with increasing Rtan,0). The beam driven current profile, however, changes dramatically with both density and Rtan,0. In Fig. 5 is plotted the ratio of the central beam current density to the central total current density for the various cases. Values of this ratio >1 imply a negative OH current and loop voltage in this central region. The ratio increases with increasing Rtan,0 and decreasing density. In order to keep values of this ratio ≤1, it would be necessary to have Rtan,0 ≤ 70 cm.

The total heating power (Fig. 6) is essentially independent of Rtan,0, as is the ratio of ion to electron heating (Fig. 7). It is not obvious why, in Fig. 7, the Pbi/Pbe values are highest for the middle density case. Finally, the half-width of the H(r) profile is plotted in Fig. 8, showing a more peaked heating and fueling source profile as Rtan,0 increases. At the highest Rtan,0 and lowest density, the H(r) profile shows an off-axis peak; hence, the large half-width. It is not known whether this is numerical (statistical) in origin.

In summary, the results of these modeling runs indicate that most of the dependences of power loss channels, heating, and current drive on Rtan,0 are weak, and in most of the cases where the dependence is strong (i.e., the losses), the variation has little consequence. The one significant consideration, however, is that as Rtan,0 increases, the heating and fueling source profiles become more peaked, and this results in a more centrally peaked driven current profile. The more peaked profiles can lead to reverse voltages and ohmic

currents in the central region. To avoid this, having Rtan,0 = 70 cm would be a reasonable design choice. Coincidentally, this is the same Rtan,0 as that in the straw man layout, which was not based on any of these quantitative considerations.

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Figure 6

Figure 7

Figure 8