April 27, 2007 – shot log – 19 plasma pulses + fiducial

Summary – obtained H-mode as desired, but no clear signatures of Type V ELMs observed on diagnostics. Saw lots of Type I and III ELMs, and some candidate Type V’s, but need more time to analyze.

Differences between these shots and normal type V regime:

- q95=5.5 to match C-MOD (normally 7-10)

-  = 1.8 to match C-MOD (normally 2 and above)

- 0 cm < drsep < -0.3 cm at L-H time, needed for H-mode access with this shape

- 0 cm < drsep < -0.8 cm in ELMy H-mode, to keep PLH low enough after ELMs to stay in H-mode (normally drsep <–1.2 cm ok for type V, but had not tested between 0 and –1 cm systematically)

Details of observations:

- Confirmed lack of H-mode with drsep=-0.6 cm and

- Obtained reliable H-mode access with drsep between 0 and –0.3 or –0.4 cm, and was able to ramp down drsep later in time at 0.3 sec

- Obtained power scan with –0.3 cm and –0.6cm. Most of the mall ELMs in these cases appear to be Type III, but there are a few unusual cases, e.g. #123660, 662 late. Pedestal did respond to power scan, i.e. pedestal beta did increase.

Diagnostics: Gas-puff imaging, divertor fast cameras, FIReTIP interferometers, reciprocating probe, fast filterscopes, IR cameras, etc. OK. We missed soft X-ray, and we may or may not have reflectometry.

#123644@0.3s

#123645@0.3s

H-mode access easier in low k, delta shape

with drsep ~  0

Power Scan with drsep = -0.2 cm shows Type III, Type I ELMs (no Type V ELMS)

Power Scan with drsep = -0.6 cm shows Type III, Type I, (Type V?) ELMs [these small ELM images actually look like some of MAST’s small ELMs]

Type V?