Access and confinement characteristics of the Enhanced Pedestal H-mode in NSTX

J.M. Canik1, R. Maingi1, R.E. Bell2, S.P. Gerhardt2, S.M. Kaye2, B.P. LeBlanc2, T.H. Osborne3, A. Diallo2, E.D. Fredrickson2, K.C. Lee4, M. Podesta2, S.A. Sabbagh5, D.R. Smith6

1Oak Ridge National Laboratory, Oak Ridge, TN, USA

2General Atomics, San Diego, CA, USA

3Princeton Plasma Physics Laboratory, Princeton, NJ, USA

4University of California-Davis, Davis, CA, USA

5Columbia University, New York, NY, USA

University of Wisconsin—Madison, Madison, WI, USA

A new high performance regime, the “Enhanced Pedestal” or EP H-mode [1,2], has been identified in NSTX experiments that has several attractive characteristics.  The pedestal temperature doubles  and  the  normalized energy confinement increases by 50% with H98y2 < 1.7 (when combined with confinement enhancement from lithium wall coatings [3]).  The  EP  H-mode  phase itself is ELM-free  in  that  any  subsequent  ELMs  return  the discharge  to  normal  H-mode, but at the same time the particle transport does not appear to be reduced to same extent as thermal transport, so that impurity accumulation is less than in conventional ELM-free H-modes.  This bears resemblance to VH-Mode [4], QH-mode [5], and I-mode [6], each of which show differentiation between the particle and thermal transport channels, and makes the EP H-mode a possible scenario for high confinement, ELM-free plasma operation, without the particle control problems of the standard ELM-free H-mode.

The transition to EP H-mode is triggered by a large ELM, either naturally occurring or externally triggered by applied 3-D fields, following which a  localized  rotation  drag  is observed at  the edge, usually near the q=3 surface, but sometimes just at the separatrix.  While short EP H-mode phases  were  previously  reported [1],  an  EP  H-mode  with  duration  up  to  three  energy  confinement times, τE, was recently observed [2], although they commonly last for less than one τE. The key to routine achievement was running at high (for NSTX) plasma current Ip=1.3 MA, low safety factor q95=6 discharges with high lithium evaporation between discharges to reduce the occurrence of natural ELMs.  Our  working  hypothesis  is that  the  magnetic  perturbation  due  to  ELMs  opens  up  a  3/1  seed  island  that  creates  the  drag, which results in a local increase in the ExB shear rate; the barrier grows inward in time and can grow  to  twice  as  wide  as  the  normal  H-mode  barrier.  Low  q95  operation  is  thought  to  be important  because  the  q=3  surface  moves  to  near  the  top  of  the  steep  gradient  region  of  the  H-mode  pedestal.  Measurements from an array of Mirnov coils show that electromagnetic turbulence clearly increases during EP  H-modes.  Progress on the characterization of transport and turbulence during the EP H-mode phase, along with conditions for access to this regime, will be presented.  *Supported in part by U.S. DOE Contracts #DE-AC05-00OR22725, DE-AC02-09CH11466, DE-FC02-04ER54698, DE-FG03-99ER54518, DE-FG02-99ER54524 and DE-FG02-89ER53296.

[1]  Maingi R., et al. J. Nucl. Mater. 390-391, 440 (2009).

[2]  Maingi R., et al. Phys. Rev. Lett. 105, 135004 (2010).

[3]  Kugel H. W., et al. Phys. Plasmas 15, 056118 (2008).

[4]  Jackson G. L., et al. Phys. Rev. Lett. 67, 3098 (1991).

[5]  Greenfield C. M., et al. Phys. Rev. Lett. 86, 4544 (2001).

[6]  McDermott R. M., et al. Phys. Plasmas 16, 056103 (2009).