| Princeton Plasma Physics Laboratory NSTX-U Machine Proposal |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Title: Commissioning of the Thomson Scattering System |  |  |  |  |  |  |
| OP-XMP-111 |  | Revision: 0 |  |  | Effective Date: 12/3/2015 Expiration Date: 12/3/2017 (2 yrs. unless otherwise stipulated) |  |
| Proposal Approvals |  |  |  |  |  |  |
| Responsible author: Benoit LeBlanc |  |  |  |  |  | Date |
| ATI (NSTX-U Physics Ops): Dennis Mueller |  |  |  |  |  | Date |
| RLM (NSTX-U Expt. Research Ops): Stefan Gerhardt |  |  |  |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |  |  |  |
| Procedure Requirements designated by RLM |  |  |  |  |  |  |
|  | NSTX Work Permit |  |  | T-MOD (OP-AD-03) |  |  |
|  | Independent Review |  |  | ES&H Review |  |  |
| RESTRICTIONS AND MINOR MODIFICATIONS Approved by RLM |  |  |  |  |  |  |

| REVIEWERS (designated by RLM) |  |  |
| --- | --- | --- |
| Organization/Position | Name | Signature |
| ATI | D. Mueller |  |
| Test Director |  |  |
| Independent Reviewer |  |  |
| NB system |  |  |
| RF systems |  |  |
| FCPC systems |  |  |
| Diagnostics |  |  |

| TRAINING (designated by RLM) Training required: No  Yes  Instructor _____________________ |  |  |  |
| --- | --- | --- | --- |
| Personnel (group, job title or individual name) | Read Only | Instruction | Hands-On |
| RLM _______________________________ |  |  |  |

NSTX-U MACHINE PROPOSAL

| TITLE: Commissioning of the Thomson Scattering System | No. OP-XMP-111 |
| --- | --- |
| AUTHORS: B. LeBlanc | DATE: |

## 1.	Overview:

The goal of this XMP is the commission the full 42 channel Thomson scattering system. L-mode plasmas, H-mode plasmas, and possibly HHFW plasmas will be used.

## 2.	Justification:

The multi-pulse Thomson scattering (MPTS) system is a critical diagnostic for NSTX-U machine operations and scientific progress; it provides the basic measures of the plasma electron density and electron temperature.

As a consequence, it is critical to commission this system.

## 3.	Plan:

3.1 L-Mode w/ NBI heating:

Reference shot:

- L-mode D2 plasma.

- Highest plasma current possible for the allowed TF at the time when this is run. This is likely in the range of an IP/BT ratio of 1.0-1.05 MA/T.

- 2-4 MW of neutral beam power.

- Outer gap of 10 cm

The exact shot number will be determined in consultation with the physics operations group.

Step 3.1.1:

Take reference shot, or record shot number if it already exists and can be used.

L-Mode Reference Shot #:________

Step 3.1.2:

Reduce the outer gap to ~5 cm and repeat 5 times.

Shot #:________

Shot #:________

Shot #:________

Shot #:________

Shot #:________

3.2 H-mode w/ NBI heating

Reference shot:

- H-mode D2 plasma, likely the standard morning H-mode fiducial shot

- Outer gap of ~10 cm

The exact shot number will be determined in consultation with the physics operations group.

Step 3.2.1:

Take reference shot, or record shot number if it already exists and can be counted toward the XMP

H-mode Reference Shot #:________

Step 3.2.2:

Reduce the outer gap to ~5 cm and repeat

Shot #:________

Shot #:________

Shot #:________

Shot #:________

Shot #:________

3.3 HHFW heated H-mode

Reference shot

- RF-heated plasma with L-mode edge and steep core Te profile.

Repeat shot five times:

Shot #:________

Shot #:________

Shot #:________

Shot #:________

Shot #:________

4.	Required machine, beam, ICRF and diagnostic capabilities:

L-mode reference shot must exist. This will likely be a 700 kA & 0.65 T scenario developed for other L-mode XMPs, but a different shot may be selected, potentially at lower IP/BT. Flat-top durations of greater than 0.5 seconds need to be achieved for this L-mode scenario.

For the H-mode shot, the morning fiducial shot will be used.

2-6 MW of NBI power must be available for steps 3.1 and 3.2. HHFW is required for step 3.3.

The MPTS system must also have demonstrated basic functionality.

## 5.  Sign off at run time:

5.1	Permission to Proceed:

Physics Operations Branch Head or Research Operations Head

5.2	Documentation of results:

Documentation of the results completed, attached to proposal and sent to Ops. Center with copies to Cognizant Physicist and Head of Physics Operations.

Cognizant Physicist/Test Director