| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: beta-suppression of Alfven Cascade |  |  |  |
| OP-XP-706 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: E. D. Fredrickson, M. Gryaznevich |  |  | Date Dec. 7, 2006 |
| ATI – ET Group Leader: S A Sabbagh |  |  | Date |
| RLM - Run Coordinator: D Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

beta-suppression of Alfven Cascade  Assess the effects of supra-Alfvénic fast ion driven instabilities on driven current in the plasma core	OP-XP-706

# 1.	Overview of planned experiment

Upward chirping modes identified as Cascade modes were discovered in very low density, low beta discharges, without MSE data and with limited reflectometer data.  We will return to conditions of these shots (120102-120104) and acquire a more complete set of data.  A well-conditioned machine is probably required to make plasmas with the target peak density on axis of ≈1 x 1013/cm3.  The first phase of this experiment be to document the q profile evolution, either by stepping source A on-time through the start-up phase or by substituting source A for source C.  The next phase will perform a density (beta) scan and document affect on range of frequency chirps.  As the density is raised, more reflectometer data will be acquired.  If additional shots are required for good reflectometer coverage, they will be taken at this time.  In the final phase, The toroidal field will be increased to 5.5 kG, and a beta scan will be performed there to test the theoretical beta scaling predictions as well as allow operation at somewhat higher density, aiding in acquisition of more complete reflectometer coverage.

This experiment cannot be done as piggyback.  It must be done in a very low-density regime.  May take some time to step source A on-time through the shot to map q-profile evolution.

# 2.	Theoretical/ empirical justification

Alfvén Cascade or rsAE modes have been studied on many conventional tokamaks.  They are found to be an accurate indicator of the qmin(t).  Their presence in reverse-shear plasmas on DIII-D, in plasmas with neutron deficits, has also raised suspicion that they may enhance fast ion transport.  Until recently, AC modes were absent from reverse shear plasmas in STs, and a theoretical explanation was developed which predicted that AC modes would only be present at very low (by ST standards) electron beta.

Alfvén Cascade-like modes were discovered at very low beta in ultra-low density plasmas during XP607 in the ’06 campaign.  The predicted suppression at high electron beta was observed. Unfortunately, MSE data was not available to confirm identification of modes as Alfvén Cascades.  A return to this experiment would allow for more complete documentation of the beta-threshold, as well as confirmation of the identification of these modes as Cascades.  The expanded array of reflectometer channels might also make possible the documentation of the expected change in mode structure as the frequency chirp saturates.

# 3.	Experimental run plan

The primary goal is to document q-profile evolution.  The secondary goal is to acquire good reflectometer data to verify modes are localized near the qmin region.  Strong velocity shear introduces uncertainty into the interpretation of frequency chirping.  A possible solution is to use n=3 braking to reduce rotation.  This should be tried early in the density scan, and if considered otherwise unperturbing, used throughout.

- Reproduce low density shot 120102/120103/120104, with source C at 90 kV	3

a) Repeat, using n=3 braking. Decide to use or not.

- Document q-profile evolution

a) Substitute source A at 90 kV for source C	1-2

b) If no Cascades are present, use Source C, step Source A every 25ms to get MSE	6-8

c) Try Argon puffing, decide whether to continue use

3	Density (beta) scan to study (Source A, if possible) to ≈ 3.5x1013/cm3 on axis	5

(Add HHFW from 160ms to 180 ms and evaluate impact, decide whether to continue)

a) If necessary, repeat q evolution documentation at intermediate density	6-8

Evaluate data acquired to this point, return to conditions where documentation is uncertain.  Verify that good reflectometer and MSE data was acquired.

4	beta (beam power) scan to look for onset frequency scaling.

1) Add source B at 70 kV to see affect of higher fast ion beta.	1-2

2) Repeat with only source B at 70 kV to lower fast ion beta.	1-2

5  Repeat, as much as possible, at higher field (5.5 kG), and then at low field (3 kG?)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Describe any prerequisite conditions, development, XPs or XMPs needed.
	Attach completed Physics Operations Request and Diagnostic Checklist

More than the standard three reflectometer channels would be desirable, but the full proposed set is not required.  MSE is a must.  N=3 braking capability is desired.

# 5.	Planned analysis

What analysis of the data will be required: EFIT, TRANSP, etc.

Apart from the initial and simple analysis of the frequency chirping, sufficient data should have been acquired for careful NOVA modeling of the Cascade mode chirps, comparison of Nova radial eigenfunctions to multipoint reflectometer data, and possibly Orbit estimates of the effect of the Cascades on fast ion confinemnt.

# 6.	Planned publication of results

What will be the final disposition of the results; where will results be published and when?

I would envision one longer paper as follow-up to the initial report on observation of Cascade modes.

PHYSICS OPERATIONS REQUEST

beta-suppression of Alfven Cascade 	OP-XP-706

Machine conditions (specify ranges as appropriate)

ITF (kA): 53	Flattop start/stop (s):  0.0/1.0

IP (MA): 0.800	Flattop start/stop (s):  0.2/0.9

Configuration: 120103

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species:  He,	Injector:  Midplane / Inner wall / Lower Dome

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90/90/90,	Duration (s): 0.90

ICRF – Power (MW): ≈2 MW,	Phasing: "Heating",	Duration (s): 0.2s

CHI:  On / Off

Either:	List previous shot numbers for setup: 120103

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

beta-suppression of Alfven Cascade 	OP-XP-706

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probe array |  |  |  |
| Magnetics - Diamagnetism |  |  |  |
| Magnetics - Flux loops |  |  |  |
| Magnetics - Locked modes |  |  |  |
| Magnetics - Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – core |  |  |  |
| Reflectometer - SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED |  |  |  |
| Thomson scattering |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |