| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Scaling of the SOL width in NSTX and extrapolation to NHTX |  |  |  |
| OP-XP-709 | Revision: | Effective Date: 02/09/07 (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: R. Maingi, J-W. Ahn, J. Boedo |  |  | Date |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates (M. Bell) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Scaling of the SOL width in NSTX and extrapolation to NHTX	OP-XP-709

# 1.	Overview of planned experiment

The goal of this experiment is to measure the Ip dependence and also the Bt dependence of the SOL ne, Te, particle and heat flux widths at the outer midplane. The first three quantities are measured by the reciprocating probe, and the latter one is obtained from the divertor IR camera data after field line mapping. The ultimate goal is to devise a physics-based technique to extrapolate the measured SOL widths for NHTX, to better predict the heat and particle flux footprints for PFC design.

# 2.	Theoretical/ empirical justification

The biggest extrapolations from NSTX to NHTX are in: Ip (1.0 -> 3.5 MA), Bt (0.55 -> 2.0 T), and Pheat (7-10 MW -> 30-50 MW). Since a big part of the NHTX proposed mission is to test the boundary physics at extremely high heat and particle fluxes, there is a need to predict the plasma footprints at the divertor strike regions. Historically such predictions are made based on analytic, 1-D, or 2-D models, which are benchmarked against experimental data from various diagnostics. Specifically the heat flux width (q) can be obtained from IR cameras measuring the divertor tile temperature and magnetic mapping, and the density, temperature, and turbulent particle flux widths (n, t, and ) can be obtained from midplane reciprocating probes.

qSOL,mid [cm]

bqSOL,mid [cm]

Fig. 1 – Dependence of midplane SOL heat flux width on (a) Ploss, and (b) Ip.

In NSTX, the heat flux widths have been measured1 as a function of heating power and Ip. At PNBI > 4 MW (Ploss > 3 MW), the heat flux width was rather insensitive to further increases in PNBI and was in the range of 0.7-1.0 cm (see figure 1a). Also, the heat flux width decreased rapidly with increasing Ip (figure 1b). Neither of these trends was well represented by three different models2 of the SOL width (red, blue, and green curves in Figure 1), highlighting the need for the consideration of other models.

Pin



b

Fig. 2 – Dependence of (a) midplane SOL ne and Te flux width on line-average density, and (b) ne profile changes during a Pin scan.

In addition, the ne and Te widths have been measured as a function of line-average density (figure 2a). Whereas the ne width drops slightly with line-average density, the Te width (curiously) does not have a monotonic dependence on line-average density. Also figure 2b shows that the density profile gets steeper with increasing input power, but the heating power range is limited to 1-2 MW.

We note that, at present, the overlap between reciprocating probe data and IR camera data has been minimal. Thus, one of the deliverables of this experiment is to obtain a consistent dataset on which to test models of the SOL width. All fluctuation diagnostics are highly desirable to better connect the measured SOL widths to the underlying turbulence.

In terms of execution of the experiment plan, the reciprocating probe can plunge to near or at the separatrix for PNBI < 3 MW, and so the baseline discharges uses that level of heating power. On the other hand, extrapolation to NHTX is best accomplished at higher N/PNBI, and so at each Ip/Bt combination, an additional discharge at PNBI =5 MW is included, mainly for IR camera data. The 3 MW base case will be accomplished by dropping source C voltage; this method is preferable to pulse-width modulation for a stable separatrix location, as needed for the reciprocating probe.

# 3.	Experimental run plan (1 day, prioritized list below)

- Reproduce baseline PF2L LSN #119083 or newer version from 2007 with 1100-1200 Torr on CS to suppress large ELMs, Bt=0.4 T, and PNBI=3 MW. Use source C at reduced voltage to get 3 MW. If discharge does not have a minimum 200ms quasi-steady period, consider either increasing Bt to 0.44 T or  to 2.2 for higher q95. (5 shots)

- Repeat above with PNBI=5 MW by adding source B. (2 shots)

- Perform I scan at approximately fixed q95 (at fixed Ip/Bt=2, or 1.818 if Bt=0.44 T is used above) and at PNBI=3 MW, 5 MW (15 shots)

- 1.1 MA, 0.55 T 	(1.0 MA, 0.55 T if lower Ip/Bt)

- 1.0 MA, 0.5 T 	(0.9 MA, 0.495 T if lower Ip/Bt)

- 0.9 MA, 0.45 T	(1.1 MA, 0.55 T to test Ip scaling)

- 0.7 MA, 0.35 T 	(0.7 MA, 0.385 T if lower Ip/Bt)

- 0.8 MA, 0.4 T 	(0.8 MA, 0.44 T if lower Ip/Bt)

Time permitting: Perform Bt scan at Ip=0.8 MA, PNBI=5 MW (6 shots)

- 0.8 MA, 0.55 T

- 0.8 MA, 0.35 T

- 0.8 MA, 0.5 T

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

This XP requires a fully operational NBI system. We desire HeGDC between shots of ~ 6.5 minutes for a 12.5 minute repetition rate.

# 5.	Planned analysis

The SOL widths will be analyzed from the data obtained, and relationships between the SOL widths will be investigated in the context of analytic cross-field transport models.

# 6.	Planned publication of results

Data and analysis will be presented at ITPA SOL group meetings and published in J. Nucl. Materials from the 2008 plasma-surface interactions meeting.

PHYSICS OPERATIONS REQUEST

Scaling of the SOL width in NSTX and extrapolation to NHTX	OP-XP-709

Machine conditions (specify ranges as appropriate)

ITF (kA): 63	Flattop start/stop (s):  _____/_____

IP (MA): 0..6-1.1	Flattop start/stop (s):  0.15/1.0 (max)

Configuration: Lower Single Null

Outer gap (m):	10 cm	Inner gap (m):	5-10 cm

Elongation :	2.0	Triangularity :	0.45

Z position (m):	0.00

Gas Species:  D,	Injector:  Inner wall Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90, 90, ~70,	Duration (s): <1 sec

ICRF – Power (MW): ____,	Phasing:      ,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: 119083 (LSN)

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Scaling of the SOL width in NSTX and extrapolation to NHTX	OP-XP-709

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast cameras |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |

1	R. Maingi, C.E. Bush, R. Kaita, H.W. Kugel, A.L. Roquemore, S.F. Paul, V.A. Soukhanovskii, and the NSTX team, et. al., J. Nucl. Materials (2007) at press.

2	G. F. Counsell, et. al., Journal of Nuclear Materials 266-269 (1999) 91.