| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: High fbs plasma at high elongation |  |  |  |
| OP-XP-710 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: D. Gates |  |  | Date |
| ATI – ET Group Leader: R. Maingi |  |  | Date |
| RLM - Run Coordinator: M. Bell (Deputy) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

High fbs plasma at high elongation	OP-XP-710

# 1.	Overview of planned experiment

This XP follows up on last year’s experiment aimed at developing high elongation discharges with high bootstrap fraction. Last year, high elongation plasmas were found to have lower beta limits, which has since been shown to have been due to reduced coupling to the passive conducting structure. High elongation was achieved by increasing the outer gap for these discharges. This had the undesirable effect of increasing the distance to the conducting structure and thereby reducing the wall stabilization.

# 2.	Theoretical/ empirical justification

The results from last year’s XP showed that high elongation was readily achieved at low current and high toroidal field. An example discharge is shown below. However high b was not achieved in these discharges. PEST calculations indicate that the plasma goes unstable when b approaches the no-wall limit. A paper has been written and submitted to Nuclear Fusion describing these results. This years experiments will focus on developing plasmas with smaller outer gaps. A target equilibrium is also shown below..

Figure 1: Equilibrium from 2006 (left) and new target equilibrium for 2007 (right)

# 3.	Experimental run plan

Day 1

Develop high  shape

1. Use 122623 as template (1 shot)

2. Raise TF to 5.5kGauss, lower Ip to 800kA (1 shot)

Note: Make sure Ip ends before TF to enabling analysis of diamagnetic data

3. Include early breakdown scenario from Menard’s XP (10 shots)

4. Raise PF1A to 16kA/MA by 200ms (6 shots)

5. Adjust outer squareness to achieve 20cm inner gap (4 shots)

6. Optimize early beam timing to maximize b avoiding b-limits (5 shots)

7. Scan Ip from 700kA-900kA (4 shots)

8. Repeat current scan at reduced Bt in attempt to make a longer pulse.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

7MW NBI, MSE, CHERS, TS

# 5.	Planned analysis

EFIT, TRANSP, PEST, DCON, etc.

# 6.	Planned publication of results

PRL, Nuclear Fusion depending on quality of results

PHYSICS OPERATIONS REQUEST

High fbs plasma at high elongation	OP-XP-710

Machine conditions (specify ranges as appropriate)

ITF (kA): 66 kA (0.55 T)	Flattop start/stop (s):  _____/_____

IP (MA): 0.7-1.0 MA	Flattop start/stop (s):  _____/_____

Configuration: Double Null

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	2.7,	Triangularity :	0.8 (lower)

Z position (m):	0.00

Gas Species:  D ,	Injector:  Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90 keV,	Duration (s): _____

ICRF – Power (MW):      ,	Phasing: Heating / CD,	Duration (s): _____

CHI:   Off

Either:	List previous shot numbers for setup: 120769 or one that has been developed more recently for 2007 small ELM experiment.

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

High fbs plasma at high elongation	OP-XP-710

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |