| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Improved break-down scenario for higher q during current ramp |  |  |  |
| OP-XP-711 | Revision: | Effective Date: 2/19/07 (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: J. Menard |  |  | Date February 19, 2007 |
| ATI – ET Group Leader: R. Maingi |  |  | Date February 19, 2007 |
| RLM - Run Coordinator: D. Gates (M. Bell) |  |  | Date February 19, 2007 |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Improved break-down scenario for higher q during current ramp	OP-XP-711

# 1.	Overview of planned experiment

This experiment will attempt to increase the safety factor during the plasma current ramp by increasing the plasma cross-sectional area during and after the plasma breakdown phase.  PF2 and PF1A field will be applied during the breakdown phase to increase the plasma elongation consistent with retaining vertical stability.  The PF3 coil current will be correspondingly decreased to maintain a large-area null during t=1-4ms.   The PF3 and PF5 programming will be modified to decrease the outer gap to 10-12cm during the current ramp.   PF2, PF1A, and PF1B coil currents will be used to divert earlier to attempt access to earlier H-mode.  The plasma current ramp-rate will be adjusted to avoid MHD instabilities during the current ramp.

# 2.	Theoretical/ empirical justification

Safety factor measurements made during the first 300ms of early H-mode discharges show that MIN(q) = 2-3 during the period when the normalized beta is increased with neutral beam heating.  During the high beta phase of such discharges the minimum safety factor is below 2.  As shown in the figure below, the plasma cross-section fills only a fraction of the vacuum chamber during the first 70ms of long-pulse NSTX discharges.  At fixed current, this small plasma size tends to reduce q at the onset of early H-mode and strong NBI heating and therefore reduces q later in the discharge.

MHD stability calculations indicate that the ideal-wall beta limit can be increased to close to 7 for the n=1 mode if MIN(q) can be increased above 2 during the high normalized beta phase with the experimentally achieved pressure profile.  Thus, it is desirable to attempt to increase the discharge safety factor to assess if the beta limit can be increased.

# 3.	Experimental run plan

  - Reproduce 700kA LSN shot 121120 which used rtEFIT control   			(4 shots)

    - Decrease TF from 5.2kG to 4.2kG to provide longer TF flat-top

    - Obtain discharge with flat-top duration of at least 300ms

  - Apply constant PF2 and PF1A field during breakdown w/ reduced PF3 for t < 0 	(12 shots)

    - Fire MPTS lasers at t=10ms and 15ms to track breakdown plasma growth

    - Adjust PF3, PF5, OH, and IP request at t=20ms as necessary for smooth plasma evolution

    - During breakdown scans in 3.b.iv-v below, improve post-breakdown plasma

      - Attempt to obtain early H-mode – adjust gas-puff and NBI as necessary

      - Eliminate outer gap oscillations – modify radial position gain as necessary

      - Increase flat-top elongation from 2.2-2.4 to 2.6

    - Determine OH & PF for good null

      - Increase PF2 current, decrease PF3 current until null lost or unstable

      - Scan field index - maximize kappa while remaining vertically stable

|  |  | PF2 scan |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IPF3U/L (kA) | 2.3 | 1.8 | 1.6 | 1.3 | 1 |
| IPF2U/L (kA) | 0 | 1 | 1.5 | 2.2 | 2.4 |
| IPF1AU/L (kA) | 0 | 0 | 0 | 0 | 0 |

    - For stable PF2 case, increase PF1A current until null lost or unstable

|  |  | PF1A scan |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IPF3U/L (kA) | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| IPF2U/L (kA) | 1.2 | 1 | 0.8 | 0.6 | 0.4 |
| IPF1AU/L (kA) | 0 | 2 | 4 | 6 | 8 |

  - Modify PF3 and PF5 IP ramp to decrease outer gap   				(5 shots)

    - Sustain larger-bore plasma from 20-80ms

    - Use MPTS and between-shots LRDFIT04s to track outer gap evolution

  - Scan IP ramp-rate and NBI timing for stable IP ramp  				(4 shots)

    - Obtain MSE data if possible to measure change in q-profile

  - Divert earlier using PF1B and PF1A coils – access earlier H-mode?   		(5 shots)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

The usual diagnostic capabilities are required, NBI voltages are A, B, C = 90, 90, 80kV

# 5.	Planned analysis

EFIT/LRDFIT, TRANSP, MPTS, CHERS, and RWM/EF sensor analysis will be performed.

# 6.	Planned publication of results

Results will be published in conference proceedings and/or journal such as Nuclear Fusion or Physics of Plasmas within one year of experiment.

PHYSICS OPERATIONS REQUEST

Improved break-down scenario for higher q during current ramp	OP-XP-711

Machine conditions (specify ranges as appropriate)

ITF (kA): 50kA		Flattop start/stop (s):  _-0.02s_/_1.6s____

IP (MA): 0.7		Flattop start/stop (s):  0.12-0.18 / 1.5s

Configuration: LSN

Outer gap (m):	12cm	Inner gap (m):	6cm

Elongation :	2.5	Triangularity :	0.5-0.7

Z position (m):	0.00

Gas Species:  D,	Injector:  CS Midplane, Outer Midplane

NBI - Species: D,	Sources: A,B,C Voltage (kV): 90,90,80kV	Duration (s): Up to 2s

ICRF – Power (MW): ____,	Phasing: N/A,	Duration (s): _____

CHI:  Off

Either:	Previous shot numbers for setup: 121120 using rtEFIT

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Improved break-down scenario for higher q during current ramp	OP-XP-711

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |