| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: HHFW Power Balance Optimization at High B Field |  |  |  |
| OP-XP-712 | Revision: | Effective Date: Feb 14, 2007 (Ref. OP-AD-97) Expiration Date: Feb 14, 2009 (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: J. Hosea, R. Bell, S. Bernabei, B. LeBlanc, C.K. Phillips, P. Ryan, D. Stutman, J. Wilgen, J.R. Wilson, et al. |  |  | Date |
| ATI – ET Group Leader: Gary Taylor |  |  | Date |
| RLM - Run Coordinator: Dave Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Title: HHFW Power Balance Optimization at High B Field	OP-XP-712

# J. Hosea, R. Bell, S. Bernabei, L. Delgado-Aparicio, S. Kubota, B. LeBlanc, F. Levinton, C.K. Phillips, P. Ryan, S. Sabbagh, K. Tritz, J. Wilgen, J.R. Wilson, et al.

# ET – Wave Physics    HHFW

# 1.	Overview of planned experiment

Goal:  Optimize heating efficiency at high B field for +/- 7m-1, 14m-1 to determine the determining parameters and in preparation for HHFW current drive/neutral beam studies

•   Scan plasma – antenna gap

- Reduce density near antenna if possible to see effect on heating efficiency

- Monitor edge density and heating effects

  - RF probe signals – amplitude and spectra from edge turbulence/PDI

  - PDI signa;ls – edge/core reflectometry, ERD

  - Thomson scattering, Chers, WEFIT, MSE, etc.

•   Upper single null versus lower single null comparison

- Determine if efficiency depends on distance along field lines to divertor

  - If radial group velocity is small relative to the parallel group velocity, the upper null configuration should provide more distance for the -7m-1 wave to penetrate the plasma edge before encountering the divertor region

# 2.	Theoretical/ empirical justification

A summary of the justification of this experiment is that it should provide additional data that can be analyzed to support the hypothesis that wave propagation in the edge of the plasma plays an important role in edge losses.  Based on last years results we should expect to be able to optimize heating and current drive efficiency at high magnetic field for a given wave-number launched by the antenna by minimizing the edge density and enhancing the wave penetration of the edge plasma.  Last year heating efficiency at -7m-1 was found to be enhanced to a level approaching that at 14m-1 for B=5.5 kG (Fig. 1) when the density at R = 156 cm was held to ~ 0.5 x 10+12 cm-3 or less (Fig. 2).

14m-1

-7m-1

7m-1

PRF = 2 MW

B = 5.5 kG

We(J)

Time (sec)

Fig. 1

Electron Density (1013 cm-3)

Time (sec)

Onset density:

14 m-1

7 m-1

R = 156 cm

Fig. 2

PRF losses at the edge of the plasma are detected through modulation of the RF power to determine W and  during the RF pulse(s), and from these the efficiency of core heating.

# 3.	Experimental run plan  -- 2 days of run time

Shots below are reduced by a factor of two to fit experiment into 1 run day.  If time runs short,

Part B will be run at another time.

- Setup conditions as for XP617 at B = 5.5 kG:

- LSN, helium L-mode, gap ~ 4 cm (same conditions as for shot 120740)

- Modulate PRF as for shot 120740 (at full 0.55 Tesla field) with k|| = -7 m-1

3 shots

- Perform wavenumber scan for gaps at ~ 4, 8 cm --1 day

Scan vs gap has been reduced to one gap change  to fit experiment into one day.

- With plasma settings as for shot 120740 (above) - Gap at ~ 4 cm:

- 1 shot @ -7 m-1 (co CD)

- 1 shot @ 14 m-1

- 1 shot @ +7m-1

- 1 shot @ -3m-1

- 1 shot @ -11m-1 (-120o)

- 1 no RF shots

(Additional shots may be required to set matching for each k||.  NB should be added 15 ms before end of third pulse)

- Change gap to ~ 8 cm:

- 1 shot @ -7 m-1 (co CD)

- 1 shot @ 14 m-1

- 1 shot @ +7m-1

- 1 shot @ -3m-1

- 1 shot @ -11m-1 (-120o)

- 1 no RF shot

- Compare edge densities and heating

- NB pulse at 90 keV is needed for CHERS and MSE

- Perform upper single null comparison – 1 day

Use ~ 4 cm gap and conditions from A and setup RF at -7m-1

3 shots

- 1 shots @ -7 m-1 (co CD)

- 1 shots @ +7 m-1

- 1 shots @ 14m-1

- 1 shots @ -3m-1

- 1 shot @ -11m-1 (-120o)

- 1 no RF shot

- Compare efficiencies with LSN data

- 1 Day only run plan:

- Setup conditions as for XP617 at B = 5.5 kG:

- LSN, helium L-mode, gap ~ 4 cm (same conditions as for shot 120740)

- Modulate PRF as for shot 120740 (at full 0.55 Tesla field) with k|| = -7 m-1

3 shots

- Perform wavenumber scan at ~ 4 cm --

Scan vs phase for one gap change to fit experiment into one day.

- With plasma settings as for shot 120740 (above) - Gap at ~ 4 cm:

- 1 shot @ 14 m-1 (180o)

- 1 shot @ -12 m-1 (-150o)

- 1 shot @ -10 m-1 (-120o)

- 1 shot @ -7 m-1 (co CD, -90o)

- 1 shot @ -5 m-1 (-60o)

- 1 no RF shot

Additional shots may be required to set matching for each k||.  NB should be added 20 ms before end of third pulse.  Start with timing of shot 120740 series: start RF at 160 msec with 100 msec pulse period (50 msec on, 50 msec off) and with total RF length of 300 msec.  Sync lasers to measure 1 msec before end of on period (equal spacing of lasers).  Place 40 msec NB 90 kV pulse to start at 390 msec, 20 msec before the end of third on period.  If it appears that we need more time for the lower k|| values, we will change the RF pulse frequency from 10 Hz to 6 Hz.  This will leave the laser timing NB timing the same and will reduce the number of RF pulses to 2 instead of 3.  The on time will then be 83.3 msec.

- Repeat A with counter CD phasing

- 1 shot @ 14 m-1 (180o)

- 1 shot @ +12 m-1 (+150o)

- 1 shot @ +10 m-1 (+120o)

- 1 shot @ +7 m-1 (+90o)

- 1 shot @ +5 m-1 (+60o)

- 1 shot @ +3m-1 (+30o)

- 1 no RF shot

(Additional shots may be required to set matching for each k||. NB should be added 20 ms before end of third pulse)

- Repeat A in deuterium.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Stable or at least reproducible plasma conditions are required for the quantitative comparisons of this XP.  Critical diagnostics include:

- Soft x-ray and Mirnov loops for stability

- EFIT with high time resolution for W

- Thomson scattering for electrons

- Edge rotation diagnostic for edge heating

- NPA for edge and core ion heating

- Radiated power diagnostic

- Reflectometry for edge density and PDI

- Reflectometry for wave measurements for opposite side from antenna

- Edge probe for PDI

- Gap RF probes for leakage

- 4 RF probe(s) for edge RF field

- CHERS for some shots for Ti  and velocities

- MSE for some shots for effects on current

# 5.	Expected results and planned analysis

Expected results:

- Heating efficiencies vs edge density and lower/upper null for 14 m-1, -7 m-1 (co CD) and       -7 m-1 (counter CD), -3m-1, -11m-1

- Core heating from EFIT W

- Core electron heating from Thomson scattering

- Ion heating and core rotation from Chers

- Edge heating/power loss

- Edge ion heating from edge rotation diagnostic

- Edge electron heating from Thomson scattering

- Rotation effects

- Initial MSE measurements (if time permits)

- Behavior of PDI characteristics and induced losses with field

- Plasma profiles, core and edge, for permitting predictions of wave propagation and damping characteristics and of PDI produced losses

- Relative surface wave amplitude for comparison to surface power loss for the explored conditions – different edge densities and different divertor configuration

Planned analysis:

- Calculation of  and W for EFIT W to obtain percent PRF deposited

- Calculation of e and We for Thomson scattering We to obtain PRF delivered to electrons

- Compare efficiencies for two gap cases and two divertor cases

- Analysis of wave propagation and damping characteristics from onset density into the core of plasma - along field and perpendicular directions of the ray path, and including collisions - for predicting surface losses

- Development of predictions for PDI losses

- Projection of heating efficiency expected at 10 kG, with and without PDI present, to compare with  DIII-D results and for higher field ST devices

# 6.	Planned publication of results

The results will be submitted for publication.

PHYSICS OPERATIONS REQUEST

Title: HHFW Power Balance Optimization at High B Field

Machine conditions (specify ranges as appropriate)

BTF (kG): 3 - 5.5	Flattop start/stop (s):  0.0 / 0.6 or as normal

IP (MA): 0.4 – 0.73	Flattop start/stop (s):  0.1 /0.6 or as for 120742

Configuration: Lower Single Null as for 120742 or as newly developed this run.

Upper single null for part B.

Outer gap (m):	4 – 8 cm,	Inner gap (m):	~ 4 cm

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species:  He,	Injector:  Inner Wall

NBI - Species: D,	Sources: A,	Voltage (kV): 90,	Duration (s): 0.04

ICRF – Power (MW): up to 3 MW Phasing: -7/+7()/14 m-1      Duration (s): 0.3 sec

(modulated) as for 120742

CHI:  On / Off

Either:	List previous shot numbers for setup: 120742

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Title: HHFW Power Balance Optimization at High B Field

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array | X |  | Whatever is needed for measuring Prad |
| Bolometer array - divertor | X |  |  |
| CHERS | X |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  | X |  |
| Edge rotation spectroscopy | X |  | Essential |
| Fast lost ion probes – IFLIP |  | X |  |
| Fast lost ion probes – SFLIP |  | X |  |
| Filtered 1D cameras |  | X |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  | X |  |
| Infrared cameras |  | X |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  | X |  |
| Langmuir probes - RF antenna | X |  |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes | X |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  | X |  |
| Mirnov coils – high frequency |  | X |  |
| Mirnov coils – poloidal array |  | X |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE | X |  | Essential for some shots |
| Neutral particle analyzer |  | X |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  | X |  |
| Reflectometer - FM/CW |  | X |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  | X |  |
| Reflectometer - HHFW/SOL | X |  |  |
| RF antenna camera |  | X |  |
| RF antenna probe | X |  |  |
| Solid State NPA |  |  |  |
| SPRED |  | X |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel | X |  |  |
| Ultrasoft X-ray arrays | X |  |  |
| Ultrasoft X-ray arrays - 2 color |  | X |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |
| High time resolution EFIT | X |  |  |
| Gap RF probes | X |  |  |
| RF probes for edge RF field | X |  |  |