| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Investigate Effect of Li Pellets on H-Mode Performance |  |  |  |
| OP-XP-718 | Revision: | Effective Date: 4/03/07 Expiration Date: 4/03/09 (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: H. Kugel |  |  | Date |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE:  	No.  OP-XP-718

AUTHOR: H. W. Kugel	DATE: 03-APR-07

# 1.	Overview of planned experiment

• Purpose

1) How will Li Pellet Injection into LSN He discharges affect subsequent D LSN H-Mode 	discharges of the same shape?

2) How will direct injection into ohmic phase of D LSN H-Mode discharges affect 	subsequent NBI phase?

2.	Theoretical/ empirical justification

1)  FY05 LPI into He conditioning shots tested only L-mode at low density LSN D.

2) FY04 LPI into LSN D NBI tested only L-mode/dithering conditions.

# 3.	Experimental run plan

| Shot # | Type | Li mg | Purpose |
| --- | --- | --- | --- |
| 1 | D/NBIa | 0 | Initial Fiducial |
| 2 | He/b | 3.4 | Deposit Li without consuming |
| 3 | D/NBI | 0 | Test if pumpout occurs due to last small amount. If yes, repeat fiducial |
| 4 | He/b | 3.4 | Deposit Li without consuming |
| 5 | He/b | 3.4 | “ |
| 6 | He/b | 3.4 | “ |
| 7 | He/b | 3.4 | “ |
| 8 | He/b | 3.4 | “ |
| 9 | D/NBIa | =17 | Test if pumpout occurs due to larger amount. If yes, repeat fiducial |
| 10 | D/NBIa | 3.4 | LPI @ ~50 ms into  before NBI. If yes, repeat |
| 11 | D/NBIa | 3.4 | LPI into NBI notch to increase penetration |
| 12 |  |  | Reserve |

a) 121323					Options:

b) 120468					1. 1.7 mg Li

2. Helium preconditioning discharges

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Prerequisites: Boronization, low recycling walls.

Required Configuration: Machine, 2-3 NBI,

Required Diagnostics: Standard.

Required Run Time~12 discharges (0.5 days)

5.	Planned analysis

UEDGE, DEGAS, TRANSP, etc.

# 6.	Planned publication of results

PSI08, IAEA08.

PHYSICS OPERATIONS REQUEST

Title: Investigate Effect of Li Pellets on H-Mode Performance	OP-XP-718

Machine conditions (specify ranges as appropriate)

ITF (kA):	Flattop start/stop (s):

IP (MA):	Flattop start/stop (s):

Configuration:

Outer gap (m): 		Inner gap (m):

Elongation :		Triangularity :

Z position (m):

Gas Species: 	Injector(s):

NBI - Species: D	Sources: 	Voltage (kV): 	Duration (s):

ICRF – Power (MW): 	Phasing: 	Duration (s):

CHI:

Either:	List previous shot numbers for setup:

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

XP718

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array | X |  |  |
| Bolometer array - divertor | X |  |  |
| CHERS | X |  |  |
| Divertor fast camera | X |  |  |
| Dust detector |  | X |  |
| EBW radiometers |  | X |  |
| Edge deposition monitor | X |  |  |
| Edge pressure gauges | X |  |  |
| Edge rotation spectroscopy |  | X |  |
| Fast lost ion probes - IFLIP |  | X |  |
| Fast lost ion probes - SFLIP |  | X |  |
| Fast X-ray pinhole camera |  | X |  |
| Filtered 1D cameras | X |  |  |
| Filterscopes | X |  |  |
| FIReTIP | X |  |  |
| Gas puff imaging | X |  |  |
| Infrared cameras | X |  |  |
| Interferometer - 1 mm |  | X |  |
| Langmuir probe array | X |  |  |
| Magnetics - Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes | X |  |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors |  | X |  |
| Mirnov coils – high frequency | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE |  |  |  |
| Neutral particle analyzer | X |  |  |
| Neutron measurements | X |  |  |
| Optical X-ray |  | X |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  | X |  |
| Reflectometer – core |  | X |  |
| Reflectometer - SOL |  | X |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED | X |  |  |
| Thomson scattering | X |  |  |
| Ultrasoft X-ray arrays | X |  |  |
| Visible bremsstrahlung det. | X |  |  |
| Visible spectrometer (VIPS) | X |  |  |
| X-ray crystal spectrometer - H | X |  |  |
| X-ray crystal spectrometer - V | X |  |  |