| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Investigate Effect of Lithium-Coated Divertor on Plasma Performance with LITER-1d |  |  |  |
| OP-XP-719 | Revision: 0 | Effective Date: 4/19/07 Expiration Date: 4/19/09 (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: H. Kugel |  |  | Date |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE:	Investigate Effect of Lithium-Coated Divertor 
on Plasma Performance with LITER-1d	No.  OP-XP-719

AUTHOR: H. Kugel	DATE: 4/19/07

# 1.	Overview of planned experiment

The motivation for this XP is to develop lithium deposition to control the spontaneous density 	rise that occurs during H-modes, and thereby to facilitate the development of long pulse 	induction-less current drive.  The approach is to investigate the effect of a lithium-coated 		divertor on plasma performance using the LITER-1d evaporator. This XP will start by 	evaporating small, pellet-size amounts of lithium between and during shots, and then 	proceed to larger amounts of 	lithium until an effect on the spontaneous density rise is 	observed. Subsequent experimental steps will focus on optimizing the effectiveness of the 	lithium deposition in controlling the 	density in long pulse discharges.

# 2.	Theoretical/ empirical justification

TFTR, CDX-U, and NSTX demonstrated the ability of lithium to control density.

# 3.	Experimental run plan

To establish baseline conditions before introduction of lithium, perform up to 3 reference discharges (D LSN H-mode shot 122740 with 2 NBI). Proceed if H-mode is obtained reliably.

Table 1 shows the nominal experimental sequence during evaporation.

- Without using HeGDC, start evaporating 30 mg of lithium at the same deposition rate between and during discharges for 3 discharges.

- On shot #4, do HeGDC for the entire between-shot evaporation or as allowed by the shot cycle.  If favorable density pump-out is observed, proceed with inter-shot HeGDC following the  evaporation rate sequence in Table 1. Proceed until locked modes prevent suitable discharges, or the H-mode low density limit is reached, or as determined by experimenters from review of diagnostic data. Note: be cautions in the range of 1 gram/shot.

  - If locked modes start to occur increase LFS gas in steps of 10 Tl/s.

  - If H-mode density threshold problems occur increase HFS gas in 200 Torr steps and adjust HFS timing as required.

- Choose best experimental conditions for maintaining a flat density waveform. Use density normalized to fueling as a figure of merit.

- Choose best condition from Step 3, and repeat at increasingly higher densities by using SGI to adjust density to be constant. Use 60Tl/s as a starting point with Plenum pressure 2500 Torr.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

D LSN H-mode shot 121323-LD with 2 NBI.

HeGDC during LITER operation if requested.

# 5.	Planned analysis

UEDGE, TRANSP, etc.

# 6.	Planned publication of results

PSI08, Nucl. Fusion, IAEA08

Table 1. Experimental Sequence

To establish baseline conditions before introduction of lithium, perform up to 3 reference discharges (D LSN H-mode shot 122740 with 2 NBI) before proceeding with the below experimental sequence.

| Shot No. | Nom. Oven Temp. °C | Between Shot mg* |  Between Shot mg | During Shot mg |  During Shot mg |
| --- | --- | --- | --- | --- | --- |
| 1 | 540 | 30 | 30 | 0.05 | 0.05 |
| 2 | 540 | 30 | 60 | 0.05 | 0.10 |
| 3 | 540 | 30 | 90 | 0.05 | 0.15 |
| 4 (HeGDC) | 540 | 30 | 120 | 0.05 | 0.20 |
| 5 | 570 | 60 | 180 | 0.10 | 0.30 |
| 6 | 570 | 60 | 240 | 0.10 | 0.40 |
| 7 | 570 | 60 | 300 | 0.10 | 0.50 |
| 8 | 600 | 120 | 420 | 0.20 | 0.70 |
| 9 | 600 | 120 | 540 | 0.20 | 0.90 |
| 10 | 600 | 120 | 660 | 0.20 | 1.10 |
| 11 | 630 | 240 | 900 | 0.40 | 1.50 |
| 12 | 630 | 240 | 1140 | 0.40 | 1.90 |
| 13 | 630 | 240 | 1380 | 0.40 | 2.30 |
| 14 | 660 | 480 | 1860 | 0.80 | 3.10 |
| 15 | 660 | 480 | 2340 | 0.80 | 3.90 |
| 16 | 660 | 480 | 2820 | 0.80 | 4.70 |
| 17 | 690 | 960 | 3780 | 1.60 | 6.30 |
| 18 | 690 | 960 | 4740 | 1.60 | 7.90 |
| 19 | 690 | 960 | 5700 | 1.60 | 9.50 |

* For 10 min of evaporation between shots

Fig.1  Between discharges, residual H2O combines with monolayer of Li in ~ 4 min

PHYSICS OPERATIONS REQUEST

Title: Investigate Effect of Lithium-Coated Divertor 
on Plasma Performance with LITER-1d	OP-XP-719

Machine conditions:

ITF (kA): -53	Flattop start/stop (s): -0.01/1.1

IP (MA): 1.0	Flattop start/stop (s): 0.2/1.0

Configuration: LSN

Outer gap (m): 		Inner gap (m):

Elongation :		Triangularity :

Z position (m):

Gas Species: D	Injector(s):  CS mid, OM #2

NBI - Species: D	Sources: A, C	Voltage (kV): 90	Duration (s): 1.0

ICRF – Power (MW): 	Phasing: 	Duration (s):

CHI:

Either:	List previous shot numbers for setup: 121323 with 2 NBI

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

XP-719

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array | X |  |  |
| Bolometer array - divertor |  | X |  |
| CHERS | X |  |  |
| Divertor fast camera |  | X |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor | X |  |  |
| Edge pressure gauges | X |  |  |
| Edge rotation spectroscopy |  | X |  |
| Fast lost ion probes - IFLIP |  | X |  |
| Fast lost ion probes - SFLIP |  | X |  |
| Fast X-ray pinhole camera |  | X |  |
| Filtered 1D cameras | X |  |  |
| Filterscopes | X |  |  |
| FIReTIP | X |  |  |
| Gas puff imaging |  | X |  |
| Infrared cameras | X |  |  |
| Interferometer - 1 mm |  | X |  |
| Langmuir probe array |  | X |  |
| Magnetics - Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes | X |  |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors |  | X |  |
| Mirnov coils – high frequency |  | X |  |
| Mirnov coils – poloidal array |  | X |  |
| Mirnov coils – toroidal array |  | X |  |
| MSE |  | X |  |
| Neutral particle analyzer |  | X |  |
| Neutron measurements | X |  |  |
| Optical X-ray |  | X |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  | X |  |
| Reflectometer – core |  | X |  |
| Reflectometer - SOL |  | X |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED | X |  |  |
| Thomson scattering | X |  |  |
| Ultrasoft X-ray arrays | X |  |  |
| Visible bremsstrahlung det. | X |  |  |
| Visible spectrometer (VIPS) | X |  |  |
| X-ray crystal spectrometer - H |  | X |  |
| X-ray crystal spectrometer - V |  | X |  |