| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Momentum Confinement and Effect of Rotation on Energy Confinement |  |  |  |
| OP-XP-723 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: S. Kaye |  |  | Date |
| ATI – ET Group Leader: K. Tritz |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Momentum Confinement and Effect of Rotation on Energy Confinement	OP-XP-723

# 1.	Overview of planned experiment

This purpose of this XP is to use the EF/RWM control coils in order to vary the plasma rotation to study momentum confinement and to study the effect of rotation on energy confinement. One plasma condition (i.e., beam power, current, toroidal field, etc) will be used for this particular study, but the current to the EF/RWM control coils will be varied. These effects will be studied both in steady-state and perturbatively.

# 2.	Theoretical/ empirical justification

The study of momentum confinement, and studies of the role of rotation on energy confinement and local transport have not been performed on NSTX in a meaningful, dedicated fashion. Results of these studies would have direct impact on high priority ITPA issues as well as on the 2008 NSTX Joule milestone.

Data from 2004 and 2005 indicated no statistical dependence of energy confinement on plasma rotation (see figure below). Plotted is the thermal energy confinement time enhancement as a function of central toroidal rotation velocity (off-axis rotation values yield the same lack of correlation).

Dedicated scans were not performed up to this point, but there is strong theoretical evidence linking the strength of rotation, and in particular ExB shear, to suppression of low-k turbulence, which may be why the ion transport on NSTX is often close to the neoclassical level.

Additionally, the momentum diffusivity, which has been tied in with ion thermal transport in conventional aspect ratio devices, appears to correlate neither with the ion nor the electron thermal transport in NSTX. This may be due to ITG modes being suppressed by the strongExB shear in NSTX, as it is the ITG mode at conventional aspect ratio that is believed to be responsible for the close relation between the momentum and the ion thermal diffusivity. Modifying the rotation (and ExB shear) in a non-resonant fashion using the EF/RWM control coils will help in our understanding the source of the momentum transport in NSTX.

# 3.	Experimental run plan

In this XP, we will use a relatively MHD quiescent discharge as baseline, similar to the discharges from XP532 (energy confinement scaling), with two sources. These discharges have relatively long “steady-state” (³400 ms), periods. Alternatively, we would use a discharge with optimal error field correction developed early in the run, if available.

There will be two phases of XP:

- Steady-state application of EF/RWM field to test effect of rotation on energy and impurity transport, determine “steady-state” momentum diffusivity/confinement

- Perturbative application of EF/RWM to determine momentum diffusivity & pinch from transients. The transients will be spaced by  (20-50 ms). Below is the suggested shot list.

- Steady-State (10-12 discharges); including baseline development

  - Determine maximum EF/RWM current for achieving near zero rotation (Imax), and the change in rotation for 1/3 Imax and 2/3 Imax. If not much, or too much change at these values, we will adjust them for the scans. For now, we will assume that the 1/3 and 2/3 values. The target Imax for attaining zero rotation is approximately 600 A (this will be verified during the scan).

  - EF/RWM on at 300 ms

  - Four conditions (2 shots/condition) – 8 discharges

ShotCondition

1IRWM=0

2 IRWM=IMAX

3IRWM=1/3 IMAX

4IRWM=2/3 IMAX

Constant EF/RWM “blips” for transient analysis (total 6 discharges)

ShotCondition

1Imax

22/3 Imax

31/3 Imax

__________________________

Step-up “blips” (total 4 shots)

ShotCondition

3 steps

2 steps

22 steps

Step-down “blips” (total 4 shots)

ShotCondition

3 steps

2 steps

Total: 24 shots (a healthy day)

# The reason for the step-up and step-down blips is to determine whether there is some hysterisis in the momentum diffusivity. In the step-up and step down transients, the width of the transient (50 ms here) will be adjusted if necessary to ensure the rotation has reached a steady-state in preparation for the next blip. Same for the spacing between the clusters of blips; we want to make sure the rotation has settled into a steady-state. The step-up and step-down blips, and the mono-amplitude blips, will be used to deduce the momentum diffusivity. The effect of rotation on confinement will be deduced primarily from the steady application of the EF/RWM current. The two-step blips are lower priority.

Below is the shot list in tabular form.

| Case | IRWM Type | IRWM | IRWM Timing |
| --- | --- | --- | --- |
| 1 | Steady-state | 0 |  |
| 2 | Steady-state | Max (600 A?) |  |
| 3 | Steady-state | 2/3 |  |
| 4 | Steady-state | 1/3 |  |
| 5 | Perturbative | Max | Vary blip widths in sequence |
| 6 | Perturbative | Max |  |
| 7 | Perturbative | 2/3 |  |
| 8 | Perturbative | 1/3 |  |
| 9 | Perturbative | 1/3, 2/3, Max | 3 step-up |
| 10 | Perturbative | Max, 2/3, 1/3 | 3 step-down |
| 11 | Perturbative | 1/3, 2/3 | 2 step-up |
| 12 | Perturbative | 2/3, 1/3 | 2 step-down |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

NPA for determining the level of fast ion loss, if any.

# 5.	Planned analysis

EFIT, TRANSP, etc.

# 6.	Planned publication of results

APS, journal article, ITPA

PHYSICS OPERATIONS REQUEST

Momentum Confinement and Effect of Rotation on Energy Confinement	OP-XP-723

Machine conditions (specify ranges as appropriate)

ITF (kA): 54 kA (0.45 T)	Flattop start/stop (s):  _____/_____

IP (MA): 0.9 MA	Flattop start/stop (s):  _____/_____

Configuration:  Lower Single Null

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	2.1,	Triangularity :	0.6 (lower)

Z position (m):	0.00

Gas Species:  D ,	Injector:  Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90 keV,	Duration (s): _____

ICRF – Power (MW):      ,	Phasing: Heating / CD,	Duration (s): _____

CHI:   Off

Either:	List previous shot numbers for setup: 120964, or a recent shot with optimized startup and flattop

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Momentum Confinement and Effect of Rotation on Energy Confinement	OP-XP-723

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  | Both toroidal and poloidal required |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |